3GPP TSG-RAN WG4 Meeting #91		R4-1905851

Reno, US, 13th– 17th May, 2019

Source:	OPPO

Title:	Discussion on capability of measurement gap patterns

Agenda Item:	6.10.3.1.2

Document for:	Discussion and Decision

Introduction

In last meeting, an LS[1] was sent to RAN4 on capability of measurement gap patterns.  RAN4 has discussed and achieved such agreements as guideline of this meeting.

In the next meeting, RAN4 decides which gap pattern(s) is mandated for cases of FR2 measurements, i.e., FR2 as PCell and EN-DC with per-FR measurement and FR2 MO configured.

Agreement:

- For SA case, if UE indicates support for any pattern 0-11 with MGL<6ms and MGRP<160ms, it means the UE can do both NR and NR+LTE measurement

- FFS for EN-DC

- FFS for LTE standalone with EN-DC capable UE

- Other issues not precluded

In this document, we further discuss the capability of measurement gap patterns.

Discussion

2.1 Background

In last meeting, an LS from RAN2 was sent to RAN4 informing that RAN2 has discussed the capability signalling of measurement gap patterns.

RAN2 agreed that current RRC signaling for gap pattern capabilities as following: - Gap patterns 0 and 1 are mandatory for all cases. - In LTE SA - LTE RRC signalling shortMeasurementGap is used to signal whether UE supports gap patterns 2 and 3. - LTE RRC signalling measGapPatterns (8 bits) is used to signal whether UE supports gap patterns 4 to 11. - In EN-DC - LTE RRC signalling shortMeasurementGap is used to signal whether UE supports gap patterns 2 and 3 (for per-UE cap and FR1 gap of per-FR gap). - LTE RRC signalling measGapPatterns is used to signal whether UE supports gap patterns 4 to 11 (for per-UE cap and FR1 gap of per-FR gap). - NR RRC signalling supportedGapPattern (22 bits) is used to signal whether UE supports gap patterns 2 to 23 (for FR2 gap of per-FR gap). - In NR SA - NR RRC signalling supportedGapPattern (22 bits) is used to signal whether UE supports gap patterns 2 to 23. - If the UE supports an optional gap pattern (#2 to #23), it supports using the gap pattern according to the corresponding applicability defined in RAN4 specifications. RAN2 would like to confirm with RAN4 that the above agreements are consistent with TS 36.133 and TS 38.133.

In TS 38.306, supportedGapPattern is defined to indicate measurement gap pattern(s) optionally supported by the UE. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3, as specified in TS 38.133 [5] and so on.

Some companies suggest to reply to RAN2 about cases that need additional gap pattern than 0&1 based on current requirements. And two options are given out in last meeting:

- Option 1:  Correct the corresponding capability signalling of measurement gap patterns in EN-DC, NE-DC, NR SA, and NR-DC modes.

- Option 2: Follow RAN2 agreements on mandatory and optional gap pattern.

From our side, if only gap pattern 0 and 1 are mandatory with other patterns optional supported, it may cause misunderstanding for UE capability signaling.  The requirements may need updated from RAN4’s perspective.

And in last meeting RAN4 also agreed that for SA case if UE indicates support for any pattern 0-11 with MGL<6ms and MGRP<160ms, it means the UE can do both NR and NR+LTE measurement, while it is FFS for EN-DC and LTE standalone with EN-DC capable UE. In our view, the similar way as NR SA can be used for EN-DC and LTE SA scenarios.

Observation 1: if only gap pattern 0 and 1 are mandatory with other patterns optional supported, it may cause misunderstanding from RAN4’s perspective.

Proposal 1: for EN-DC and LTE standalone with EN-DC capable UE, if UE indicates support for any pattern 0-11 with MGL<6ms and MGRP<160ms, it means the UE can do both NR and NR+LTE measurement.

2.2 Mandatory gap pattern for FR2

As a guideline of this meeting, RAN4 is encouraged to discuss which gap pattern(s) is mandated for cases of FR2 measurements, i.e., FR2 as PCell and EN-DC with per-FR measurement and FR2 MO configured.

In our understanding, RAN4 specifies the corresponding requirements for different conditions in TS 36.133 or TS 38.133, while RAN2 defines the corresponding RRC signaling and decides which is optional or mandatory. However, only gap patterns 0 and 1 are mandatory for all cases, with no applicable gap pattern mandatory to FR2 measurement for both per UE and per FR gaps.

For SA case, a UE which is supporting SA operation on FR2 needs to support some of GP12-23 from a RAN4 requirements perspective.

Table 9.1.2-3: Applicability for Gap Pattern Configurations supported by the UE with NR standalone operation (with single carrier, NR CA and NR-DC configuration)

| Measurement gap pattern configuration | Serving cell | Measurement Purpose NOTE 2 | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE measurement gap | FR1 NOTE5, or FR1 + FR2 | E-UTRA onlyNOTE3 | 0,1,2,3 |
|  |  | FR1 and/or FR2 | 0-11 |
|  |  | E-UTRAN and FR1 and/or FR2 NOTE3 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 NOTE5 | E-UTRA only NOTE3 | 0,1,2,3 |
|  |  | FR1 only | 0-11 |
|  |  | FR1 and FR2 | 0-11 |
|  |  | E-UTRAN and FR1 and/or FR2 NOTE3 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  |  | FR2 only | 12-23 |
| Per FR measurement gap | FR1 if configured | E-UTRA only NOTE3 | 0,1,2,3 |
|  | FR2 if configured |  | No gap |
|  | FR1 if configured | FR1 only | 0-11 |
|  | FR2 if configured |  | No gap |
|  | FR1 if configured | FR2 only | No gap |
|  | FR2 if configured |  | 12-23 |
|  | FR1 if configured | E-UTRA and FR1 NOTE3 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | No gap |
|  | FR1 if configured | FR1 and FR2 | 0-11 |
|  | FR2 if configured |  | 12-23 |
|  | FR1 if configured | E-UTRA and FR2 NOTE3 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | 12-23 |
|  | FR1 if configured | E-UTRA and FR1 and FR2 NOTE3 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | 12-23 |
| NOTE 1: When E-UTRA inter-RAT RSTD measurements are configured and the UE requires measurement gaps for performing such measurements, only Gap Pattern #0 can be used. NOTE 2: Measurement purpose which includes E-UTRA measurements includes also inter-RAT E-UTRA RSRP and RSRQ measurements for E-CID NOTE 3: The measurement gap pattern #2, 3, 6, 7, 8, 10 are supported by the UEs which indicate the capability signalling of supportedGapPattern to network. NOTE4: If per-UE measurement gap is configured with MG timing advance of TMG ms, the measurement gap starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap among all serving cells subframes. If per-FR measurenet gap for FR1 is configured with MG timing advance of TMG ms, the measurement gap for FR1 starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap among serving cells subframes in FR1. If per-FR measurenet gap for FR2 is configured with MG timing advance of TMG ms, the measurement gap for FR2 starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap among serving cells subframes in FR2. TMG is the MG timing advance value provided in mgta according to [2]. In determining the measurement gap starting point, UE shall use the DL timing of the latest subframe occurring immediately before the configured measurement gap among serving cells. NOTE 5: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. |  |  |  |

For EN-DC with per-FR measurement and FR2 MO configured, Gap patterns 12-24 also need to be supported by UE.

Table 9.1.2-2: Applicability for Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE or NR-E-UTRA dual connectivity UE

| Measurement gap pattern configuration | Serving cell | Measurement Purpose | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE measurement gap | E-UTRA + FR1, or E-UTRA + FR2, or E-UTRA + FR1 + FR2 | non-NR RAT Note1,2 | 0,1,2,3 |
|  |  | FR1 and/or FR2 | 0-11 |
|  |  | non-NR RATNote1,2 and FR1 and/or FR2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
| Per FR measurement gap | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 | 0,1,2,3 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 only | 0-11 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR2 only | No gap |
|  | FR2 if configured |  | 12-23 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR1 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 and FR2 | 0-11 |
|  | FR2 if configured |  | 12-23 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | 12-23 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR1 and FR2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | 12-23 |
| Note: In E-UTRA-NR dual connectivity mode, if GSM or UTRA TDD or UTRA FDD inter-RAT frequency layer is configured to be monitered, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. NOTE 1: In E-UTRA-NR dual connectivity mode, non-NR RAT includes E-UTRA, UTRA and/or GSM. In NR-E-UTRA dual connectivity mode, non-NR RAT means E-UTRA. NOTE 2: In E-UTRA-NR dual connectivity mode, the gap patterns with short MGL (gap pattern #2, 3, 6, 7, 8, 10) are supported by UEs which support shortMeasurementGap-r14. In NR-E-UTRA dual connectivity mode, the measurement gap pattern #2, 3, 6, 7, 8, 10 are supported by the UEs which indicate the capability signalling of supportedGapPattern to network. NOTE 3: When E-UTRA inter-frequency RSTD measurements are configured and the UE requires measurement gaps for performing such measurements, only Gap Pattern #0 can be used. |  |  |  |

Thus, support of some common gap patterns by different UE implementations is important for network interoperability. Similar to LTE’s gap pattern 0 and 1, gaps with a longer MGL and MGRP<160ms can be set as mandatory.

Proposal 2: If gap pattern(s) is decided to be mandated for FR2 measurements, gaps 12-14 with 5.5ms MGL and MGRP<160ms are suggested to be mandatory.

Table 9.1.2-1: Gap Pattern Configurations

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |

Conclusion

In this contribution, we would like to propose the followings:

Observation 1: if only gap pattern 0 and 1 are mandatory with other patterns optional supported, it may cause misunderstanding from RAN4’s perspective.

Proposal 1: for EN-DC and LTE standalone with EN-DC capable UE, if UE indicates support for any pattern 0-11 with MGL<6ms and MGRP<160ms, it means the UE can do both NR and NR+LTE measurement.

Proposal 2: If gap pattern(s) is decided to be mandated for FR2 measurements, gaps 12-14 with 5.5ms MGL and MGRP<160ms are suggested to be mandatory.

References

- R4-1904781, Ad hoc minutes for NR RRM measurement, Ericsson