3GPP TSG-RAN WG4 Meeting #91		R4-1905893

# Reno, Nevada, USA, May 13 – May 17, 2019

Agenda item:	6.5.6

Source:	Intel Corporation

Title:	NBB requirements for re-farming bands

Document for:	Approval

# Introduction

The current specification TS 38.101-1 has still TBD values for the narrow band blocking requirement for re-farming bands in FR1. In this contribution we provide our view on the calculation of the narrow band blocking to keep consistency in the specification.

# Narrow Band Blocking for re-farming bands

Narrow band blocking has been only defined for re-farming bands, however the values for SCS 15 kHz for 30 MHz and the values for SCS 30 kHz are missing. In this contribution we provide our view on the calculation of the NBB.

| NR band | Parameter | Unit | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1, n2, n3, n5, n7, n8, n12, n20, n25, n28, n34, n38, n39, n40, n41, n50, n51, n66, n70, n71, n74, n75, n76 | Pw | dBm |  | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 |
|  | Fuw (offset SCS= 15 kHz) | MHz | 2.7075 | 5.2125 | 7.7025 | 10.2075 | 13.0275 | TBD | 20.5575 | NA | NA | NA | NA | NA |
|  | Fuw (offset SCS= 30 kHz) | MHz | NA | NA | NA | NA | NA | NA | NA | TBD | TBD | TBD | TBD | TBD |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## NBB for re-farming bands

For the calculation of the NBB requirement we have considered the difference of spectral utilization for NR compared to LTE. The higher spectral utilization in NR compared to LTE has an impact on the complexity for the implementation of the digital filter. In NR the digital filter design has to satisfy more constraints for the wider BW.

Additionally, the narrower separation between the wanted signal and the NBB blockers the higher the limitation on the steepness of the filter. Hence, the frequency offset of the narrow band blocker shall not be lower than the offset of the first ACS subcarrier.

Figure 1: Narrow Band Blocking (NBB)

Observation 1: The frequency offset of the narrow band blocker shall not be lower than the offset of the first ACS subcarrier.

Figure 1 illustrates an example with a wanted signal of 25 MHz bandwidth and an ACS blocker of 5 MHz bandwidth. The narrow band blocker allocation is calculated with the following formula:

NBBoffset= ACSoffset-ACSBW2+SCS2

ACSoffset is the distance between the centre of the wanted channel and the ACS blocker. ACSBW is the bandwidth of the ACS blocker which for SCS 15 kHz is 4.5 MHz and for SCS 30 kHz is 3.96 MHz considering the number of RBs from [1].  For the example of 25 MHz wanted channel bandwidth with 15 kHz SCS, the ACS offset is 15 MHz and the ACS bandwidth is 4.5 MHz. The calculated NBB offset is 12.7575 MHz for a SCS of 15 kHz and a CBW of 25 MHz when applying the equation above.

In Table 1 we provide the values for the the NBB requirement for re-farming bands, considering the same numbers as in LTE for the CBW between 5 MHz and 20 MHz. For the CBW larger than 20 MHz the above NBB offset equation applies and the power level is 16 dBm up to 100 MHz.

Table 1: Narrow-Band Blocking for re-farming bands

| NR band | Parameter | Unit | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1, n2, n3, n5, n7, n8, n12, n20, n25, n28, n34, n38, n39, n40, n41, n50, n51, n66, n70, n71, n74, n75, n76 | Pw | dBm |  | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 |
|  | Fuw (offset SCS= 15 kHz) | MHz | 2.7075 | 5.2125 | 7.7025 | 10.2075 | 12.7575 | 15.2575 | 20.2575 | NA | NA | NA | NA | NA |
|  | Fuw (offset SCS= 30 kHz) | MHz | NA | NA | NA | NA | NA | NA | NA | 25,535 | 30.535 | 40.535 | 45.535 | 50.535 |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 1:	Use Table 1 for the Narrow-Band blocking requirement for re-farming bands

# Conclusions

Observation 1: The frequency offset of the narrow band blocker shall not be lower than the offset of the first ACS subcarrier.

Proposal 1:	Use table 1 for the Narrow-Band blocking requirement for re-farming bands

# References

- 3GPP TS 38.101-1 V15.5.0 (2019-03)