3GPP TSG RAN WG4 Meeting #91		R4-1905929

Reno, USA, 13th – 17th May 2019

Source: 	NTT DOCOMO, INC.

Title: 	Further clarification on RAN4 UE feature list

Agenda item:	6

Document for:	Approval

- Introduction

Currently, RAN2 are preparing an internal TR for capturing all of RAN1/2/4 UE feature lists.

R2-1901813 Discussion on the necessity of documenting UE feature list in TS/TR NTT DOCOMO, INC. discussion Rel-15 NR_newRAT-Core R2-1819115 => The RAN1/2/4 feature lists will be captured in an internal TR at the end of Rel-15 (i.e. when late drop is finished and we have confidence that the information is very unlikely to change). The TR will state that it captures the status at a certain date and will not be maintained after that date.

The TR skeleton was endorsed in the last RAN2 meeting [1]. This TR will be submitted in the next RAN meeting in June for approval. In this contribution, we further update the RAN4 UE feature list for the clarification of some UE features, especially modulation scheme. The modified version is also attached in this contribution.

- Discussion

2.1.	Supported modulation scheme for CA/DC

For modulation scheme, the following agreements were made in the RAN1 and RAN4 UE feature lists [2, 3].

Table 1. Summary of modulation scheme in RAN1/4 UE feature lists

| Frequency range | Direction | Mandatory |  | Optional with capability |
| --- | --- | --- | --- | --- |
|  |  | without capability | with capability |  |
| FR1 | DL | QPSK, 16QAM, 64QAM | 256QAM (per UE) |  |
|  | UL | QPSK, 16QAM, 64QAM |  | 256QAM (per band), Pi/2 BPSK (per UE) |
| FR2 | DL | QPSK, 16QAM, 64QAM |  | 256QAM (per band) |
|  | UL | QPSK, 16QAM, 64QAM | Pi/2 BPSK (per UE) | 256QAM (per band) |

QPSK, 16QAM and 64QAaM are mandatory without capability signalling and Pi/2 BPSK and 256QAM are mandatory or optional with capability signalling. From the UE capability signalling in RAN2 [4, 5], on the other hand, UE is allowed to report any modulation order as “maximum supported modulation scheme” in per CC basis when the UE supports a CA configuration. Thus, we would like to discuss the relationship of the RAN4 decision and the UE capability signalling in CA configuration. Note that the introduction of this UE capability signalling was decided in RAN2 in parallel with the discussion on the UE feature list in RAN4, e.g. [6].

The information element of UE capability is signalled in DL and UL respectively to indicate supported modulation scheme in per CC basis, and the information element for DL is indicated below as an example [5].

FeatureSetDownlinkPerCC ::=         SEQUENCE {

supportedSubcarrierSpacingDL        SubcarrierSpacing,

supportedBandwidthDL                SupportedBandwidth,

channelBW-90mhz                     ENUMERATED {supported}                               OPTIONAL,

maxNumberMIMO-LayersPDSCH           MIMO-LayersDL                                        OPTIONAL,

supportedModulationOrderDL          ModulationOrder                                      OPTIONAL

}

ModulationOrder ::= ENUMERATED {bpsk-halfpi, bpsk, qpsk, qam16, qam64, qam256}

Figure 1. UE capability signalling for supported modulation schemes per CC (in DL as example)

It is also noted that even if UE reports “QPSK” as maximum supported modulation scheme in a CC in a CA configuration, it does not mean that 16/64QAM are not supported in the CC. In other words, the reported value is NOT related to the actual supported modulation scheme in the CC. This is justified in [4] as follows.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| supportedModulationOrderDL Indicates the maximum supported modulation order to be applied for downlink in the carrier in the max data rate calculation as defined in 4.1.2. The network may use a modulation order on this serving cell which is higher than the value indicated in this field as long as UE supports the modulation of higher value for downlink. | FSPC | Tbd | No | Tbd |

The reported modulation schemes are just used for the calculation of supported max date rate specified in the section 4.1.2 of [4].

### 4.1.2 Supported max data rate For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows. wherein J is the number of aggregated component carriers in a band or band combination Rmax = 948/1024 For the j-th CC, is the maximum number of supported layers given by higher layer parameter maxNumberMIMO-LayersPDSCH for downlink and maximum of higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink. is the maximum supported modulation order given by higher layer parameter supportedModulationOrderDL for downlink and higher layer parameter supportedModulationOrderUL for uplink. is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4. is the numerology (as defined in TS 38.211 [6]) is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed. is the maximum RB allocation in bandwidth with numerology , as defined in 5.3 TS 38.101-1 [2] and 5.3 TS 38.101-2 [3], where is the UE supported maximum bandwidth in the given band or band combination. is the overhead and takes the following values 0.14, for frequency range FR1 for DL 0.18, for frequency range FR2 for DL 0.08, for frequency range FR1 for UL 0.10, for frequency range FR2 for UL

The calculated data rate is like “UE category” in LTE. The supported max data rate is calculated per frequency range (within cell group of NR), and the network needs to configure actual MCS level in PDSCH or PUSCH transmission such that the total number of the transmitted bits at a certain time duration should not exceed the calculated data rate [7][8]. Although the max data rate is calculated by the reported modulation scheme via supportedModulationOrder (DL/UL are omitted), it is still possible to configure a higher modulation scheme than the reported modulation scheme unless the transmitted bits does not exceed the max data rate. This is quite reasonable in order not to limit system performance of NR. If the actual modulation order for PDSCH or PUSCH is limited by supportedModulationOrder in each CC, the system performance is greatly degraded in some scenario as shown in Fig. 2.

Figure 2. Impact of limited modulation order on performance

It is noted that if higher layer parameter processingType2Enabled of PDSCH-ServingCellConfig IE is configured for the serving cell and set to enable, the network needs to configure the actual MCS so that the transmitted bits size is not larger than the supported max data rate per CC [7]. This is mainly for URLLC like UE.

Considering above specifications, in the UE feature list, it is better to clarify that the signalled values via supportedModulationOrder IEs are independent from “functional support” of modulation scheme in each CC in a CA configuration. Also, it should be clarified that if UE supports a CA configuration, “functional support” of QPSK/16QAM/64QAM are mandatory without capability signalling in all CCs. For Pi/2 BPSK and 256QAM, per UE or per band UE capability signalling is introduced in RAN2, e.g. pdsch-256QAM-FR1, pdsch-256QAM-FR2 in [3]. Whether to support those modulation schemes in each CC is just followed by the UE capability signalling. Without those clarifications, there is no way for network to configure higher modulation scheme than the reported values in supportedModulationOrder IEs to UE since the network cannot know the actual supported modulation scheme in each CC at UE side based on the current UE capability signaling.

Proposal 1:

  - The current agreements on modulation schemes in the RAN4 UE feature list are applicable not only for single carrier case but also for CA or EN-DC configuration, i.e.,

    - If UE support a CA or EN-DC configuration,

      - UE shall support QPSK, 16QAM and 64QAM in all NR CCs without UE capability signalling

      - Whether to support 256QAM in a NR CC is followed by pdsch-256QAM-FR1 (per UE) or pdsch-256QAM-FR2 (per band) IEs

      - Whether to support Pi/2 BPSK in a NR CC is followed by pusch-HalfPi-BPSK (per UE) and/or pucch-F3-4-HalfPi-BPSK (per UE) IEs

  - Note: Above are only related to functionality of each modulation scheme, and NOT related to the reported values in supportedModulationOrderDL and supportedModulationOrderUL IEs for the calculation of supported max data rate in the section 4.1.2 of [3]

Besides, as mentioned above, the value in supportedModulationOrderDL (or UL) can limit the total number of transmitted bits in a certain time duration. However, the benefit to report lower value than 64QAM via this IE is quite unclear. One possible motivation is to adjust the transmitted bit size considering the actual memory size at UE side, especially in higher order CA configuration with wider channel bandwidths, such as100MHz x 8CCs, by indicating a low modulation scheme. For this purpose, however, RAN1 already introduced the scaling factor, f = {1, 0.8, 0.75, and 0.4}, for the calculation of the maximum data rate as indicated above. Also, similar to LTE, the supported number of MIMO layers in each CC can also be used by this purpose. Hence, reporting lower modulation scheme would bring unnecessary flexibility in UE side, and it is equivalent to allow the huge number of UE category for NR UEs. If the signalling flexibility is needed for the supported modulation scheme, in this case UE shall report “1” as scaling factor f. Based on above, the following observation and proposals were reached:

Observation: The necessity to report lower value than 64QAM via supportedModulationOrderDL (or UL) is unclear. To adjust the value of supported max data rate in [4], the scaling factor, f = {1, 0.8, 0.75, and 0.4}, was already introduced.

Proposal 2:

  - Add the UE feature “Modulation order supported for each CC for calculating maximum date rate” for DL and UL into the RAN4 UE feature list

    - Optional with capability singling

    - Per CC per band per band combination signaling (Type3)

    - Supported values: {pi/2-BPSK, BPSK, QPSK, 16QAM, 64QAM, 256QAM}

    - Note: RAN2 signalling is already introduced

Proposal 3:

  - Consider either of the following two restrictions for supportedModulationOrderDL (or UL) and scaling factor f to limit flexibility of supported max data rate in the section 4.1.2 of [3]:

    - Alt.1: UE shall report “1” as scaling factor f, but supportedModulationOrderDL (or UL) are not restricted

    - Alt.2: Scaling factor f is not restricted, but supportedModulationOrderDL (or UL) are restricted. Specifically, UE shall report “64QAM” in CC(s) where 256QAM is not supported. In CCs where 256QAM is supported, UE shall report “256QAM”.

  - Note that this restriction is applicable only if processingType2Enabled is not enable.

  - Some relaxations to this requirement may be applicable in the future (including in Rel-15).

2.2.	Interruption of “Mandatory to report”

In the “RAN decision” column in [2], the wording “Mandatory to report” is used. More specifically, this wording is used for the following UE features:

  - FG1-8, Active BWP switching delay

  - FG1-11, Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective

  - FG2-3, Non-contiguous intra-band CA frequency separation class for FR2

  - FG2-8, UE power class

  - FG2-16, Maximum uplink duty cycle for FR2 power class 3 UE

  - FG2-17, PA architectures for intra-band EN-DC

  - FG2-18, PA architectures for intra-band UL CA

The wording is well aligned with the RAN2/4 specifications, but the meaning is slightly different among the features. For example, for FG2-8 (UE power class), the decision is “Mandatory to report non-default power class if UE supports”. This means that UE power class 3 (default power class) is mandatory without capability signalling but other classes are optional with capability signalling. For FG1-8 (Active BWP switching delay), the decision is “Mandatory to report type 1 or type 2”. This just means that UE shall support either type 1 or type 2. To avoid confusion and for better understanding, we propose the followings.

Proposal 4: The RAN decision part including the wording “Mandatory to report” can be interrupted as Table 2.

Table 2: Interruption of “Mandatory to report”

| Feature group |  | RAN decision | Interruption |
| --- | --- | --- | --- |
| 1-8 | Active BWP switching delay | Mandatory to report type 1 or type 2 | Mandatory to support either type 1 or type 2 |
| 1-11 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Mandatory to report switching time type 1 or type 2 if UE reports its capability in 1-10 as 1) LTE and NR UL Transmission in the shared carrier via TDM only, or 3) LTE and NR UL transmission in the shared carrier via FDM or TDM | Mandatory to support either type 1 or type 2 if UE reports its capability in 1-10 as 1) LTE and NR UL Transmission in the shared carrier via TDM only, or 3) LTE and NR UL transmission in the shared carrier via FDM or TDM |
| 2-3 | Non-contiguous intra-band CA frequency separation class for FR2 | Mandatory to report for UE to support non-continuous CA in FR2 | Mandatory to support a frequency separation class within {c1, c2, c3} specified in TS38.101-2 if UE supports non-contiguous CA in FR2 |
| 2-9 | UE power class | Mandatory to report non-default power class if UE supports | Default power class is mandatory without capability signalling. Other power classes are optional with capability signalling |
| 2-16 | Maximum uplink duty cycle for FR2 power class 3 UE | Mandatory to report non-default uplink duty cycle if UE supports | Optional with capability signalling. If this capability signalling is absent, the UE shall ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies under the default UL duty cycle. |
| 2-17 | PA architectures for intra-band EN-DC | Mandatory to report dual PA architecture if UE supports | Mandatory to support either single or dual PA architectures if UE supports intra-band EN-DC configuration in uplink. UE shall report its capability only if dual PA architecture is supported. |
| 2-18 | PA architectures for intra-band UL CA | Mandatory to report dual PA architecture if UE supports | Mandatory to support either single or dual PA architectures if UE supports intra-band CA configuration in uplink. UE shall report its capability only if dual PA architecture is supported. |

Proposal 5:

  - For the “Mandatory/Optional” column in the internal TR for UE capability, it is up to TR editor whether to adopt the original description in the RAN decision column or above interruption.

  - Send LS to RAN2 to inform above proposal.

2.3.	Others (minor updates)

- For FG2-16 (Maximum uplink duty cycle for FR2 power class 3 UE), the value range is updated based on the previous agreement which was made in the last RAN4 meeting [9].

- For FG3-4 (SU-MIMO Interference Mitigation advanced receiver), one “TBD” remains for the type of advanced receiver. We just remove this TBD and insert type “X” receiver based on the latest TS38.101-4.

- Conclusion

In this contribution, we further update the RAN4 UE feature list. The modified version is also attached in this contribution. Our observation and proposals are summarized blow.

Proposal 1:

  - The current agreements on modulation schemes in the RAN4 UE feature list are applicable not only for single carrier case but also for CA or EN-DC configuration, i.e.,

    - If UE support a CA or EN-DC configuration,

      - UE shall support QPSK, 16QAM and 64QAM in all NR CCs without UE capability signalling

      - Whether to support 256QAM in a NR CC is followed by pdsch-256QAM-FR1 (per UE) or pdsch-256QAM-FR2 (per band) IEs

      - Whether to support Pi/2 BPSK in a NR CC is followed by pusch-HalfPi-BPSK (per UE) and/or pucch-F3-4-HalfPi-BPSK (per UE) IEs

  - Note: Above are only related to functionality of each modulation scheme, and NOT related to the reported values in supportedModulationOrderDL and supportedModulationOrderUL IEs for the calculation of supported max data rate in the section 4.1.2 of [3]

Observation: The necessity to report lower value than 64QAM via supportedModulationOrderDL (or UL) is unclear. To adjust the value of supported max data rate in [4], the scaling factor, f = {1, 0.8, 0.75, and 0.4}, was already introduced.

Proposal 2:

  - Add the UE feature “Modulation order supported for each CC for calculating maximum date rate” for DL and UL into the RAN4 UE feature list

    - Optional with capability singling

    - Per CC per band per band combination signaling (Type3)

    - Supported values: {pi/2-BPSK, BPSK, QPSK, 16QAM, 64QAM, 256QAM}

    - Note: RAN2 signalling is already introduced

Proposal 3:

  - Consider either of the following two restrictions for supportedModulationOrderDL (or UL) and scaling factor f to limit flexibility of supported max data rate in the section 4.1.2 of [3]:

    - Alt.1: UE shall report “1” as scaling factor f, but supportedModulationOrderDL (or UL) are not restricted

    - Alt.2: Scaling factor f is not restricted, but supportedModulationOrderDL (or UL) are restricted. Specifically, UE shall report “64QAM” in CC(s) where 256QAM is not supported. In CCs where 256QAM is supported, UE shall report “256QAM”.

  - Note that this restriction is applicable only if processingType2Enabled is not enable.

  - Some relaxations to this requirement may be applicable in the future (including in Rel-15).

Proposal 4: The RAN decision part including the wording “Mandatory to report” can be interrupted as Table 2.

Proposal 5:

  - For the “Mandatory/Optional” column in the internal TR for UE capability, it is up to TR editor whether to adopt the original description in the RAN decision column or above interruption.

  - Send LS to RAN2 to inform above proposal.

Based on the above proposals, we update the UE feature list in the attachment.

Proposal 6: Approve the updated UE feature list in the attachment, and send LS to RAN2 in RAN4#91

# Reference

- R2-1904720, NTT DOCOMO, TR skeleton and work plan on Rel-15 UE feature list, Apr. 2019

- R1-1905917, NTT DOCOMO, LS on RAN1 NR UE features update, Apr. 2019

- RP‑190704, NTT DOCOMO, Update of RAN4 UE feature list for REL-15 NR, Mar. 2019

- TS38.306, 15.5.0, Apr. 2019

- TS38.331, V15.5.1, Apr. 2019

- R4-1803563, Qualcomm Incorporated, Reply LS on Baseband Processing Capabilities, Feb. 2018.

- TS38.214, V15.5.0, Apr. 2019

- R1-1905887, Ericsson, CR to 38.214 clarifying calculation of DataRate and DataRateCC, Apr. 2019

- draft_RAN4#90bis_meeting report_v1, Apr. 2019