3GPP TSG-RAN WG4 Meeting #91	R4-1905942

# Reno, May, May 13th – 17th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TCI State Switching Delay Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction CR to section 8.10 capturing UE TCI swith delay requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR clarifies the UE requirements in section 8.10 related to Active TCI state switching delay. Missing requirements for TCI switch delay requirements are introduced including: - known conditions for TCI state - MAC-CE base TCI state switch delay - DCI based TCI switch delay - RRC based TCI based switch delay (new) - MAC-CE based active TCI state set switch delay (new) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Section 8.10 on TCI stte delay requirements is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sections 8.10.2, 8.10.3, 8.10.4, 8.10.5 (new) and 8.10.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

## 8.10	Active TCI state switching delay

### 8.10.1	Introduction

The requirements in this section apply for a UE configured with one or more TCI state configurations on serving cell in MR-DC or standalone NR. UE shall complete the switch of active TCI state within the delay defined in this section.

### 8.10.2	Known conditions for TCI state

The TCI state is known if it has been meeting the following conditions:

- During the period (measured in slots) equal to:

  - For a periodic or semi-persistent CSI report on PUCCH and PUSCH, for the TCI State, the period as configured by the higher layer parameter reportSlotConfig.

  - For an aperiodic CSI report, for the TCI State, the period as configured by the higher layer parameter reportSlotOffsetList.

    - The UE has sent an L1-RSRP/L3-RSRP report for the target TCI state.

    - The reported TCI state remain detectable according to the cell detection requirements in section 9.2.

    - The SSB or the CSI-RS measured and reported remains detectable during the TCI activation delay according to the cell identification conditions specified in section 9.2

Otherwise, the TCI state is unknown.

### 8.10.3	MAC-CE based TCI state switch delay

If the target TCI state is known to UE, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ THARQ +3ms, where THARQ is the timing between DL data transmission and acknowledgement as specified in [7].

Otherwise, the UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ THARQ +3ms + TRS, where TRS is one occasion of either SSB, if SSB is used as RS for the TCI state, or CSI-RS, if CSI-RS is used as RS for the TCI state.

### 8.10.4	DCI based TCI switch delay

If the target TCI state is known to UE, when a UE is configured with the higher layer parameter tci-PresentInDCI which is set as 'enabled' for the CORESET scheduling the PDSCH at slot n, UE shall be able to receive PDSCH or transmit PUSCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+timeDurationForQCL, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], the value of TQCL_time is defined in TS 38.306[14].

The known condition for TCI state defined in section 8.7.2 is applied.

### 8.10.5	RRC based TCI state switch delay

If the target TCI state is known to UE, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at n+TTCI_Delay_RRC:

- TTCI_Delay_RRC = TRRC_delay + 3 Nslotsubframe,μ + 1

Where:

- TRRC_delay is the RRC procedure delay as specified in [2].

Otherwise, the UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ TTCI_Delay_RRC + TRS, where TRS is one occasion of either SSB, if SSB is used as RS for the TCI state, or CSI-RS, if CSI-RS is used as RS for the TCI state.

If the target TCI state is known to UE the UE shall be able to operate using its current TCI state without interrupts until the UE switches to the new TCI state at n+TTCI_Delay_RRC.

### 8.10.6	MAC-CE based active TCI state set switch delay

Upon receiving a MAC-CE UE with a UE specific PDSCH MAC CE indicating a new active TCI state set for PDSCH in slot n, the UE shall apply the mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' starting from slot n+3*Nslotsubframe,μ + 1.

Next Modified Subclause