3GPP TSG-RAN WG4 Meeting #91	R4-1905954

# May, USA, May 13th –17th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0066 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of band n48 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_n48-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of band n48 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of band n48 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | band n48 is not introduced |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

### 3.5.2	NR operating bands in FR1

NR frequency bands grouping for FR1 is specified in Table 3.5.2-1.

Table 3.5.2-1: NR frequency band groups for FR1

| Group | NR FDD |  | NR TDD |  | NR SDL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation |  | Operating bands |
| A | NR_FDD_FR1_A | n1, n70, n744 | NR_TDD_FR1_A | n34, n38, n39, n40, n50, n51 | NR_SDL_FR1_A |  | n75, n76 |
| B | NR_FDD_FR1_B | n65, n66, n743 | NR_TDD_FR1_B | - | NR_SDL_FR1_B | - |  |
| C | NR_FDD_FR1_C | - | NR_TDD_FR1_C | n48, n771, n78, n79 | NR_SDL_FR1_C | - |  |
| D | NR_FDD_FR1_D | n28 | NR_TDD_FR1_D | n772 | NR_SDL_FR1_D | - |  |
| E | NR_FDD_FR1_E | n2, n5, n7 | NR_TDD_FR1_E | n41 | NR_SDL_FR1_E | - |  |
| F | NR_FDD_FR1_F | - | NR_TDD_FR1_F | - | NR_SDL_FR1_F | - |  |
| G | NR_FDD_FR1_G | n3, n8, n12, n20, n71 | NR_TDD_FR1_G | - | NR_SDL_FR1_G | - |  |
| H | NR_FDD_FR1_H | n25 | NR_TDD_FR1_H | - | NR_SDL_FR1_H | - |  |
| NOTE 1: Except 3.8 GHz to 4.2 GHz. NOTE 2: Only 3.8 GHz to 4.2 GHz. NOTE 3: Except 1475.9 MHz to 1510.9 MHz. NOTE 4: Only when the band is confined in 1475.9 MHz to 1510.9 MHz. |  |  |  |  |  |  |  |

Next Modified Subclause