3GPP TSG-RAN WG4 Meeting #91		     R4-1906020

# Reno, US, 13th– 17th May, 2019

Agenda Item:   8.5.2

Source:            AT&T

Title:                 Co-existence scenarios for IAB

Document for: Discussion/Approval

# Introduction

As part of the Work Item on Integrated Access and Backhaul for NR [1], 3GPP has agreed to support solutions for efficient operation of integrated access and wireless backhaul for NR. This document provides an overview of the key co-existence scenarios for IAB for evaluations and the definition of requirements in RAN4.

# Co-existence scenarios for IAB

Evaluating the impact of IAB operation on adjacent spectrum bands is an important objective of the WI. Selecting the simulation assumptions is critical to balance the need for realistic modelling with a reasonable workload for conducting the evaluations. During the SI, RAN1 agreed on detailed simulation assumptions for system-level simulation evaluations covering multiple deployment scenarios including regular and random IAB node deployment, FR1 and FR2 bands, and different densities of donor nodes.

Considering the trade-offs of the different scenarios, it is proposed to focus on a typical deployment for IAB where the gains are most readily realized and is a natural extension of existing NR deployments. In our view this corresponds to IAB nodes on a fixed hexagonal grid of 19 sites with 3 donor sites since this results in a topology with a large number of nodes 1 or 2 hops from the donor with up to 3 or 4 hops in a small percentage of cases as well. Simulations with 7 donors result in very few multi-hop links which may not be realistic for early deployments and 1 donor site may result in some topologies with a very large number of hops (7 or more) which is unrealistic from a latency perspective. In terms of carrier frequency and bandwidth assumptions, the simulations should consider FR2 (30 GHz) with 100MHz CC bandwidth (which may be aggregated up to 400MHz).

Proposal 1: Adopt the following parameters for co-existence simulations based on Table A.1-1 of TR 38.874:

| Parameters | Homogeneous scenario (urban micro) |
| --- | --- |
| Layout | Non-IAB deployment Hex. Grid 19 sites IAB deployment Hex. Grid (100% shift relative to Non-IAB deployment) 19 sites Number of IAB-donors (Ndonor) 3 Number of IAB-nodes is 19 – Ndonor |
| Inter-BS distance | 200m |
| Topology formation | Max RSRP of candidate parent nodes |
| Carrier frequency | 30GHz |
| Duplex mode | TDD |
| Aggregated system bandwidth (access + backhaul) | Up to 400MHz (DL+UL) |
| Simulation bandwidth | Per CC BW is 100MHz |
| Large-scale channel parameters | - Micro-to-UE: UMi-Street canyon - Micro-to-Micro: UMi-Street canyon (hUE =10m) - UE-to-UE: UMi-Street canyon (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 of TR38.802 The path loss for links between the IAB-node and candidate serving IAB-nodes/donors is determined based on N =3 independent large-scale channel realizations (taking into account LOS/NLOS probability and shadow fading). The realization that results in the minimum pathloss between the IAB-node and the associated serving IAB-node/donor is selected. |
| Fast fading parameters | - Micro to Micro: UMi-Street canyon O-to-O (hUE =10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 |
| BS Tx power | 33dBm EIRP should not exceed 68 dBm (*) |
| BS antenna configurations | See Table A.2.1-4 of TR38.802. At least for the purpose of IAB evaluations, when the IAB-node has multiple panels, access and backhaul traffic can be sent on any panel, subject to the per IAB-node half duplex constraint. |
| BS antenna height | 10 m |
| BS antenna element gain + connector loss | See Table A.2.1-4 of TR38.802 |
| BS receiver noise figure | 7dB |

One additional question which is especially relevant for IAB is the activity level of the different access and backhaul links served by the IAB nodes and IAB donor nodes. Due to the nature of the multi-hop topology, it is a reasonable assumption that the activity level of a given IAB node is the distance from the donor (e.g. the probability that an IAB node is active decreases as hop order increases). Also, typically the IAB donor will be almost 100% active on the DL and UL resources assuming a moderate to large topology is managed underneath it. However, the contribution of the donor nodes to the adjacent channel interference is no different than in the case a non-IAB network is evaluated, since the donor nodes do not have their own UL transmissions (e.g. no MT functionality), so the key aspect is to accurately model the IAB node activity level. A statistical model can be derived by running basic system-level simulations once the basic assumptions are agreed.

Proposal 2:  For the purpose of co-existence evaluations, the IAB donor nodes can be assumed to be always active and the probability that given IAB node is active should be a function of hop order and subject to the half-duplex constraint.

FFS: The activity distribution, pending evaluations based on the TBD simulation assumptions.

# Conclusion

This document provides an overview of the key co-existence scenarios for IAB. The following proposals were made:

Proposal 1: Adopt the following parameters for co-existence simulations based on Table A.1-1 of TR 38.874:

| Parameters | Homogeneous scenario (urban micro) |
| --- | --- |
| Layout | Single layer Micro layer: Hex. Grid 19 sites Number of IAB-donors (Ndonor) 3 Number of IAB-nodes is 19 – Ndonor |
| Inter-BS distance | 200m |
| Topology formation | Max RSRP of candidate parent nodes |
| Carrier frequency | 30GHz |
| Duplex mode | TDD |
| Aggregated system bandwidth (access + backhaul) | Up to 400MHz (DL+UL) |
| Simulation bandwidth | Per CC BW is 100MHz |
| Large-scale channel parameters | - Micro-to-UE: UMi-Street canyon - Micro-to-Micro: UMi-Street canyon (hUE =10m) - UE-to-UE: UMi-Street canyon (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 of TR38.802 The path loss for links between the IAB-node and candidate serving IAB-nodes/donors is determined based on N =3 independent large-scale channel realizations (taking into account LOS/NLOS probability and shadow fading). The realization that results in the minimum pathloss between the IAB-node and the associated serving IAB-node/donor is selected. |
| Fast fading parameters | - Micro to Micro: UMi-Street canyon O-to-O (hUE =10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 |
| BS Tx power | 33dBm EIRP should not exceed 68 dBm (*) |
| BS antenna configurations | See Table A.2.1-4 of TR38.802. At least for the purpose of IAB evaluations, when the IAB-node has multiple panels, access and backhaul traffic can be sent on any panel, subject to the per IAB-node half duplex constraint. |
| BS antenna height | 10 m |
| BS antenna element gain + connector loss | See Table A.2.1-4 of TR38.802 |
| BS receiver noise figure | 7dB |

Proposal 2:  For the purpose of co-existence evaluations, the IAB donor nodes can be assumed to be always active and the probability that given IAB node is active should be a function of hop order and subject to the half-duplex constraint.

FFS: The activity distribution, pending evaluations based on the TBD simulation assumptions.

# Reference

[1] RP-182882, New WID: Integrated Access and Backhaul for NR, Qualcomm