3GPP TSG-RAN WG4 #91 Meeting                                        	R4-1906042

Reno, USA, 13 – 17 May, 2019

Source: 	LG Electronics

Title: 	MSD test results for new x bands DL (x=3, 4, 5) with 2 bands UL CA in Rel-16

Agenda item:	7.6.2

Document for:	Approval

1	Introduction

Based on the self-desense analysis from RAN4 #90Bis, MSD needs to be determined for those who have self-interference problems.

For 3 bands DL and 2 bands UL CA band combinations, MSD results are provided for one of three downlink bands which is impacted by harmonics and IMD products during dual uplink transmission.

For 4 bands DL with 2 bands UL and 5 bands DL with 2 bands UL CA band combinations, there is no need to study more for the case when harmonics and IMDs impact to their own receiving bands (4th and 5th) since all self-desense problems will be covered in 2 bands DL/2 bands UL and 3 bands DL/2 bands UL CA band combinations.

### 2.	Summary of interference studies

Table 1 shows 3 bands DL/ 2 bands UL CA band combinations with self-interference problems in Rel-16 from [1] after RAN4#90Bis meeting.

Table 1: Summary of Self-interference analysis for 3 bands DL/ 2 bands UL CA

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_3A-11A-18A | CA_3A-11A | - | 5th IMD | - | 4.9 dB |
| CA_3A-11A-26A | CA_3A-11A | - | 5th IMD | - | 4.9 dB |
| CA_1A-3A-42C | CA_1A-42C | - | - | - | N/A |
| CA_1A-3A-42C | CA_3A-42C | - | - | - | N/A |
| CA_2A-4A-13A | CA_2A-13A | - | 4th IMD | - | 7.6 dB |
|  | CA_4A-13A | - | 4th IMD | - | 6.2 dB |
| CA_2A-2A-4A-5A | CA_2A-5A | - | 4th IMD | - | 7.6 dB |
|  | CA_4A-5A | - | 2nd IMD 5th IMD | - | - 2nd and 5th IMD problems were already covered in Table 7.3.1A-0f:2DL/2UL_4A-5A of TS36.101. |
| CA_2A-2A-5A-66A-66A CA_2A-5A-66A, CA_2A-5A-66B, CA_2A-5A-66C, CA_2A-5B-66A, CA_2A-5B-66B, CA_2A-5B-66C, CA_2A-2A-5A-66A, CA_2A-2A-5A-66B, CA_2A-2A-5A-66C, CA_2A-5A-66A-66A, CA_2A-5B-66A-66A | CA_2A-5A | - | 4th IMD | - | 7.2 dB |
|  | CA_5A-66A | - | - | - | N/A |
| CA_2A-5A-46D | CA_2A-5A | 3rd Harmonic | 4th IMD 5th IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 2.4 dB for IMD4 - 2.7 dB for IMD5 |
| CA_5A-46D-66A | CA_5A_46A | - | 5th IMD | - | 0.3 dB |
|  | CA_5A_66A | 3rd Harmonic | 4th IMD 5th IMD | - | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 2.5 dB for IMD4 - 0 dB for IMD5 |
| CA_2A-13A-66A-66B CA_2A-13A-66A, CA_2A-2A-13A-66A, CA_2A-13A-66A-66A, CA_2A-13A-66B, CA_2A-13A-66C, | CA_2A-13A | - | 4th IMD | - | 7.2 dB |
|  | CA_13A-66A | - | 4th IMD | - | 6.2 dB |
| CA_2A-13A-48A-48C | CA_2A-13A | 2nd Harmonic at high frequency band edge | - | - | 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. |
| CA_13A-46D-66A | CA_13A-66A | 3rd Harmonic | 4th IMD 5th IMD | - | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 7.2 dB for IMD4 - 0 dB for IMD5 |
| CA_2A-13A-46D | CA_2A-13A | 3rd Harmonic | 4th IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 2.5 dB |
| CA_1A-3A-38A | CA_1A-3A | - | - | - | N/A |
| CA_2A-12A-66A | CA_2A-12A | 3rd Harmonic impact from B12 to B66 | - | - | - 3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_2A-66A | - | - | - | N/A |
|  | CA_12A-66A | 3rd Harmonic impact from B12 to B66 | Side lobe impact of 4th IMD | - | - 3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. - 0 dB |
| CA_1A-3A-42D | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 4th IMD | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 4th IMD problem was already covered in Table 7.3.1A0g:3DL/2UL_CA_1A-3A-42A/CA_1A-3A of TS36.101. |
|  | CA_1A-42A | - | - | - | N/A |
|  | CA_3A-42A | - | - | - | N/A |
|  | CA_1A-42C | - | - | - | N/A |
|  | CA_3A-42C | - | - | - | N/A |
| CA_2A-46E-48A, CA_2A-46D-48A | CA_2A-48A | 3rd Harmonic | 2nd IMD 3rd IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS 36.101 - No need to study for IMDs since Reference sensitivity for CA with band 46 is specified in Table 7.3.1A-0eA of TS 36.101 |
| CA_2A-48A-66A | CA_48A-66A | - | 2nd IMD 5th IMD | - | FFS |
| CA_3A-8A-38A | CA_3A-8A | - | 2nd IMD 3rd IMD | - | FFS |
| CA_1A-1A-3C-5A | CA_1A-3A | - | - | - | N/A |
|  | CA_1A-5A | - | - | - | N/A |
|  | CA_3A-5A | - | - | - | N/A |
| CA_1A-1A-3C-28A | CA_1A-3A | - | - | - | N/A |
|  | CA_1A-28A | - | 5th IMD | - | FFS |
|  | CA_3A-28A | 3rd Harmonic | 4th IMD | - | - 3rd harmonic impact from B28 to B1 was covered in Table 7.3.1A-0a of TS36.101. FFS |
| CA_1A-3A-3A-7A, CA_1A-3A-3A-7A-7A | CA_1A-3A | - | - | - | N/A |
|  | CA_1A-7A | - | - | - | N/A |
|  | CA_3A-7A | - | - | - | N/A |

For MSD analysis, UE RF component parameters in Table 2 and each isolation level of RF components in Table 3 are considered.

Table 2: UE RF Front-end component parameters

| UE ref. architecture | Cascaded Diplexer Architecture |  |  |  |
| --- | --- | --- | --- | --- |
| Component | All CA band combos |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 |
| Diplexer | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 |
| Quadplexer | 110 | 72 | 55 | 52 |
| PA Forward | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 |

Table 3: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Diplexer | 25 | High/low band isolation |
| Quadplexer | 15 | Adjacent Tx-Rx atten level |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

In Table 4, MSD levels are for 3 bands DL/2 bands UL inter-band CA band are proposed. If several interested companies brings different MSD levels, then they will be averaged to finalize the values.

Table 4: Proposed MSD levels for 3 bands DL/2 bands UL inter-band CA in rel-16

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-48A-66A | CA_48A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | 28.3 | FDD-TDD | IMD2 |
|  |  | 48 | 3695 | 5 | 25 | 3695 | 5 | N/A |  |  |
|  |  | 66 | 1735 | 5 | 25 | 2135 | 5 | N/A |  |  |
|  |  | 2 | 1895 | 5 | 25 | 1975 | 5 | 0 |  | IMD5 |
|  |  | 48 | 3620 | 5 | 25 | 3620 | 5 | N/A |  |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | N/A |  |  |
| CA_3A-8A-38A | CA_3A-8A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | IMD2 |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 | N/A | FDD |  |
|  |  | 38 | 2610 | 5 | 25 | 2610 | 5 | 26.4 | TDD |  |
|  | CA_3A-8A | 3 | 1750 | 5 | 25 | 1845 | 5 | N/A | FDD | IMD3 |
|  |  | 8 | 900 | 5 | 25 | 945 | 5 | N/A | FDD |  |
|  |  | 38 | 2600 | 5 | 25 | 2600 | 5 | 15.7 | TDD |  |

3		Conclusion

In this contribution, we propose MSD levels for 3 bands DL/2 bands UL inter-band CAs that have potential IMD problems. Also, these MSD levels can be merged with other MSD levels from companies.

=== Text Proposal for TR36.716-03-02 section 6.x ===

6.16 LTE-A inter-band CA: Band 2 and Band 48 and Band 66 DL with 2 bands UL

----- Unchanged sections omitted -----

6.16.1.3 MSD

When uplink CA (band 48 and band 66) is paired with downlink CA (band 2 and band 48 and band 66), the 2nd and 5th order IMD products by band 48 and band 66 falls into the own Rx frequency band 2. In Table 6.x.1.3-1, evaluated MSD values of 3 bands (band 2 and band 48 and band 66) DL with 2 bands (band 48 and band 66) UL is shown.

Table 6.16.1.3-1: MSD summary for UL CA band 48 and band 66 and DL CA band 2 and band 48 and band 66

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-48A-66A | CA_48A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | 28.3 | FDD-TDD | IMD2 |
|  |  | 48 | 3695 | 5 | 25 | 3695 | 5 | N/A |  |  |
|  |  | 66 | 1735 | 5 | 25 | 2135 | 5 | N/A |  |  |
|  |  | 2 | 1895 | 5 | 25 | 1975 | 5 | 0 |  | IMD5 |
|  |  | 48 | 3620 | 5 | 25 | 3620 | 5 | N/A |  |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | N/A |  |  |

----- Unchanged sections omitted -----

6.18 LTE-A inter-band CA: Band 3 and Band 8 and Band 38 DL with 2 bands UL

----- Unchanged sections omitted -----

6.18.1.3 MSD

When uplink CA_3A_8A is paired with downlink CA_3A-8A-38A, the MSD issue is analysed in this proposal [3] from Huawei. In Table 6.18.1.3-1, the MSD analysis results from interested companies are summarized.

Table 6.18.1.3-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | Huawei | LGE | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CA_3A-8A-38A | B3 | IMD2 | ¦fB3 + fB8¦ | 24.1 | 28.6 | 26.4 |
|  | B8 |  |  |  |  |  |
| CA_3A-8A-38A | B3 | IMD3 | ¦2*fB3 - fB8¦ | 15.7 | 15.7 | 15.7 |
|  | B8 |  |  |  |  |  |

And the MSD results are listed as below.

Table 6.18.1.3-2: MSD summary for UL CA_3A-8A and DL CA_3A-8A-38A

| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| CA_3A-8A-38A | CA_3A-8A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | N/A |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 | N/A | FDD | N/A |
|  |  | 38 | 2610 | 5 | 25 | 2610 | 5 | 26.4 | TDD | IMD2 |
|  | CA_3A-8A | 3 | 1750 | 5 | 25 | 1845 | 5 | N/A | FDD | N/A |
|  |  | 8 | 900 | 5 | 25 | 945 | 5 | N/A | FDD | N/A |
|  |  | 38 | 2600 | 5 | 25 | 2600 | 5 | 15.7 | TDD | IMD3 |

----- Unchanged sections omitted -----

6.19 LTE-A inter-band CA: Band 1 and Band 3 and Band 28 DL with 2 bands UL

----- Unchanged sections omitted -----

6.19.1.3 MSD

When uplink CA_3A_28A is paired with downlink CA_1A-1A-3C-28A, the 4th order IMD products by band 28 and band 3 falls into the own Rx frequency band 1.

When uplink CA_1A_28A is paired with downlink CA_1A-1A-3C-28A, the 5th order IMD products by band 1 and band 28 falls into the own Rx frequency band 3.

There is no need to study for the IMD4 and IMD5 due to the fact that it was already covered by Table 7.3.1A-0g: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations in TS 36.101.

For UL CA_1A-3A paired with downlink CA_1A-1A-3C-28A, there is no IMD products falls into the own Rx band.

----- Unchanged sections omitted -----

----- End of TP -----

References

- TR 36.716-03-02 v0.5.0