TSG-RAN Working Group 4 (Radio) meeting #91	R4-1906171

Reno, US, 13th – 17th May 2019

Source:	Ericsson

Title:	On TDD OFF power and carrier configuration aspects

Agenda item:	6.8.5.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#90bis in Xian) the final detail related to TDD OFF power requirement was settled for FR2. As an improvement it was suggested to improve the specification text specifying the measurement time required to measure the noise spectral density. A proposal was presented in [1].

In this contribution we describe the technical background for the measurement time part of the TDD OFF power requirement test procedure.

# Discussion

The technical background to the measurement integration time is to filter the signal to get a smooth up and down ramp of the energy to minimize switch transient spectrum effects. The guard symbol is related to the Sub-Carrier Spacing (SCS), therefore is the measurement time determined by the SCS configuration.

In the current test procedure in TS 38.141-2, the following text can be found:

Measure the mean power spectral density as power sum over two orthogonal polarizations over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

This means that the integration time required to measure the noise spectral density is varying as function of configured SCS. The current version of TS 38.104 defines the sub-carrier-spacing (SCS) configurations as listed for FR1 and FR2 in Table 2-1 and Table 2-2.

Table 2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 121 | 135 |

Table 2-2: Transmission bandwidth configuration NRB for FR2

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

With different SCS configurations the integration time will vary as function of configured carrier SCS. The integration time in s is currently defined as:

t=70SCS15

, where SCS is the sub-carrier-spacing in kHz. The integration time is listed for supported SCS in Table 2-3.

Table 2-3: Integration time

| SCS (kHz) | Integration time (s) |
| --- | --- |
| 15 | 70 |
| 30 | 35 |
| 60 | 17.5 |
| 120 | 8.75 |

At the last meeting is was suggested to replace N with 2, where  is the subcarrier configuration. This change gives the equivalent results under the assumption of the definition of  to be as described in Table 2-4.

Table 2-4: Subcarrier configuration

| SCS (kHz) | Subcarrier configuration,  |
| --- | --- |
| 15 | 0 |
| 30 | 1 |
| 60 | 2 |
| 120 | 3 |

Currently, the definition of subcarrier configuration is not very clear in TS 38.104, TS 38.141-1 and TS 38.141-2.

In TS 38.211, the subcarrier configuration,  is defined as:

Δf=2μ∙15 (kHz)

Even though, subcarrier configuration is defined in the RAN1 specification TS 38.211, it is not used in RAN4 specifications. From an RF perspective it is better to keep the current definition based on SCS in TS 38.104, TS 38.141-1 and TS 38.141-2, since SCS is used everywhere else in RAN4 specifications.

It shall also be noticed that for the conformance test specification, TS 38.141-1, subclause 4.8.3 and TS 38.141-2, subclause 4.8.2 states following;

Unless otherwise stated, single carrier configuration (SC) tests shall be performed using signal with narrowest supported BS channel bandwidth with the smallest supported subcarrier spacing declared per operating band.

# Conclusion

In this contribution we present some technical background information for TDD OFF power requirement and details related to the configuration of the measurement receiver. The integration time to be used depends on the configuration of the carrier to be measured on.

Since SCS is a key parameter in RAN4 specifications it is suggested to keep the definition of the integration time based on SCS, instead of using the parameter  defined in TS 38.211.

Proposal:

For TDD OFF use SCS as parameter in TS 38.104, TS 38.141-1 and TS 38.141-2.

# References

[1]	R4-1903263, “Transmit ON/OFF power”, CATT, RAN4#90bis