3GPP TSG-RAN4 Meeting #91	R4-1906181

# Reno, US,  13th- 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-2: Improving language on “narrowest beam” in subclause 6.2 and subclause 6.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A beamwidth is a number, and cannot be narrow. Smallest beamwidth is correct langauge. In this CR, the specification text is updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Language improved for Radiated Transmit Power (6.2) and OTA Occupied Bandwidth (6.7). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the specification text will be unclear and difficult to understand. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 6.2 and subclause 6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.145-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--- Start of text proposal

## 6.2	Radiated transmit power

### 6.2.1	Definition and applicability

Radiated transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction.

For each declared beam, the requirement is based on declarations captured in subclause 4.6 for a beam identifier (D.3), reference beam direction pair (D.8), rated beam EIRP (D.11) at the beam's reference direction pair, OTA peak directions set (D.9), the beam direction pairs at the maximum steering directions (D.10) and their associated rated beam EIRP and beamwidth(s) for reference beam direction pair and maximum steering directions (D.12).

For a declared beam identifier and beam direction pair, the rated beam EIRP level is the maximum power that the BS is declared to radiate at the associated beam peak direction during the transmitter ON period.

For each beam peak direction associated with a beam direction pair within the OTA peak directions set, a specific rated beam EIRP level may be claimed. Any claimed value shall be met within the accuracy requirement as described below. Rated beam EIRP is only required to be declared for the beam direction pairs subject to conformance testing as detailed in subclause 6.2.4.1.

NOTE 1:	The OTA peak directions set for a beam is the complete continuous or discrete set of all beam direction for which the EIRP accuracy is intended to be achieved for the beam.

NOTE 2:	A beam direction pair consists of a beam centre direction and an associated beam peak direction.

NOTE 3:	A declared EIRP value is a value provided by the manufacturer for verification according to the conformance specification declaration requirements, whereas a claimed EIRP value is provided by the manufacturer to the equipment user for normal operation of the equipment and is not subject to formal conformance testing.

For operating bands where the supported fractional bandwidth (FBW) is larger than 6%, two rated carrier EIRP may be declared by manufacturer:

-	Prated,c,FBWlow for lower supported frequency range, and

-	Prated,c,FBWhigh for higher supported frequency range.

For frequencies in between FFBWlow and FFBWhigh the rated carrier EIRP is:

-	Prated,c,FBWlow, for the carrier whose carrier frequency is within frequency range FFBWlow ≤ f < (FFBWlow +FFBWhigh) / 2,

-	Prated,c,FBWhigh, for the carrier whose carrier frequency is within frequency range (FFBWlow +FFBWhigh) / 2 ≤ f ≤FFBWhigh.

Radiated transmit power is directional requirement applicable to BS type 1-H, BS type 1-O and BS type 2-O.

### 6.2.2	Minimum requirement

Radiated transmit power minimum requirement for BS type 1-H and BS type 1-O is defined in TS 38.104 [2], subclause 9.2.2.

Radiated transmit power minimum requirement for BS type 2-O is defined in TS 38.104 [2], subclause 9.2.3.

### 6.2.3	Test purpose

The test purpose is to verify the ability to accurately generate and direct radiated power per beam, across the frequency range and under normal conditions, for all declared beams of the BS type 1-H, BS type 1-O and BS type 2-O.

### 6.2.4	Method of test

#### 6.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: B, M and T; see subclause 4.9.1.

Base station RF bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW, MRFBW and TRFBW in single-band operation, see subclause 4.9.1,

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

Directions to be tested:

-	OTA peak directions set reference beam direction pair (D.8), and

-	OTA peak directions set maximum steering directions (D.10).

Beams to be tested:	Declared beam with the highest intended EIRP for the smallest intended BeWθ, or for the smallest intended BeWϕ (D.3, D.11).

In addition for the BS type 1-O and BS type 2-O, a single test case shall be performed under extreme test environment as defined in annex B.3. In this case, it is sufficient to test on a single combination of one NR-ARFCN, one RF bandwidth position and with only one applicable test configuration defined in subclause 4.7. Direction to be tested is only at OTA peak directions set reference beam direction pair (D.8).

NOTE:	Tests under extreme power supply also test extreme temperature.

--- End of text proposal

--- Start of text proposal

### 6.7.2	OTA occupied bandwidth

#### 6.7.2.1	Definition and applicability

The OTA occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. See also recommendation ITU-R SM.328 [13].

The value of /2 shall be taken as 0.5%.

The OTA occupied bandwidth requirement applies during the transmitter ON period for a single transmitted carrier. The minimum requirement below may be applied regionally. There may also be regional requirements to declare the OTA occupied bandwidth according to the definition in the present clause.

The OTA occupied bandwidth is defined as a directional requirement and shall be met in the manufacturer’s declared OTA coverage range at the RIB.

#### 6.7.2.2	Minimum requirement

The minimum requirement for BS type 1-O and BS type 2-O is in TS 38.104 [2], subclause 9.7.2.2.

#### 6.7.2.3	Test purpose

The test purpose is to verify that the emission at the RIB does not occupy an excessive bandwidth for the service to be provided and is, therefore, not likely to create interference to other users of the spectrum beyond undue limits.

#### 6.7.2.4	Method of test

##### 6.7.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M; see subclause 4.9.1.

Directions to be tested: OTA coverage range reference direction (D.35).

Beams to be tested: Declared beam with the highest intended EIRP for the smallest intended BeWθ, or for the smallest intended BeWϕ (D.3, D.11).

Aggregated Channel Bandwidth positions to be tested for contiguous carrier aggregation: MBW Channel CA; see subclause 4.9.1.

For a BS declared to be capable of single carrier operation, start transmission according to the applicable test configuration in subclause 4.8 using the corresponding test model NR-FR1-TM1.1 for BS type 1-O or NR-FR2-TM1.1 for BS type 2-O in subclause 4.9.2 at manufacturers declared rated carrier output EIRP (PRated,c,EIRP, D.11).

For a BS declared to be capable of contiguous carrier aggregation operation, set the base station to transmit according to NR-FR1-TM1.1 for BS type 1-O or NR-FR2-TM1.1 for BS type 2-O in subclause 4.9.2 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclauses 4.7.2.3.1 and 4.8.

--- End of text proposal