3GPP TSG-RAN WG4 Meeting #91	R4-1906272

# Reno, NV, 13 – 17 May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | <CR#> | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to MPR/A-MPR and additional requirements for intra-band EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The applicability of the A-MPR when the appropriate NS value is not indicated (as the MPR) as specified by table notes in 6.2B.3 is not consistent with recent changes of the MPR for intra-band EN-DC. MPR should apply when NS_01 is indicated in both cell groups and band-agnostic requirements apply for EN-DC. In general, the standalone requirements also applies unless otherwise stated, from clause 4.1: “Terminal that supports EN-DC configuration shall meet E-UTRA requirements as specified in TS 36.101 [4] and NR requirements as in TS 38.101-1 [2] and TS 38.101-2 [3] unless otherwise specified in this specification”. These standalone requirements may not be met for UEs supporting simultaneous UL transmissions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2B.3.1: it is clarified that the A-MPR applies for UEs configured with EN-DC. For these the A-MPR applies when the applicable combinations of NS values are indicated (NOTE 2 modified). NOTE 3 and NOTE 4 are ambigous and removed – if the combination of NS values is not applicable the A-MPR is not allowed and the requirements undefined unless NS_01 is indicated on both CGs (then band-agnostic requirements apply). Clauses 6.5B.2.1.2, 6.5B.2.2.1: the stand.alone requirements do not apply for UEs configured with EN-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specification of MPR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3.1, 6.5B.2.1.2, 6.5B.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 6.2B.3	UE additional maximum output power reduction for EN-DC

#### 6.2B.3.1	Intra-band contiguous EN-DC

##### 6.2B.3.1.0	General

For EN-DC band combinations with additional requirements the allowed A-MPR is specified in Table 6.2B.3.1.0-1 for UEs configured with EN-DC and combinations of network signalling values indicated in the E-UTRA and NR cell groups.

Unless otherwise stated the A-MPR specified in sub-clause 6.2B.3.1 for intra-band contiguous EN-DC configurations is the total power reduction allowed including MPR.

Table 6.2B.3.1.0-1: Additional maximum power reduction for Intra-band contiguous EN-DC

| DC configuration | Requirement (sub-clause) | E-UTRA network signalling value | NR network signalling value | A-MPR (subclause) |
| --- | --- | --- | --- | --- |
| DC_(n)71AA | 6.5B.2.1.2.1 | NS_35 | NS_35 | 6.2B.3.1.13 |
| DC_(n)41AA1 | 6.5B.2.1.2.2 | NS_01 or NS_04 | NS_04 | 6.2B.3.1.24 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. NOTE 2: The additional emission requirement applies when the combination of network signalling values in the two CGs is set (only for UEs configured with EN-DC). |  |  |  |  |

##### 6.2B.3.1.1	A-MPR for DC_(n)71AA

< text omitted >

##### 6.5B.2.1.2	Additional spectrum emissions mask

###### 6.5B.2.1.2.1	Requirements for network signalled value "NS_35"

When NS_35 is indicated in the MCG and NS_35 is indicated in the SCG, the requirements in Table 6.5B.2.1.2.1-1 apply in the frequency ranges immediately adjacent and outside the aggregated sub-blocks of the EN-DC configuration for DC_(n)71AA.

Table 6.5B.2.1.2.1-1: Additional requirements

| ΔfOOB (MHz) | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1 MHz | 0.015 MHz f_offset < 0.085 MHz | -13 | 30 kHz |
| 0.1 MHz f < ENBW | 0.15 MHz f_offset < ENBW – 0.05 MHz | -13 | 100 kHz |
| ENBW f < ENBW + 5 MHz | ENBW+0.5 MHz f_offset < ENBW + 4.5 MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |  |

For UEs configured with EN-DC, the additional requirements in sub-clause 6.6.2.2.7 of [4] for stand-alone operation of the MCG with NS_35 indicated and the additional requirements in 6.5.2.3.1 of [2] for stand-alone operation of the SCG with NS_35 indicated do not apply.

###### 6.5B.2.1.2.2	Requirements for network signalled value "NS_04"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The Band 41/n41 SEM transition point from -13 dBm/MHz to -25 dBm/MHz is based on the emission bandwidth. The emission bandwidth is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power.  Since the 26 dB emission bandwidth is implementation dependent, the transmission bandwidths occupied by RBs is used for the SEM. The emission bandwidth for LTE carriers is document in TS 36.101 [4], and the emission bandwidth for NR carriers is documented in 38.101-1 [2]. The total emission bandwidth for contiguous intra-band EN-DC is the sum of the emission bandwidth for each CC plus the guard band between contiguous CCs.

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.2.1.2.2-1.

Table 6.5B.2.1.2.2-1: DC_(n)41 SEM with NS_04

|  | Spectrum emission limit (dBm) / measurement bandwidth for each ENBW |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz |  | 15 MHz | 20 MHz | 40 MHz | 50 MHz | > 50 MHz | Measurement bandwidth |
| ± 0 - 1 |  | -10 | -10 | -10 |  |  | 2 % ENBW |
|  |  |  |  |  | -10 |  | 1 MHz |
| ± 1 - 5 | -10 |  |  |  |  |  | 1 MHz |
| ± 5 - X | -13 |  |  |  |  |  |  |
| ± X - (ENBW+ 5 MHz) | -25 |  |  |  |  |  |  |
| NOTE: X is defined as the sum of the emission bandwidth of the component carriers plus the guard band between contiguous CCs. |  |  |  |  |  |  |  |

For UEs configured with EN-DC, the additional requirements in sub-clause 6.6.2.2.2 of [4] for stand-alone operation of the MCG with NS_04 indicated and the additional requirements in 6.5.2.3.2 of [2] for stand-alone operation of the SCG with NS_04 indicated do not apply.

##### 6.5B.2.1.3	Adjacent channel leakage ratio

< text omitted >

#### 6.5B.2.2	Intra-band non-contiguous EN-DC

##### 6.5B.2.2.1	Spectrum emissions mask

For UEs configured with EN-DC, the spectral emission mask for intra-band non-contiguous EN-DC is a composite of the emission mask for each CC with the level set to the maximum value from each mask for each frequency outside of the transmission bandwidth of either carrier. A composite spectrum emission mask is a combination of individual CC spectrum emissions masks. Where two masks overlap the most relaxed limit is used notwithstanding the requirements in [2] and [4] for stand-alone operation of each CG. Composite spectrum emission mask applies to frequencies up to  ΔfOOB starting from the edges of the sub-blocks. If for some frequency an individual CC spectrum emission mask overlaps with the bandwidth of another CC then the emission mask does not apply for that frequency.

##### 6.5B.2.2.2	Additional spectrum emissions mask

< end of changes >