3GPP TSG-RAN4 Meeting # 91	R4-1906321

# Reno, Nevada, USA,  13 May – 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.133: TCI state switch delay requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - MAC CE based and RRC based active TCI state switch delay requirements are not completed - TCI state known condition is open - Active TCI state update for PDSCH is not introdcued yet |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Active TCI state switch requirements are introduced for MAC CE based and RRC based. - TCI state known condition is introduced. - Active TCI state update for PDSCH is introduced - Reference correction for THARQ - Format Correction for Note 4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Active TCI state switch requirements are not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.10	Active TCI state switching delay

### 8.10.1	Introduction

The requirements in this section apply for a UE configured with one or more TCI state configurations on serving cell in MR-DC or standalone NR. UE shall complete the switch of active TCI state within the delay defined in this section.

### 8.10.2	Known conditions for TCI state

The TCI state is known if it has been meeting the following conditions:

- The target TCI state remains detectable during the TCI state activation delay

- The UE reports L1-RSRP/L3-RSRP in last 500ms for the target TCI state

Otherwise, the TCI state is unknown.

Note 1: FFS on the side condition for TCI state detection.

### 8.10.3	MAC-CE based TCI state switch delay

If the target TCI state is known to UE, for UE supporting power class 1, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+  ceil [(THARQ + 3ms)/ NR slot length (ms)], where THARQ is the timing between DL data transmission and acknowledgement as specified in [7].

If the target TCI state is not known to UE, for UE supporting power class 1, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ ceil [(THARQ +3ms + TSMTC)/ NR slot length (ms)] where THARQ is the timing between DL data transmission and acknowledgement as specified in [3].

Note 2: If the target TCI state is known to UE, FFS the delay requirement for UE supporting power class 2/3/4.

Note 3: If the target TCI state is unknown to UE, FFS the delay requirement for UE supports power class 1/2/3/4.

### 8.10.4	DCI based TCI state switch delay

If the target TCI state is known to UE, for UE supporting power class 1, when a UE is configured with the higher layer parameter tci-PresentInDCI which is set as 'enabled' for the CORESET scheduling the PDSCH at slot n, UE shall be able to receive PDSCH or transmit PUSCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+timeDurationForQCL, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], the value of TQCL_time is defined in TS 38.306[26].

The known condition for TCI state defined in section 8.7.2 is applied.

If the target TCI state is not known to the UE, DCI based TCI state switch is not supported for any power class.

Note 4: If the target TCI state is known to UE, FFS the delay requirement for UE supports power class 2/3/4.

### 8.10.5	RRC based TCI state switch delay

If the target TCI state is known to UE, for UE supporting power class 1, upon receiving PDSCH carrying RRC Reconfiguration message at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ ceil [TRRCprocessingDelay/ NR slot length (ms)], where TRRCprocessingDelay is the length of the RRC procedure delay in millisecond as defined in clause 12 in TS 38.331 [2].

If the target TCI state is not known to UE, for UE supporting power class 1, upon receiving PDSCH carrying RRC Reconfiguration message at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ ceil [(TRRCprocessingDelay + TSMTC)/ NR slot length (ms)], where TRRCprocessingDelay is the length of the RRC procedure delay in millisecond as defined in clause 12 in TS 38.331 [2].

The known condition for TCI state defined in section 8.7.2 is applied.

### 8.10.6 	Active TCI states update for PDSCH

If the UE is configured with more than one TCI state for PDSCH by RRC, active TCI states is updated by MAC CE. Upon receiving PDSCH active TCI states update by MAC CE command at slot n, UE shall be able to receive on any of the TCI state from the active TCI states from the slot n+ ceil [(THARQ + 3ms)/ NR slot length (ms)], where THARQ is the timing between DL data transmission and acknowledgement as specified in [3].

9	Measurement Procedure

## 9.1	General measurement requirement