3GPP TSG-RAN WG4 Meeting #91	R4-1906385

Reno, US, 13 - 17 May, 2019

Agenda item:	7.13.2

Source:	Dish Network

Title:	Considerations on NB-IoT/NR Co-existence

Document for:	Approval

# 1	Introduction

A WF this the NB-IoT/NR co-existence was agreed in RAN4#90bis [1]. This contribution discusses definitions of NB-IoT in case when NB-IoT is deployed within NR channel bandwidth.

# 2	Discussion

According to agreements in [1]:

- WF on system scenarios

  - To clarify the current definition of NB-IoT operation modes when NB-IoT is deployed within NR  channel bandwidth, including when spectrum is refarmed from E-UTRA to NR

      - whether NB-IoT in-band or guard band operation without a hosting E-UTRA carrier would have backward compatibility impact on Rel’13/14/15 NB-IoT UE, which may expect the presence of a hosting E-UTRA carrier, especially for NB-IoT in-band operation

- Agreement on NB-IoT standalone operating

  - There is no issue for NB-IoT standalone coexistence with NR and no further study is needed.

  - simulation results will be added in TR annex

  - Nokia/Huawei will provide TP in next meeting

- WF on power boosting

  - Further investigate if currently specified power boosting could be supported, including when NB-IoT is located at NR edge positions for any channel bandwidth.

  - At least 3dB (currently specified RE power dynamic range) power boosting should be feasible.

- Agreement on channel raster

  - for NR bands with 100 kHz channel raster and 15 kHz SCS, subcarrier and PRB alignment can be achieved for NB-IoT operating in NR in-band.

  - for NR bands with 15 kHz channel raster and 15 kHz SCS, subcarrier and PRB alignment can be achieved for NB-IoT operating in NR in-band.

  - Ericsson/ZTE will provide TP in next meeting

- Agreement on mixed numerology

  - no requirement is defined for mixed numerology between NR and NB-IoT similar as NR mixed numerology itself.

  - ZTE will provide TP in next meeting

- Agreement on bands

  - NB-IoT shall be supported on all NR bands refarmed from E-UTRA bands supporting NB-IoT.

FFS topics are:

- NB-IoT operating in NR guard band

- TAGs misalignment

- TDD configuration for sync between NR and NB-IoT

- Specification and conformance impact

We consider current definitions of NB IoT operating modes.

NB-IoT is originally defined in TS 36.101 as follows:

Category NB1/NB2 stand-alone operation: category NB1/NB2 is operating standalone when it utilizes its own spectrum, for example the spectrum used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment.

Category NB1/NB2 guard band operation: category NB1/NB2 is operating in guard band when it utilizes the unused resource block(s) within a E-UTRA carrier’s guard-band.

Category NB1/NB2 in-band operation: category NB1/NB2 is operating in-band when it utilizes the resource block(s) within a normal E-UTRA carrier.

For both in-band and guard band cases, the current specification (LTE 36 series) talks about an E-UTRA carrier. This is natural as during NB-IoT work a few years ago NR standard was not specified at all.

Basically, the problem setting is as follows. In LTE, deployment where an NB-IoT carrier is located in-band LTE is called “In-band operation mode” and deployment where an NB-IoT carrier is located in LTE guard band is called “Guard band operation mode”.

The question is how are in-band and guard band operation modes called when there is no E-UTRA carrier but and NR carrier instead?

Are the modes still “In-band operation mode” and “guard band operation mode”, are they “Stand-alone mode” or are they something else, such as “NB-IoT in-band NR”?

If deployment where NB-IoT is within NR guard band or in-band is defined as standalone NB-IoT, then so problems arise. In this case there would be two different kind of standalone NB-IoT SA operation types, with different performances. Another example of differences is that within E-UTRA carrier, the TX-RX separation for both in-band and guard band has been flexible while in standalone, the nominal TX-RX separation is fixed.

On the other hand, new operation mode definitions (e.g. NB-IoT operation within NR guard band or in-band) would require some specification work outside the TS 36.101 section 3.1 (Definitions).

We suggest to look if the current definitions of in-band and guard band operation could be amended to include NR carrier as well. We note that the intention of this amendment is not to cause any changes to existing deployments or requirements.

Suggestion: Look if the following naming conventions could be done into TS 36.101/TS38.101 and other relevant specifications.

Category NB1/NB2 in-band operation: category NB1/NB2 is operating in-band when it utilizes the resource block(s) within a normal E-UTRA or NR carrier.

Category NB1/NB2 guard band operation: category NB1/NB2 is operating in guard band when it utilizes the unused resource block(s) within a E-UTRA or NR carrier’s guard-band.

# 3	Conclusions

Definitions of NB-IoT modes were discussed, with the following suggestion.

Suggestion: Look if the following naming conventions could be done into TS 36.101/TS38.101 and other relevant specifications.

Category NB1/NB2 in-band operation: category NB1/NB2 is operating in-band when it utilizes the resource block(s) within a normal E-UTRA or NR carrier.

Category NB1/NB2 guard band operation: category NB1/NB2 is operating in guard band when it utilizes the unused resource block(s) within a E-UTRA or NR carrier’s guard-band.

# 3	References

[1] R4-1905203 “WF on coexistence of NB-IoT with NR”, RAN4#90bis, Huawei, HiSilicon, Ericsson, CHTTL, SGS Wireless, Nokia, ZTE, Dish, T-Mobile USA