3GPP TSG RAN WG4 meeting #91 							                                   R4-1906590

# Reno, USA, 13 – 17 May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4867 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to demodulation performance for NPRACH FDD preamble format 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Perf |  |  |  |  |  | Date: |  |  | 03/05/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections to preliminary demodulation performance for new NPRACH FDD preamble format 2 according to R4-1906588 and R4-1906589 are needed. Correction to SNR definition for NPRACH is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 8.5.3.2, corrections to preliminary demodulation performance for new NPRACH FDD preamble format 2 are done to specify the final performance requirements. In subclause 8.1, the SNR is clarified to be related to the transmission duration of one random access symbol group for NPRACH, which is not aligned to the subframe duration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demodulation requirements for Rel-15 Further NB-IoT enhancements for FDD cannot be finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.141 CR 1224 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |

1st Modification

# 8	Performance requirement

## 8.1	General

Performance requirements for the BS are specified for the fixed reference channels defined in Annex A and the propagation conditions in Annex B. The requirements only apply to those FRCs that are supported by the base station.

Unless stated otherwise, performance requirements apply for a single carrier only. Performance requirements for a BS supporting carrier aggregation are defined in terms of single carrier requirements. For FDD operation the requirements in clause 8 shall be met with the transmitter(s) on.

NOTE:	In normal operating conditions the BS in FDD operation is configured to transmit and receive at the same time. The transmitter may be off for some of the tests as specified in 36.141 [4].

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in the subframe, or, in case of NPRACH, in the random access symbol group, on a 	single antenna port.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe, or, in case of NPRACH, in the random access symbol group.

For enhanced performance requirements type A and type B, the SINR used in this clause is specified based on a single carrier and defined as:

Where:

is the total signal energy of one intra-cell UE in the subframe on a single antenna port.

is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a single antenna port. The respective energy of each inter-cell interferer relative to  is defined by its associated DIP value.

2nd Modification

### 8.5.3	Performance requirements for NPRACH

#### 8.5.3.1	NPRACH False alarm probability

The false alarm requirement is valid for any number of receive antennas, for all repetition numbers and for any number of subcarriers.

The false alarm probability is the conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

##### 8.5.3.1.1	Minimum requirement

The false alarm probability shall be less than or equal to 0.1%.

#### 8.5.3.2	NPRACH detection requirements

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. A timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 3.646us. The strongest path for the timing estimation error refers to the strongest path in the power delay profile.

The requirements for TDD are optional and only valid for base stations supporting TDD.

Table 8.5.3.2-1: Test preambles for NPRACH

| Parameter | Value |
| --- | --- |
| Narrowband physical layer cell identity | 0 |
| Initial subcarrier index | 0 |
| Uplink-downlink configuration for TDD | 1 |
| Special subframe configuration for TDD | 7 |

##### 8.5.3.2.1	Minimum requirements

The probability of detection shall be equal to or exceed 99% for the SNR levels listed in table 8.5.3.2.1-1.

Table 8.5.3.2.1-1: NPRACH missed detection requirements for FDD

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR[dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 2 |
| 1 | 2 | 8 | AWGN | 0 | -2.1 | -2.1 | -1.0 |
|  |  |  | EPA1 Low | 200 Hz | 6.1 | 6.1 | 9.0 |
|  |  | 32 | AWGN | 0 | -6.8 | -6.8 | -5.1 |
|  |  |  | EPA1 Low | 200 Hz | 0.5 | 0.5 | 2.2 |

Table 8.5.3.2.1-2: NPRACH missed detection requirements for TDD

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR[dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 0-a | Preamble format 1-a |
| 1 | 2 | 8 | AWGN | 0 | [-17.8] | [-20.4] | [-19.3] | [-23.0] |
|  |  |  | EPA1 Low | 200 Hz | [-8.6] | [-11.1] | [-9.5] | [-13.3] |
|  |  | 32 | AWGN | 0 | [-20.2] | [-22.8] | [-20.1] | [-23.0] |
|  |  |  | EPA1 Low | 200 Hz | [-13.6] | [-17.5] | [-13.3] | [-17.7] |

End of Modifications