3GPP TSG-RAN WG4 Meeting #91	R4-1906747

Reno, Nevada, US, 13 May – 17 May 2019

Source:	Ericsson, Telstra

Title:	TP for TR 37.716-21-11: MSD values for 3A-28A_n5A and 3C-28A_n5A

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 2 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-21-11 v0.5.0 to add the missing MSD values for 3A-28A_n5A and 3C-28A_n5A

# 2. Text Proposal

# ---Start of changes---

## 5.1.46	DC_3A-28A_n5A
DC_3C-28A_n5A

### 5.1.46.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-28_n5 | CA_3-28 | n5 | DC_3_n5 |

### 5.1.46.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-28A_n5A DC_3C-28A_n5A | DC_3A_n5A DC_3C_n5A DC_28A_n5A | CA_3A-28A CA_3C-28A | n5 |

#### 5.1.46.3	Co-existence Studies

Co-existence analysis from DC_28_n5 shows that there is IMD4 impact from DC_28_n5 UL to Band 3 DL.

Co-existence analysis from DC_3_n5 shows that there is IMD4 impact from DC_3_n5 UL to Band 28 DL.

### 5.1.46.4		∆TIB and ∆RIB values

For DC_3-28_n5, the TIB,c and RIB,c values are reused from the LTE combination CA_3-5-28, and are given in the tables below.

Table 5.1.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n5 | 3 | 0.3 |
|  | 28 | 0.5 |
|  | n5 | 0.5 |

Table 5.1.46.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n5 | 3 | 0 |
|  | 28 | 0.1 |
|  | n5 | 0.1 |

### 5.1.46.5	REFSENS requirements

Co-existence analysis from shows that there is IMD4 impact from DC_28_n5 UL to Band 3 DL, as well as IMD4 impact from DC_3_n5 UL to Band 28 DL. Therefore, MSD values are needed to be introduced in 38.101-3.

MSD values are reused from DC_20_n28-n78 which has IMD4 in n28 DL and n78 DL.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-28A_n5A DC_3C-28A_n5A | 3 | 1735 | 5 | 25 | 1830 | 8.7 | FDD | IMD4 |
|  | 28 | 705 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n5 | 845 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | 28 | 730 | 5 | 25 | 785 | 9.4 | FDD | IMD4 |
|  | n5 | 845 | 5 | 25 | 874 | N/A | FDD | N/A |

---End of changes---

# Reference

[1]	RP-190379, “Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei