3GPP TSG-RAN WG4 Meeting #91	R4-1906835

# Reno, US, 13 - 17 May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5480 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Rel-16 LTE inter-band CA for 2 bands DL with 1 band UL combinations in TS36101 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R16_2BDL_1BUL-Core |  |  |  |  |  | Date: |  |  | 2018-04-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding approved LTE inter-band CA combinations with 2 bands DL and 1 band UL from RAN4 90bis Adding approved LTE inter-band CA combinations with 2 bands DL and 1 band UL from RAN4 91 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the following LTE inter-band CA combinations with 2 bands DL and 1 band UL: From RAN4 90bis meeting: CA_25A-41E_BCS0 CA_25A-41F_BCS0 CA_25A-25A-41E_BCS0 CA_25A-25A-41F_BCS0 CA_28A-66A_BCS0 CA_3A-3A-5A_BCS0 CA_3A-3A-46C_BCS0 From RAN4 91 meeting: CA_7C-46E |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for inter-band CA combinations with 2 bands DL and 1 band UL are not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5, 5.6, 6.2, 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

Table 5.5A-2: Inter-band CA operating bands (two bands)

| E-UTRA CA Band |  | E-UTRA Band (Table 5.5.1) |  |
| --- | --- | --- | --- |
| CA_1-3 |  | 1, 3 |  |
| CA_1-1-3 |  | 1,3 |  |
| CA_1-1-5 |  | 1,5 |  |
| CA_1-1-7 |  | 1,7 |  |
| CA_1-1-28 |  | 1,28 |  |
| CA_1-3-3 |  | 1, 3 |  |
| CA_1-5 |  | 1, 5 |  |
| CA_1-7 |  | 1, 7 |  |
| CA_1-7-7 |  | 1, 7 |  |
| CA_1-8 |  | 1, 8 |  |
| CA_1-11 |  | 1, 11 |  |
| CA_1-18 |  | 1, 18 |  |
| CA_1-19 |  | 1, 19 |  |
| CA_1-20 |  | 1, 20 |  |
| CA_1-21 |  | 1, 21 |  |
| CA_1-26 |  | 1, 26 |  |
| CA_1-28 |  | 1, 28 |  |
| CA_1-32 |  | 1, 32 |  |
| CA_1-38 |  | 1, 38 |  |
| CA_1-40 |  | 1, 40 |  |
| CA_1-41 |  | 1, 41 |  |
| CA_1-42 |  | 1, 42 |  |
| CA_1-42-42 |  | 1, 42 |  |
| CA_1-43 |  | 1, 43 |  |
| CA_1-46 |  | 1, 46 |  |
| CA_2-4 |  | 2, 4 |  |
| CA_2-2-4 |  | 2, 4 |  |
| CA_2-2-4-4 |  | 2, 4 |  |
| CA_2-4-4 |  | 2, 4 |  |
| CA_2-5 |  | 2, 5 |  |
| CA_2-2-5 |  | 2, 5 |  |
| CA_2-2-7 |  | 2, 7 |  |
| CA_2-7 |  | 2, 7 |  |
| CA_2-7-7 |  | 2, 7 |  |
| CA_2-12 |  | 2, 12 |  |
| CA_2-2-12 |  | 2, 12 |  |
| CA_2-2-12-12 |  | 2, 12 |  |
| CA_2-2-29 |  | 2, 29 |  |
| CA_2-7-46 |  | 2, 7, 46 |  |
| CA_2-12-12 |  | 2, 12 |  |
| CA_2-13 |  | 2, 13 |  |
| CA_2-2-13 |  | 2, 13 |  |
| CA_2-14 |  | 2, 14 |  |
| CA_2-2-14 |  | 2, 14 |  |
| CA_2-17 |  | 2, 17 |  |
| CA_2-28 |  | 2, 28 |  |
| CA_2-29 |  | 2, 29 |  |
| CA_2-30 |  | 2, 30 |  |
| CA_2-2-30 |  | 2, 30 |  |
| CA_2-46 |  | 2, 46 |  |
| CA_2-2-46 |  | 2, 46 |  |
| CA_2-46-46 |  | 2, 46 |  |
| CA_2-48-48 |  | 2, 48 |  |
| CA_2-49 |  | 2, 49 |  |
| CA_2-66 |  | 2, 66 |  |
| CA_2-48 |  | 2, 48 |  |
| CA_2-2-66 |  | 2, 66 |  |
| CA_2-2-66-66 |  | 2, 66 |  |
| CA_2-66-66 |  | 2, 66 |  |
| CA_2-66-66-66 |  | 2, 66 |  |
| CA_2-71 |  | 2, 71 |  |
| CA_2-2-71 |  | 2, 71 |  |
| CA_3-5 |  | 3, 5 |  |
| CA_3-3-5 |  | 3, 3, 5 |  |
| CA_3-7 |  | 3, 7 |  |
| CA_3-3-7 |  | 3, 7 |  |
| CA_3-3-7-7 |  | 3, 7 |  |
| CA_3-7-7 |  | 3, 7 |  |
| CA_3-8 |  | 3, 8 |  |
| CA_3-3-8 |  | 3, 8 |  |
| CA_3-11 |  | 3, 11 |  |
| CA_3-18 |  | 3, 18 |  |
| CA_3-19 |  | 3, 19 |  |
| CA_3-3-19 |  | 3, 19 |  |
| CA_3-20 |  | 3, 20 |  |
| CA_3-3-20 |  | 3, 20 |  |
| CA_3-3-21 |  | 3, 21 |  |
| CA_3-3-28 |  | 3, 28 |  |
| CA_3-3-41 |  | 3, 41 |  |
| CA_3-3-42 |  | 3, 42 |  |
| CA_3-21 |  | 3, 21 |  |
| CA_3-26 |  | 3, 26 |  |
| CA_3-27 |  | 3, 27 |  |
| CA_3-28 |  | 3, 28 |  |
| CA_3-31 |  | 3, 31 |  |
| CA_3-32 |  | 3, 32 |  |
| CA_3-38 |  | 3, 38 |  |
| CA_3-40 |  | 3, 40 |  |
| CA_3-40-40 |  | 3, 40 |  |
| CA_3-41 |  | 3, 41 |  |
| CA_3-42 |  | 3, 42 |  |
| CA_3-42-42 |  | 3, 42 |  |
| CA_3-43 |  | 3, 43 |  |
| CA_3-46 |  | 3, 46 |  |
| CA_3-3-46 |  | 3, 46 |  |
| CA_3-69 |  | 3, 69 |  |
| CA_4-5 |  | 4, 5 |  |
| CA_4-4-5 |  | 4, 5 |  |
| CA_4-7 |  | 4, 7 |  |
| CA_4-4-7 |  | 4, 7 |  |
| CA_4-7-7 |  | 4, 7 |  |
| CA_4-12 |  | 4, 12 |  |
| CA_4-4-12 |  | 4, 12 |  |
| CA_4-4-12-12 |  | 4, 12 |  |
| CA_4-12-12 |  | 4, 12 |  |
| CA_4-13 |  | 4, 13 |  |
| CA_4-4-13 |  | 4, 13 |  |
| CA_4-17 |  | 4, 17 |  |
| CA_4-27 |  | 4, 27 |  |
| CA_4-28 |  | 4, 28 |  |
| CA_4-29 |  | 4, 29 |  |
| CA_4-4-29 |  | 4, 29 |  |
| CA_4-30 |  | 4, 30 |  |
| CA_4-4-30 |  | 4, 30 |  |
| CA_4-46 |  | 4, 46 |  |
| CA_4-46-46 |  | 4, 46 |  |
| CA_4-48 |  | 4, 48 |  |
| CA_4-71 |  | 4,71 |  |
| CA_4-4-71 |  | 4, 71 |  |
| CA_5-5-40 |  | 5, 40 |  |
| CA_5-7 |  | 5, 7 |  |
| CA_5-7-7 |  | 5, 7 |  |
| CA_5-12 |  | 5, 12 |  |
| CA_5-12-12 |  | 5, 12 |  |
| CA_5-13 |  | 5 ,13 |  |
| CA_5-17 |  | 5, 17 |  |
| CA_5-25 |  | 5, 25 |  |
| CA_5-28 |  | 5, 28 |  |
| CA_5-29 |  | 5, 29 |  |
| CA_5-30 |  | 5, 30 |  |
| CA_5-38 |  | 5, 38 |  |
| CA_5-40 |  | 5, 40 |  |
| CA_5-40-40 |  | 5, 40 |  |
| CA_5-41 |  | 5, 41 |  |
| CA_5-46 |  | 5, 46 |  |
| CA_5-48 |  | 5, 48 |  |
| CA_5-66 |  | 5, 66 |  |
| CA_5-5-66 |  | 5, 66 |  |
| CA_5-66-66 |  | 5, 66 |  |
| CA_5-5-66-66 |  | 5, 66 |  |
| CA_7-8 |  | 7, 8 |  |
| CA_7-7-8 |  | 7, 8 |  |
| CA_7-12 |  | 7, 12 |  |
| CA_7-20 |  | 7, 20 |  |
| CA_7-22 |  | 7, 22 |  |
| CA_7-26 |  | 7, 26 |  |
| CA_7-7-26 |  | 7, 26 |  |
| CA_7-28 |  | 7, 28 |  |
| CA_7-7-28 |  | 7, 28 |  |  |
| CA_7-29 |  | 7,29 |  |  |
| CA_7-7-29 |  | 7,29 |  |  |
| CA_7-30 |  | 7, 30 |  |
| CA_7-32 |  | 7, 32 |  |
| CA_7-40 |  | 7, 40 |  |
| CA_7-42 |  | 7, 42 |  |
| CA_7-42-42 |  | 7, 42 |  |
| CA_7-46 |  | 7, 46 |  |
| CA_7-7-46 |  | 7, 46 |  |
| CA_7-7-66 |  | 7, 66 |  |
| CA_7-66 |  | 7, 66 |  |
| CA_7-66-66 |  | 7, 66 |  |
| CA_7-7-66-66 |  | 7, 66 |  |
| CA_8-11 |  | 8, 11 |  |
| CA_8-20 |  | 8, 20 |  |
| CA_8-27 |  | 8, 27 |  |
| CA_8-28 |  | 8, 28 |  |
| CA_8-32 |  | 8, 32 |  |
| CA_8-38 |  | 8, 38 |  |
| CA_8-39 |  | 8, 39 |  |
| CA_8-40 |  | 8, 40 |  |
| CA_8-41 |  | 8, 41 |  |
| CA_8-42 |  | 8, 42 |  |
| CA_8-46 |  | 8, 46 |  |
| CA_11-18 |  | 11, 18 |  |
| CA_11-26 |  | 11, 26 |  |
| CA_11-28 |  | 11, 28 |  |
| CA_11-41 |  | 11, 41 |  |
| CA_11-42 |  | 11, 42 |  |
| CA_11-46 |  | 11, 46 |  |
| CA_12-25 |  | 12, 25 |  |
| CA_12-30 |  | 12, 30 |  |
| CA_12-46 |  | 12, 46 |  |
| CA_12-48 |  | 12, 48 |  |
| CA_12-48 |  | 12, 48 |  |
| CA_12-66 |  | 12, 66 |  |
| CA_12-66-66 |  | 12, 66 |  |
| CA_13-46 |  | 13, 46 |  |
| CA_13-48 |  | 13, 48 |  |
| CA_13-48-48 |  | 13, 48 |  |
| CA_13-66 |  | 13, 66 |  |
| CA_13-66-66 |  | 13, 66 |  |
| CA_14-66 |  | 14, 66 |  |
| CA_14-66-66 |  | 14, 66 |  |
| CA_14-66-66-66 |  | 14, 66 |  |
| CA_14-30 |  | 14, 30 |  |
| CA_18-281 |  | 18, 28 |  |
| CA_18-42 |  | 18, 42 |  |  |
| CA_19-21 |  | 19, 21 |  |
| CA_19-282 |  | 19, 28 |  |
| CA_19-42 |  | 19, 42 |  |
| CA_19-46 |  | 19, 46 |  |
| CA_20-281 |  | 20, 28 |  |
| CA_20-31 |  | 20, 31 |  |
| CA_20-32 |  | 20, 32 |  |
| CA_20-38 |  | 20, 38 |  |
| CA_20-40 |  | 20, 40 |  |
| CA_20-40-40 |  | 20, 40 |  |
| CA_20-42 |  | 20, 42 |  |
| CA_20-42-42 |  | 20, 42 |  |
| CA_20-43 |  | 20, 43 |  |
| CA_20-67 |  | 20, 67 |  |
| CA_20-75 |  | 20, 75 |  |  |
| CA_20-76 |  | 20, 76 |  |  |
| CA_21-28 |  | 21, 28 |  |
| CA_21-42 |  | 21, 42 |  |
| CA_21-46 |  | 21, 46 |  |
| CA_23-29 |  | 23, 29 |  |
| CA_25-26 |  | 25, 26 |  |
| CA_25-25-26 |  | 25, 26 |  |
| CA_25-41 |  | 25, 41 |  |
| CA_25-25-41 |  | 25, 41 |  |
| CA_25-46 |  | 25, 46 |  |
| CA_26-41 |  | 26, 41 |  |
| CA_26-46 |  | 26, 46 |  |
| CA_26-48 |  | 26,48 |  |
| CA_26-48-48 |  | 26,48 |  |
| CA_28-32 |  | 28, 32 |  |  |
| CA_28-38 |  | 28,38 |  |
| CA_28-40 |  | 28, 40 |  |
| CA_28-41 |  | 28, 41 |  |
| CA_28-42 |  | 28, 42 |  |
| CA_28-42-42 |  | 28, 42 |  |  |
| CA_28-46 |  | 28, 46 |  |
| CA_28-66 |  | 28, 66 |  |
| CA_29-30 |  | 29, 30 |  |
| CA_29-66 |  | 29, 66 |  |
| CA_29-66-66 |  | 29, 66 |  |
| CA_29-70 |  | 29, 70 |  |
| CA_30-66 |  | 30, 66 |  |
| CA_30-66-66 |  | 30, 66 |  |
| CA_32-42 |  | 32, 42 |  |
| CA_32-43 |  | 32, 43 |  |
| CA_34-39 |  | 34, 39 |  |
| CA_34-41 |  | 34, 41 |  |
| CA_38-40 |  | 38, 40 |  |
| CA_38-40-40 |  | 38, 40 |  |
| CA_39-41 |  | 39, 41 |  |
| CA_39-40 |  | 39, 40 |  |
| CA_39-42 |  | 39, 42 |  |
| CA_39-46 |  | 39, 46 |  |
| CA_40-41 |  | 40, 41 |  |
| CA_40-42 |  | 40, 42 |  |
| CA_40-43 |  | 40, 43 |  |
| CA_40-46 |  | 40, 46 |  |
| CA_41-42 |  | 41, 42 |  |
| CA_41-42-42 |  | 41, 42 |  |  |
| CA_41-46 |  | 41, 46 |  |
| CA_41-48 |  | 41, 48 |  |
| CA_42-43 |  | 42,43 |  |
| CA_42-46 |  | 42, 46 |  |
| CA_46-48 |  | 46, 48 |  |
| CA_46-48-48 |  | 46, 48 |  |
| CA_46-66 |  | 46, 66 |  |
| CA_46-46-66 |  | 46, 66 |  |
| CA_46-66-66 |  | 46, 66 |  |
| CA_46-70 |  | 46, 70 |  |
| CA_46-71 |  | 46, 71 |  |
| CA_48-66 |  | 48, 66 |  |
| CA_48-66-66 |  | 48, 66 |  |
| CA_48-66-66-66 |  | 48, 66 |  |
| CA_48-48-66 |  | 48, 66 |  |
| CA_48-71 |  | 48, 71 |  |
| CA_48-48-71 |  | 48, 71 |  |
| CA_66-70 |  | 66,70 |  |
| CA_66-66-70 |  | 66,70 |  |
| CA_66-71 |  | 66, 71 |  |
| CA_66-66-71 |  | 66, 71 |  |
| CA_70-71 |  | 70, 71 |  |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 2: The frequency range in band 28 is restricted for this CA band combination to 718-748 MHz for the UL and 773-803 MHz for the DL |  |  |  |

---Text omitted---

Table 5.6A.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (two bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | E-UTRA Bands | 1.4 MHz |  |  | 3 MHz |  |  |  |  | 5 MHz |  |  | 10 MHz |  |  |  |  |  | 15 MHz |  |  |  |  | 20 MHz |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A | CA_1A-3A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 3 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-1A-3A | - | 1 | See CA_1A-1A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-1A-7A | - | 1 | See CA_1A-1A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-3A-3A | CA_1A-3A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3C | CA_1A-3A, CA_3C | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-1A-3C | - | 1 | See CA_1A-1A Bandwidth Combination Set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-5A | CA_1A-5A | 1 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 5 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 1 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-1A-5A | - | 1 | See CA_1A-1A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1C-5A | - | 1 | See CA_1C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A | CA_1A-7A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-7A-7A | CA_1A-7A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - | 1 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  | 60 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7C | CA_1A-7A, CA_7C | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_1A-7A, CA_7C | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-8A | CA_1A-8A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 8 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-11A | CA_1A-11A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-18A | CA_1A-18A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 18 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 18 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-19A | CA_1A-19A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 19 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-20A | CA_1A-20A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-21A | CA_1A-21A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-26A | CA_1A-26A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 26 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 26 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-28A | CA_1A-28A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-1A-28A | - | 1 | See CA_1A-1A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-32A | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 32 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-38A | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 38 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-40A | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-40C | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-41A | CA_1A-41A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-41A8 | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-41C8 | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-41D8 | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41D Bandwidth combination set 0 at Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42A | CA_1A-42A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-42A-42A | CA_1A-42A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42C | CA_1A-42A, CA_1A-42C, CA_42C | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42A-42C | CA_1A-42A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42C-42C | CA_1A-42A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42D | CA_1A-42A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42E | CA_1A-42A | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-43A | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 43 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-46A | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-46C | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-46D | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 1 |
|  |  | 46 | See CA_46D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-46E | - | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 1 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1C-3A | - | 1 | See CA_1C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-4A | CA_2A-4A | 2 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 2 |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-2A-4A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-4A-4A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-4A-4A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A | CA_2A-5A | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-46D | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-5A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5B | - | 2 | See CA_2A-2A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-5B | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-7A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-7A | CA_2A-7A | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-7A-7A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7C | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See the CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12A | CA_2A-12A | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 1 |
|  |  | 12 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 2 |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-12A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12A-12A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-12A-12A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12B | CA_2A-12A | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-12B | - | 2 | See CA_2A-2A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-12A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A | CA_2A-13A | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-13A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-14A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 14 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-14A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 14 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-17A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 17 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-28A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-29A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 29 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-29A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-29A | - | 2 | See CA_2C Bandwidth Combination Set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-30A | CA_2A-30A | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-30A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A | CA_2A-46A | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-2A-46A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-46A-46C | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46A-46C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46C | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-46C | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46D | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46E | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A-46A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A-46D | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46A-46D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-48A-48A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48C | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48A-48C | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 48 | See the CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48A-48D | - | 2 |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48A-48D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48C-48C | - | 2 |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48C-48C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48D | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48E | - | 2 |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | Yes |  | 100 | 0 |
|  |  | 48 | See CA_48E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-49A | CA_2A-49A | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 49 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-66A | CA_2A-66A | 2 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 2 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-66B | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66C | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66D | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-2A-66A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-66A-66B | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-66A-66C | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66A-66A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66A-66A-66A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66A-66B | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66A-66C |  | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-66B | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-66C | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-66D |  | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-66A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_2C-66A-66A |  | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-71A | - | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-71A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-5A | CA_3A-5A | 3 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 3 |
|  |  | 5 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 4 |
|  |  | 5 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-5A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-5A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A | CA_3A-7A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-7A | CA_3A-7A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 1 |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-7A-7A | CA_3A-7A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-7C | 7C | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 of 36.101 Bandwidth combination set 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-42D | CA_3A-42A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-7A | CA_3A-7A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 50 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 2 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7B | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 7 | See CA_7B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7C | CA_3A-7A CA_7C | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-7A | CA_3A-7A CA_3C | 3 | See CA_3C Bandwidth Combination Set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-7C | CA_3A-7A, CA_3C, CA_7C | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-8A | CA_3A-8A | 3 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 8 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 3 |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-8A | CA_3A-8A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-8A | CA_3A-8A, CA_3C | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-11A | CA_3A-11A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-18A | CA_3A-18A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 18 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-19A | CA_3A-19A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 19 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-19A | CA_3A-19A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 19 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-20A | CA_3A-20A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-20A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-20A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-21A | CA_3A-21A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-21A | CA_3A-21A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-26A | CA_3A-26A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 26 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 26 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-27A | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 27 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A | CA_3A-28A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-28A | - | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |
| CA_3C-28A | CA_3C | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-31A | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 25 | 0 |
|  |  | 31 |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-32A | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 32 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-32A | - | 3 | See the CA_3C Bandwidth combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 32 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-38A | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 38 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-38A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 38 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-40A | CA_3A-40A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 3 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-40A-40A | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-40C | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-40D | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-40E | - | 3 |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  | Yes |  |  | 100 | 0 |
|  |  | 40 | See CA_40E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-40A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-40C | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41A | CA_3A-41A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-41A | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-41C | CA_3A-41A, CA_3A-41C, CA_41C | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41D | CA_3A-41A, CA_41C | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-41A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-41C | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-41D | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42A | CA_3A-42A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-42A | CA_3A-42A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-42C | CA_3A-42A, CA_42C CA_3A-42C | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42D | CA_3A-42A | 3 |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  | 80 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-42C | CA_3A-42A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42A-42A | CA_3A-42A | 3 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42A-42C | CA_3A-42A, CA_42C | 3 |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42C-42C | CA_3A-42A, CA_42C | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42E | CA_3A-42A | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-43A | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 43 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-46A | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-46C | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - | 3 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-46D | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 1 |
|  |  | 46 | See CA_46D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-46E | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 1 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-46A |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-46C | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-46A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-46C | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-46D | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-69A | - | 3 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 69 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-5A | CA_4A-5A | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 1 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-5A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5B | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-5B | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-7A | CA_4A-7A | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-4A-7A | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-7A-7A | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-7C | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-12A | CA_4A-12A | 4 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 1 |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 12 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 3 |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 4 |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 20 | 5 |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-12A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-12A-12A | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-12A-12A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-12B | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-12B | CA_4A-12A | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-13A | CA_4A-13A | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-13A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-17A | CA_4A-17A | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 17 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-27A | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 27 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-28A | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-29A | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 29 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-29A | - | 4 | See CA_4A-4A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-30A | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-30A | - | 4 | See CA_4A-4A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-46A | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-46A-46A | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-46A-46C | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46A-46C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-46C | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-46D | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-46A-46D | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46A-46D Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-48A | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-48C | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-48D | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-48E | - | 4 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  | 100 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-71A | - | 4 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-4A-71A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-7A | CA_5A-7A | 5 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 1 |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-7A-7A | CA_5A-7A | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-7C | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12A | CA_5A-12A | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12A-12A | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12B | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-13A | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-17A | CA_5A-17A | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 17 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-25A | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 25 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-28A | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-29A | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-30A | CA_5A-30A | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-30A | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-38A | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 38 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-40A | CA_5A-40A | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 1 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-5A-40A | - | 5 | See CA_5A-5A Bandwidth Combination Set 0 in table 6.140.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-40A-40A | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-40C | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 1 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-41A | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-46A | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 1 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-46C | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-46D | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 1 |
|  |  | 46 | See CA_46D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-46E | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46E of Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 1 |
|  |  | 46 | See CA_46E of Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-46A | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5B-46C | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-46D | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-46E | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-48A | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-48C | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-48D | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66A | CA_5A-66A | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-5A-66A | - | 5 | See CA_5A-5A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-5A-66A-66A | - | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-5A-66A-66B | - | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-5A-66A-66C | - | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth Combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-5A-66B | - | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-5A-66C | - | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-5A-66D | - | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66A-66A | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66A-66C | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66B | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66C | - | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66D |  | 5 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-66A | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_5B-66A-66A |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66A-66B |  | 5 |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-66A-66B | - | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-66A-66C | - | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-66B |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-66C |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-8A | CA_7A-8A | 7 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 8 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 1 |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-7A-8A | CA_7A-8A | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-12A | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-12B | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 12 | See CA_12B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-20A | CA_7A-20A | 7 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 2 |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7C-20A | - | 7 | See CA_7C Bandwidth Combination Set 1 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-22A | - | 7 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 22 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-26A | CA_7A-26A | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 26 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_7A-7A-26A | CA_7A-26A | 7 | See CA_7A-7A bandwidth combination set 3 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 26 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_7A-28A | CA_7A-28A | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-7A-28A | - | 7 | See CA_7A-7A Bandwidth combination set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7B-28A | - | 7 | See CA_7B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7C-28A | CA_7A-28A CA_7C | 7 | See CA_7C bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 1 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-29A | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-7A-29A | - | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-29A | - | 7 | See CA_7C Bandwidth combination set 2 in table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-30A | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-32A | - | 7 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 32 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-40A | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-40C | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-40D | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-40E | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 40 | See CA_40E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-42A | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-42A-42A | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-46A | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-7A-46C | - | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-46C | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-46D | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 1 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-46E | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-7A-46E | - | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in table 5.6A.1-3 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-46C | - | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-46D | - | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-46E | - | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-7A-46A | - | 7 | See CA_7A-7A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-7A-46D | - | 7 | See CA_7A-7A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-66A | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-7A-66A-66A | - | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-66A | - | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7C-46A | - | 7 | See CA_7C Bandwidth Combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-7A-66A | - | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-66A-66A | - | 7 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-66A-66A | - | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-11A | - | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-20A | - | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 2 |
|  |  | 20 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-27A | - | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 27 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-28A | - | 8 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-32A | - | 8 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 32 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-38A | - | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 38 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-39A | CA_8A-39A | 8 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 39 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-39C | - | 8 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 45 | 0 |
|  |  | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8B-39A | - | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 39 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_8B-39C | - | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-40A | - | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  | - | 8 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 1 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-40C | - | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-41A | CA_8A-41A | 8 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 8 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 1 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-41C | - | 8 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 41 | See CA_41C bandwidth combination set 3 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-41D | - | 8 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 41 | See CA_41D bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8B-41A | - | 8 | See CA_8B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8B-41C | - | 8 | See CA_8B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 41 | See CA_41C bandwidth combination set 3 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8B-41D | - | 8 | See CA_8B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41D bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-42A | - | 8 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-42C | - | 8 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-46A | - | 8 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-46C | - | 8 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-46D | - | 8 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-46E | - | 8 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8B-46A | - | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8B-46C | - | 8 | See CA_8B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8B-46D | - | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_11A-18A | CA_11A-18A | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 18 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_11A-26A | CA_11A-26A | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 26 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_11A-28A | - | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_11A-41A | - | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_11A-41C | - | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 41 | See CA_41C bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_11A-42A | - | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_11A-42C | - | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_11A-46A | - | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_11A-46C | - | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_11A-46D | - | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_11A-46E | - | 11 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-25A | - | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 25 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_12A-30A | CA_12A-30A | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-46A | - | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_12A-48A |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_12A-46C | - | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-46D | - | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-46E | - | 12 |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-48C | - | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-48D | - | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-48E | - | 12 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-66A | CA_12A-66A | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 66 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 1 |
|  |  | 66 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 2 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 3 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 4 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 12 |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 5 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_12A-66A-66A | - | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-66C | - | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12B-66A | - | 12 | See CA_12B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_12B-66A-66A | - | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46A | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_13A-46C | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46D | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46E | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48A | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_13A-48A-48A | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48A-48C | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See the CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48A-48D | - | 13 |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48A-48D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48C-48C | - | 13 |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48C-48C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48C | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48D | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48E | - | 13 |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-66A | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_13A-66A-66A | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-66A-66B | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-66A-66C | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-66B | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-66C | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-66D | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_14A-66A | - | 14 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_14A-66A-66A | - | 14 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_14A-66A-66A-66A | - | 14 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 66 | See CA_66A-66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_14A-30A | - | 14 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_18A-28A | CA_18A-28A | 18 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_18A-42A | - | 18 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_18A-42C | - | 18 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 42 | See the CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-21A | CA_19A-21A | 19 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_19A-28A | - | 19 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-42A | CA_19A-42A | 19 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_19A-42C | CA_19A-42A | 19 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-42D | - | 19 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-46A | - | 19 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_19A-46C | - | 19 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-46D | - | 19 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-46E | - | 19 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-28A7 | - | 20 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-31A | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 25 | 0 |
|  |  | 31 |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-32A | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 32 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 32 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-38A | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 38 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-38C | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 38 | See CA_38C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-40A | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 35 | 1 |
|  |  | 40 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-40A-40A | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-40C | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-40D | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 40 | See CA_40D Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-42A | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-42A-42A | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-43A | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 43 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-67A | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 67 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-75A | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 75 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-76A | - | 20 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 25 | 0 |
|  |  | 76 |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-28A | CA_21A-28A | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-42A | CA_21A-42A | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_21A-42C | CA_21A-42A | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-42D | - | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-42E | - | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 42 | See CA_42E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-46A | - | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_21A-46C | - | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-46D | - | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-46E | - | 21 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_23A-29A | - | 23 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 29 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 23 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 29 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-26A | - | 25 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 26 | Yes |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 25 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 26 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 25 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 2 |
|  |  | 26 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-26A | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 45 | 0 |
|  |  | 26 |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-41A8 | - | 25 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_25A-25A-41A | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_25A-41C8 | - | 25 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-41C | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-41D8 | - | 25 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-41D | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 41 | See CA_41D bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-41E | - | 25 |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 41 | See CA_41E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-41E | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 41 | See CA_41E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-41F | - | 25 |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  | 120 | 0 |
|  |  | 41 | See CA_41F Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-41F | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 | 0 |
|  |  | 41 | See CA_41F Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-46A | - | 25 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_25A-46C | - | 25 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-46D | - | 25 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_26A-41A | - | 26 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_26A-41C | - | 26 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_26A-46A | CA_26A-46A | 26 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_26A-48A | CA_26A-48A | 26 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_26A-48C | CA_26A-48A | 26 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_26A-48A-48A | CA_26A-48A | 26 |  |  |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-32A | - | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 32 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-38A |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 38 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-40A | - | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-40C | - | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-40D | - | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-41A | CA_28A-41A | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-41C |  | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-42A | CA_28A-42A | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-42C | CA_28A-42A, CA_42C | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-42A-42A | - | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-42D | - | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 42 | See CA_42D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-42A-42C | CA_42C | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-42C-42C | CA_42C | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-46A | - | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-46C | - | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-46D | - | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-46E | - | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-66A | - | 28 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_29A-30A | - | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-66A | - | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_29A-66C |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-66A-66A |  | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-70A | - | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 70 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_29A-70C | - | 29 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 70 | See CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_30A-66A | CA_30A-66A | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_30A-66A-66A |  | 30 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_32A-42A | - | 32 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_32A-43A | - | 32 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 43 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_34A-39A |  | 34 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 39 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_34A-41A |  | 34 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_38A-40A | - | 38 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 38 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 40 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_38A-40A-40A | - | 38 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  | 60 | 1 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_38A-40C | - | 38 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_38A-40D | - | 38 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40D Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-40A | - | 39 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_39A-40C | - | 39 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-40D | - | 39 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-40E | - | 39 |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  | 100 | 0 |
|  |  | 40 | See the CA_40E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39C-40A | - | 39 | See CA_39C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_39C-40C | - | 39 | See CA_39C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39C-40D | - | 39 | See the CA_39C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 40 | See the CA_40D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-41A | CA_39A-41A | 39 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39A-41C | CA_41C CA_39A-41A CA_39A-41C | 39 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39A-41D | CA_41C CA_39A-41A | 39 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39C-41A | CA_39C CA_39A-41A CA_39C-41A | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39C-41C | CA_39C CA_41C CA_39A-41A | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39C-41D | - | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-42A | - | 39 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_39A-42C | - | 39 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-42D | - | 39 |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  | 80 | 0 |
|  |  | 42 | See CA_42D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-42E | - | 39 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See the CA_42E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39C-42A | - | 39 | See CA_39C Bandwidth Combination Set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_39C-42C | - | 39 | See CA_39C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39C-42D | - | 39 | See the CA_39C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 42 | See the CA_42D Bandwidth combination set 1 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-46A | - | 39 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39A-46C | - | 39 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-46D | - | 39 |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-46E | - | 39 |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39C-46A | - | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39C-46C | - | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39C-46D | - | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40A-41A | - | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_40A-42A | CA_40A-42A | 40 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_40A-42C | - | 40 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40C-42C | - | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40A-43A | - | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 43 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_40A-46A | - | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_40A-46C | - | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40A-46D | - | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 1 |
|  |  | 46 | See CA_46D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40A-46E | - | 40 |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 1 |
|  |  | 46 | See CA_46E Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40C-42A | - | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_40C-46A | - | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_40C-46C | - | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40C-46D | - | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40D-46A | - | 40 | See CA_40D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_40D-46C | - | 40 | See CA_40D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A9-42A9 | CA_41A-42A | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_41A-42C | CA_41A-42A, CA_42C, CA_41A-42C | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-42A-42A | - | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-42D | - | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-42A-42C | CA_42C | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-42C-42C | CA_42C | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-42A | CA_41A-42A, CA_41C, CA_41C-42A | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_41C-42C | CA_41A-42A, CA_41C, CA_42C, CA_41C-42C | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-42A-42A | - | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-42D | - | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-42A-42C | CA_42C | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-42C-42C | CA_42C | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41D-42A | - | 41 | See CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_41D-42C | - | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-46A | - | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_41A-46C | - | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-46D | - | 41 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-46E | - | 41 |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-46A | - | 41 | See CA_41C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_41C-46C | - | 41 | See CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-46D | - | 41 | See the CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41D-46A | - | 41 | See CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_41D-46C | - | 41 | See the CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-48A | - | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_41A-48C | - | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-48D | - | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-48A | CA_41C | 41 | See the CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |
| CA_41C-48C | CA_41C | 41 | See the CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-48D | CA_41C | 41 | See the CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41D-48A | CA_41C | 41 | See the CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |
| CA_41D-48C | CA_41C | 41 | See the CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_42A-43A | - | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 43 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_42A-46A | - | 42 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_46A-48A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_46A-48A-48A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46A-48C | CA_48C | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | 60 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46C-48A | - | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |
| CA_46C-48A-48A | - | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46C-48C | CA_48C | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46A-48D | CA_48C | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | 80 | 0 |
|  |  | 48 | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46D-48A | - | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |
| CA_46A-46A-66A | - | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_46A-46C-66A | - | 46 | See CA_46A-46C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_46A-46D-66A | - | 46 | See CA_46A-46D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |
| CA_46A-48E | CA_48C | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | 100 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46C-48D | CA_48C | 46 | See CA_46C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46D-48A-48A | - | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46D-48C | - | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46E-48A | - | 46 | See CA_46E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |
| CA_46C-66A | - | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_46A-66A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_46A-66A-66A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See the CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46C-66A-66A | - | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46A-66C | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See the CA_66C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46D-66A | - | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_46D-66A-66A | - | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46C-48E | CA_48C | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46E-48C | - | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46E-66A | - | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_46E-66A-66A | - | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46A-70A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 70 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_46A-71A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_46C-71A | - | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_46D-71A | - | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-66A | - | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-48A-66A | - | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-48C-66A | - | 48 | See the CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-48C-66B | - | 48 | See CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-48C-66C | - | 48 | See CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-48D-66A | - | 48 | See CA_48A-48D Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_48C-48C-66A | - | 48 | See CA_48C-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-66A-66A | - | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-48A-66A-66A | - | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-48A-66B | - | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-48A-66C | - | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48C-66A-66A | - | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48C-66B | - | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48C-66C | - | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-66B | - | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-66C | - | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48C-66A | - | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_48D-66A | - | 48 | See the CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_48E-66A | - | 48 | See CA_48E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-71A | - | 48 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_48C-71A | - | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-48A-71A | - | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_66A-70A | - | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 70 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_66A-66A-70A | - | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 70 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_66A-70C | - | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 45 | 0 |
|  |  | 70 | See CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_66A-66A-70C | - | 66 | See the CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 65 | 0 |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_66C-70A | - | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 70 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_66C-70C | - | 66 | See the CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 65 | 0 |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_66A-71A | - | 66 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_66C-71A | - | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_66A-66A-71A | - | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_70A-71A |  | 70 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| CA_70C-71A | - | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 45 | 0 |
|  |  | 71 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 4: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 5: For TDD inter-band Carrier Aggregation only non-simultaneous Rx/Tx uplink CA configurations can be supported by UE supporting corresponding DL CA configuration without simultaneous Rx/Tx. NOTE 6: Void NOTE 7: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. NOTE 8: For the corresponding CA configuration, UE may not support Pcell transmissions in this E-UTRA band. NOTE 9: 8Rx Requirements are applicable for this band configuration if UE supports 8Rx. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

---Text omitted---

Table 6.2.5-2: ΔTIB,c (two bands)

| E-UTRA operating band combination |  | E-UTRA Band |  | ΔTIB,c [dB] |  |
| --- | --- | --- | --- | --- | --- |
| CA_1-3, CA_1-1-3, CA_1-3-3 |  | 1 |  | 0.3 |  |
|  |  | 3 |  | 0.3 |  |
| CA_1-5 |  | 1 |  | 0.3 |  |
|  |  | 5 |  | 0.3 |  |
| CA_1-7, CA_1-1-7, CA_1-7-7 |  | 1 |  | 0.5 |  |
|  |  | 7 |  | 0.6 |  |
| CA_1-8 |  | 1 |  | 0.3 |  |
|  |  | 8 |  | 0.3 |  |
| CA_1-11 |  | 1 |  | 0.3 |  |
|  |  | 11 |  | 0.3 |  |
| CA_1-18 |  | 1 |  | 0.3 |  |
|  |  | 18 |  | 0.3 |  |
| CA_1-19 |  | 1 |  | 0.3 |  |
|  |  | 19 |  | 0.3 |  |
| CA_1-20 |  | 1 |  | 0.3 |  |
|  |  | 20 |  | 0.3 |  |
| CA_1-21 |  | 1 |  | 0.3 |  |
|  |  | 21 |  | 0.3 |  |
| CA_1-26 |  | 1 |  | 0.3 |  |
|  |  | 26 |  | 0.3 |  |
| CA_1-28, CA_1-1-28 |  | 1 |  | 0.3 |  |
|  |  | 28 |  | 0.6 |  |
| CA_1-32 |  | 1 |  | 0.5 |  |
|  |  | 32 |  | N/A |  |
| CA_1-38 |  | 1 |  | 0.5 |  |
|  |  | 38 |  | 0.5 |  |
| CA_1-40 |  | 1 |  | 0.5 |  |
|  |  | 40 |  | 0.5 |  |
| CA_1-418 |  | 1 |  | 0.5 |  |
|  |  | 41 |  | 0.5 |  |
| CA_1-42, CA_1-42-42 |  | 1 |  | 0.3 |  |
|  |  | 42 |  | 0.8 |  |
| CA_1-43 |  | 1 |  | 0.3 |  |
|  |  | 43 |  | 0.8 |  |
| CA_1-46 |  | 1 |  | 0 |  |
| CA_2-4, CA_2-2-4, CA_2-4-4, CA_2-2-4-4 |  | 2 |  | 0.5 |  |
|  |  | 4 |  | 0.5 |  |
| CA_2-5, CA_2-2-5 |  | 2 |  | 0.3 |  |
|  |  | 5 |  | 0.3 |  |
| CA_2-7, CA_2-2-7, CA_2-7-7 |  | 2 |  | 0.5 |  |
|  |  | 7 |  | 0.5 |  |
| CA_2-12, CA_2-2-12, CA_2-12-12, CA_2-2-12-12 |  | 2 |  | 0.3 |  |
|  |  | 12 |  | 0.3 |  |
| CA_2-13, CA_2-2-13 |  | 2 |  | 0.3 |  |
|  |  | 13 |  | 0.3 |  |
| CA_2-14, CA_2-2-14 |  | 2 |  | 0.3 |  |
|  |  | 14 |  | 0.3 |  |
| CA_2-17 |  | 2 |  | 0.3 |  |
|  |  | 17 |  | 0.8 |  |
| CA_2-28 |  | 2 |  | 0.3 |  |
|  |  | 28 |  | 0.3 |  |
| CA_2-29, CA_2-2-29 |  | 2 |  | 0.3 |  |
| CA_2-30, CA_2-2-30 |  | 2 |  | 0.5 |  |
|  |  | 30 |  | 0.3 |  |
| CA_2-46, CA_2-2-46, CA_2-46-46 |  | 2 |  | 0 |  |
| CA_2-48, CA_2-48-48 |  | 2 |  | 0.6 |  |
|  |  | 48 |  | 0.8 |  |
| CA_2-49 |  | 2 |  | 0.6 |  |
| CA_2-66, CA_2-2-66, CA_2-66-66, CA_2-2-66-66, CA_2-66-66-66 |  | 2 |  | 0.5 |  |
|  |  | 66 |  | 0.5 |  |
| CA_2-71, CA_2-2-71 |  | 2 |  | 0.3 |  |
|  |  | 71 |  | 0.3 |  |
| CA_3-5, CA_3-3-5 |  | 3 |  | 0.3 |  |
|  |  | 5 |  | 0.3 |  |
| CA_3-7, CA_3-3-7, CA_3-7-7, CA_3-3-7-7 |  | 3 |  | 0.5 |  |
|  |  | 7 |  | 0.5 |  |
| CA_3-8, CA_3-3-8 |  | 3 |  | 0.3 |  |
|  |  | 8 |  | 0.3 |  |
| CA_3-11 |  | 3 |  | 0.8 |  |
|  |  | 11 |  | 0.9 |  |
| CA_3-18 |  | 3 |  | 0.3 |  |
|  |  | 18 |  | 0.3 |  |
| CA_3-19, CA_3-3-19 |  | 3 |  | 0.3 |  |
|  |  | 19 |  | 0.3 |  |
| CA_3-20, CA_3-3-20 |  | 3 |  | 0.3 |  |
|  |  | 20 |  | 0.3 |  |
| CA_3-21, CA_3-3-21 |  | 3 |  | 0.8 |  |
|  |  | 21 |  | 0.9 |  |
| CA_3-26 |  | 3 |  | 0.3 |  |
|  |  | 26 |  | 0.3 |  |
| CA_3-27 |  | 3 |  | 0.3 |  |
|  |  | 27 |  | 0.3 |  |
| CA_3-28 |  | 3 |  | 0.3 |  |
|  |  | 28 |  | 0.3 |  |
| CA_3-31 |  | 3 |  | 0.3 |  |
|  |  | 31 |  | 0.6 |  |
| CA_3-32 |  | 3 |  | 0.5 |  |
| CA_3-38 |  | 3 |  | 0,5 |  |
|  |  | 38 |  | 0,5 |  |
| CA_3-40, CA_3-40-40 |  | 3 |  | 0.5 |  |
|  |  | 40 |  | 0.5 |  |
| CA_3-41, CA_3-3-41 |  | 3 |  | 0.5 |  |
|  |  | 41 |  | 0.310 |  |
|  |  |  |  | 0.811 |  |
| CA_3-42, CA_3-3-42, CA_3-42-42 |  | 3 |  | 0.6 |  |
|  |  | 42 |  | 0.8 |  |
| CA_3-43 |  | 3 |  | 0.3 |  |
|  |  | 43 |  | 0.8 |  |
| CA_3-46, CA_3-3-46 |  | 3 |  | 0 |  |
| CA_3-69 |  | 3 |  | 0.5 |  |
| CA_4-5, CA_4-4-5 |  | 4 |  | 0.3 |  |
|  |  | 5 |  | 0.3 |  |
| CA_4-7, CA_4-4-7, CA_4-7-7 |  | 4 |  | 0.5 |  |
|  |  | 7 |  | 0.5 |  |
| CA_4-12, CA_4-4-12, CA_4-12-12, CA_4-4-12-12 |  | 4 |  | 0.3 |  |
|  |  | 12 |  | 0.8 |  |
| CA_4-13, CA_4-4-13 |  | 4 |  | 0.3 |  |
|  |  | 13 |  | 0.3 |  |
| CA_4-17 |  | 4 |  | 0.3 |  |
|  |  | 17 |  | 0.8 |  |
| CA_4-27 |  | 4 |  | 0.3 |  |
|  |  | 27 |  | 0.3 |  |
| CA_4-28 |  | 4 |  | 0.3 |  |
|  |  | 28 |  | 0.6 |  |
| CA_4-29, CA_4-4-29 |  | 4 |  | 0.3 |  |
| CA_4-30, CA_4-4-30 |  | 4 |  | 0.5 |  |
|  |  | 30 |  | 0.3 |  |
| CA_4-46, CA_4-46-46 |  | 4 |  | 0 |  |
| CA_4-48 |  | 4 |  | 0.3 |  |
|  |  | 48 |  | 0.8 |  |
| CA_4-71, CA_4-4-71 |  | 4 |  | 0.3 |  |
|  |  | 71 |  | 0.3 |  |
| CA_5-7, CA_5-7-7 |  | 5 |  | 0.3 |  |
|  |  | 7 |  | 0.3 |  |
| CA_5-12, CA_5-12-12 |  | 5 |  | 0.8 |  |
|  |  | 12 |  | 0.4 |  |
| CA_5-13 |  | 5 |  | 0.5 |  |
|  |  | 13 |  | 0.5 |  |
| CA_5-17 |  | 5 |  | 0.8 |  |
|  |  | 17 |  | 0.4 |  |
| CA_5-25 |  | 5 |  | 0.3 |  |
|  |  | 25 |  | 0.3 |  |
| CA_5-28 |  | 5 |  | 0.5 |  |
|  |  | 28 |  | 0.5 |  |
| CA_5-29 |  | 5 |  | 0.5 |  |
| CA_5-30 |  | 5 |  | 0.3 |  |
|  |  | 30 |  | 0.3 |  |
| CA_5-38 |  | 5 |  | 0.3 |  |
|  |  | 38 |  | 0.3 |  |
| CA_5-40, CA_5-5-40, CA_5-40-40 |  | 5 |  | 0.3 |  |
|  |  | 40 |  | 0.3 |  |
| CA_5-41 |  | 5 |  | 0.3 |  |
|  |  | 41 |  | 0.3 |  |
| CA_5-46 |  | 5 |  | 0 |  |
| CA_5-48 |  | 5 |  | 0.3 |  |
|  |  | 48 |  | 0.3 |  |
| CA_5-66, CA_5-5-66, CA_5-66-66, CA_5-5-66-66 |  | 5 |  | 0.3 |  |
|  |  | 66 |  | 0.3 |  |
| CA_7-8, CA_7-7-8 |  | 7 |  | 0.3 |  |
|  |  | 8 |  | 0.6 |  |
| CA_7-12 |  | 7 |  | 0.3 |  |
|  |  | 12 |  | 0.3 |  |
| CA_7-20 |  | 7 |  | 0.3 |  |
|  |  | 20 |  | 0.3 |  |
| CA_7-22 |  | 7 |  | 0.5 |  |
|  |  | 22 |  | 0.8 |  |
| CA_7-26, CA_7-7-26 |  | 7 |  | 0.3 |  |
|  |  | 26 |  | 0.3 |  |
| CA_7-28, CA_7-7-28 |  | 7 |  | 0.3 |  |
|  |  | 28 |  | 0.3 |  |
| CA_7-29, CA_7-7-29 |  | 7 |  | 0.3 |  |
| CA_7-30 |  | 7 |  | 0.5 |  |
|  |  | 30 |  | 0.5 |  |
| CA_7-32 |  | 7 |  | 0.7 |  |
| CA_7-40 |  | 7 |  | 0.5 |  |
|  |  | 40 |  | [0.6] |  |
| CA_7-42, CA_7-42-42 |  | 7 |  | 0.5 |  |
|  |  | 42 |  | 0.8 |  |
| CA_7-46, CA_7-7-46 |  | 7 |  | 0 |  |
| CA_7-66, CA_7-7-66, CA_7-66-66, CA_7-7-66-66 |  | 7 |  | 0.5 |  |
|  |  | 66 |  | 0.5 |  |
| CA_8-11 |  | 8 |  | 0.3 |  |
|  |  | 11 |  | 0.4 |  |
| CA_8-20 |  | 8 |  | 0.4 |  |
|  |  | 20 |  | 0.4 |  |
| CA_8-27 |  | 8 |  | 0.8 |  |
|  |  | 27 |  | 0.8 |  |
| CA_8-2814 |  | 8 |  | 0.6 |  |
|  |  | 28 |  | 0.5 |  |
| CA_8-32 |  | 8 |  | 0.3 |  |
| CA_8-38 |  | 8 |  | 0.3 |  |
|  |  | 38 |  | 0.3 |  |
| CA_8-39 |  | 8 |  | 0,3 |  |
|  |  | 39 |  | 0,3 |  |
| CA_8-40 |  | 8 |  | 0.3 |  |
|  |  | 40 |  | 0.3 |  |
| CA_8-41 |  | 8 |  | 0.3 |  |
|  |  | 41 |  | 0.3 |  |
| CA_8-42 |  | 8 |  | 0.6 |  |
|  |  | 42 |  | 0.8 |  |
| CA_8-46 |  | 8 |  | 0 |  |
| CA_11-18 |  | 11 |  | 0.3 |  |
|  |  | 18 |  | 0.3 |  |
| CA_11-26 |  | 11 |  | 0.3 |  |
|  |  | 26 |  | 0.3 |  |
| CA_11-28 |  | 11 |  | 0.4 |  |
|  |  | 28 |  | 0.6 |  |
| CA_11-41 |  | 11 |  | 0.3 |  |
|  |  | 41 |  | 0.3 |  |
| CA_11-42 |  | 11 |  | 0.4 |  |
|  |  | 42 |  | 0.8 |  |
| CA_11-46 |  | 11 |  | 0 |  |
| CA_12-25 |  | 12 |  | 0.3 |  |
|  |  | 25 |  | 0.3 |  |
| CA_12-30 |  | 12 |  | 0.3 |  |
|  |  | 30 |  | 0.3 |  |
| CA_12-46 |  | 12 |  | 0 |  |
|  |  | 46 |  | 0 |  |
| CA_12-48 |  | 12 |  | 0.3 |  |
|  |  | 48 |  | 0.3 |  |
| CA_12-66, CA_12-66-66 |  | 12 |  | 0.8 |  |
|  |  | 66 |  | 0.3 |  |
| CA_13-46 |  | 13 |  | 0 |  |
| CA_13-48, CA_13-48-48 |  | 13 |  | 0.3 |  |
|  |  | 48 |  | 0.3 |  |
| CA_13-66, CA_13-66-66 |  | 13 |  | 0.3 |  |
|  |  | 66 |  | 0.3 |  |
| CA_14-30 |  | 14 |  | 0.3 |  |
|  |  | 30 |  | 0.3 |  |
| CA_14-66, CA_14-66-66, CA_14-66-66-66 |  | 14 |  | 0.3 |  |
|  |  | 66 |  | 0.3 |  |
| CA_18-289 |  | 18 |  | 0.5 |  |
|  |  | 28 |  | 0.5 |  |
| CA_19-21 |  | 19 |  | 0.3 |  |
|  |  | 21 |  | 0.4 |  |
| CA_19-289 |  | 19 |  | 0.5 |  |
|  |  | 28 |  | 0.5 |  |
| CA_18-42 |  | 18 |  | 0.3 |  |
|  |  | 42 |  | 0.8 |  |
| CA_19-42 |  | 19 |  | 0.3 |  |
|  |  | 42 |  | 0.8 |  |
| CA_19-46 |  | 19 |  | 0 |  |
| CA_20-28 |  | 20 |  | 0.5 |  |
|  |  | 28 |  | 0.5 |  |
| CA_20-31 |  | 20 |  | 0.5 |  |
|  |  | 31 |  | 0.5 |  |
| CA_20-32 |  | 20 |  | 0.3 |  |
| CA_20-38 |  | 20 |  | 0.3 |  |
|  |  | 38 |  | 0.3 |  |
| CA_20-40, CA_20-40-40 |  | 20 |  | 0.3 |  |
|  |  | 40 |  | 0.3 |  |
| CA_20-42, CA_20-42-42 |  | 20 |  | 0.6 |  |
|  |  | 42 |  | 0.8 |  |
| CA_20-43 |  | 20 |  | 0.3 |  |
|  |  | 43 |  | 0.8 |  |
| CA_20-67 |  | 20 |  | 0.5 |  |
| CA_20-75 |  | 20 |  | 0.3 |  |
| CA_20-76 |  | 20 |  | 0.3 |  |
| CA_21-28 |  | 21 |  | 0.4 |  |
|  |  | 28 |  | 0.3 |  |
| CA_21-42 |  | 21 |  | 0.4 |  |
|  |  | 42 |  | 0.8 |  |
| CA_21-46 |  | 21 |  | 0 |  |
| CA_23-29 |  | 23 |  | 0.3 |  |
| CA_25-26, CA_25-25-26 |  | 25 |  | 0.3 |  |
|  |  | 26 |  | 0.3 |  |
| CA_25-41, CA_25-25-41 |  | 25 |  | 0.5 |  |
|  |  | 41 |  | 0.410 |  |
|  |  |  |  | 0.911 |  |
| CA_25-46 |  | 25 |  | 0 |  |
|  |  | 46 |  | 0 |  |
| CA_26-41 |  | 26 |  | 0.3 |  |
|  |  | 41 |  | 0.3 |  |
| CA_26-46 |  | 26 |  | 0 |  |
| CA_26-48, CA_26-48-48 |  | 26 |  | 0.3 |  |
|  |  | 48 |  | 0.3 |  |
| CA_28-32 |  | 28 |  | 0.3 |  |
| CA_28-38 |  | 28 |  | 0.3 |  |
|  |  | 38 |  | 0.3 |  |
| CA_28-40 |  | 28 |  | 0.3 |  |
|  |  | 40 |  | 0.3 |  |
| CA_28-41 |  | 28 |  | 0.3 |  |
|  |  | 41 |  | 0.3 |  |
| CA_28-42, CA_28-42-42 |  | 28 |  | 0.5 |  |
|  |  | 42 |  | 0.8 |  |
| CA_28-46 |  | 28 |  | 0 |  |
| CA_28-66 |  | 28 |  | 0.6 |  |
|  |  | 66 |  | 0.3 |  |
| CA_29-30 |  | 30 |  | 0.3 |  |
| CA_29-66, CA_29-66-66 |  | 66 |  | 0.3 |  |
| CA_29-70 |  | 70 |  | 0.3 |  |
| CA_30-66, CA_30-66-66 |  | 30 |  | 0.3 |  |
|  |  | 66 |  | 0.5 |  |
| CA_32-42 |  | 42 |  | 0.8 |  |
| CA_32-43 |  | 43 |  | 0.8 |  |
| CA_34-39 |  | 34 |  | 01 |  |
|  |  | 39 |  | 01 |  |
| CA_34-41 |  | 34 |  | 01 |  |
|  |  | 41 |  | 01 |  |
| CA_38-40, CA_38-40-40 |  | 38 |  | 04 |  |
|  |  | 40 |  | 04 |  |
| CA_39-40 |  | 39 |  | 04 |  |
|  |  | 40 |  | 04 |  |
| CA_39-41 |  | 39 |  | 04 |  |
|  |  | 41 |  | 04 |  |
| CA_39-41 |  | 39 |  | 0.57 |  |
|  |  | 41 |  | 0.57 |  |
| CA_39-42 |  | 39 |  | 04 |  |
|  |  | 42 |  | 0.54 |  |
| CA_39-46 |  | 39 |  | 0 |  |
| CA_40-41 |  | 40 |  | 0.54 |  |
|  |  | 41 |  | 0.54 |  |
| CA_40-42 |  | 40 |  | 04 |  |
|  |  | 42 |  | 0.54 |  |
| CA_40-43 |  | 40 |  | 04 |  |
|  |  | 43 |  | 0.54 |  |
| CA_40-46 |  | 40 |  | 0 |  |
| CA_41-42, CA_41-42-42 |  | 41 |  | 04 |  |
|  |  | 42 |  | 0.54 |  |
| CA_41-42, CA_41-42-42 |  | 41 |  | 0.37 |  |
|  |  | 42 |  | 0.87 |  |
| CA_41-46 |  | 41 |  | 0 |  |
| CA_41-48 |  | 41 |  | 04 |  |
|  |  | 48 |  | 0.54 |  |
| CA_42-43 |  | 42 |  | 04 |  |
|  |  | 43 |  | 04 |  |
| CA_42-46 |  | 42 |  | [0.5] |  |
| CA_46-48, CA_46-48-48 |  | 48 |  | 0.8 |  |
| CA_46-66, CA_46-46-66, CA_46-66-66 |  | 66 |  | 0 |  |
| CA_46-70 |  | 70 |  | 0 |  |
| CA_46-71 |  | 71 |  | 0 |  |
| CA_48-66, CA_48-48-66, CA_48-66-66, CA_48-48-66-66 |  | 48 |  | 0.8 |  |
|  |  | 66 |  | 0.6 |  |
| CA_48-71, CA_48-48-71 |  | 48 |  | 0.3 |  |
|  |  | 71 |  | 0.3 |  |
| CA_66-70, CA_66-66-70 |  | 66 |  | 0.5 |  |
|  |  | 70 |  | 0.5 |  |
| CA_66-71, CA_66-66-71 |  | 66 |  | 0.3 |  |
|  |  | 71 |  | 0.3 |  |
| CA_70-71 |  | 70 |  | 0.3 |  |
|  |  | 71 |  | 0.6 |  |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above 2DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 2DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the 2DL tolerances above, truncated to one decimal place for that operating band among the supported 2DL CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 2DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 2DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 2DL CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 5: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 6: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 7: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in the FDD band. NOTE 9: For Band 28, the requirements only apply for the restricted frequency range specified for this CA configuration (Table 5.5A-2). NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 12: For UE supporting E-UTRA band 65 and CA configurations including Band 1, the Band 65 ΔTIB,c is the max(Band 65 ΔTIB,c , Band 1 ΔTIB,c) NOTE 13: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔTIB,c in Band 42, 43, or 48 is the max(Band 42 ΔTIB,c , Band 43 ΔTIB,c, Band 48 ΔTIB,c). NOTE 14: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8. |  |  |  |  |  |

NOTE:	The above additional tolerances do not apply to supported UTRA operating bands with frequency range below 1 GHz that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations when such bands are belonging only to band combination(s) where one band is <1GHz and another band is >1.7GHz and there is no harmonic relationship between the low band UL and high band DL. Otherwise the above additional tolerances also apply to supported UTRA operating bands that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations.

NOTE:	To meet the TIB,c requirements for CA_3A-7A with state-of-the-art technology, an increase in power consumption of the UE may be required. It is also expected that as the state-of-the-art technology evolves in the future, this possible power consumption increase can be reduced or eliminated.

---Text omitted---

Table 7.3.1-1A: ΔRIB,c (two bands)

| E-UTRA operating band combination |  | E-UTRA Band |  | ΔRIB,c [dB] |  |
| --- | --- | --- | --- | --- | --- |
| CA_1-3, CA_1-1-3, CA_1-3-3 |  | 1 |  | 0 |  |
|  |  | 3 |  | 0 |  |
| CA_1-5, CA_1-1-5 |  | 1 |  | 0 |  |
|  |  | 5 |  | 0 |  |
| CA_1-7, CA_1-1-7, CA_1-7-7 |  | 1 |  | 0 |  |
|  |  | 7 |  | 0 |  |
| CA_1-8 |  | 1 |  | 0 |  |
|  |  | 8 |  | 0 |  |
| CA_1-11 |  | 1 |  | 0 |  |
|  |  | 11 |  | 0 |  |
| CA_1-18 |  | 1 |  | 0 |  |
|  |  | 18 |  | 0 |  |
| CA_1-19 |  | 1 |  | 0 |  |
|  |  | 19 |  | 0 |  |
| CA_1-20 |  | 1 |  | 0 |  |
|  |  | 20 |  | 0 |  |
| CA_1-21 |  | 1 |  | 0 |  |
|  |  | 21 |  | 0 |  |
| CA_1-26 |  | 1 |  | 0 |  |
|  |  | 26 |  | 0 |  |
| CA_1-28, CA_1-1-28 |  | 1 |  | 0 |  |
|  |  | 28 |  | 0.2 |  |
| CA_1-32 |  | 1 |  | 0 |  |
|  |  | 32 |  | 0 |  |
| CA_1-38 |  | 1 |  | 0 |  |
|  |  | 38 |  | 0 |  |
| CA_1-40 |  | 1 |  | 0 |  |
|  |  | 40 |  | 0 |  |
| CA_1-418 |  | 1 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_1-42, CA_1-42-42 |  | 1 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_1-43 |  | 1 |  | 0 |  |
|  |  | 43 |  | 0.5 |  |
| CA_1-46 |  | 1 |  | 0 |  |
| CA_2-4, CA_2-2-4, CA_2-4-4, CA_2-2-4-4 |  | 2 |  | 0.3 |  |
|  |  | 4 |  | 0.3 |  |
| CA_2-5, CA_2-2-5 |  | 2 |  | 0 |  |
|  |  | 5 |  | 0 |  |
| CA_2-7, CA_2-2-7, CA_2-7-7 |  | 2 |  | 0 |  |
|  |  | 7 |  | 0 |  |
| CA_2-12, CA_2-2-12, CA_2-12-12, CA_2-2-12-12 |  | 2 |  | 0 |  |
|  |  | 12 |  | 0 |  |
| CA_2-13, CA_2-2-13 |  | 2 |  | 0 |  |
|  |  | 13 |  | 0 |  |
| CA_2-14, CA_2-2-14 |  | 2 |  | 0 |  |
|  |  | 14 |  | 0 |  |
| CA_2-17 |  | 2 |  | 0 |  |
|  |  | 17 |  | 0.5 |  |
| CA_2-28 |  | 2 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_2-29, CA_2-2-29 |  | 2 |  | 0 |  |
| CA_2-30, CA_2-2-30 |  | 2 |  | 0.4 |  |
|  |  | 30 |  | 0.5 |  |
| CA_2-46, CA_2-2-46 |  | 2 |  | 0 |  |
| CA_2-48, CA_2-48-48 |  | 2 |  | 0.2 |  |
|  |  | 48 |  | 0.5 |  |
| CA_2-49 |  | 2 |  | 0.2 |  |
| CA_2-66, CA_2-2-66, CA_2-66-66, CA_2-2-66-66, CA_2-66-66-66 |  | 2 |  | 0.3 |  |
|  |  | 66 |  | 0.3 |  |
| CA_2-71, CA_2-2-71 |  | 2 |  | 0 |  |
|  |  | 71 |  | 0 |  |
| CA_3-5, CA_3-3-5 |  | 3 |  | 0 |  |
|  |  | 5 |  | 0 |  |
| CA_3-7, CA_3-3-7, CA_3-7-7, CA_3-3-7-7 |  | 3 |  | 0 |  |
|  |  | 7 |  | 0 |  |
| CA_3-8, CA_3-3-8 |  | 3 |  | 0 |  |
|  |  | 8 |  | 0 |  |
| CA_3-11 |  | 3 |  | 0.3 |  |
|  |  | 11 |  | 0.5 |  |
| CA_3-18 |  | 3 |  | 0 |  |
|  |  | 18 |  | 0 |  |
| CA_3-19, CA_3-3-19 |  | 3 |  | 0 |  |
|  |  | 19 |  | 0 |  |
| CA_3-20, CA_3-3-20 |  | 3 |  | 0 |  |
|  |  | 20 |  | 0 |  |
| CA_3-21, CA_3-3-21 |  | 3 |  | 0.3 |  |
|  |  | 21 |  | 0.5 |  |
| CA_3-26 |  | 3 |  | 0 |  |
|  |  | 26 |  | 0 |  |
| CA_3-27 |  | 3 |  | 0 |  |
|  |  | 27 |  | 0 |  |
| CA_3-28 |  | 3 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_3-31 |  | 3 |  | 0 |  |
|  |  | 31 |  | 0.2 |  |
| CA_3-32 |  | 3 |  | 0 |  |
|  |  | 32 |  | 0 |  |
| CA_3-38 |  | 3 |  | 0 |  |
|  |  | 38 |  | 0 |  |
| CA_3-40, CA_3-40-40 |  | 3 |  | 0 |  |
|  |  | 40 |  | 0 |  |
| CA_3-41, CA_3-3-41 |  | 3 |  | 0 |  |
|  |  | 41 |  | 010 |  |
|  |  |  |  | 0.511 |  |
| CA_3-42, CA_3-3-42, CA_3-42-42 |  | 3 |  | 0.2 |  |
|  |  | 42 |  | 0.5 |  |
| CA_3-43 |  | 3 |  | 0 |  |
|  |  | 43 |  | 0.5 |  |
| CA_3-46, CA_3-3-46 |  | 3 |  | 0 |  |
| CA_4-5, CA_4-4-5 |  | 4 |  | 0 |  |
|  |  | 5 |  | 0 |  |
| CA_4-7, CA_4-4-7, CA_4-7-7 |  | 4 |  | 0.5 |  |
|  |  | 7 |  | 0.5 |  |
| CA_4-12, CA_4-4-12, CA_4-12-12, CA_4-4-12-12 |  | 4 |  | 0 |  |
|  |  | 12 |  | 0.5 |  |
| CA_4-13, CA_4-4-13 |  | 4 |  | 0 |  |
|  |  | 13 |  | 0 |  |
| CA_4-17 |  | 4 |  | 0 |  |
|  |  | 17 |  | 0.5 |  |
| CA_4-27 |  | 4 |  | 0 |  |
|  |  | 27 |  | 0 |  |
| CA_4-28 |  | 4 |  | 0 |  |
|  |  | 28 |  | 0.2 |  |
| CA_4-29, CA_4-4-29 |  | 4 |  | 0 |  |
| CA_4-30, CA_4-4-30 |  | 4 |  | 0.4 |  |
|  |  | 30 |  | 0.5 |  |
| CA_4-46 |  | 4 |  | 0 |  |
| CA_4-48 |  | 4 |  | 0 |  |
|  |  | 48 |  | 0.5 |  |
| CA_4-71, CA_4-4-71 |  | 4 |  | 0 |  |
|  |  | 71 |  | 0 |  |
| CA_5-7, CA_5-7-7 |  | 5 |  | 0 |  |
|  |  | 7 |  | 0 |  |
| CA_5-12, CA_5-12-12 |  | 5 |  | 0.5 |  |
|  |  | 12 |  | 0.3 |  |
| CA_5-13 |  | 5 |  | 0 |  |
|  |  | 13 |  | 0 |  |
| CA_5-17 |  | 5 |  | 0.5 |  |
|  |  | 17 |  | 0.3 |  |
| CA_5-25 |  | 5 |  | 0 |  |
|  |  | 25 |  | 0 |  |
| CA_5-28 |  | 5 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_5-29 |  | 5 |  | 0 |  |
| CA_5-30 |  | 5 |  | 0 |  |
|  |  | 30 |  | 0 |  |
| CA_5-38 |  | 5 |  | 0 |  |
|  |  | 38 |  | 0 |  |
| CA_5-40, CA_5-5-40, CA_5-40-40 |  | 5 |  | 0 |  |
|  |  | 40 |  | 0 |  |
| CA_5-41 |  | 5 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_5-48 |  | 5 |  | 0 |  |
|  |  | 48 |  | 0 |  |
| CA_5-66, CA_5-5-66, CA_5-66-66, CA_5-5-66-66 |  | 5 |  | 0 |  |
|  |  | 66 |  | 0 |  |
| CA_7-8, CA_7-7-8 |  | 7 |  | 0 |  |
|  |  | 8 |  | 0.2 |  |
| CA_7-12 |  | 7 |  | 0 |  |
|  |  | 12 |  | 0 |  |
| CA_7-20 |  | 7 |  | 0 |  |
|  |  | 20 |  | 0 |  |
| CA_7-22 |  | 7 |  | 0 |  |
|  |  | 22 |  | 0.5 |  |
| CA_7-26, CA_7-7-26 |  | 7 |  | 0 |  |
|  |  | 26 |  | 0 |  |
| CA_7-28, CA_7-7-28 |  | 7 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_7-29, CA_7-7-29 |  | 7 |  | 0 |  |
| CA_7-30 |  | 7 |  | 0.5 |  |
|  |  | 30 |  | 0.5 |  |
| CA_7-32 |  | 7 |  | 0 |  |
|  |  | 32 |  | 0 |  |
| CA_7-40 |  | 7 |  | 0 |  |
|  |  | 40 |  | 0.5 |  |
| CA_7-42, CA_7-42-42 |  | 7 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_7-46, CA_7-7-46 |  | 7 |  | 0 |  |
| CA_7-66, CA_7-7-66, CA_7-66-66, CA_7-7-66-66 |  | 7 |  | 0.5 |  |
|  |  | 66 |  | 0.5 |  |
| CA_8-11 |  | 8 |  | 0 |  |
|  |  | 11 |  | 0 |  |
| CA_8-20 |  | 8 |  | 0 |  |
|  |  | 20 |  | 0 |  |
| CA_8-27 |  | 8 |  | 0.3 |  |
|  |  | 27 |  | 0.3 |  |
| CA_8-2813 |  | 8 |  | 0.2 |  |
|  |  | 28 |  | 0.1 |  |
| CA_8-32 |  | 8 |  | 0 |  |
|  |  | 32 |  | 0 |  |
| CA_8-38 |  | 8 |  | 0 |  |
|  |  | 38 |  | 0 |  |
| CA_8-39 |  | 8 |  | 0 |  |
|  |  | 39 |  | 0 |  |
| CA_8-40 |  | 8 |  | 0 |  |
|  |  | 40 |  | 0 |  |
| CA_8-41 |  | 8 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_8-42 |  | 8 |  | 0.2 |  |
|  |  | 42 |  | 0.5 |  |
| CA_8-46 |  | 8 |  | 0 |  |
| CA_11-18 |  | 11 |  | 0 |  |
|  |  | 18 |  | 0 |  |
| CA_11-26 |  | 11 |  | 0 |  |
|  |  | 26 |  | 0 |  |
| CA_11-28 |  | 11 |  | 0 |  |
|  |  | 28 |  | 0.2 |  |
| CA_11-41 |  | 11 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_11-42 |  | 11 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_11-46 |  | 11 |  | 0 |  |
| CA_12-25 |  | 12 |  | 0 |  |
|  |  | 25 |  | 0 |  |
| CA_12-30 |  | 12 |  | 0 |  |
|  |  | 30 |  | 0 |  |
| CA_12-46 |  | 12 |  | 0 |  |
|  |  | 46 |  | 0 |  |
| CA_12-48 |  | 12 |  | 0 |  |
|  |  | 48 |  | 0 |  |
| CA_12-66, CA_12-66-66 |  | 12 |  | 0.5 |  |
|  |  | 66 |  | 0 |  |
| CA_13-48, CA_13-48-48 |  | 13 |  | 0 |  |
|  |  | 48 |  | 0 |  |
| CA_13-66, CA_13-66-66 |  | 13 |  | 0 |  |
|  |  | 66 |  | 0 |  |
| CA_14-30 |  | 14 |  | 0 |  |
|  |  | 30 |  | 0 |  |
| CA_14-66, CA_14-66-66, CA_14-66-66-66 |  | 14 |  | 0 |  |
|  |  | 66 |  | 0 |  |
| CA_18-289 |  | 18 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_18-42 |  | 18 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_19-21 |  | 19 |  | 0 |  |
|  |  | 21 |  | 0 |  |
| CA_19-289 |  | 19 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_19-42 |  | 19 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_19-46 |  | 19 |  | 0 |  |
| CA_20-28 |  | 20 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_20-31 |  | 20 |  | 0 |  |
|  |  | 31 |  | 0 |  |
| CA_20-32 |  | 20 |  | 0 |  |
| CA_20-38 |  | 20 |  | 0 |  |
|  |  | 38 |  | 0 |  |
| CA_20-40, CA_20-40-40 |  | 20 |  | 0 |  |
|  |  | 40 |  | 0 |  |
| CA_20-42, CA_20-42-42 |  | 20 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_20-43 |  | 20 |  | 0 |  |
|  |  | 43 |  | 0.5 |  |
| CA_20-67 |  | 20 |  | 0 |  |
| CA_20-75 |  | 20 |  | 0 |  |
| CA_20-76 |  | 20 |  | 0 |  |
| CA_21-28 |  | 21 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_21-42 |  | 21 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_21-46 |  | 21 |  | 0 |  |
| CA_23-29 |  | 23 |  | 0 |  |
| CA_25-26, CA_25-25-26 |  | 25 |  | 0 |  |
|  |  | 26 |  | 0 |  |
| CA_25-41, CA_25-25-41 |  | 25 |  | 0 |  |
|  |  | 41 |  | 010 |  |
|  |  |  |  | 0.511 |  |
| CA_25-46 |  | 25 |  | 0 |  |
|  |  | 46 |  | 0 |  |
| CA_26-41 |  | 26 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_26-46 |  | 26 |  | 0 |  |
| CA_26-48, CA_26-48-48 |  | 26 |  | 0 |  |
|  |  | 48 |  | 0 |  |
| CA_28-32 |  | 28 |  | 0 |  |
| CA_28-38 |  | 28 |  | 0 |  |
|  |  | 38 |  | 0 |  |
| CA_28-40 |  | 28 |  | 0 |  |
|  |  | 40 |  | 0 |  |
| CA_28-41 |  | 28 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_28-42, CA_28-42-42 |  | 28 |  | 0.2 |  |
|  |  | 42 |  | 0.5 |  |
| CA_28-46 |  | 28 |  | 0 |  |
| CA_28-66 |  | 28 |  | 0.2 |  |
|  |  | 66 |  | 0 |  |
| CA_29-30 |  | 30 |  | 0 |  |
| CA_29-66, CA_29-66-66 |  | 66 |  | 0 |  |
| CA_29-70 |  | 70 |  | 0 |  |
| CA_30-66, CA_30-66-66 |  | 30 |  | 0.5 |  |
|  |  | 66 |  | 0.4 |  |
| CA_32-42 |  | 42 |  | 0.5 |  |
| CA_32-43 |  | 43 |  | 0.5 |  |
| CA_34-39 |  | 34 |  | 0.21 |  |
|  |  | 39 |  | 0.21 |  |
| CA_34-41 |  | 34 |  | 0.21 |  |
|  |  | 41 |  | 0.21 |  |
| CA_38-40, CA_38-40-40 |  | 38 |  | 0.54 |  |
|  |  | 40 |  | 0.54 |  |
| CA_39-40 |  | 39 |  | 0.34 |  |
|  |  | 40 |  | 0.34 |  |
| CA_39-41 |  | 39 |  | 0.24 |  |
|  |  | 41 |  | 0.24 |  |
| CA_39-41 |  | 39 |  | 0.27 |  |
|  |  | 41 |  | 0.27 |  |
| CA_39-42 |  | 39 |  | 04 |  |
|  |  | 42 |  | 0.54 |  |
| CA_39-46 |  | 39 |  | 0 |  |
| CA_40-41 |  | 40 |  | 04 |  |
|  |  | 41 |  | 04 |  |
| CA_40-42 |  | 40 |  | 0.44 |  |
|  |  | 42 |  | 0.54 |  |
| CA_40-43 |  | 40 |  | 0.44 |  |
|  |  | 43 |  | 0.54 |  |
| CA_40-46 |  | 40 |  | 0 |  |
| CA_41-42, CA_41-42-42 |  | 41 |  | 0.44 |  |
|  |  | 42 |  | 0.54 |  |
| CA_41-42, CA_41-42-42 |  | 41 |  | 07 |  |
|  |  | 42 |  | 0.57 |  |
| CA_41-46 |  | 41 |  | 0 |  |
| CA_41-48 |  | 41 |  | 04 |  |
|  |  | 48 |  | 0.54 |  |
| CA_42-43 |  | 42 |  | 04 |  |
|  |  | 43 |  | 04 |  |
| CA_42-46 |  | 42 |  | [0] |  |
| CA_46-48, CA_46-48-48 |  | 48 |  | 0.5 |  |
| CA_46-66, CA_46-66-66 |  | 66 |  | 0 |  |
| CA_46-70 |  | 70 |  | 0 |  |
| CA_46-71 |  | 71 |  | 0 |  |
| CA_48-66, CA_48-48-66, CA_48-66-66, CA_48-48-66-66 |  | 48 |  | 0.5 |  |
|  |  | 66 |  | 0.2 |  |
| CA_48-71, CA_48-48-71 |  | 48 |  | 0 |  |
|  |  | 71 |  | 0 |  |
| CA_66-70, CA_66-66-70 |  | 66 |  | 0 |  |
|  |  | 70 |  | 0 |  |
| CA_66-71, CA_66-66-71 |  | 66 |  | 0 |  |
|  |  | 71 |  | 0 |  |
| CA_70-71 |  | 70 |  | 0 |  |
|  |  | 71 |  | 0 |  |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in intra-band and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above 2DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 2DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the 2DL tolerances in Table 7.3.1-1A, truncated to one decimal place that would apply for that operating band among the supported 2DL CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 2DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum 2DL tolerance in Table 7.3.1-1A that would apply for that operating band among the supported 2DL CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 5: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 6: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 7: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in the FDD band. NOTE 9: For Band 28, the requirements only apply for the restricted frequency range specified for this CA configuration (Table 5.5A-2). NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 12: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔRIB,c in Band 42, 43, or 48 is the max(Band 42 ΔRIB,c , Band 43 ΔRIB,c, Band 48 ΔRIB,c). NOTE 13: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8. |  |  |  |  |  |

---Text omitted---

Table 7.3.1A-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-5A-41A28 | 41 |  |  |  |  |  | N/A | TDD |
| CA_1A-3A-7A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-7A-8A4,5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_1A-3C-7A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3C-7A-8A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_1A-3A-7A-8A-20A5,6 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-7A-8A-20A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_1A-3A-7A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-7A-20A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-7A-20A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-7A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-7A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-7A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-11A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-20A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-11A-28A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-28A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-38A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-11A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-11A-28A9,10 | 11 |  |  | -75.2 | -75.2 |  |  | FDD |
| CA_1A-3A-8A-40A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-18A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-18A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-19A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-19A-42A11 | 42 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-20A-32A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-20A-32A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-20A-32A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_1A-3A-21A-28A4,21 | 21 |  |  | N/A | N/A | N/A |  | FDD |
| CA_1A-3A-21A-42A22,23 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-21A-42A24 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-28A-42A22,23 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-28A-42A24 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-28A5,6 CA_1A-1A-3A-28A CA_1A-3A-3A-28A | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-32A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-32A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-42A-43A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-42A-43A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-32A-42A-43A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-32A-42A-43A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-41A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-41A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-3A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-3A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-5A-41A8 | 41 |  |  |  |  |  | N/A | TDD |
| CA_1A-7A-8A5,6, CA_1A-7A-7A-8A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_1A-7A-8A-20A | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-7A-8A-40A5,6 | 733 |  |  |  | -87.1 | -86.7 | -86.4 | FDD |
| CA_1A-7A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-7A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-8A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-11A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-11A-28A9,10 | 11 |  |  | -75.2 | -75.2 |  |  | FDD |
| CA_1A-18A-28A14 | 1 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-19A-28A14 | 133 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-20A-28A | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-20A-32A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_1A-21A-28A4,21 | 21 |  |  | N/A | N/A | N/A |  | FDD |
| CA_1A-21A-28A-42A4,21 | 21 |  |  | N/A | N/A | N/A |  | FDD |
| CA_1A-28A5,6,14 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-1A-28A5,6,14 | 1 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-28A-40A15,16 CA_1A-28A-40C15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_1A-28A-40A5,6 CA_1A-28A-40C5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-28A-42A5,6,17,18 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 4233 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_2A-46A15,16 | 2 |  |  | -70 | -67 | -65.2 | -64 | FDD |
| CA_2A-2A-46A15,16 | 2 |  |  | -70 | -67 | -65.2 | -64 | FDD |
| CA_2A-48A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-71A36 | 2 |  |  | -93.4 | -94 | -92.5 | -91.4 | FDD |
| CA_2A-71A37 | 2 |  |  | -96.8 | -94 | -92.5 | -91.4 | FDD |
| CA_2A-71A15,16 | 71 |  |  | -70.4 | -70.4 | -70.4 | -70.4 | FDD |
| CA_2A-2A-71A36 | 2 |  |  | -93.4 | -94 | -92.5 | -91.4 | FDD |
| CA_2A-2A-71A37 | 2 |  |  | -96.8 | -94 | -92.5 | -91.4 | FDD |
| CA_2A-2A-71A15,16 | 71 |  |  | -70.4 | -70.4 | -70.4 | -70.4 | FDD |
| CA_2A-4A-12A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-4A-28A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-4A-71A36, CA_2A-2A-4A-71A36 | 2 |  |  | -93.1 | -93.7 | -92.2 | -91.1 | FDD |
| CA_2A-4A-71A37, CA_2A-2A-4A-71A37 | 2 |  |  | -96.5 | -93.7 | -92.2 | -91.1 | FDD |
| CA_2A-4A-71A15,16, CA_2A-2A-4A-71A15,16 | 71 |  |  | -70.4 | -70.4 | -70.4 | -70.4 | FDD |
| CA_2A-4A-5A-12A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-4A-7A-12A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-4A-12A-30A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-5A-12A-66A 5,6 | 66 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-7A-12A-66A5,6 | 6633 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-12A-66A5,6 | 6633 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-13A-48A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-13A-48A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-13A-48A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-13A-48A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48A-48A10,31 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A-48A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48A-48C10,31 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A-48C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48C9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48D9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48D11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48A-66A10,32, CA_2A-48C-66A10,32 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A-66A11, CA_2A-48C-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-66A-71A36 CA_2A-2A-66A-71A36 CA_2A-66A-66A-71A36 CA_2A-66C-71A36 | 2 |  |  | -93.1 | -93.7 | -92.2 | -91.1 | FDD |
| CA_2A-66A-71A37 CA_2A-2A-66A-71A37 CA_2A-66A-66A-71A37 CA_2A-66C-71A37 | 2 |  |  | -96.5 | -93.7 | -92.2 | -91.1 | FDD |
| CA_2A-66A-71A15,16 CA_2A-2A-66A-71A15,16 CA_2A-66A-66A-71A15,16 CA_2A-66C-71A15,16 | 71 |  |  | -70.4 | -70.4 | -70.4 | -70.4 | FDD |
| CA_3A-5A-41A28 | 41 |  |  |  |  |  | N/A | TDD |
| CA_3A-7A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-8A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_3C-7A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3C-7A-8A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_3A-7A-8A-20A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-8A-20A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_1A-3A-8A-20A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-8A-38A 4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-8A-40A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-8A-40A5,6 | 733 |  |  |  | -87.1 | -86.7 | -86.4 | FDD |
| CA_3A-7A-20A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-7A-20A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-7A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-7A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-11A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-11A-28A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-32A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-38A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3C-8A-38A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-40A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-40C4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-11A-28A9,10 | 11 |  |  | -75.2 | -75.2 |  |  | FDD |
| CA_3A-18A-42A9,10 CA_3A-18A-42C9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-18A-42A11 CA_3A-18A-42C11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-19A-21A-42A25,26 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-19A-21A-42A27 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-19A-21A-42C25,26 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-19A-21A-42C27 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-19A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-19A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-20A-32A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-20A-32A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-20A-32A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_3A-20A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-20A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-21A-28A4,21 | 21 |  |  | N/A | N/A | N/A |  | FDD |
| CA_3A-21A-28A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-21A-28A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-21A-28A-42C9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-21A-28A-42C11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-21A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-21A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-40A15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_3A-28A-41A-42A9,10,29 | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 |  |
| CA_3A-28A-41A-42A11,29 | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |
|  | 4233 |  |  |  | -94.7 | -93.2 | -92.5 |  |
| CA_3A-28A-41A-42A17,18, 29 | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |
|  | 4233 |  |  |  | -85.4 | -85.1 | -84.9 |  |
| CA_3A-28A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-28A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-42A17,18 | 4233 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_3A-31A12,13 | 333 |  |  | -86.9 | -86.4 | -86 | -85.6 | FDD |
| CA_3A-32A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-32A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-42A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-3A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-42A-43A9. 10 | 42 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42A-43A11 | 42 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-32A-42A-43A9. 10 | 42 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-32A-42A-43A11 | 42 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_4A-5A-12A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-7A-12A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-7A -28A5,6 | 433 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_4A-12A5,6 | 433 | -89.2 | -89.2 | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-12A-30A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-17A5,6 | 433 |  |  | -90 | -89.5 |  |  | FDD |
| CA_4A-28A5,6 | 433 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_5A-12A-66A5,6 | 6633 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_5A-38A19 | 38 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_5A-41A8,19 | 41 |  |  |  |  |  | N/A | TDD |
| CA_5A-40A-41A8 | 41 |  |  |  |  |  | N/A | TDD |
| CA_5A-48A-66A9,10 CA_5A-48A-66A-66A9,10 | 4833 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_5A-48A-66A11 CA_5A-48A-66A-66A11 | 4833 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_7A-8A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_7A-8A-20A5,6 | 733 |  |  |  | -87.4 | -87 | -86.7 | FDD |
| CA_7A-12A-66A5,6 | 6633 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_7A-12B-66A5,6 | 6633 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_7A-20A-38A19 | 38 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_7A-28A-40A15,16 CA_7A-28A-40C15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_8A-28A-41A28 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_8A-39A-41A4 | 41 |  |  |  |  |  | N/A | TDD |
| CA_8A-41A8 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_8A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_11A-28A9,10 | 1133 |  |  | -75.2 | -75.2 |  |  | FDD |
| CA_12A-30A-66A5,6 | 6633 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_12A-66A5,6 | 6633 | -88.7 | -88.7 | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_13A-48A-66A9,10 CA_13A-48A-66A-66A9,10 CA_13A-48A-66B9,10 CA_13A-48A-66C9,10 CA_13A-48A-48A-66A9,10 CA_13A-48C-66A9,10 CA_13A-48D-66A9,10 CA_13A-48A-48C-66A9,10 | 4833 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_13A-48A-66A11 CA_13A-48A-66A-66A11 CA_13A-48A-66B11 CA_13A-48A-66C11 CA_13A-48A-48A-66A11 CA_13A-48C-66A11 CA_13A-48D-66A11 CA_13A-48A-48C-66A11 CA_13A-48E-66A | 4833 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_20A-32A-42A12,13 | 20 |  |  | -97 |  |  |  | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_20A-38A19 | 20 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_20A-38A-40D15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-38C19 | 38 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_20A-40A15,16 | 2033 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_20A-40A-40A15,16 | 2033 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-38A-40A15,16 | 2033 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-40C15,16 | 2033 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-40D15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-38A-40A-40A15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-38A-40C15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-42A12,13, CA_20A-42A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_21A-28A4,21 | 21 |  |  | N/A | N/A | N/A |  | FDD |
| CA_21A-28A-42A4,21 | 21 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_25A-26A-41A38 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_25A-25A-26A-41A8 CA_25A-26A-41C8 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_26A-41A8,19 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_28A-32A9,10 | 32 |  |  | -72.2 | -72.2 | -72.2 | -72.2 | FDD |
| CA_28A-32A11 | 32 |  |  | -97.6 | -95.2 | -93.7 | -93.0 | FDD |
| CA_28A-40A15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_28A-41A-42A17,18 | 4233 |  |  |  | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-42A17,18 | 4233 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-66A5,6 | 66 |  |  |  |  |  | -89.5 |  | -88.9 |  | -88.5 |  | -88.2 |  | FDD |  |
| CA_48A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-66B9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-66B11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-66C9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-66C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-66A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-66A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48A-66A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48A-66A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48A-66B9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48A-66B11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48A-66C9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48A-66C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48C-66A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_48C-66A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_48C-66B9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48C-66B11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48C-66C9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48C-66C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48C-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48C-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48E-66A 9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48E-66A 11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_66A-70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
| CA_66A-66A-70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
| CA_66A-70C-71A5,6, 35 | 70 |  |  | -90 | -89.5 | -89.2 | -89 | FDD |
| CA_66A-66A-70C-71A5,6,35 | 70 |  |  | -90 | -89.5 | -89.2 | -89 | FDD |
| CA_66C-70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
| CA_66C-70C-71A5,6,35 | 70 |  |  | -90 | -89.5 | -89.2 | -89 | FDD |
| CA_70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
| CA_70C-71A5,6,35 | 70 |  |  | -90 | -89.5 | -89.2 | -89 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1. NOTE 3: The signal power is specified per port NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 7: Void. NOTE 8: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_5A-48A-66A, CA_5A-48A-66A-66A, CA_13A-48A-66A, CA_13A-48A-66A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_13A-48D-66A, CA_13A-48A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-18A-42A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_1A-3A-41A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A, CA_3A-28A-41A-42A, CA_3A-18A-42A and CA_3A-18A-42C. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 12: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 13: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 14: For the UE that supports CA_1A-18A-28A or CA_1A-19A-28A, no requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 15: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 16: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 17: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 18: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 19: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 20: Void NOTE 21: No requirements apply when there is at least one individual RE on band 28 uplink outside frequencies 728 – 738 MHz. The reference sensitivity is only verified when all configured RE’s are confined within frequencies 728 – 738 MHz (the requirements specified in clause 7.3.1 of [6] apply). NOTE 22: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_1A-3A-21A-42A. NOTE 23: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 24: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 25: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-19A-21A-42A. NOTE 26: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 27: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 28: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of either Band 5 or Band 8 for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of Band 41. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 29: The B41 requirements are modified by -0.1dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 30: Void NOTE 31: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2A-48A-48A and CA_2A-48A-48C NOTE 32: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2A-48A-66A, CA_2A-48A-66A-66A and CA_2A-48C-66A NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 34: Void NOTE 35: These exceptions for the intra-band class C carriers should be verified one carrier at a time, according to note 6 frequency arrangements. No exceptions apply for the carrier which is not under REFSENS exception test. NOTE 36: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 37: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. Note 38: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 3nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 15 MHz for CA_26A-41A, CA_25A-26A-41A. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0b: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3C-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-7A-8A-20A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-7A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-7A-20A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-7A-28A | 28 |  |  |  | 16 | 25 | 25 | FDD |
| CA_1A-3A-7A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-11A-28A4 | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-11A-28A5 | 28 |  |  | 12 | 25 |  |  | FDD |
| CA_1A-3A-18A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-19A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-28A CA_1A-1A-3A-28A CA_1A-3A-3A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-3A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-21A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-28A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-41A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-32A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-42A-43A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-7A-8A-20A CA_1A-7A-7A-8A | 8 |  |  | 8 | 16 |  |  | FDD |
| CA_1A-7A-8A-40A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-7A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-7A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-8A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-11A-28A4 | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-11A-28A5 | 28 |  |  | 12 | 25 |  |  | FDD |
| CA_1A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-20A-32A-42A | 20 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-20A-32A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
|  | 20 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-32A-42A-43A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-1A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-28A-42A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_2A-46A | 46 |  |  |  |  |  | 100 | TDD |
| CA_2A-2A-46A | 46 |  |  |  |  |  | 100 | TDD |
| CA_2A-71A | 71 |  |  | 25 | 251 | 201 | 201 | FDD |
| CA_2A-71A | 2 |  |  | 25 | 50 | 50 | 50 | FDD |
| CA_2A-2A-71A | 71 |  |  | 25 | 50 | 50 | 50 | FDD |
| CA_2A-4A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_2A-4A-28A | 28 |  |  | 8 | 16 | 25 |  | FDD |
| CA_2A-4A-71A CA_2A-2A-4A-71A | 71 |  |  | 25 | 50 | 50 | 50 | FDD |
| CA_2A-4A-5A-12A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-4A-7A-12A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-4A-12A-30A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-5A-12A-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_2A-7A-12A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-12A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-13A-48A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_2A-13A-48A-66A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 66 |  |  | 121 | 251 | 361 | 501 |  |
| CA_2A-48A CA_2A-48A-48A CA_2A-48A-48C CA_2A-48D | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_2A-48A-66A CA_2A-48C-66A CA_2A-48A-66A-66A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_2A-66A-71A CA_2A-2A-66A-71A CA_2A-66A-66A-71A CA_2A-66C-71A | 71 |  |  | 25 | 50 | 50 | 50 | FDD |
| CA_3A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3C-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-7A-20A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-7A-8A-40A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |  |
| CA_3A-7A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-8A-32A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-11A-28A | 28 |  |  | 12 | 25 |  |  | FDD |
| CA_3A-18A-42A CA_3A-18A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-19A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-19A-21A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-20A-32A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
|  | 20 |  |  | 8 | 16 | 25 | 25 |  |
| CA_3A-19A-21A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-20A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-21A-28A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-21A-28A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-21A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-28A-41A-42A | 3 |  |  |  | 25 | 36 | 50 | FDD |
| CA_3A-28A-41A-42A | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_3A-31A | 31 |  |  | 5 | 5 | 5 | 5 | FDD |
| CA_3A-32A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-32A-42A-43A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-3A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-42A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-42A-43A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_4A-5A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-7A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-7A-28A | 28 |  |  | [8] | [16] | [25] | [25] | FDD |
| CA_4A-12A | 12 | 2 | 5 | 8 | 16 | 20 | 20 | FDD |
| CA_4A-12A-30A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-17A | 17 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-28A | 28 |  |  | [8] | [16] | [25] | [25] | FDD |
| CA_5A-12A-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_5A-48A-66A CA_5A-48A-66A-66A | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_7A-8A-20A | 8 |  |  |  | 16 | 25 | 25 | FDD |
| CA_7A-12A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_7A-12B-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_7A-28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_8A-42A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_11A-28A | 28 |  |  | 12 | 25 |  |  | FDD |
| CA_12A-30A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_12A-66A | 12 | 2 | 5 | 8 | 16 | 20 | 20 | FDD |
| CA_13A-48A-66A CA_13A-48A-66A-66A CA_13A-48A-66B CA_13A-48A-66C CA_13A-48A-48A-66A CA_13A-48C-66A CA_13A-48D-66A CA_13A-48A-48C-66A CA_13A-48E-66A | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_20A-32A-42A | 20 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_20A-40A3 CA_20A-40D3 CA_20A-38A-40A-40A3 CA_20A-38A-40C3 | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_20A-40C3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_20A-40A-40A3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_20A-38A-40A3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_20A-38A-40D3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_20A-42A, CA_20A-42A-42A | 20 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_28A-32A | 28 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_28A-41A-42A | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_28A-42A | 28 |  |  | 5 | 10 | 15 | 20 | FDD |
| CA_28A-66A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_48A-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48A-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48A-66A-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48A-66B | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48A-66C | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48C-66A-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48C-66B | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48C-66C | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-66A-66A | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_48A-66B | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_48A-66C | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_48C-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48E-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_66A-70A-71A | 71 |  |  | 8 | 16 | 20 |  | FDD |
| CA_66A-66A-70A-71A | 71 |  |  | 8 | 16 | 20 |  | FDD |
| CA_66A-70C-71A | 71 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_66A-66A-70C-71A | 71 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_66C-70A-71A | 71 |  |  | 8 | 16 | 20 |  | FDD |
| CA_66C-70C-71A | 71 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_70A-71A | 71 |  |  | 8 | 16 | 20 |  | FDD |
| CA_70C-71A | 71 |  |  | 8 | 16 | 20 | 20 | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: 3 refers to the UL resource blocks shall be located between 2373-2400MHz. NOTE 4: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a high band. |  |  |  |  |  |  |  |  |

---Text omitted---

Table 7.3.1A-0bE: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |  |
| CA_1A-3A-5A-40A | 4019 |  |  |  | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_1A-3A-5A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
| CA_1A-3A-5A-41A5 | 41 |  |  |  |  |  | [-88.1] | TDD | 3 |  |
| CA_1A-3A-7A-20A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
| CA_1A-3A-7A-20A-42A | 719 |  |  |  | -93 | -91.3 | -90.1 | FDD | 42 |  |
| CA_1A-3A-7A-38A CA_1A-3C-7A-38A | 719 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | FDD | 1 |  |
|  | 3819 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-38A CA_1A-3C-7A-38A | 719 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | FDD | 3 |  |
|  | 3819 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-40A15 | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |  |
|  | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-40A16 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |  |
|  | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-40A | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_1A-3A-7A-40A | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-3A-7A-40A | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_1A-3A-7A-40C | 40 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_1A-3A-7A-40C | 40 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-3A-7A-40C | 1 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 7 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_1A-3A-7A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 719 |  |  |  | -93 | -91.3 | -90.1 | FDD | 42 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
| CA_1A-3A-8A-40A | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_1A-3A-8A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
| CA_1A-3A-38A12 CA_1A-3C-38A12 | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD | 3 |  |
|  | 119 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |  |
|  | 319 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |  |
| CA_1A-3A-38A13 CA_1A-3C-38A13 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD | 3 |  |
|  | 119 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |  |
|  | 319 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |  |
| CA_1A-3A-40A | 4019 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_1A-3A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
| CA_1A-3A-40C | 4019 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_1A-3A-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
| CA_1A-3C-40A | 4019 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_1A-3C-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
| CA_1A-3A-41A12,14 CA_1A-3A-41C12,14 CA_1A-3A-41D12,14 | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD | 3 |  |
| CA_1A-3A-41A13,14 CA_1A-3A-41C13,14 CA_1A-3A-41D13,14 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD | 3 |  |
| CA_1A-3A-41A-42A12,14 CA_1A-3A-41A-42C12,14 CA_1A-3A-41C-42A12,14 CA_1A-3A-41C-42C12,14 | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD | 3 |  |
| CA_1A-3A-41A-42A13,14 CA_1A-3A-41A-42C13,14 CA_1A-3A-41C-42A13,14 CA_1A-3A-41C-42C13,14 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD | 3 |  |
| CA_1A-3C-40C | 4019 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_1A-3C-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
| CA_1A-5A-40A | 4019 |  |  |  | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-5A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
| CA_1A-7A-8A-40A | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD | 1 |  |
| CA_1A-7A-8A-40A | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-7A-8A-40A | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_1A-7A-8A-40C | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD | 1 |  |
| CA_1A-7A-8A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-7A-8A-40C | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_1A-7A-20A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
| CA_1A-7A-20A-42A | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
| CA_1A-7A-38A | 719 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | FDD | 1 |  |
|  | 38 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-7A-40A | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD | 1 |  |
| CA_1A-7A-40A | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-7A-40A | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_1A-7A-40C | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD | 1 |  |
| CA_1A-7A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-7A-40C | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_1A-7A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
| CA_1A-8A-40A | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-8A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
| CA_1A-8A-40C | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-8A-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
| CA_1A-28A-40A CA_1A-28A-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD |  |  |
| CA_1A-28A-40A CA_1A-28A-40C | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-28A-40A CA_1A-28A-40C | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_1A-40A | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
| CA_1A-40C | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
| CA_3A-5A-40A | 4019 |  |  |  | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-5A-40A | 319 |  | [-95.3] | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-5A-41A5 | 41 |  |  |  |  |  | [-88.1] | TDD | 3 |  |
| CA_3A-5A-41A5,18 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  |  |  |  | -91.5 | TDD |  |  |
| CA_3A-7A-8A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3A-7A-8A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 8 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3C-7A-8A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3C-7A-8A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] |  | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] |  |  |  |
| CA_3A-7A-8A-40A | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_3A-7A-8A-40A | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_3A-7A-8A-40C | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_3A-7A-8A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_3A-7A-8A-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_3A-7A-8A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_3A-7A-28A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3A-7A-28A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 28 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3C-7A-28A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3C-7A-28A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3A-7A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3C-7A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3A-7A-40A | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_3A-7A-40A | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_3A-7A-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 719 |  |  | -96.8 | -94 | -92.4 | -91.2 |  |  |  |
| CA_3A-7A-40C | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_3A-7A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_3A-7A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 719 |  |  | -96.8 | -94 | -92.4 | -91.2 |  |  |  |
| CA_3A-7A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 3 |  |  | [-96.6] | [-93.6] | [-91.8] | [-90.6] | FDD | 42 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 |  |  |  |
| CA_3A-8A-40A | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-8A-40C | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-8A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-8A-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-20A-32A | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD | 3 |  |
| CA_3A-28A-40A | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-28A-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-28A-40A | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_3A-28A-40C | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-28A-40C | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_3A-28A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |  |
| CA_3A-28A-40D | 4019 |  |  |  | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-28A-40D | 4019 |  |  |  | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_3A-28A-40D | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |  |
| CA_3A-28A-41A5 | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
| CA_3A-28A-41C5 | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |
|  | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
| CA_3A-28A-41A-42A | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 41 |  |  |  | -94.517 | -92.717 | -91.517 | TDD |  |
|  | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |
| CA_3A-28A-41A-42C | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  |  | -94.517 | -92.717 | -91.517 | TDD |  |  |
| CA_3A-28A-41C-42A | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  |  | -94.517 | -92.717 | -91.517 | TDD |  |  |
| CA_3A-40A | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-40A | 319 | [-97.4] | [-95.3] | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-40A-40A | 4019 |  |  |  | -92.9 |  | -90.2 | TDD | 3 |  |
| CA_3A-40A-40A | 319 |  |  | -94.2 | -91.2 |  |  | FDD | 40 |  |
| CA_3A-40C | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-40D | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |
| CA_3A-40D | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |
| CA_3A-40E | 4019 |  |  |  |  | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-40E | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3C-40A | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3C-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3C-40C | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3C-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-41A5 | 319 |  | [-95.3] | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3A-41C5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3A-41D5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3A-3A-41A | 3 |  | [-95.3] | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |
| CA_3C-41A5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3C-41C5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3C-41D5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |
| CA_3A-41A-42A5,6,7,8 | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  | -71.7 | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41A-42A5,6,9 | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41A-42A5,6,10 | 319 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
| CA_3A-41A-42C5,6,7,8 | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  |  | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41A-42C5,6,9 | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  |  | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41A-42C5,6,10 | 319 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 | TDD |  |  |
| CA_3A-41C-42A5,6,7,8 | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  |  | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41C-42A5,6,9 | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 42 |  |  |  | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41C-42A5,6,10 | 319 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
| CA_3A-41C-42C5,6,7,8 | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  | -71.7 | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41C-42C5,6,9 | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41C-42C5,6,10 | 319 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
| CA_7A-8A-38A | 719 |  |  |  | [-91.2] | [-89.7] | [-88.6] | FDD | 8 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_7A-8A-40A | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -88 | -87.4 | -87 | -86.7 | FDD | 8 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-8A-40C | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -88 | -87.4 | -87 | -86.7 | FDD | 8 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-20A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
| CA_7A-28A-38A | 719 |  |  |  | [-91.2] | [-89.7] | [-88.6] | FDD | 8 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_7A-28A-40A CA_7A-28A-40C | 719 |  |  | -96.8 | -94 | -92.4 | -91.2 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD |  |  |
| CA_7A-28A-40A CA_7A-28A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_7A-28A-40A CA_7A-28A-40C | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_7A-40A | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-40C | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-40D | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-40E | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
| CA_7A-42A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
| CA_25A-41E, CA_25A-25A-41E, CA_25A-41F, CA_25A-25A-41F | 25 |  |  | [-95.9] | [-92.9] | [-91.1] | [-89.9] | FDD | 41 |  |
| CA_28A-40A | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_28A-40A | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
| CA_28A-40C | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_28A-40C | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
| CA_28A-40D | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_28A-40D | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply regardless of the channel bandwidth and the location of UL band. NOTE 5: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 6: The antenna isolation for MSD calculation is assumed as 10 dB. For conducted mode REFSENS test such antenna isolation is not observed as the antennas are disconnected. Additionally antenna isolation assumption is under discussion depending on the frequency range NOTE 7: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-42C, CA_3A-42D, CA_3A-3A-42C, CA_3A-42A-42C, CA_1A-3A-42A, CA_1A-3A-42C, CA_1A-3A-42D, CA_3A-19A-42A, CA_3A-19A-42C, CA_1A-3A-19A-42A, CA 3A-41A-42A, CA_3A-41A-42C, CA_3A-41C-42A, and CA_3A-41C-42C. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 11: Void NOTE 12: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 13: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 14: The B41 requirements also apply to the supported CA_1A-41A. NOTE 15: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1 NOTE 16: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 17: The B41 requirements are modified by -0.1dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 18: No requirements apply when there is at least one individual RE within the downlink transmission bandwidth of band5 for which the 3rd receiver harmonic is within the uplink transmission bandwidth of band41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 19: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.1A-0bF: Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3A-5A-41A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_1A-3A-7A-20A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | FDD |
| CA_1A-3A-7A-38A CA_1A-3C-7A-38A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_1A-3A-7A-40A CA_1A-3A-7A-40C | 11,3 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,4 |  |  | 25 | 45 | 45 | 45 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-7A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-38A CA_1A-3C-38A | 11,3 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,4 |  |  | 25 | 45 | 45 | 45 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 38 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-41A CA_1A-3A-41C CA_1A-3A-41D | 1 |  |  | 253 | 251,3 | 251,3 | 251,3 | FDD |
|  |  |  |  | 254 | 451,4 | 451,4 | 451,4 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_1A-3A-41A-42A CA_1A-3A-41A-42C CA_1A-3A-41C-42A CA_1A-3A-41C-42C | 1 |  |  | 253 | 251,3 | 251,3 | 251,3 | FDD |
|  |  |  |  | 254 | 451,4 | 451,4 | 451,4 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_1A-5A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-8A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-8A-40C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-8A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-8A-40C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-20A-42A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 20 |  |  | 25 | 201 | 205 | 205 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-38A | 1 |  |  | 25 | 45 | 451 | 451 | FDD |
| CA_1A-7A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-40C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-42A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-28A-40A CA_1A-28A-40C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-5A-41A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  |  |  |  | 100 | TDD |
| CA_3A-7A-8A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3C-7A-8A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3A-7A-8A-40A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-8A-40C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-28A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3C-7A-28A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3A-7A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3C-7A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3A-7A-40A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-40C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-8A-40A | 3 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-8A-40C | 3 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-28A-41A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-28A-41C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-28A-41A-42A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 28 |  |  | 25 | 251 |  |  | FDD |
|  | 41 |  |  |  | 50 | 75 | 100 | TDD |
|  | 42 |  |  |  | 50 | 75 | 100 | TDD |
| CA_3A-28A-41A-42C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  |  | 50 | 75 | 100 | TDD |
| CA_3A-28A-41C-42A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  |  | 50 | 75 | 100 | TDD |
| CA_3A-40A CA_3A-40C CA_3A-40D CA_3A-40E CA_1A-3A-40A CA_1A-3C-40A CA_1A-3A-40C CA_1A-3C-40C CA_3A-5A-40A CA_3A-28A-40A CA_3A-28A-40C CA_1A-3A-5A-40A CA_1A-3A-8A-40A | 3 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-40A-40A | 3 |  |  | 25 | 50 |  |  | FDD |
|  | 40 |  |  |  | 50 |  | 100 | TDD |
| CA_3A-41A CA_3A-3A-41A | 3 |  | 15 | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41C CA_3C-41A CA_3C-41D | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41D | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3C-41C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41A-42A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-8A-38A | 8 |  |  | 25 | 50 |  |  | FDD |
| CA_7A-8A-40A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 8 |  |  | 8 | 16 |  |  |  |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-8A-40C | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 8 |  |  | 8 | 16 |  |  |  |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-20A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 20 |  |  | 25 | 50 | 75 | 100 |  |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-28A-38A | 28 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_7A-28A-40A CA_7A-28A-40C | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-40A, CA_7A-40C, CA_7A-40D, CA_7A-40E | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-42A, CA_7A-42A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_25A-41E, CA_25A-25A-41E, CA_25A-41F, CA_25A-25A-41F | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_28A-40A, CA_28A-40C | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. NOTE 4: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. NOTE 5: 5 refers to Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16 |  |  |  |  |  |  |  |  |

---Text omitted---

Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-46A CA_1A-3A-46C CA_1A-3A-46D CA_1A-3A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_1A-5A-7A-46A CA_1A-5A-7A-46C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-5A-46A CA_1A-5A-46C CA_1A-5A-46D | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-5A-46A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 46 |  |  |  |  |  | -90 |  |
| CA_2A-5A-46A-66A CA_2A-5A-46C-66A CA_2A-5A-46D-66A CA_2A-5A-46E-66A CA_2A-5A-46A-66A-66A CA_2A-5A-46C-66A-66A CA_2A-5A-46D-66A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-7A-46A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-13A-46A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 13 |  |  | -97 | -94 |  |  |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-7A-46A CA_1A-7A-46C CA_1A-7A-46D CA_1A-7A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-46A CA_1A-46C CA_1A-46D CA_1A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_2A-5A-46C | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-5A-46D CA_2A-5A-46E | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 46 |  |  |  |  |  | -90 |  |
| CA_2A-7A-46A CA_2A-7A-7A-46A CA_2A-7A-46C CA_2A-7A-7A-46C CA_2A-7A-46D CA_2A-7A-7A-46D CA_2A-7A-46E CA_2A-7A-7A-46E | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_2A-13A-46C | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-13A-46A-66A CA_2A-13A-46C-66A CA_2A-13A-46D-66A CA_2A-13A-46A-66A-66A CA_2A-13A-46C-66A-66A CA_2A-13A-46D-66A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-46A CA_2A-46E CA_2A-2A-46A CA_2A-2A-46D CA_2A-46C CA_2A-46A-46A CA_2A-2A-46C CA_2A-46A-46C CA_2A-46D CA_2A-46A-46D | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-46A-48A10,11 CA_2A-46A-48C10,11 CA_2A-46A-48D10,11 CA_2A-46A-48E10,11 CA_2A-46C-48A10,11 CA_2A-46C-48C10,11 CA_2A-46C-48D10,11 CA_2A-46C-48E10,11 CA_2A-46D-48A10,11 CA_2A-46D-48C10,11 CA_2A-46E-48A10,11 CA_2A-46E-48C10,11 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-46A-48A12 CA_2A-46A-48C12 CA_2A-46A-48D12 CA_2A-46A-48E12 CA_2A-46C-48A12 CA_2A-46C-48C12 CA_2A-46C-48D12 CA_2A-46C-48E12 CA_2A-46D-48A12 CA_2A-46E-48A12 CA_2A-46D-48C12 CA_2A-46E-48C12 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-46A-48A-66A10,11 CA_2A-46A-48C-66A10,11 CA_2A-46A-48D-66A10,11 CA_2A-46C-48A-66A10,11 CA_2A-46C-48C-66A10,11 CA_2A-46C-48D-66A10,11 CA_2A-46D-48A-66A10,11 CA_2A-46D-48C-66A10,11 CA_2A-46E-48A-66A10,11 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| CA_2A-46A-48A-66A12 CA_2A-46A-48D-66A12 CA_2A-46C-48C-66A12 CA_2A-46C-48D-66A12 CA_2A-46D-48A-66A 12 CA_2A-46D-48C-66A12 CA_2A-46E-48A-66A12 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| CA_2A-46A-48C-66A12 CA_2A-46C-48A-66A12 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |  |
|  | 46 |  |  |  |  |  | -83 | TDD |  |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |  |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |  |
| CA_2A-46A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-46A-46A-66A CA_2A-46C-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |  |
|  | 46 |  |  |  |  |  | -90 | TDD |  |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |  |
| CA_2A-46A-46C-66A CA_2A-46D-66A CA_2A-46E-66A CA_2A-46A-66A-66A, CA_2A-46C-66A-66A, CA_2A-46D-66A-66A, CA_2A-46E-66A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-49A9 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 49 |  |  |  | -95.5 |  | -92.5 | TDD |
| CA_3A-7A-46C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-7A-46D | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-7A-46E | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-46A CA_3A-46C CA_3A-46D CA_3A-46E | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_3C-46A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-3A-46A CA_3C-46C CA_3C-46D CA_3A-3A-46C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-7A-46A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-32A-46A CA_3A-32A-46C CA_3A-32A-46D CA_3A-32A-46E | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | SDL |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-7A-32A-46A CA_3A-7A-32A-46C CA_3A-7A-32A-46D CA_3A-7A-32A-46E | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_4A-46A CA_4A-46C CA_4A-46A-46A CA_4A-46A-46C CA_4A-46D CA_4A-46A-46D | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-7A-46A CA_5A-7A-46C CA_5A-7A-46D | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-12A-46A, CA_5A-12A-46C, CA_5A-12A-46D | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 12 |  |  | -97 | -94 |  |  |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-46A CA_5A-46C CA_5A-46D CA_5A-46E CA_5B-46A CA_5B-46C CA_5B-46D CA_5B-46E | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_5A-46A-66A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 |  |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_5A-46C-66A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_5A-46D-66A, CA_5A-46E-66A, CA_5A-46A-66A-66A, CA_5A-46C-66A-66A, CA_5A-46D-66A-66A, CA_5A-46E-66A-66A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 |  |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_7A-46A CA_7A-46C CA_7A-7A-46C CA_7A-46D CA_7A-46E CA_7C-46A CA_7C-46C CA_7C-46D CA_7C-46E | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_7A-7A-46A CA_7A-7A-46D CA_7A-7A-46E | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_7A-32A-46A CA_7A-32A-46C CA_7A-32A-46D CA_7A-32A-46E | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | SDL |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_7A-46A-66A | 7 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
|  | 66 |  |  | -99 | -96 | -94.2 | -93 | FDD |
| CA_8A-46A CA_8A-46D CA_8A-46E CA_8B-46C CA_8B-46D | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8A-46C | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8B-46A | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46A | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46C | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46D | 11 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46E | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_12A-46A CA_12A-46C CA_12A-46D | 12 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_12A-46E | 12 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_13A-46A CA_13A-46C CA_13A-46D CA_13A-46E | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_13A-46A-66A CA_13A-46C-66A CA_13A-46D-66A CA_13A-46A-66A-66A CA_13A-46C-66A-66A CA_13A-46D-66A-66A CA_13A-46E-66A | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_19A-46A CA_19A-46C CA_19A-46D CA_19A-46E | 19 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_21A-46A CA_21A-46C CA_21A-46D CA_21A-46E | 21 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_25A-46A CA_25A-46C CA_25A-46D | 25 | -101.2 | -98.2 | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_26A-46A | 26 |  | -99.7 | -97.57 | -94.57 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_28A-46A CA_28A-46C CA_28A-46D CA_28A-46E | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_29A-46A-66A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_39A-46A | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39A-46C | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39C-46A | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39A-46E CA_39C-46C CA_39C-46D | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 |  |
| CA_40A-46A CA_40A-46D CA_40A-46E CA_40C-46C CA_40C-46D CA_40D-46A CA_40D-46C | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_40A-46C | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_40C-46A | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_41A-46A CA_41A-46C CA_41A-46D CA_41A-46E CA_41C-46A CA_41C-46C CA_41C-46D CA_41D-46A CA_41D-46C | 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_42A-46A | 42 |  |  | -99 | -96 | -94.2 | -93 | TDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
| CA_46A-48A CA_46A-48A-48A CA_46A-48C CA_46A-48D CA_46A-48E CA_46C-48A CA_46C-48A-48A CA_46C-48C CA_46C-48D CA_46C-48E CA_46D-48A CA_46D-48C CA_46D-48A-48A CA_46E-48A CA_46E-48C | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| CA_46A-48A-66A10,11 CA_46A-48C-66A10,11 CA_46A-48D-66A10,11 CA_46A-48E-66A10,11 CA_46C-48A-66A10,11 CA_46C-48C-66A10,11 CA_46C-48D-66A10, 11 CA_46C-48E-66A10,11 CA_46D-48A-66A10,11 CA_46D-48C-66A 10,11 CA_46E-48A-66A10,11 CA_46E-48C-66A10,11 | 46 |  |  |  |  |  | -83 | FDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_46A-48A-66A12 CA_46A-48C-66A12 CA_46A-48D-66A12 CA_46A-48E-66A12 CA_46C-48A-66A12 CA_46C-48C-66A12 CA_46C-48D-66A12 CA_46C-48E-66A12 CA_46D-48A-66A12 CA_46D-48C-66A 12 CA_46E-48A-66A12 CA_46E-48C-66A12 | 46 |  |  |  |  |  | -83 | FDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_46A-48A-71A CA_46A-48A-48A-71A CA_46A-48C-71A CA_46C-48A-71A CA_46C-48C-71A CA_46C-48A-48A-71A | 46 |  |  |  |  |  | -83 | TDD |  |
|  | 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |  |
|  | 71 |  |  | -97.2 | -94.2 | -92.0 | -87.5 | FDD |  |
| CA_46A-66A CA_46A-46A-66A CA_46A-66A-66A CA_46A-66C CA_46A-46C-66A CA_46A-46D-66A CA_46C-66A CA_46C-66A-66A CA_46D-66A CA_46D-66A-66A CA_46E-66A CA_46E-66A-66A | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_46A-70A | 46 |  |  |  |  |  | -90 | TDD |
|  | 70 |  |  | -100 | -97 | -95.2 |  | FDD |
| CA_46A-71A CA_46C-71A CA_46D-71A | 46 |  |  |  |  |  | -90 | FDD |
|  | 71 |  |  | -97.2 | -94.2 | -92.0 | -87.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1. NOTE 3: The signal power is specified per port. NOTE 4: Void NOTE 5: The requirement for B46 does not apply when there is at least one individual RE within the B46 downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3.1A-0eC. NOTE 6: Void NOTE 7: 7 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 8: When Band 46 have self interference problems by dual uplink CA, then the requirements not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. NOTE 9: The requirement for B49 does not apply when there is at least one individual RE within the B49 downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3.1A-0eD. NOTE 10: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band which excludes band 46 and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2-46-48, CA_46-48-66, and CA_2-46-48-66. For harmonic issue not related with band 46, the uplink configuration of CA_2-48, CA_48-66 and CA_2-48-66 in Table 7.3.1A-0b can be used. NOTE 11: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 12: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_1A-3A-46A; CA_1A-3A; 2*fc_1A + fc_3A; 2*BW_1A + BW_3A. CA_1A-3A-46A; CA_1A-3A; fc_1A – 2*fc_3A; BW_1A + 2*BW_3A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_1A-5A-46A; CA_1A-5A; 2*fc_1A + 2*fc_5A; 2*BW_1A + 2*BW_5A. CA_1A-7A-46A; CA_1A-7A; 3*fc_7A - fc_1A; 3*BW_7A + BW_1A. CA_5A-7A-46A; CA_5A-7A; 2*fc_7A + fc_5A; 2*BW_7A + BW_5A. |

---End of changes---