# 3GPP TSG-RAN WG4 Meeting #91															R4-1906875

# Reno, Nevada, USA, 13th – 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to 38.101-3: ULSUP-TDM related clarifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ULSUP-TDM related requirements and constraints are not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following corrections and clarifications are made: - UE Single UL behaviour for TDM based ULSUP is defined. - Single UL operations are allowed for TDM based ULSUP in EN-DC. In case of TDM based ULSUP single UL is always allowed, not only for difficult configurations of difficult EN-DC band combinations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network may have wrong interpretations of UE requirements related to TDM based UL sharing (ULSUP-TDM) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 5.5B.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ---------------------------------------Start of Change 1-------------------------------

## 4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal

d)	Terminal that supports EN-DC configuration shall meet E-UTRA requirements as specified in TS 36.101 [4] and NR requirements as in TS 38.101-1 [2] and TS 38.101-2 [3] unless otherwise specified in this specification

e)	All the requirements for intra-band contiguous and non-contiguous EN-DC apply under the assumption of the same uplink-downlink and special subframe configurations in the E-UTRA and slot format indicated by UL-DL-configuration-common  in the NR for the EN-DC.

f)	For EN-DC combinations with CA configuarations for E-UTRA and/or NR, all the requirements for E-UTRA and/or NR all the requirements for E-UTRA and/or NR intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configuration-common in the PSCell and SCells for NR and the same uplink-downlink and special subframe configurations in Pcell and SCells for E-UTRA.

g)	For a terminal that supports TDM based ULSUP, the current version of the specification assumes the terminal is not required to transmit simultaneously NR SUL and E-UTRA UL on the same channel, which is shared by E-UTRA and NR and the UE requirements for single carrier shall apply for the active NR SUL or E-UTRA UL accordingly.

A terminal which supports an inter-band EN-DC configuration shall support all apecified E-UTRA bandwidth combination set that belong to the E-UTRA CA configuration part of E-UTRA – NR DC and shall support all apecified NR bandwidth combination set that belong to the NR CA configuration part of E-UTRA – NR DC.

A terminal which supports an inter-band EN-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.

Terminal that supports inter-band NR-DC  between FR1 and FR2 configuration shall meet the requirements for corresponding CA configuration (suffix A), unless otherwise specified.

## 4.3	Specification suffix information

### ---------------------------------------End of Change 1-------------------------------

### ---------------------------------------Start of Change 2-------------------------------

#### 5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n28A | DC_1A_n28A DC_3A_n28A |  |  |
| DC_1A-3A_n77A5 DC_1A-3A_n77C5 | DC_1A_n77A DC_3A_n77A |  |  |
| DC_1A-3A_n78A5 DC_1A-3A_n78C5 DC_1A-3C_n78A5 | DC_1A_n78A DC_3A_n78A |  |  |
| DC_1A-3A_n79A5 DC_1A-3A_n79C5 | DC_1A_n79A DC_3A_n79A |  |  |
|  | DC_1A_n78A DC_3A_n78A |  |  |
| DC_1A-5A_n78A5 | DC_1A_n78A DC_5A_n78A |  |  |
| DC_1A-7A_n28A5 | DC_1A_n28A DC_7A_n28A |  |  |
| DC_1A-7A_n78A5 | DC_1A_n78A DC_7A_n78A |  |  |
| DC_1A-7A-7A_n78A5 | DC_1A_n78A DC_7A_n78A |  |  |
| DC_1A-8A_n78A5 | DC_1A_n78A DC_8A_n78A |  |  |
| DC_1A-18A_n77A5 | DC_1A_n77A DC_18A_n77A |  |  |
| DC_1A-18A_n78A5 | DC_1A_n78A DC_18A_n78A |  |  |
| DC_1A-18A_n79A | DC_1A_n79A DC_18A_n79A |  |  |
| DC_1A-19A_n77A5 DC_1A-19A_n77C5 | DC_1A_n77A DC 19A_n77A |  |  |
| DC_1A-19A_n78A5 DC_1A-19A_n78C5 | DC_1A_n78A DC_19A_n78A |  |  |
| DC_1A-19A_n79A5 DC_1A-19A_n79C5 | DC_1A_n79A DC_19A_n79A |  |  |
| DC_1A-20A_n28A6 | DC_1A_n28A DC_20A_n28A |  |  |
| DC_1A-20A_n78A5 | DC_1A_n78A DC_20A_n78A |  |  |
| DC_1A-21A_n77A5 DC_1A-21A_n77C5 | DC_1A_n77A DC_21A_n77A |  |  |
| DC_1A-21A_n78A5 DC_1A-21A_n78C5 | DC_1A_n78A DC_21A_n78A |  |  |
| DC_1A-21A_n79A5 DC_1A-21A_n79C5 | DC_1A_n79A DC_21A_n79A |  |  |
| DC_1A-28A_n77A5 DC_1A-28A_n77C5 | DC_1A_n77A DC_28A_n77A |  |  |
| DC_1A-28A_n78A5 DC_1A-28A_n78C5 | DC_1A_n78A DC_28A_n78A |  |  |
| DC_1A_n28A-n78A5 | DC_1A_n28A DC_1A_n78A |  |  |
| DC_1A-28A_n79A DC_1A-28A_n79C | DC_1A_n79A DC_28A_n79A |  |  |
| DC_1A-41A_n77A DC_1A-41C_n77A | DC_1A_n77A DC_41A_n77A DC_41C_n77A |  |  |
| DC_1A-41A_n78A DC_1A-41C_n78A | DC_1A_n78A DC_41A_n78A DC_41C_n78A |  |  |
| DC_1A-41C_n79A | DC_1A_n79A DC_41C_n79A |  |  |
| DC_1A-42A_n77A DC_1A-42A_n77C DC_1A-42C_n77A DC_1A-42C_n77C DC_1A-42D_n77A DC_1A-42E_n77A | DC_1A_n77A |  |  |
| DC_1A-42A_n78A DC_1A-42A_n78C DC_1A-42C_n78A DC_1A-42C_n78C DC_1A-42D_n78A DC_1A-42E_n78A | DC_1A_n78A |  |  |
| DC_1A-42A_n79A DC_1A-42A_n79C DC_1A-42C_n79A DC_1A-42C_n79C DC_1A-42D_n79A DC_1A-42E_n79A | DC_1A_n79A |  |  |
| DC_1A_n77A-n79A | DC_1A_n77A DC_1A_n79A |  |  |
| DC_1A_n78A-n79A | DC_1A_n78A DC_1A_n79A |  |  |
| DC_1A_SUL_n78A-n84A5 | DC_1A_n78A, DC_1A_n84A_ULSUP-TDM_n78A7, DC_1A_n84A_ULSUP-FDM_n78A |  |  |
| DC_2A-5A_n66A | DC_2A_n66A DC_5A_n66A |  |  |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A |  |  |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A |  |  |
| DC_2A-66A_n71A | DC_2A_n71A DC_66A_n71A |  |  |
| DC_2A-(n)71AA | DC_2A_n71A DC_(n)71AA |  |  |
| DC_3A_n3A-n77A | DC_3A_n77A DC_3A_n3A2 |  |  |
| DC_3A_n3A-n78A | DC_3A_n78A DC_3A_n3A2 |  |  |
| DC_3A-5A_n78A5 | DC_3A_n78A DC_5A_n78A |  |  |
| DC_3A-7A_n28A | DC_3A_n28A DC_7A_n28A |  |  |
| DC_3A-7A_n78A5 DC_3C-7A_n78A5 | DC_3A_n78A DC_7A_n78A |  |  |
| DC_3A-7C_n78A5 DC_3C-7C_n78A5 | DC_3A_n78A DC_7C_n78A |  |  |
| DC_3A-7A-7A_n78A5 | DC_3A_n78A DC_7A_n78A |  |  |
| DC_3A-8A_n78A | DC_3A_n78A DC_8A_n78A |  |  |
| DC_3A-19A_n77A5 DC_3A-19A_n77C5 | DC_3A_n77A DC_19A_n77A |  |  |
| DC_3A-19A_n78A5 DC_3A-19A_n78C5 | DC_3A_n78A DC_19A_n78A |  |  |
| DC_3A-19A_n79A5 DC_3A-19A_n79C5 | DC_3A_n79A DC_19A_n79A |  |  |
| DC_3A-20A_n28A5,6 | DC_3A_n28A DC_20A_n28A |  |  |
| DC_3A-20A_n78A5 DC_3C-20A_n78A5 | DC_3A_n78A DC_20A_n78A |  |  |
| DC_3A-21A_n77A5 DC_3A-21A_n77C5 | DC_3A_n77A DC_21A_n77A |  |  |
| DC_3A-21A_n78A5 DC_3A-21A_n78C5 | DC_3A_n78A DC_21A_n78A |  |  |
| DC_3A-21A_n79A5 DC_3A-21A_n79C5 | DC_3A_n79A DC_21A_n79A |  |  |
| DC_3A-28A_n77A DC_3A-28A_n77C | DC_3A_n77A DC_28A_n77A |  |  |
| DC_3A-28A_n78A5 DC_3A-28A_n78C5 | DC_3A_n78A DC_28A_n78A |  |  |
| DC_3A_n28A-n78A5 | DC_3A_n28A DC_3A_n78A |  |  |
| DC_3A-28A_n79A DC_3A-28A_n79C | DC_3A_n79A DC_28A_n79A |  |  |
| DC_3A-38A_n78A | DC_3A_n78A |  |  |
| DC_3A-41A_n78A | DC_3A_n78A DC_41A_n78A |  |  |
| DC_3A-42A_n77A DC_3A-42A_n77C DC_3A-42C_n77A DC_3A-42C_n77C DC_3A-42D_n77A DC_3A-42E_n77A | DC_3A_n77A |  |  |
| DC_3A-42A_n78A DC_3A-42A_n78C DC_3A-42C_n78A DC_3A-42C_n78C DC_3A-42D_n78A DC_3A-42E_n78A | DC_3A_n78A |  |  |
| DC_3A-42A_n79A DC_3A-42A_n79C DC_3A-42C_n79A DC_3A-42C_n79C DC_3A-42D_n79A DC_3A-42E_n79A | DC_3A_n79A |  |  |
| DC_3A_n77A-n79A | DC_3A_n77A DC_3A_n79A |  |  |
| DC_3A_n78A-n79A | DC_3A_n78A DC_3A_n79A |  |  |
| DC_3A_SUL_n78A-n80A5 | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A7 DC_3A_n80A_ULSUP-FDM_n78A |  |  |
| DC_3A_SUL_n78A-n82A5 | DC_3A_n78A DC_3A_n82A |  |  |
| DC_3A_SUL_n79A-n80A5 | DC_3A_n79A, DC_3A_n80A_ULSUP-TDM_n79A7, DC_3A_n80A_ULSUP-FDM_n79A |  |  |
| DC_5A-7A_n78A | DC_5A_n78A DC_7A_n78A |  |  |
| DC_5A-7A-7A_n78A | DC_5A_n78A DC_7A_n78A |  |  |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A |  |  |
| DC_7A-20A_n28A6 | DC_7A_n28A DC_20A_n28A |  |  |
| DC_7A-20A_n78A5 | DC_7A_n78A DC_20A_n78A |  |  |
| DC_7A-28A_n78A5 | DC_7A_n78A DC_28A_n78A |  |  |
| DC_7C-28A_n78A5 | DC_7C_n78A DC_28A_n78A |  |  |
| DC_7A_n28A-n78A5 | DC_7A_n28A, DC_7A_n78A |  |  |
| DC_7A-46A_n78A3 DC_7A-46C_n78A3 DC_7A-46D_n78A3 DC_7A-46E_n78A3 | DC_7A_n78A |  |  |
| DC_8A_SUL_n78A-n81A5 | DC_8A_n78A, DC_8A_n81A_ULSUP-TDM_n78A7, DC_8A_n81A_ULSUP-FDM_n78A |  |  |
| DC_8A_SUL_n79A-n81A5 | DC_8A_n79A, DC_8A_n81A_ULSUP-TDM_n79A7, DC_8A_n81A_ULSUP-FDM_n79A |  |  |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A |  |  |
| DC_18A-28A_n77A5 | DC_18A_n77A DC_28A_n77A |  |  |
| DC_18A-28A_n78A5 | DC_18A_n78A DC_28A_n78A |  |  |
| DC_18A-28A_n79A5 | DC_18A_n79A DC_28A_n79A |  |  |
| DC_19A-21A_n78A5 DC_19A-21A_n78C5 | DC_19A_n78A DC_21A_n78A |  |  |
| DC_19A-21A_n79A5 DC_19A-21A_n79C5 | DC_19A_n79A DC_21A_n79A |  |  |
| DC_19A-21A_n77A5 DC_19A-21A_n77C5 | DC_19A_n77A DC_21A_n77A |  |  |
| DC_19A-42A_n77A DC_19A-42A_n77C | DC_19A_n77A |  |  |
| DC_19A-42A_n78A DC_19A-42A_n78C | DC_19A_n78A |  |  |
| DC_19A-42A_n79A DC_19A-42A_n79C | DC_19A_n79A |  |  |
| DC_19A-42C_n77A DC_19A-42C_n77C | DC_19A_n77A |  |  |
| DC_19A-42C_n78A DC_19A-42C_n78C | DC_19A_n78A |  |  |
| DC_19A-42C_n79A DC_19A-42C_n79C | DC_19A_n79A |  |  |
| DC_19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |  |  |
| DC_19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |  |  |
| DC_20A_n8A-n75A6 | DC_20A_n8A |  |  |
| DC_20A_n28A-n75A6 | DC_20A_n28A |  |  |
| DC_20A_n28A-n78A5,6 | DC_20A_n28A DC_20A_n78A |  |  |
| DC_20A_n75A-n78A5 | DC_20A_n78A |  |  |
| DC_20A_n76A-n78A5 | DC_20A_n78A |  |  |
| DC_20A_SUL_n78A-n82A5 | DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A7 DC_20A_n82A_ULSUP-FDM_n78A |  |  |
| DC_20A_SUL_n78A-n83A5 | DC_20A_n78A DC_20A_n83A |  |  |
| DC_21A-28A_n77A DC_21A-28A_n77C | DC_21A_n77A DC_28A_n77A |  |  |
| DC_21A-28A_n78A DC_21A-28A_n78C | DC_21A_n78A DC_28A_n78A |  |  |
| DC_21A-28A_n79A DC_21A-28A_n79C | DC_21A_n79A DC_28A_n79A |  |  |
| DC_21A-42A_n77A DC_21A-42A_n77C DC_21A-42C_n77A DC_21A-42C_n77C | DC_21A_n77A |  |  |
| DC_21A-42A_n78A DC_21A-42A_n78C DC_21A-42C_n78A DC_21A-42C_n78C | DC_21A_n78A |  |  |
| DC_21A-42A_n79A DC_21A-42A_n79C DC_21A-42C_n79A DC_21A-42C_n79C | DC_21A_n79A |  |  |
| DC_21A_n77A-n79A | DC_21A_n77A DC_21A_n79A |  |  |
| DC_21A_n78A-n79A | DC_21A_n78A DC_21A_n79A |  |  |
| DC_28A-42A_n77A DC_28A-42A_n77C DC_28A-42C_n77A | DC_28A_n77A |  |  |
| DC_28A-42A_n78A DC_28A-42A_n78C DC_28A-42C_n78A | DC_28A_n78A |  |  |
| DC_28A-42A_n79A DC_28A-42A_n79C DC_28A-42C_n79A | DC_28A_n79A |  |  |
| DC_28A_SUL_n78A-n83A5 |  |  | DC_28A_n78A, DC_28A_n83A_ULSUP-TDM_n78A7, DC_28A_n83A_ULSUP-FDM_n78A |  |  |  |  |  |  |
| DC_41A-42A_n77A DC_41A-42C_n77A DC_41C-42A_n77A DC_41C-42C_n77A | DC_41A_n77A |  |  |
| DC_41A-42A_n78A DC_41A-42C_n78A DC_41C-42A_n78A DC_41C-42C_n78A | DC_41A_n78A |  |  |
| DC_41A-42A_n79A DC_41A-42C_n79A DC_41C-42A_n79A DC_41C-42C_n79A | DC_41A_n79A |  |  |
| DC_66A_(n)71AA | DC_66A_n71A DC_(n)71AA |  |  |
| DC_66A_SUL_n78A-n86A5 | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A7 DC_66A_n86A_ULSUP-FDM_n78A |  |  |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported in Rel.15 NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 4: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 6: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 7: If a UE supports ULSUP-TDM and is configured with ULSUP-TDM for the same channel, which is shared by E-UTRA and NR , single UL is allowed between E-UTRA UL and NR SUL on this channel. |  |  |  |

#### 5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)

### ---------------------------------------End of Change 2-------------------------------