3GPP TSG-RAN WG4 Meeting #91	R4-1906904

Reno, Nevada, USA, 13 – 17 May 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Further system simulation results for Rel-16 HST

Agenda item:	7.15.2.2

Document for:	Discussion

# 1	Introduction

During the RAN4#90bis meeting, the way forward on RRM enhancement for HST [1] was approved. In [2],

initial mobility performance results for Rel-16 HST were provided, which focused on RRM mobility performance with various DRX values.

In this document, further system simulation results for mobility performance for trains travelling at 500 km/h are provided. The same simulation assumptions as in [2] were used. The conclusion based on our simulation results is presented for the Pcell reselection and identification requirement.

# 2	Discussion

From [1], the WF for Pcell detection and measurement requirement is as follows:

- Cell detection/identification requirement for DRX 1.28s

  - Option 1(Huawei): 6 DRX cycle when DRX cycle is 1.28s.

  - Option 2(Intel): keep original requirement (10 DRX cycle)

- Cell detection/identification requirement for DRX 2.56s

  - Option 1(Huawei): remove requirement for DRX

  - Option 2(CMCC): keep requirement for DRX 2.56s

System level simulations were used to investigate the impact of DRX on the Pcell detection and measurement requirements. Table 1 in Appendix A lists the system level assumptions used, which are adopted from [1]. In general, the simulation assumptions are similar to those used in Rel-14 HST WI except for the increase UE speed.

With the simulations we want to investigate at least following two aspects:

Assuming Rel-14 HST flag is On:

- Are the current HST requirements sufficient to ensure robust mobility?

- Are the current SCell requirements sufficient for activated and deactivated or do they need to be tightened?

In order to analyse the performance achievable with the Rel-14 HST requirements we have performed a number of simulations in a fully dynamic system simulator. In the simulator the UEs are moving according to the assumptions in Appendix A, and the goal is to analyse if it is possible with current the Rel-14 HST minimum performance requirements, to ensure robust mobility in terms of handover and whether the existing minimum requirements for deactivated SCell are sufficient.

The performance metrics used in the simulation are as follows:

- HO failure ratio: number of failed connection attempts (HOF+RLF) over the total number of connection attempts (HO+ HOF+RLF)

- ping-pong HO ratio: the number of HOs happening back and forth between sectors of the same eNB or between eNBs within 1 second

- HO per UE per second ratio: the number of HOs per UE per second

Figure 1: HO failure ratio (main beam direction at ±11.3 degrees)

Figure 2: Ping-pong HO ratio (main beam direction at ±11.3 degrees)

Figure 3: HO per UE per second ratio (main beam direction at ±11.3 degrees)

Figure 4: UE HO positions vs DRX for the case of A3 threshold 3 dB (main beam direction at ±11.3 degrees)

The simulation results with Rel-14 HST assumption for cell detection and measurement are shown with different DRX cycles, A3 threshold options and background traffic load. Figures 1-3 are for HO failure ratio, ping-pong HO ratio and HO per UE per second ratio, respectively, for the main beam direction at ±11.3 degrees.

From Figure 1, it can be seen that the HO performance degrades to an unacceptable level with DRX greater than 160 ms for 75% loading and all the considered A3 threshold options.

Another interesting observation is in Figure 2, where we can find there are ping-pongs in the HST scenario, which is worse for small DRX values and low A3 thresholds. This is also shown in Figure 3. One reason is that in HST scenario the HO related parameters (Hysteresis, Threshold and TTT) are usually set aggressively, which will make the filtered RSRP for serving and neighbouring cells become closer to each other and less reliable in the middle between the sectors.

Observation 1: HO failure rates increase with long DRX cycle length (as expected)

Observation 2: A3 threshold has little or no effect on HO failure rates but is effective in reducing ping-pong HO behaviour and the number of HO per UE per second

Figure 4 shows the position of UE when HO occurred for different DRX values for A3 threshold of 3 dB, assuming 50% UE traffic load. As observed in the figure, without DRX, the UE HO position occurs in the area between adjacent sectors of eNodeBs. With 160 ms DRX, the UE HO is slightly delayed as the HO position is shifted to the direction of train mobility. This is due to the late measurement reports as a result of longer measurement intervals. For 640 ms DRX, the UE HO is completely failed.

Figures 5 and 6 show the HO failure ratio and HO per UE per second ratio for the main beam direction pointing at ±45 degrees. As can be observed, there is no significant performance difference the main beam direction pointing at ±45 degrees and ±11.3 degrees.

Figure 5: HO failure ratio (main beam direction at ±45 degrees)

Figure 6: Ping-pong HO ratio (main beam direction at ±45 degrees)

Observation 3: mobility performance is not significantly affected by the direction of the main beam as long as the beam is directed towards railway tracks

Based on our simulation results and observations, we can make the following conclusions related to the Rel-14 HST RRM requirements when increasing the UE speed to 500 km/h:

- in general, using DRX cycles of 160 ms or less will ensure robust mobility without noticeable performance degradation

  - One exception is for the full traffic load case

- when DRX cycle = 640 ms is used, it is not possible to ensure robust mobility even at 50% traffic load

- at 75% load, robust mobility suffers even further even at DRX cycles of 320 ms

- at full load, only non-DRX (or up to the assumed measure interval of 40 ms) can guarantee robust mobility (even 80 ms leads to about 5% HO failure rates)

- to avoid excessive ping-pong effects (even for short DRX cycles), HO threshold should be used (e.g., at least 2 dB)

From the simulations it can also be observed, that for measurement of deactivated SCell using a measCycleScell = 160ms will enable the UE to perform timely detection and measurements of new intra-frequency cell on the SCell carrier. This is assuming that the HST flag – when configured – applies for all serving cells including deactivated SCells. Only at 100% load we see that measurement cycle of 160ms is not working (DRX=measCycleSCell=160ms). Based on the results we observe:

Observation 4: Deactivated SCell measurement cycle of 160ms is sufficient for timely SCell management.

All of the above observation are mostly observations related to configuration parameters. I.e. we observe that assuming Rel-14 HST minimum requirements, the network can configure the UE such that robust PCell mobility is ensured also with 500kmh.

Additionally, we observe that using the shortest measurement cycle of 160ms for deactivated SCell in a CA configuration, it is possible ensue timely detection, measurements and reporting of SCell’s. This is assuming that Rel-14 HST flag applies also for activated and deactivated SCell’s.

Proposal 1: Rel-14 HST flag shall apply also for activated and deactivated SCell’s – i.e. all serving cells.

Assuming Proposal 1 we additionally propose:

Proposal 2: No change to existing Rel-14 HST RRM requirements.

Proposal 3: No change to existing measCycleScell cycles for deactivated SCell measurements.

# 3	Conclusions

The document has provided further system simulation results for Rel-16 HST scenario at 500 km/h for connected mode. Based on our simulation results and observations, we can make the following conclusions and proposals:

- in general, using DRX cycles of 160 ms or less will ensure robust mobility without noticeable performance degradation

  - One exception is for the full traffic load case

- when DRX cycle = 640 ms is used, it is not possible to ensure robust mobility even at 50% traffic load

- at 75% load, robust mobility suffers even further even at DRX cycles of 320 ms

- at full load, only non-DRX (or up to the assumed measure interval of 40 ms) can guarantee robust mobility (even 80 ms leads to about 5% HO failure rates)

- to avoid excessive ping-pong effects (even for short DRX cycles), HO threshold should be used (e.g., at least 2 dB)

Proposal 1: Rel-14 HST flag shall apply also for activated and deactivated SCell’s – i.e. all serving cells.

Proposal 2: No change to existing Rel-14 HST RRM requirements.

Proposal 3: No change to existing measCycleScell cycles for deactivated SCell measurements.

# References

- R4-1904804, Way forward on RRM enhancement for HST, Qualcomm

- R4-1904383, Initial system simulation results for Rel-16 HST, Nokia, Nokia Shanghai Bell

# A

Table 1: System level simulation assumptions

| Parameters | Value | Comments |
| --- | --- | --- |
| Cell layout | 6 cells | 3 sites; 2 cells per site 2 wrap-around areas from the left and from the right |
| Network synchronization | Synchronized |  |
| Inter-site distance (Ds) | 1000 m |  |
| Minimum distance between eNB and railroad track (Dmin) | 50 m |  |
| Carrier frequency | 2.0 GHz |  |
| Antenna deployment | 11 degrees horizontal 6 degrees down-tilt |  |
| Antenna configuration | 3D antenna, 1x2 SIMO in DL Isotropic, 1x2 SIMO in UL |  |
| Receiver types | MRC in DL MRC in UL |  |
| Antenna gain | BS: 17dBi UE: 0dBi |  |
| Antenna height | BS: 35m UE: 1.5m |  |
| Main beam angle with respect to the x axis | ±5.7, ±11, and ±45 degrees | Bi-directional coverage consists of two main beams pointing in opposite direction on railway tracks |
| User speed | 500 km/h (138.9 m/s) |  |
| UE distribution | All UEs are generated consecutively in the left most point with the scenario in 1.44 seconds (500 km/h, 30 UE/s). | This is analogical of generating UEs in the train of 200m length. |
| Number of UEs | 63 |  |
| User mobility model | Constant speed, wrap around |  |
| Distance-dependent path loss | Simple logarithmic | 128.1+37.6log10(d) dB |
| Penetration loss | 20 dB |  |
| Slow fading/Shadowing | Not used |  |
| Fast fading | Not used |  |
| Traffic type | No data traffic in UL and DL | One packet is sent only in the beginning and in the end of simulations |
| Background load | Fixed as 0, 25%, 50%, 75%, 100% |  |
| Cell detection delay | PSS/SSS detection delay shall be within 5* DRX cycles when SINR>=-6dB |  |
| L1 measurement period | Measurement period shall be within 3* DRX cycles when SINR>=-6dB |  |
| T310 | 1s |  |
| T312 | Disabled |  |
| HO | A3-based |  |
| A3 event parameters | Hysteresis: 0 dB Threshold: 0, 3 dB Time To Trigger: 0 ms |  |
| HO Preparation delay | Constant delay of 50ms |  |
| L3 filtering | OFF |  |
| DRX | Long cycle values: OFF, 40ms, 80ms, 160ms, 320ms, 640ms | Other parameters: DRX.inactivityTimeInTTIs: 10 DRX.onDurationInTTIs: 5 |
| RACH parameters | Constant delay of 40ms |  |
| RRC measurement quantity | RSRP |  |
| RSRP Measurement Error Std. | 2dB | Random error of +-2dB with a normal distribution is added to the RSRP measurement of each cell. |
| Threshold Qout,SNR | -8 dB |  |
| Threshold Qin,SNR | -6 dB |  |
| RRC messages sent over the air | HO command, HO complete Measurement report Re-Establishment (request, response, complete) |  |
| RLM evaluation period | 10 DRX cycles | Per requirements in 36.133 |
| Simulation time | 10 min |  |