3GPP TSG-RAN WG4 Meeting #91		R4-1906943

2019

Agenda item:	8.4.1

Source:	Qualcomm Inc.

Title:	On Band Definition of NR V2X

Document for:	Discussion

# Introduction

In the WID [1], it has been requested to study the coexistence with other V2X technologies in ITS spectrum in 5.9 GHz. Also, 5GAA has identified 40 MHz in B47 for advanced vehicular services in their FCC filing [2]. This band definition for NR V2X is in line with these two guidelines.

# Discussion

As per the objectives in the WID [1] of NR V2X Work Item

- UE Tx and Rx RF requirement [RAN4]

  - This requirement should ensure

    - coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum

  - coexistence with other V2X technologies in the adjacent channel in ITS spectrum in 5.9 GHz, without assuming that 5.9 GHz spectrum will be universally available nor that it will be universally available in sufficient quantity to support NR V2X advanced use cases

It is clearly understood that the scope of Coexistence study only includes license spectrum and ITS spectrum in 5.9GHz. 5.9 GHz is the common spectrum used for vehicular communication in Europe, US and China. In a recent filing with FCC [2], 5GAA is also proposing to allocate 40MHz spectrum at this frequency range for 5G V2X communication.

For these reasons, we feel that it is important that RAN4 define a band at ITS spectrum in 5.9GHz frequency for NR V2X Communication Service. Indeed, the same frequency range has been defined as band 47 for LTE V2X Communication Service. So, it is a natural extension to define a re-farmed NR band n47 to use for NR V2X Communication Service. The Band definition should be as follow

Table 1: NR V2X operating bands in FR1

| NR operating band | Sidelink (SL) Transmission operating band F_low – F_high | Sidelink (SL) Reception operating band F_low – F_high | Duplex Mode | Interface |
| --- | --- | --- | --- | --- |
| n47 | 5855 MHz – 5925 MHz | 5855 MHz – 5925 MHz | TDD | PC5 |

Proposal 1: RAN4 to define re-farmed band n47 for NR V2X Communication Service as in Table 1

# Conclusions

Proposal 1: RAN4 to define re-farmed band n47 for NR V2X Communication Service as in Table 1

Table 1: NR V2X operating bands in FR1

| NR operating band | Sidelink (SL) Transmission operating band F_low – F_high | Sidelink (SL) Reception operating band F_low – F_high | Duplex Mode | Interface |
| --- | --- | --- | --- | --- |
| n47 | 5855 MHz – 5925 MHz | 5855 MHz – 5925 MHz | TDD | PC5 |

# References

- RP-190766 New WID on 5G V2X with NR sidelink. LG Electronics, Huawei

- 5GAA Petition for Waiver to Allow Deployment of Cellular Vehicle-to-Everything (C-V2X) Technology in the 5.9 GHz Band; GN Docket No. 18-357. https://ecfsapi.fcc.gov/file/104030451515194/5GAA%20Band%20Plan%20Ex%20Parte%20-%20FINAL.pdf