3GPP TSG-RAN WG4 Meeting #91 	R4-1907070

# Reno, Nevada, USA, 13th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction in SDL bands requirements applicability (sections 3.5.2, 3.6.5, and B.2.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Incomplete requirements for SDL bands |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: Adding the SDL group in the conditions for UE transmit timing requirements Change #2: Correcting the references in the applicability for SDL bands Change #3: Adding a reference in the SDL bands grouping |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete requirements for SDL bands |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.6, 3.6.5, 3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--- start of change 1 ---

## B.2.6	Conditions for UE transmit timing

### B.2.6.1	Conditions for SSB based UE transmit timing

This section defines the following conditions for UE transmit timing adjustment performed based on SSBs: SSB_RP and SSB Ês/Iot and applicable for a corresponding operating band.

The conditions are defined in Table B.2.6.1-1 for FR1 SSB.

Table B.2.6.1-1: Conditions for UE transmit timing in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB=15 kHz | SCSSSB=30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | [-3] |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

The conditions are defined in Table B.2.6.1-2 for FR2 SSB.

Table B.2.6.1-2: Conditions for UE transmit timing in FR2

| Parameter | NR operating band groupsNote1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB=120 kHz | SCSSSB=240 kHz |  |
| Conditions | NR_TDD_FR2_A | TBD | TBD | TBD |
|  | NR_TDD_FR2_B | TBD | TBD |  |
|  | NR_TDD_FR2_F | TBD | TBD |  |
|  | NR_TDD_FR2_G | TBD | TBD |  |
|  | NR_TDD_FR2_T | TBD | TBD |  |
|  | NR_TDD_FR2_Y | TBD | TBD |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.3. |  |  |  |  |

### B.2.6.2	Conditions for CSI-RS based UE transmit timing

This section defines the following conditions for UE transmit timing adjustment based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot and applicable for a corresponding operating band.

The conditions are defined in Table B.2.6.2-1 for FR1 CSI-RS.

Table B.2.6.2-2: Conditions for UE transmit timing in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS RSRP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS=15 kHz | SCSCSI-RS=30 kHz |  | SCSCSI-RS=60 kHz |  |  |  |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 |  | -121 |  | [-3] |  |  |
|  | NR_FDD_FR1_B | -126.5 |  | -123.5 |  | -120.5 |  |  |  |
|  | NR_TDD_FR1_C | -126 | -123 |  | -120 |  |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  | -119.5 |  |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  | -119 |  |  |  |  |
|  | NR_FDD_FR1_G | -124 | -121 |  | -118 |  |  |  |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  | -117.5 |  |  |  |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

The conditions are defined in Table B.2.6.2-2 for FR2 CSI-RS.

Table B.2.6.2-2: Conditions for UE transmit timing in FR2

| Parameter | NR operating band groupsNote1 | Minimum CSI-RS_RP |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  | dB |
|  |  | SCSCSI-RS=60 kHz | SCSCSI-RS=120 kHz |  |
| Conditions | NR_TDD_FR2_A | TBD | TBD | TBD |
|  | NR_TDD_FR2_B | TBD | TBD |  |
|  | NR_TDD_FR2_F | TBD | TBD |  |
|  | NR_TDD_FR2_G | TBD | TBD |  |
|  | NR_TDD_FR2_T | TBD | TBD |  |
|  | NR_TDD_FR2_Y | TBD | TBD |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.3. |  |  |  |  |

--- end of change 1 ---

--- start of change 2 ---

### 3.6.5	Applicability for SDL bands

The measurements accuracy requirements for SDL bands in this version of specification in Section 10.1 shall apply for NR intra-frequency measurements on SCC (SS-RSRP, SS-RSRQ, SS-SINR, and L1-RSRP) and inter-frequency measurements (SS-RSRP, SS-RSRQ, and SS-SINR).

--- end of change 2 ---

## --- start of change 3 ---

### 3.5.2	NR operating bands in FR1

NR frequency bands grouping for FR1 is specified in Table 3.5.2-1.

Table 3.5.2-1: NR frequency band groups for FR1

| Group | NR FDD |  | NR TDD |  | NR SDL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation |  | Operating bands |
| A | NR_FDD_FR1_A | n1, n70, n744 | NR_TDD_FR1_A | n34, n38, n39, n40, n50, n51 | NR_SDL_FR1_A |  | n75, n76 |
| B | NR_FDD_FR1_B | n65, n66, n743 | NR_TDD_FR1_B | - | NR_SDL_FR1_B | - |  |
| C | NR_FDD_FR1_C | - | NR_TDD_FR1_C | n771, n78, n79 | NR_SDL_FR1_C | - |  |
| D | NR_FDD_FR1_D | n28 | NR_TDD_FR1_D | n772 | NR_SDL_FR1_D | - |  |
| E | NR_FDD_FR1_E | n2, n5, n7 | NR_TDD_FR1_E | n41 | NR_SDL_FR1_E | - |  |
| F | NR_FDD_FR1_F | - | NR_TDD_FR1_F | - | NR_SDL_FR1_F | - |  |
| G | NR_FDD_FR1_G | n3, n8, n12, n20, n71 | NR_TDD_FR1_G | - | NR_SDL_FR1_G | - |  |
| H | NR_FDD_FR1_H | n25 | NR_TDD_FR1_H | - | NR_SDL_FR1_H | - |  |
| NOTE 1: Except 3.8 GHz to 4.2 GHz. NOTE 2: Only 3.8 GHz to 4.2 GHz. NOTE 3: Except 1475.9 MHz to 1510.9 MHz. NOTE 4: Only when the band is confined in 1475.9 MHz to 1510.9 MHz. NOTE 5: These bands are used only in NR carrier aggregation with other NR bands according to NR CA band combinations specified in TS 38.101-1 [18] and TS 38.101-3 [20]. |  |  |  |  |  |  |  |

--- end of change 3 ---