3GPP TSG-RAN WG4 Meeting #91 	R4-1907078

# Reno, Nevada, USA, 13th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of inter-RAT measurement requirements for late-drop deployments (section 9.4) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Inter-RAT measurement requirements are currently specified for SA deployment only |  |  |  |  |  |  |  |  |
| Summary of change: |  | The applicability of inter-RAT measurement requirements is extended to cover also for NR-DC and NE-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-RAT measurement requirements in 38.133 are specified for SA deployment only |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--- start of change 1 ---

## 9.4	Inter-RAT measurements

### 9.4.1	Introduction

The requirements in this section are specified for NR−E-UTRAN FDD and NR−E-UTRAN TDD measurements and are applicable without an explicit E-UTRAN neighbour cell list containing physical layer cell identities, for a UE:

-	in RRC_CONNECTED state, and

-	configured with SA or NR-DC operation mode or configured in NE-DC operation mode by PCell with NRE-UTRAN FDD or TDD measurement (RSRP, RSRQ, RS-SINR, RSTD, or E-CID) on E-UTRA non-serving frequency carrier, and

-	configured with an appropriate measurement gap pattern according to Table 9.1.2-3.

When the UE is in NE-DC operation mode and an NRE-UTRAN FDD or TDD measurement (RSRP, RSRQ, RS-SINR, RSTD, or E-CID) configured by NR PCell is on a E-UTRA serving frequency carrier, then the corresponding E-UTRA intra-frequency measurements requirements specified in TS 36.133 [15, section 8.19] shall apply.

Parameter TInter1 used in inter-RAT requirements in Section 9.4 is specified in Table 9.4.1-1.

Table 9.4.1-1: Minimum available time for inter-RAT measurements

| Gap Pattern Id | MeasurementGap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) | Minimum available time for inter-frequency and inter-RAT measurements during 480ms period (Tinter1, ms) |
| --- | --- | --- | --- |
| 0 | 6 | 40 | 60 |
| 1 | 6 | 80 | 30 |
| 2 | 3 | 40 | 24Note 1 |
| 3 | 3 | 80 | 12Note 1 |
| 4 | 6 | 20 | 120 Note 1 |
| 6 | 4 | 20 | 72 Note 1,3,6 |
| 7 | 4 | 40 | 36 Note 1,4,6 |
| 8 | 4 | 80 | 18Note 1,5,6 |
| 10 | 3 | 20 | 48 Note 1 |
| NOTE 1: When determing UE requirements using Tinter1 for gap pattern IDs 2 , 3, 4, 6, 7, 8, 10, Tinter1 = 60 for gap pattern IDs 2, 4, 6, 7, 10, and Tinter1 = 30 for gap pattern IDs 3 and 8 shall be used. NOTE 2: Measurement gaps pattern configurations applicability is as specified in Table 9.1.2-1. NOTE 3: When this gap pattern is used, the Tinter for E-UTRA inter-frequency measurements is 48 ms corresponding to the first 3 ms of the 4 ms gap NOTE 4: When this gap pattern is used, the Tinter for E-UTRA inter-frequency measurements is 24 ms corresponding to the first 3 ms of the 4 ms gap NOTE 5: When this gap pattern is used, the Tinter for E-UTRA inter-frequency measurements is 12 ms corresponding to the first 3 ms of the 4 ms gap NOTE 6: This gap pattern is applicable for E-UTRA inter-frequency measurements only if gap based NR measurements are also configured. |  |  |  |

Editor’s note: a note to be added in Table 9.4.1-1 on that measurement gap patterns #2 #3, #6, #7, #8, #10 are supported only by the UEs which have a corresponding capability once RAN2 specifies the capability.

A UE configured with gap pattern Id 2, 3 or 10, shall be able to detect a target cell if the E-UTRA subframe #0 or #5 of the target E-UTRAN cell begins no earlier than 500 s from the start of the measurement gap and if the E-UTRA subframe #0 or #5 of the target E-UTRAN cell ends no later than 500 s before the end of the measurement gap in case of FDD, and no later than 750 s before the end of measurement gap in case of TDD.

A UE configured with gap pattern Id 6, 7 or 8 shall be able to detect a target cell if the E-UTRA subframe #0 or #5 of the target E-UTRAN cell begins no earlier than 500 s from the start of the measurement gap and if the E-UTRA subframe #0 or #5 of the target E-UTRAN cell ends no later than 1500 s before the end of the measurement gap in case of FDD, and no later than 1750 s before the end of measurement gap in case of TDD.

### 9.4.2	NR − E-UTRAN FDD measurements

--- unchanged sections ---

### 9.4.3	NR − E-UTRAN TDD measurements

--- unchanged sections ---

### 9.4.4	Inter-RAT RSTD measurements

#### 9.4.4.1	NR − E-UTRAN FDD RSTD measurements

--- unchanged sections ---

#### 9.4.4.2	NR − E-UTRAN TDD RSTD measurements

--- unchanged sections ---

### 9.4.5	Inter-RAT E-CID measurements

--- unchanged sections ---

--- end of change 1 ---