3GPP TSG-RAN WG4 Meeting #91	R4-1907157

Reno, US, 12th – 17th May 2019

Agenda item:	6.12.2.4	PRACH	[NR_newRAT-Perf]

Source: 		NTT DOCOMO, INC.

Title: 	NR PRACH for high speed in Rel.15

Document for:	Approval

- Introduction

In the past meetings, the following agreements were reached for BS performance under HST scenarios [1, 2]. In this contribution, we provide parameters for NR PRACH restricted set type A and B.

RAN#81 [1] The UE and BS demodulation performance requirements under HST scenarios are treated in TEI15 after December 2018. RAN4#90bis [2] - Overview - Release to define NR BS HST requirements - Option 1: Release 15 - Option 2: Release 16 with starting point of 350km/h on the same Band 1 as LTE - One of the above options will be decided based on the outcome of the evaluation. - The performance requirements for HST are optional, and the applicability depends on the declaration. - Assumptions for the following HST evaluations are for NR Rel-15 HST for RAN4#91 meeting. Bases on the evaluation results, companies can share their views on the HST requirements definition. - Way forward - Companies are encourage to evaluate PUSCH and PRACH performance under HST for RAN4#91. - If no technical issues are identified and no concern is raised on the evaluation assumptions, Draft CRs implementing framework of PUSCH for HST and PRACH for restricted set type A or/and B, for TS 38.104, TS 38.141-1 and TS 38.141-2, might be provided in RAN4 #91.

2.	Discussion

In the current RAN4 specification [3], PRACH requirements for only normal mode are defined and support frequency offset up to 400Hz.

TS 38.104 [3] Table 8.4.2.2-1: PRACH missed detection requirements for Normal Mode, 1.25 kHz SCS Number of TX antennas; Number of RX antennas; Propagation conditions and correlation matrix (Annex [TBD]); Frequency offset; SNR (dB). Burst format 0. 1; 2; AWGN; 0; [-15.1]. [TDLC300-100]; 400 Hz; [-6.7]. 4; AWGN; 0; [-17.3]. [TDLC300-100]; 400 Hz; [-12.2]. 8; AWGN; 0; [-19.6]. [TDLC300-100]; 400 Hz; [-16.3]. TS 38.211 [4] Table 6.3.3.1-1: PRACH preamble formats for and kHz. Format; Support for restricted sets. 0; 839; 1.25 kHz; Type A, Type B. 1; 839; 1.25 kHz; Type A, Type B. 2; 839; 1.25 kHz; Type A, Type B. 3; 839; 5 kHz; Type A, Type B.

As mentioned in [5], for support HST, test cases for restricted set type A and B with larger frequency offset corresponding to the assumed maximum Doppler shift should be defined. In order to reduce the work, the test configurations should be reused from E-UTRA as much as possible. Based on above, it is proposed to introduce PRACH tests for format 0 with restricted set type A and B.

Proposal 1: Introduce PRACH performance requirements for format 0 with restricted set type A and B.

Table 1: Proposed parameters for NR high speed train test

| Parameter | value |  |
| --- | --- | --- |
|  | Restricted set type A | Restricted set type B |
| Number of Tx | 1 | 1 |
| Number of Rx | 2, 4, 8 | 2, 4, 8 |
| Preamble format | Format 0 | Format 0 |
| Frequency offset / Propagation condition | 0Hz: AWGN 400Hz: TDLC300-100 625Hz: AWGN 1340Hz: AWGN | 0Hz: AWGN 400Hz: TDLC300-100 625Hz: AWGN [2000Hz]: AWGN |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Testing metric | False alarm probability: 0.1% missed detection: 99% | False alarm probability: 0.1% missed detection: 99% |

For introduction of NR HST, we provide draft CRs to capture in TS 38.104, TS 38.141-1 and TS 38.141-2.

Proposal 2: Agree on draft CRs to introduce PRACH format 0 with restricted set type A and B.

3.	Conclusion

In this contribution, we provide our views on PRACH for HST. The following proposals are obtained.

Proposal 1: Introduce PRACH performance requirements for format 0 with restricted set type A and B.

Table 1: Proposed parameters for NR high speed train test

| Parameter | value |  |
| --- | --- | --- |
|  | Restricted set type A | Restricted set type B |
| Number of Tx | 1 | 1 |
| Number of Rx | 2, 4, 8 | 2, 4, 8 |
| Preamble format | Format 0 | Format 0 |
| Frequency offset / Propagation condition | 0Hz: AWGN 400Hz: TDLC300-100 625Hz: AWGN 1340Hz: AWGN | 0Hz: AWGN 400Hz: TDLC300-100 625Hz: AWGN [2000Hz]: AWGN |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Testing metric | False alarm probability: 0.1% missed detection: 99% | False alarm probability: 0.1% missed detection: 99% |

Proposal 2: Agree on draft CRs to introduce PRACH format 0 with restricted set type A and B.

References

- RP-182149, “Way forward on RAN4 work for Core and Perf. part for NR”, RAN4 Chairman, RAN #81, September 2018

- R4-1904720, “WF on high speed related requirements for NR BS demodulation in Rel.15”, NTT DOCOMO, INC., China Telecom, RAN4 #90bis, April 2019

- “3GPP TS 38.104: NR; Base Station (BS) radio transmission and reception”, 3GPP

- “3GPP TS 38.211: NR; Physical channels and modulation”, 3GPP

- R4-1903572, “NR PRACH for high speed in Rel.15”, NTT DOCOMO, INC., RAN4 #90bis, April 2019