3GPP TSG-RAN WG4 Meeting #91	R4-1907158

Reno, US, 12th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | <CR#> | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.104: Introduction of PRACH performance requirements for HST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-5-3 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to introduce BS performance requirements for HST to BS specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduction of PRACH performance requirements for restricted set type A and B. The changes are highlighted in yelllow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR does not support HST |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.141-1, TS38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------------Start of text proposal-------------

8.4.2.2	Minimum requirements

The probability of detection shall be equal to or exceed 99% for the SNR levels listed in Tables 8.4.2.2-1 to 8.4.2.2-3.

The requirements for high speed mode restricted set type A (table 8.4.2.2-4) and high speed mode restricted set type B (table 8.4.2.2-5) are only valid for the base stations supporting high speed mode restricted set A and restricted set type B respectively.

Table 8.4.2.2-1: PRACH missed detection requirements for Normal Mode, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | AWGN | 0 | [-14.5] |
|  |  | TDLC300-100 Low | 400 Hz | [-6.6] |
|  | 4 | AWGN | 0 | [-16.7] |
|  |  | TDLC300-100 Low | 400 Hz | [-11.9] |
|  | 8 | AWGN | 0 | [-18.9] |
|  |  | TDLC300-100 Low | 400 Hz | [-15.8] |

Table 8.4.2.2-2: PRACH missed detection requirements for Normal Mode, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | [-9.3] | [-12.6] | [-14.3] | [-16.9] | [-6.3] | [-12.5] |
|  |  | TDLC300-100 Low | 400 Hz | [-2.2] | [-4.8] | [-6.8] | [-8.8] | [0.7] | [-4.9] |
|  | 4 | AWGN | 0 | [-11.7] | [-14.6] | [-16.4] | [-19.1] | [-8.8] | [-14.5] |
|  |  | TDLC300-100 Low | 400 Hz | [-7.2] | [-10.2] | [-11.6] | [-13.8] | [-4.3] | [-10.2] |
|  | 8 | AWGN | 0 | [-14.0] | [-17.1] | [-18.7] | [-21.2] | [-11.1] | [-17.1] |
|  |  | TDLC300-100 Low | 400 Hz | [-11.1] | [-14.0] | [-15.4] | [-17.3] | [-8.2] | [-14.0] |

Table 8.4.2.2-3: PRACH missed detection requirements for Normal Mode, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | [-9.1] | [-12.0] | [-13.9] | [-16.6] | [-6.1] | [-12.0] |
|  |  | TDLC300-100 Low | 400 Hz | [-2.8] | [-5.8] | [-7.5] | [-10.0] | [0.0] | [-5.7] |
|  | 4 | AWGN | 0 | [-11.6] | [-14.5] | [-16.3] | [-18.9] | [-8.6] | [-14.3] |
|  |  | TDLC300-100 Low | 400 Hz | [-7.2] | [-10.4] | [-12.0] | [-14.5] | [-4.4] | [-10.5] |
|  | 8 | AWGN | 0 | [-13.8] | [-16.9] | [-18.5] | [-21.0] | [-11.0] | [-16.6] |
|  |  | TDLC300-100 Low | 400 Hz | [-10.7] | [-13.8] | [-15.4] | [-17.6] | [-8.0] | [-13.9] |

Table 8.4.2.2-4: PRACH missed detection requirements for High speed Mode restricted set type A, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | AWGN | 0 | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |
|  | 4 | AWGN | 0 | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |
|  | 8 | AWGN | 0 | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |

Table 8.4.2.2-5: PRACH missed detection requirements for High speed Mode restricted set type B, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | AWGN | 0 | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | [2000 Hz] | TBD |
|  | 4 | AWGN | 0 | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | [2000 Hz] | TBD |
|  | 8 | AWGN | 0 | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | [2000 Hz] | TBD |

--------------End of text proposal-------------