3GPP TSG RAN WG4 Meeting #91	R4-1907223

Reno, Nevada, US, May 13 – 17, 2019

Agenda Item:	8.2.3

Source: 	Ericsson

Title:	Simulation results for SRS-RSRP measurements

Document for:	Discussion

# Introduction

At RAN4#90bis simulation assumptions for SRS-RSRP measurements were agreed, [1]. In this contribution we provide initial simulation results based on the agreed simulation assumptions.

# Simulation results

## Overview

Simulation results for FR1 with UE1 and UE2 having same timing and same cyclic shift (Option 1) are provided in:

Table 1: Simulation results for FR1, AWGN, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 20 RBs

Table 2: Simulation results for FR1, TDL-A10, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 20 RBs

Table 3: Simulation results for FR1, TDL-C100, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 20 RBs

Table 4: Simulation results for FR1, AWGN, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 40 RBs

Table 5: Simulation results for FR1, TDL-A10, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 40 RBs

Table 6: Simulation results for FR1, TDL-C100, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 40 RBs

Simulation results for FR2 with UE1 and UE2 having same timing and same cyclic shift (Option 1) are provided in:

Table 7: Simulation results for FR2, AWGN, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 20 RBs

Table 8: Simulation results for FR2, TDL-A75, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 20 RBs

Table 9: Simulation results for FR2, AWGN, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 40 RBs

Table 10: Simulation results for FR2, TDL-A75, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 40 RBs

Simulation results for FR1 with UE1 and UE2 having a relative timing offset and different cyclic shifts (Option 2) are provided in:

Table 11: Simulation results for FR1, AWGN, UE1 and UE2 different cyclic shifts: 0 and 5, respectively, timing offset UE1 – UE2 2 µs, measurement bandwidth 20 RBs

Table 12: Simulation results for FR1, AWGN, UE1 and UE2 different cyclic shifts: 0 and 5, respectively, timing offset UE1 – UE2 2 µs, measurement bandwidth 40 RBs

Simulation results for FR2 with UE1 and UE2 having a relative timing offset and different cyclic shifts (Option 2) are provided in:

Table 13: Simulation results for FR2, AWGN, UE1 and UE2 different cyclic shifts: 0 and 5, respectively, timing offset UE1 – UE2 1 µs, measurement bandwidth 20 RBs

Table 14: Simulation results for FR2, AWGN, UE1 and UE2 different cyclic shifts: 0 and 5, respectively, timing offset UE1 – UE2 1 µs, measurement bandwidth 40 RBs

## Simulation results for FR1, UE1 and UE2 same timing and cyclic shift

### Measurement bandwidth 20 RBs

Table 1: Simulation results for FR1, AWGN, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 20 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 15 | 1 | 1 | 0 | -18.7 | -0.01 | 3.01 |
|  |  |  | 1 | -19.25 | -0.1 | 2.99 |
|  |  |  | 2 | -21.45 | -0.04 | 3 |
|  |  |  | 10 | -18.01 | -0.39 | 2.66 |
|  |  | 3 | 0 | -5.03 | -0.01 | 2.34 |
|  |  |  | 1 | -5.54 | -0.24 | 2.37 |
|  |  |  | 2 | -5.46 | -0.07 | 2.27 |
|  |  |  | 10 | -5.44 | -0.38 | 1.97 |
|  |  | 5 | 0 | -2.98 | 0.08 | 1.88 |
|  |  |  | 1 | -3.38 | -0.09 | 1.76 |
|  |  |  | 2 | -3.52 | 0.02 | 1.81 |
|  |  |  | 10 | -3.47 | -0.39 | 1.47 |
|  | -1 | 1 | 0 | -19.44 | -0.06 | 3 |
|  |  |  | 1 | -19.07 | -0.13 | 3.01 |
|  |  |  | 2 | -21.2 | -0.08 | 3.01 |
|  |  |  | 10 | -19.48 | -0.35 | 2.65 |
|  |  | 3 | 0 | -5.04 | -0.09 | 2.26 |
|  |  |  | 1 | -5.08 | -0.04 | 2.23 |
|  |  |  | 2 | -5.18 | -0.2 | 2.24 |
|  |  |  | 10 | -5.67 | -0.44 | 1.91 |
|  |  | 5 | 0 | -3.36 | 0.01 | 1.67 |
|  |  |  | 1 | -3.49 | -0.02 | 1.78 |
|  |  |  | 2 | -3.04 | -0.15 | 1.93 |
|  |  |  | 10 | -3.54 | -0.28 | 1.37 |
|  | -3 | 1 | 0 | -21.07 | 0.14 | 3.05 |
|  |  |  | 1 | -21.01 | -0.16 | 3.04 |
|  |  |  | 2 | -19.36 | 0.04 | 3 |
|  |  |  | 10 | -19.91 | -0.17 | 2.71 |
|  |  | 3 | 0 | -4.91 | 0.1 | 2.34 |
|  |  |  | 1 | -5.12 | -0.08 | 2.29 |
|  |  |  | 2 | -4.68 | -0.05 | 2.15 |
|  |  |  | 10 | -6.04 | -0.23 | 1.93 |
|  |  | 5 | 0 | -3.14 | 0.07 | 1.89 |
|  |  |  | 1 | -3.7 | 0.04 | 1.68 |
|  |  |  | 2 | -3.25 | -0.02 | 1.88 |
|  |  |  | 10 | -3.56 | -0.25 | 1.57 |
| 30 | 1 | 1 | 0 | -19.81 | 0.04 | 3.02 |
|  |  |  | 1 | -19.61 | 0.22 | 3.01 |
|  |  |  | 2 | -20.73 | -0.34 | 3.01 |
|  |  |  | 10 | -14.27 | -0.95 | 1.95 |
|  |  | 3 | 0 | -5.47 | 0.15 | 2.26 |
|  |  |  | 1 | -4.77 | 0.13 | 2.31 |
|  |  |  | 2 | -5.66 | -0.17 | 2.35 |
|  |  |  | 10 | -5.59 | -0.95 | 1.23 |
|  |  | 5 | 0 | -3.25 | 0.04 | 1.79 |
|  |  |  | 1 | -2.76 | 0.16 | 1.78 |
|  |  |  | 2 | -3.34 | -0.08 | 1.75 |
|  |  |  | 10 | -3.82 | -1.09 | 0.81 |
|  | -1 | 1 | 0 | -21.23 | -0.09 | 3.01 |
|  |  |  | 1 | -20.6 | -0.02 | 3.01 |
|  |  |  | 2 | -21.62 | -0.32 | 3.01 |
|  |  |  | 10 | -14.13 | -0.94 | 2.02 |
|  |  | 3 | 0 | -5.41 | -0.11 | 2.21 |
|  |  |  | 1 | -5.73 | -0.03 | 2.3 |
|  |  |  | 2 | -5.33 | -0.08 | 2.26 |
|  |  |  | 10 | -5.91 | -0.92 | 1.24 |
|  |  | 5 | 0 | -3.16 | -0.06 | 1.74 |
|  |  |  | 1 | -2.87 | -0.15 | 1.87 |
|  |  |  | 2 | -3.11 | -0.21 | 1.9 |
|  |  |  | 10 | -4.36 | -0.87 | 0.83 |
|  | -3 | 1 | 0 | -19.68 | -0.15 | 3.03 |
|  |  |  | 1 | -20.17 | -0.04 | 3.03 |
|  |  |  | 2 | -21.05 | 0.05 | 3.03 |
|  |  |  | 10 | -15.49 | -1.02 | 2.02 |
|  |  | 3 | 0 | -5.15 | -0.06 | 2.43 |
|  |  |  | 1 | -5.32 | -0.1 | 2.26 |
|  |  |  | 2 | -5.03 | -0.06 | 2.35 |
|  |  |  | 10 | -6.14 | -0.99 | 1.3 |
|  |  | 5 | 0 | -3.42 | -0.03 | 1.85 |
|  |  |  | 1 | -3.21 | -0.04 | 1.79 |
|  |  |  | 2 | -2.95 | 0.02 | 1.98 |
|  |  |  | 10 | -4.07 | -0.97 | 0.92 |

Table 2: Simulation results for FR1, TDL-A10, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 20 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 15 | 1 | 1 | 0 | -7.8 | -0.6 | 3.59 |
|  |  |  | 1 | -7.78 | -0.62 | 3.64 |
|  |  |  | 2 | -7.86 | -0.6 | 3.61 |
|  |  |  | 10 | -8.1 | -1 | 3.29 |
|  |  | 3 | 0 | -7.24 | -0.61 | 3.63 |
|  |  |  | 1 | -7.27 | -0.63 | 3.61 |
|  |  |  | 2 | -7.21 | -0.66 | 3.59 |
|  |  |  | 10 | -7.46 | -0.92 | 3.24 |
|  |  | 5 | 0 | -6.19 | -0.38 | 3.29 |
|  |  |  | 1 | -6.4 | -0.42 | 3.31 |
|  |  |  | 2 | -6.36 | -0.39 | 3.3 |
|  |  |  | 10 | -6.68 | -0.76 | 3 |
|  | -1 | 1 | 0 | -7.76 | -0.57 | 3.63 |
|  |  |  | 1 | -7.81 | -0.64 | 3.59 |
|  |  |  | 2 | -7.84 | -0.67 | 3.65 |
|  |  |  | 10 | -7.98 | -0.98 | 3.33 |
|  |  | 3 | 0 | -7.29 | -0.62 | 3.64 |
|  |  |  | 1 | -7.07 | -0.64 | 3.65 |
|  |  |  | 2 | -7.15 | -0.61 | 3.6 |
|  |  |  | 10 | -7.5 | -0.92 | 3.31 |
|  |  | 5 | 0 | -6.24 | -0.42 | 3.28 |
|  |  |  | 1 | -6.19 | -0.45 | 3.36 |
|  |  |  | 2 | -6.54 | -0.4 | 3.28 |
|  |  |  | 10 | -6.73 | -0.78 | 2.99 |
|  | -3 | 1 | 0 | -7.88 | -0.62 | 3.64 |
|  |  |  | 1 | -7.76 | -0.6 | 3.66 |
|  |  |  | 2 | -7.65 | -0.6 | 3.68 |
|  |  |  | 10 | -8.24 | -1.01 | 3.28 |
|  |  | 3 | 0 | -7.31 | -0.6 | 3.65 |
|  |  |  | 1 | -7.24 | -0.62 | 3.63 |
|  |  |  | 2 | -7.12 | -0.6 | 3.62 |
|  |  |  | 10 | -7.44 | -0.92 | 3.27 |
|  |  | 5 | 0 | -6.35 | -0.41 | 3.27 |
|  |  |  | 1 | -6.38 | -0.45 | 3.29 |
|  |  |  | 2 | -6.4 | -0.45 | 3.29 |
|  |  |  | 10 | -6.66 | -0.79 | 3.01 |
| 30 | 1 | 1 | 0 | -7.2 | -0.44 | 3.73 |
|  |  |  | 1 | -7.43 | -0.44 | 3.71 |
|  |  |  | 2 | -7.05 | -0.46 | 3.7 |
|  |  |  | 10 | -7.86 | -1.45 | 2.74 |
|  |  | 3 | 0 | -7.03 | -0.45 | 3.72 |
|  |  |  | 1 | -7.03 | -0.42 | 3.65 |
|  |  |  | 2 | -7.12 | -0.46 | 3.68 |
|  |  |  | 10 | -7.97 | -1.41 | 2.77 |
|  |  | 5 | 0 | -6.54 | -0.39 | 3.48 |
|  |  |  | 1 | -6.79 | -0.36 | 3.58 |
|  |  |  | 2 | -6.49 | -0.42 | 3.48 |
|  |  |  | 10 | -7.25 | -1.44 | 2.7 |
|  | -1 | 1 | 0 | -7.08 | -0.41 | 3.79 |
|  |  |  | 1 | -7.23 | -0.43 | 3.73 |
|  |  |  | 2 | -7.32 | -0.45 | 3.69 |
|  |  |  | 10 | -7.95 | -1.4 | 2.76 |
|  |  | 3 | 0 | -7 | -0.45 | 3.73 |
|  |  |  | 1 | -7.13 | -0.45 | 3.71 |
|  |  |  | 2 | -7.13 | -0.38 | 3.61 |
|  |  |  | 10 | -7.65 | -1.37 | 2.74 |
|  |  | 5 | 0 | -6.48 | -0.39 | 3.6 |
|  |  |  | 1 | -6.76 | -0.38 | 3.44 |
|  |  |  | 2 | -6.61 | -0.34 | 3.55 |
|  |  |  | 10 | -7.4 | -1.36 | 2.72 |
|  | -3 | 1 | 0 | -7.32 | -0.42 | 3.71 |
|  |  |  | 1 | -7.2 | -0.47 | 3.67 |
|  |  |  | 2 | -7.18 | -0.45 | 3.71 |
|  |  |  | 10 | -7.76 | -1.44 | 2.81 |
|  |  | 3 | 0 | -7.07 | -0.41 | 3.65 |
|  |  |  | 1 | -7.18 | -0.46 | 3.65 |
|  |  |  | 2 | -7.07 | -0.45 | 3.72 |
|  |  |  | 10 | -7.55 | -1.43 | 2.67 |
|  |  | 5 | 0 | -6.73 | -0.4 | 3.58 |
|  |  |  | 1 | -6.69 | -0.43 | 3.54 |
|  |  |  | 2 | -6.37 | -0.42 | 3.61 |
|  |  |  | 10 | -7.4 | -1.4 | 2.69 |

Table 3: Simulation results for FR1, TDL-C100, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 20 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 15 | 1 | 1 | 0 | -6.1 | -0.8 | 3.17 |
|  |  |  | 1 | -5.99 | -0.85 | 3.17 |
|  |  |  | 2 | -5.97 | -0.84 | 3.17 |
|  |  |  | 10 | -6.45 | -1.18 | 2.82 |
|  |  | 3 | 0 | -5.59 | -0.78 | 3.05 |
|  |  |  | 1 | -5.55 | -0.83 | 3.04 |
|  |  |  | 2 | -5.66 | -0.78 | 3.01 |
|  |  |  | 10 | -6.01 | -1.13 | 2.76 |
|  |  | 5 | 0 | -4.72 | -0.81 | 2.97 |
|  |  |  | 1 | -4.77 | -0.83 | 2.96 |
|  |  |  | 2 | -4.77 | -0.75 | 2.93 |
|  |  |  | 10 | -5.33 | -1.15 | 2.53 |
|  | -1 | 1 | 0 | -6.25 | -0.83 | 3.14 |
|  |  |  | 1 | -6.15 | -0.83 | 3.13 |
|  |  |  | 2 | -6.11 | -0.88 | 3.17 |
|  |  |  | 10 | -6.61 | -1.14 | 2.74 |
|  |  | 3 | 0 | -5.69 | -0.75 | 3 |
|  |  |  | 1 | -5.72 | -0.83 | 3.03 |
|  |  |  | 2 | -5.57 | -0.76 | 3.05 |
|  |  |  | 10 | -6.1 | -1.09 | 2.68 |
|  |  | 5 | 0 | -4.99 | -0.85 | 2.89 |
|  |  |  | 1 | -5.1 | -0.84 | 2.95 |
|  |  |  | 2 | -4.96 | -0.84 | 2.9 |
|  |  |  | 10 | -5.19 | -1.21 | 2.59 |
|  | -3 | 1 | 0 | -6.19 | -0.8 | 3.17 |
|  |  |  | 1 | -6.39 | -0.88 | 3.2 |
|  |  |  | 2 | -6.19 | -0.85 | 3.15 |
|  |  |  | 10 | -6.68 | -1.21 | 2.78 |
|  |  | 3 | 0 | -5.7 | -0.78 | 3.05 |
|  |  |  | 1 | -5.84 | -0.78 | 3.05 |
|  |  |  | 2 | -5.65 | -0.85 | 3.08 |
|  |  |  | 10 | -6.16 | -1.16 | 2.68 |
|  |  | 5 | 0 | -5 | -0.8 | 2.9 |
|  |  |  | 1 | -4.98 | -0.87 | 2.95 |
|  |  |  | 2 | -4.87 | -0.85 | 2.86 |
|  |  |  | 10 | -5.43 | -1.26 | 2.52 |
| 30 | 1 | 1 | 0 | -5.05 | -0.55 | 3.16 |
|  |  |  | 1 | -5.03 | -0.57 | 3.14 |
|  |  |  | 2 | -5.11 | -0.6 | 3.14 |
|  |  |  | 10 | -6.33 | -1.69 | 2.09 |
|  |  | 3 | 0 | -4.96 | -0.58 | 3.12 |
|  |  |  | 1 | -4.88 | -0.53 | 3.11 |
|  |  |  | 2 | -5.04 | -0.55 | 3.14 |
|  |  |  | 10 | -6.2 | -1.6 | 2.02 |
|  |  | 5 | 0 | -4.95 | -0.58 | 3.07 |
|  |  |  | 1 | -4.91 | -0.56 | 3.06 |
|  |  |  | 2 | -4.96 | -0.62 | 3.12 |
|  |  |  | 10 | -6.05 | -1.67 | 2.05 |
|  | -1 | 1 | 0 | -5.23 | -0.63 | 3.15 |
|  |  |  | 1 | -5.28 | -0.64 | 3.13 |
|  |  |  | 2 | -5.32 | -0.65 | 3.21 |
|  |  |  | 10 | -6.31 | -1.66 | 2.08 |
|  |  | 3 | 0 | -5.03 | -0.56 | 3.11 |
|  |  |  | 1 | -5.17 | -0.67 | 3.14 |
|  |  |  | 2 | -5.09 | -0.65 | 3.17 |
|  |  |  | 10 | -6.14 | -1.64 | 2.07 |
|  |  | 5 | 0 | -4.92 | -0.57 | 3.02 |
|  |  |  | 1 | -5.16 | -0.65 | 3.07 |
|  |  |  | 2 | -5.06 | -0.66 | 3.09 |
|  |  |  | 10 | -5.89 | -1.7 | 2.03 |
|  | -3 | 1 | 0 | -5.4 | -0.66 | 3.16 |
|  |  |  | 1 | -5.4 | -0.64 | 3.2 |
|  |  |  | 2 | -5.36 | -0.66 | 3.21 |
|  |  |  | 10 | -6.5 | -1.68 | 2.04 |
|  |  | 3 | 0 | -5.26 | -0.65 | 3.09 |
|  |  |  | 1 | -5.27 | -0.64 | 3.16 |
|  |  |  | 2 | -5.22 | -0.64 | 3.11 |
|  |  |  | 10 | -6.28 | -1.62 | 1.98 |
|  |  | 5 | 0 | -5.05 | -0.62 | 3.07 |
|  |  |  | 1 | -5.11 | -0.54 | 3.05 |
|  |  |  | 2 | -5.21 | -0.57 | 3.11 |
|  |  |  | 10 | -5.99 | -1.74 | 1.93 |

### Measurement bandwidth 40 RBs

Table 4: Simulation results for FR1, AWGN, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 40 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 15 | 1 | 1 | 0 | -19.58 | -0.11 | 2.97 |
|  |  |  | 1 | -20.38 | 0.02 | 2.98 |
|  |  |  | 2 | -19.98 | -0.07 | 2.98 |
|  |  |  | 10 | -19.13 | -0.29 | 2.62 |
|  |  | 3 | 0 | -5.61 | -0.07 | 2.19 |
|  |  |  | 1 | -5.44 | 0.06 | 2.25 |
|  |  |  | 2 | -5.05 | -0.15 | 2.38 |
|  |  |  | 10 | -5.85 | -0.3 | 1.88 |
|  |  | 5 | 0 | -3.16 | -0.04 | 1.63 |
|  |  |  | 1 | -3.44 | 0.05 | 1.78 |
|  |  |  | 2 | -3.22 | -0.08 | 1.72 |
|  |  |  | 10 | -3.64 | -0.29 | 1.43 |
|  | -1 | 1 | 0 | -21.25 | 0.2 | 2.98 |
|  |  |  | 1 | -19.51 | -0.01 | 3 |
|  |  |  | 2 | -19.12 | -0.13 | 3 |
|  |  |  | 10 | -18.25 | -0.45 | 2.61 |
|  |  | 3 | 0 | -4.38 | 0.11 | 2.31 |
|  |  |  | 1 | -5.29 | 0.09 | 2.3 |
|  |  |  | 2 | -6.04 | -0.06 | 2.19 |
|  |  |  | 10 | -5.3 | -0.32 | 1.79 |
|  |  | 5 | 0 | -2.81 | 0.05 | 1.78 |
|  |  |  | 1 | -3.59 | 0.03 | 1.99 |
|  |  |  | 2 | -3.71 | -0.02 | 1.85 |
|  |  |  | 10 | -3.62 | -0.38 | 1.42 |
|  | -3 | 1 | 0 | -23.39 | -0.15 | 3 |
|  |  |  | 1 | -22.38 | 0.12 | 3 |
|  |  |  | 2 | -24.44 | 0.22 | 3.01 |
|  |  |  | 10 | -21.25 | -0.38 | 2.64 |
|  |  | 3 | 0 | -5.34 | -0.05 | 2.31 |
|  |  |  | 1 | -4.66 | -0.05 | 2.28 |
|  |  |  | 2 | -5.58 | 0.09 | 2.15 |
|  |  |  | 10 | -5.66 | -0.41 | 1.91 |
|  |  | 5 | 0 | -3.57 | -0.05 | 1.8 |
|  |  |  | 1 | -3.13 | -0.02 | 1.9 |
|  |  |  | 2 | -3.27 | 0.11 | 1.77 |
|  |  |  | 10 | -3.52 | -0.34 | 1.36 |
| 30 | 1 | 1 | 0 | -19.27 | -0.15 | 2.98 |
|  |  |  | 1 | -18.06 | 0.2 | 2.99 |
|  |  |  | 2 | -18.38 | -0.03 | 2.98 |
|  |  |  | 10 | -14.8 | -0.88 | 1.91 |
|  |  | 3 | 0 | -4.53 | -0.12 | 2.26 |
|  |  |  | 1 | -4.66 | 0.1 | 2.24 |
|  |  |  | 2 | -4.62 | -0.11 | 2.16 |
|  |  |  | 10 | -5.76 | -1.04 | 1.13 |
|  |  | 5 | 0 | -3.31 | -0.08 | 1.67 |
|  |  |  | 1 | -3.21 | 0.06 | 1.91 |
|  |  |  | 2 | -2.78 | -0.08 | 1.72 |
|  |  |  | 10 | -3.71 | -1.05 | 0.76 |
|  | -1 | 1 | 0 | -24.06 | -0.11 | 3.01 |
|  |  |  | 1 | -18.88 | -0.09 | 2.99 |
|  |  |  | 2 | -20.15 | 0.17 | 3 |
|  |  |  | 10 | -14.34 | -0.79 | 1.95 |
|  |  | 3 | 0 | -5.94 | -0.13 | 2.35 |
|  |  |  | 1 | -4.55 | -0.08 | 2.28 |
|  |  |  | 2 | -5.37 | 0.05 | 2.31 |
|  |  |  | 10 | -5.64 | -0.96 | 1.22 |
|  |  | 5 | 0 | -3.5 | -0.04 | 1.74 |
|  |  |  | 1 | -3.2 | -0.04 | 1.83 |
|  |  |  | 2 | -3.01 | 0.02 | 1.97 |
|  |  |  | 10 | -3.83 | -0.97 | 0.98 |
|  | -3 | 1 | 0 | -23.58 | 0 | 3.02 |
|  |  |  | 1 | -19.74 | 0.16 | 3.03 |
|  |  |  | 2 | -23.93 | -0.08 | 2.98 |
|  |  |  | 10 | -17.73 | -1.36 | 1.98 |
|  |  | 3 | 0 | -5.2 | -0.02 | 2.39 |
|  |  |  | 1 | -4.72 | 0.1 | 2.16 |
|  |  |  | 2 | -5.57 | -0.11 | 2.21 |
|  |  |  | 10 | -6.44 | -1.12 | 1.14 |
|  |  | 5 | 0 | -3.47 | 0.03 | 2.02 |
|  |  |  | 1 | -2.85 | 0.1 | 1.68 |
|  |  |  | 2 | -3.32 | -0.06 | 1.77 |
|  |  |  | 10 | -4.4 | -1.16 | 0.54 |

Table 5: Simulation results for FR1, TDL-A10, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 40 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 15 | 1 | 1 | 0 | -7.6 | -0.62 | 3.63 |
|  |  |  | 1 | -7.53 | -0.6 | 3.58 |
|  |  |  | 2 | -7.6 | -0.64 | 3.61 |
|  |  |  | 10 | -7.81 | -0.95 | 3.19 |
|  |  | 3 | 0 | -6.95 | -0.64 | 3.59 |
|  |  |  | 1 | -6.98 | -0.63 | 3.59 |
|  |  |  | 2 | -7.02 | -0.62 | 3.6 |
|  |  |  | 10 | -7.3 | -0.99 | 3.19 |
|  |  | 5 | 0 | -6.24 | -0.38 | 3.3 |
|  |  |  | 1 | -6.12 | -0.42 | 3.35 |
|  |  |  | 2 | -6.09 | -0.41 | 3.32 |
|  |  |  | 10 | -6.7 | -0.8 | 2.96 |
|  | -1 | 1 | 0 | -7.55 | -0.62 | 3.6 |
|  |  |  | 1 | -7.52 | -0.61 | 3.61 |
|  |  |  | 2 | -7.62 | -0.62 | 3.63 |
|  |  |  | 10 | -7.93 | -0.91 | 3.26 |
|  |  | 3 | 0 | -7.07 | -0.58 | 3.6 |
|  |  |  | 1 | -6.92 | -0.62 | 3.6 |
|  |  |  | 2 | -6.96 | -0.62 | 3.61 |
|  |  |  | 10 | -7.31 | -0.92 | 3.24 |
|  |  | 5 | 0 | -6.14 | -0.38 | 3.31 |
|  |  |  | 1 | -6.34 | -0.37 | 3.38 |
|  |  |  | 2 | -6.31 | -0.36 | 3.34 |
|  |  |  | 10 | -6.73 | -0.73 | 2.97 |
|  | -3 | 1 | 0 | -7.53 | -0.59 | 3.58 |
|  |  |  | 1 | -7.54 | -0.62 | 3.58 |
|  |  |  | 2 | -7.57 | -0.59 | 3.6 |
|  |  |  | 10 | -7.79 | -0.98 | 3.27 |
|  |  | 3 | 0 | -7.05 | -0.63 | 3.56 |
|  |  |  | 1 | -6.95 | -0.63 | 3.58 |
|  |  |  | 2 | -7.05 | -0.61 | 3.58 |
|  |  |  | 10 | -7.41 | -0.96 | 3.23 |
|  |  | 5 | 0 | -6.12 | -0.42 | 3.33 |
|  |  |  | 1 | -6.15 | -0.39 | 3.36 |
|  |  |  | 2 | -6.16 | -0.37 | 3.32 |
|  |  |  | 10 | -6.8 | -0.76 | 3.01 |
| 30 | 1 | 1 | 0 | -6.66 | -0.4 | 3.67 |
|  |  |  | 1 | -6.67 | -0.4 | 3.64 |
|  |  |  | 2 | -6.77 | -0.44 | 3.63 |
|  |  |  | 10 | -7.4 | -1.41 | 2.61 |
|  |  | 3 | 0 | -6.58 | -0.38 | 3.63 |
|  |  |  | 1 | -6.49 | -0.37 | 3.6 |
|  |  |  | 2 | -6.53 | -0.41 | 3.6 |
|  |  |  | 10 | -7.33 | -1.38 | 2.58 |
|  |  | 5 | 0 | -6.03 | -0.4 | 3.46 |
|  |  |  | 1 | -6.16 | -0.38 | 3.45 |
|  |  |  | 2 | -6.24 | -0.4 | 3.5 |
|  |  |  | 10 | -7.1 | -1.41 | 2.55 |
|  | -1 | 1 | 0 | -6.69 | -0.4 | 3.68 |
|  |  |  | 1 | -6.67 | -0.44 | 3.66 |
|  |  |  | 2 | -6.69 | -0.41 | 3.67 |
|  |  |  | 10 | -7.41 | -1.43 | 2.66 |
|  |  | 3 | 0 | -6.5 | -0.37 | 3.59 |
|  |  |  | 1 | -6.5 | -0.38 | 3.6 |
|  |  |  | 2 | -6.5 | -0.43 | 3.61 |
|  |  |  | 10 | -7.39 | -1.43 | 2.57 |
|  |  | 5 | 0 | -6.03 | -0.4 | 3.47 |
|  |  |  | 1 | -6.12 | -0.4 | 3.46 |
|  |  |  | 2 | -6.23 | -0.42 | 3.46 |
|  |  |  | 10 | -7.08 | -1.48 | 2.59 |
|  | -3 | 1 | 0 | -6.74 | -0.43 | 3.71 |
|  |  |  | 1 | -6.73 | -0.41 | 3.69 |
|  |  |  | 2 | -6.61 | -0.38 | 3.65 |
|  |  |  | 10 | -7.65 | -1.43 | 2.69 |
|  |  | 3 | 0 | -6.56 | -0.4 | 3.65 |
|  |  |  | 1 | -6.75 | -0.38 | 3.58 |
|  |  |  | 2 | -6.54 | -0.38 | 3.57 |
|  |  |  | 10 | -7.27 | -1.45 | 2.68 |
|  |  | 5 | 0 | -6.26 | -0.43 | 3.47 |
|  |  |  | 1 | -6.19 | -0.4 | 3.47 |
|  |  |  | 2 | -6.11 | -0.42 | 3.48 |
|  |  |  | 10 | -7.12 | -1.39 | 2.57 |

Table 6: Simulation results for FR1, TDL-C100, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 40 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 15 | 1 | 1 | 0 | -4.91 | -0.75 | 2.96 |
|  |  |  | 1 | -4.94 | -0.76 | 2.98 |
|  |  |  | 2 | -4.85 | -0.77 | 3 |
|  |  |  | 10 | -5.28 | -1.12 | 2.61 |
|  |  | 3 | 0 | -4.76 | -0.7 | 2.88 |
|  |  |  | 1 | -4.82 | -0.68 | 2.84 |
|  |  |  | 2 | -4.81 | -0.72 | 2.83 |
|  |  |  | 10 | -5.22 | -1.13 | 2.48 |
|  |  | 5 | 0 | -4.28 | -0.67 | 2.6 |
|  |  |  | 1 | -4.27 | -0.7 | 2.62 |
|  |  |  | 2 | -4.25 | -0.69 | 2.6 |
|  |  |  | 10 | -4.72 | -1.03 | 2.23 |
|  | -1 | 1 | 0 | -5.07 | -0.79 | 2.93 |
|  |  |  | 1 | -5.05 | -0.8 | 2.93 |
|  |  |  | 2 | -5.07 | -0.76 | 2.97 |
|  |  |  | 10 | -5.48 | -1.17 | 2.63 |
|  |  | 3 | 0 | -4.89 | -0.74 | 2.82 |
|  |  |  | 1 | -4.9 | -0.75 | 2.86 |
|  |  |  | 2 | -4.92 | -0.69 | 2.8 |
|  |  |  | 10 | -5.19 | -1.13 | 2.46 |
|  |  | 5 | 0 | -4.26 | -0.73 | 2.63 |
|  |  |  | 1 | -4.31 | -0.71 | 2.57 |
|  |  |  | 2 | -4.32 | -0.73 | 2.63 |
|  |  |  | 10 | -4.68 | -1.07 | 2.31 |
|  | -3 | 1 | 0 | -5.28 | -0.86 | 2.94 |
|  |  |  | 1 | -5.16 | -0.83 | 3.01 |
|  |  |  | 2 | -5.07 | -0.81 | 2.95 |
|  |  |  | 10 | -5.67 | -1.18 | 2.55 |
|  |  | 3 | 0 | -5.09 | -0.76 | 2.79 |
|  |  |  | 1 | -5.2 | -0.79 | 2.86 |
|  |  |  | 2 | -4.97 | -0.77 | 2.84 |
|  |  |  | 10 | -5.48 | -1.23 | 2.43 |
|  |  | 5 | 0 | -4.32 | -0.72 | 2.56 |
|  |  |  | 1 | -4.31 | -0.74 | 2.58 |
|  |  |  | 2 | -4.44 | -0.78 | 2.64 |
|  |  |  | 10 | -4.8 | -1.09 | 2.22 |
| 30 | 1 | 1 | 0 | -3.86 | -0.25 | 2.58 |
|  |  |  | 1 | -3.8 | -0.25 | 2.58 |
|  |  |  | 2 | -3.82 | -0.25 | 2.53 |
|  |  |  | 10 | -5.18 | -1.3 | 1.48 |
|  |  | 3 | 0 | -3.79 | -0.23 | 2.5 |
|  |  |  | 1 | -3.7 | -0.24 | 2.53 |
|  |  |  | 2 | -3.77 | -0.17 | 2.51 |
|  |  |  | 10 | -5.08 | -1.25 | 1.47 |
|  |  | 5 | 0 | -3.65 | -0.18 | 2.5 |
|  |  |  | 1 | -3.62 | -0.24 | 2.56 |
|  |  |  | 2 | -3.65 | -0.2 | 2.56 |
|  |  |  | 10 | -4.94 | -1.26 | 1.49 |
|  | -1 | 1 | 0 | -3.94 | -0.28 | 2.56 |
|  |  |  | 1 | -3.93 | -0.26 | 2.53 |
|  |  |  | 2 | -3.88 | -0.3 | 2.51 |
|  |  |  | 10 | -5.06 | -1.29 | 1.47 |
|  |  | 3 | 0 | -3.85 | -0.24 | 2.53 |
|  |  |  | 1 | -3.88 | -0.21 | 2.55 |
|  |  |  | 2 | -3.82 | -0.26 | 2.42 |
|  |  |  | 10 | -4.97 | -1.29 | 1.5 |
|  |  | 5 | 0 | -3.84 | -0.16 | 2.52 |
|  |  |  | 1 | -3.73 | -0.2 | 2.46 |
|  |  |  | 2 | -3.78 | -0.22 | 2.43 |
|  |  |  | 10 | -5.01 | -1.27 | 1.51 |
|  | -3 | 1 | 0 | -4.02 | -0.3 | 2.5 |
|  |  |  | 1 | -3.94 | -0.27 | 2.52 |
|  |  |  | 2 | -3.99 | -0.3 | 2.54 |
|  |  |  | 10 | -5.21 | -1.37 | 1.42 |
|  |  | 3 | 0 | -3.9 | -0.32 | 2.51 |
|  |  |  | 1 | -3.93 | -0.27 | 2.41 |
|  |  |  | 2 | -3.95 | -0.29 | 2.47 |
|  |  |  | 10 | -5.07 | -1.31 | 1.43 |
|  |  | 5 | 0 | -3.81 | -0.24 | 2.48 |
|  |  |  | 1 | -3.93 | -0.18 | 2.51 |
|  |  |  | 2 | -3.9 | -0.2 | 2.49 |
|  |  |  | 10 | -5 | -1.28 | 1.41 |

## Simulation results for FR2, UE1 and UE2 same timing and cyclic shift

### Measurement bandwidth 20 RBs

Table 7: Simulation results for FR2, AWGN, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 20 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 60 | 1 | 1 | 0 | -18.51 | -0.38 | 2.99 |
|  |  |  | 0.5 | -19.44 | 0.12 | 2.96 |
|  |  |  | 1 | -20.33 | -0.04 | 3.02 |
|  |  |  | 5 | -14.41 | -1.02 | 1.98 |
|  |  | 3 | 0 | -5.75 | -0.06 | 2.24 |
|  |  |  | 0.5 | -4.91 | 0.08 | 2.16 |
|  |  |  | 1 | -5 | 0.01 | 2.17 |
|  |  |  | 5 | -5.91 | -0.85 | 1.16 |
|  |  | 5 | 0 | -3.79 | -0.17 | 1.82 |
|  |  |  | 0.5 | -3.13 | 0.03 | 1.83 |
|  |  |  | 1 | -3.18 | 0.06 | 1.89 |
|  |  |  | 5 | -4.46 | -0.9 | 0.85 |
|  | -1 | 1 | 0 | -21.35 | -0.16 | 3.01 |
|  |  |  | 0.5 | -19.38 | 0.1 | 3 |
|  |  |  | 1 | -19.75 | 0.42 | 3.01 |
|  |  |  | 5 | -14.4 | -0.89 | 2.02 |
|  |  | 3 | 0 | -4.67 | -0.13 | 2.25 |
|  |  |  | 0.5 | -5.13 | 0.14 | 2.21 |
|  |  |  | 1 | -5.15 | 0.18 | 2.42 |
|  |  |  | 5 | -5.72 | -0.84 | 1.29 |
|  |  | 5 | 0 | -3.06 | -0.06 | 1.78 |
|  |  |  | 0.5 | -2.77 | -0.01 | 1.82 |
|  |  |  | 1 | -3.05 | 0.24 | 2.04 |
|  |  |  | 5 | -3.71 | -0.9 | 0.86 |
|  | -3 | 1 | 0 | -20.79 | 0.02 | 3.04 |
|  |  |  | 0.5 | -20.49 | -0.02 | 3.03 |
|  |  |  | 1 | -19.93 | -0.07 | 3.02 |
|  |  |  | 5 | -17.76 | -1.04 | 2.11 |
|  |  | 3 | 0 | -4.99 | 0.04 | 2.33 |
|  |  |  | 0.5 | -5.97 | 0.07 | 2.3 |
|  |  |  | 1 | -5.57 | 0.01 | 2.2 |
|  |  |  | 5 | -6.23 | -0.98 | 1.09 |
|  |  | 5 | 0 | -3.42 | -0.06 | 1.85 |
|  |  |  | 0.5 | -3.1 | -0.03 | 1.85 |
|  |  |  | 1 | -3.45 | -0.08 | 1.82 |
|  |  |  | 5 | -4.19 | -1 | 0.87 |
| 120 | 1 | 1 | 0 | -18.82 | -0.01 | 2.99 |
|  |  |  | 0.5 | -17.89 | 0.22 | 2.99 |
|  |  |  | 1 | -18.68 | -0.04 | 2.82 |
|  |  |  | 5 | -8.66 | -2.27 | 0.43 |
|  |  | 3 | 0 | -5.53 | -0.02 | 2.23 |
|  |  |  | 0.5 | -4.65 | 0.2 | 2.19 |
|  |  |  | 1 | -5.66 | -0.01 | 2.19 |
|  |  |  | 5 | -5.27 | -2.16 | -0.23 |
|  |  | 5 | 0 | -3.36 | -0.06 | 2.06 |
|  |  |  | 0.5 | -2.55 | 0.1 | 1.82 |
|  |  |  | 1 | -3.06 | -0.1 | 1.58 |
|  |  |  | 5 | -4.49 | -2.2 | -0.65 |
|  | -1 | 1 | 0 | -18.57 | -0.02 | 3.01 |
|  |  |  | 0.5 | -21.29 | 0.15 | 3.03 |
|  |  |  | 1 | -18.39 | -0.49 | 2.77 |
|  |  |  | 5 | -8.84 | -2.25 | 0.47 |
|  |  | 3 | 0 | -5.21 | 0.11 | 2.36 |
|  |  |  | 0.5 | -4.67 | -0.08 | 2.19 |
|  |  |  | 1 | -5.54 | -0.35 | 1.95 |
|  |  |  | 5 | -5.69 | -2.21 | -0.3 |
|  |  | 5 | 0 | -2.79 | -0.07 | 1.84 |
|  |  |  | 0.5 | -3.09 | 0.04 | 1.87 |
|  |  |  | 1 | -3.5 | -0.45 | 1.67 |
|  |  |  | 5 | -4.4 | -2.22 | -0.73 |
|  | -3 | 1 | 0 | -18.37 | 0.01 | 3.05 |
|  |  |  | 0.5 | -20.94 | -0.04 | 3.04 |
|  |  |  | 1 | -18.44 | -0.07 | 2.86 |
|  |  |  | 5 | -9.29 | -2.13 | 0.51 |
|  |  | 3 | 0 | -5.37 | 0.04 | 2.2 |
|  |  |  | 0.5 | -5.78 | 0.04 | 2.27 |
|  |  |  | 1 | -4.81 | 0.01 | 2.03 |
|  |  |  | 5 | -5.45 | -2.12 | -0.19 |
|  |  | 5 | 0 | -3.05 | 0.01 | 1.83 |
|  |  |  | 0.5 | -3.08 | 0.03 | 1.79 |
|  |  |  | 1 | -3.4 | -0.1 | 1.64 |
|  |  |  | 5 | -4.31 | -2.15 | -0.62 |

Table 8: Simulation results for FR2, TDL-A75, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 20 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 60 | 1 | 1 | 0 | -4.97 | -0.72 | 3.01 |
|  |  |  | 0.5 | -4.96 | -0.78 | 3 |
|  |  |  | 1 | -4.95 | -0.71 | 3.02 |
|  |  |  | 5 | -6.32 | -1.78 | 2.08 |
|  |  | 3 | 0 | -4.16 | -0.64 | 2.52 |
|  |  |  | 0.5 | -4.23 | -0.61 | 2.54 |
|  |  |  | 1 | -4.13 | -0.63 | 2.5 |
|  |  |  | 5 | -5.5 | -1.68 | 1.68 |
|  |  | 5 | 0 | -3.61 | -0.45 | 2.27 |
|  |  |  | 0.5 | -3.68 | -0.43 | 2.31 |
|  |  |  | 1 | -3.62 | -0.48 | 2.25 |
|  |  |  | 5 | -4.96 | -1.5 | 1.38 |
|  | -1 | 1 | 0 | -5.17 | -0.84 | 2.97 |
|  |  |  | 0.5 | -5.21 | -0.84 | 2.98 |
|  |  |  | 1 | -5.23 | -0.86 | 2.99 |
|  |  |  | 5 | -6.53 | -1.92 | 2.03 |
|  |  | 3 | 0 | -4.4 | -0.74 | 2.49 |
|  |  |  | 0.5 | -4.33 | -0.73 | 2.5 |
|  |  |  | 1 | -4.34 | -0.73 | 2.47 |
|  |  |  | 5 | -5.65 | -1.73 | 1.67 |
|  |  | 5 | 0 | -3.77 | -0.47 | 2.19 |
|  |  |  | 0.5 | -3.86 | -0.48 | 2.24 |
|  |  |  | 1 | -3.79 | -0.45 | 2.28 |
|  |  |  | 5 | -5.28 | -1.63 | 1.11 |
|  | -3 | 1 | 0 | -5.37 | -0.86 | 2.99 |
|  |  |  | 0.5 | -5.44 | -0.9 | 2.93 |
|  |  |  | 1 | -5.33 | -0.89 | 3 |
|  |  |  | 5 | -6.97 | -2.01 | 2.02 |
|  |  | 3 | 0 | -4.63 | -0.75 | 2.47 |
|  |  |  | 0.5 | -4.54 | -0.75 | 2.51 |
|  |  |  | 1 | -4.48 | -0.79 | 2.53 |
|  |  |  | 5 | -5.95 | -1.79 | 1.64 |
|  |  | 5 | 0 | -3.99 | -0.59 | 2.16 |
|  |  |  | 0.5 | -4.09 | -0.53 | 2.22 |
|  |  |  | 1 | -4.05 | -0.59 | 2.18 |
|  |  |  | 5 | -5.34 | -1.64 | 1.26 |
| 120 | 1 | 1 | 0 | -4.88 | -0.65 | 3.03 |
|  |  |  | 0.5 | -4.91 | -0.61 | 3.07 |
|  |  |  | 1 | -5.17 | -0.94 | 2.8 |
|  |  |  | 5 | -7.63 | -3.01 | 0.82 |
|  |  | 3 | 0 | -4.68 | -0.59 | 3 |
|  |  |  | 0.5 | -4.62 | -0.6 | 3 |
|  |  |  | 1 | -4.98 | -0.82 | 2.76 |
|  |  |  | 5 | -7.16 | -3.04 | 0.82 |
|  |  | 5 | 0 | -4.36 | -0.5 | 2.63 |
|  |  |  | 0.5 | -4.35 | -0.48 | 2.63 |
|  |  |  | 1 | -4.57 | -0.81 | 2.49 |
|  |  |  | 5 | -6.53 | -2.87 | 0.57 |
|  | -1 | 1 | 0 | -5.22 | -0.7 | 2.93 |
|  |  |  | 0.5 | -5.27 | -0.8 | 3.01 |
|  |  |  | 1 | -5.6 | -1.07 | 2.74 |
|  |  |  | 5 | -8.24 | -3.27 | 0.86 |
|  |  | 3 | 0 | -4.78 | -0.72 | 2.88 |
|  |  |  | 0.5 | -5.02 | -0.74 | 2.85 |
|  |  |  | 1 | -5.35 | -1.05 | 2.67 |
|  |  |  | 5 | -7.72 | -3.14 | 0.71 |
|  |  | 5 | 0 | -4.67 | -0.56 | 2.57 |
|  |  |  | 0.5 | -4.69 | -0.64 | 2.7 |
|  |  |  | 1 | -4.99 | -0.98 | 2.26 |
|  |  |  | 5 | -7.2 | -3.2 | 0.48 |
|  | -3 | 1 | 0 | -5.84 | -0.87 | 2.9 |
|  |  |  | 0.5 | -5.79 | -0.89 | 2.9 |
|  |  |  | 1 | -6.12 | -1.21 | 2.64 |
|  |  |  | 5 | -8.54 | -3.23 | 0.88 |
|  |  | 3 | 0 | -5.35 | -0.8 | 2.79 |
|  |  |  | 0.5 | -5.19 | -0.83 | 2.84 |
|  |  |  | 1 | -5.62 | -1.12 | 2.56 |
|  |  |  | 5 | -8.09 | -3.11 | 0.78 |
|  |  | 5 | 0 | -5.14 | -0.69 | 2.5 |
|  |  |  | 0.5 | -4.92 | -0.75 | 2.47 |
|  |  |  | 1 | -5.17 | -1.11 | 2.34 |
|  |  |  | 5 | -7.38 | -3.21 | 0.36 |

### Measurement bandwidth 40 RBs

Table 9: Simulation results for FR2, AWGN, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 40 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 60 | 1 | 1 | 0 | -20.14 | -0.28 | 2.99 |
|  |  |  | 0.5 | -21.34 | -0.04 | 3.01 |
|  |  |  | 1 | -20.43 | -0.16 | 2.96 |
|  |  |  | 5 | -14.24 | -1.12 | 1.94 |
|  |  | 3 | 0 | -5.54 | -0.19 | 2.24 |
|  |  |  | 0.5 | -5.18 | 0 | 2.26 |
|  |  |  | 1 | -5.67 | -0.15 | 2.29 |
|  |  |  | 5 | -5.94 | -1.14 | 1.18 |
|  |  | 5 | 0 | -3.14 | -0.18 | 1.74 |
|  |  |  | 0.5 | -3.29 | -0.03 | 1.77 |
|  |  |  | 1 | -3.26 | -0.04 | 1.77 |
|  |  |  | 5 | -4.2 | -1.08 | 0.86 |
|  | -1 | 1 | 0 | -19.18 | 0.14 | 2.99 |
|  |  |  | 0.5 | -20.36 | 0.16 | 2.99 |
|  |  |  | 1 | -20.67 | -0.03 | 3 |
|  |  |  | 5 | -14.23 | -0.99 | 1.97 |
|  |  | 3 | 0 | -4.62 | -0.09 | 2.37 |
|  |  |  | 0.5 | -4.89 | 0.06 | 2.27 |
|  |  |  | 1 | -5.59 | -0.09 | 2.2 |
|  |  |  | 5 | -5.76 | -1.01 | 1.15 |
|  |  | 5 | 0 | -3.57 | 0.03 | 1.88 |
|  |  |  | 0.5 | -3.02 | -0.07 | 1.92 |
|  |  |  | 1 | -3.3 | -0.02 | 1.77 |
|  |  |  | 5 | -4.02 | -0.96 | 0.66 |
|  | -3 | 1 | 0 | -20.55 | 0.16 | 3.01 |
|  |  |  | 0.5 | -21.78 | -0.02 | 3.01 |
|  |  |  | 1 | -19.15 | 0.09 | 3.01 |
|  |  |  | 5 | -16.51 | -0.97 | 1.96 |
|  |  | 3 | 0 | -4.81 | 0.06 | 2.37 |
|  |  |  | 0.5 | -5.09 | 0.01 | 2.07 |
|  |  |  | 1 | -5.52 | 0.16 | 2.34 |
|  |  |  | 5 | -5.77 | -0.98 | 1.24 |
|  |  | 5 | 0 | -3.08 | 0.1 | 1.88 |
|  |  |  | 0.5 | -3.14 | 0 | 1.64 |
|  |  |  | 1 | -3.09 | 0.11 | 1.82 |
|  |  |  | 5 | -4.23 | -0.97 | 0.98 |
| 120 | 1 | 1 | 0 | -18.4 | 0.39 | 2.98 |
|  |  |  | 0.5 | -20.1 | -0.29 | 2.99 |
|  |  |  | 1 | -18.17 | -0.18 | 2.77 |
|  |  |  | 5 | -8.51 | -2.02 | 0.41 |
|  |  | 3 | 0 | -5.14 | 0.16 | 2.33 |
|  |  |  | 0.5 | -5.83 | -0.19 | 2.23 |
|  |  |  | 1 | -5.75 | -0.26 | 2.07 |
|  |  |  | 5 | -5.39 | -2.11 | -0.27 |
|  |  | 5 | 0 | -3.11 | 0.22 | 1.88 |
|  |  |  | 0.5 | -3.42 | -0.13 | 1.71 |
|  |  |  | 1 | -3.81 | -0.19 | 1.57 |
|  |  |  | 5 | -4.86 | -2.01 | -0.72 |
|  | -1 | 1 | 0 | -20.24 | -0.03 | 3 |
|  |  |  | 0.5 | -20.62 | 0.04 | 2.99 |
|  |  |  | 1 | -20.02 | -0.21 | 2.77 |
|  |  |  | 5 | -8.27 | -2.09 | 0.42 |
|  |  | 3 | 0 | -5.07 | -0.01 | 2.15 |
|  |  |  | 0.5 | -4.58 | -0.08 | 2.12 |
|  |  |  | 1 | -5.57 | -0.13 | 1.97 |
|  |  |  | 5 | -5.55 | -2.17 | -0.23 |
|  |  | 5 | 0 | -3.06 | 0.05 | 1.82 |
|  |  |  | 0.5 | -3.26 | -0.06 | 1.76 |
|  |  |  | 1 | -3.21 | -0.15 | 1.6 |
|  |  |  | 5 | -4.15 | -2.2 | -0.63 |
|  | -3 | 1 | 0 | -22.97 | -0.16 | 2.99 |
|  |  |  | 0.5 | -25.25 | -0.17 | 3 |
|  |  |  | 1 | -21.28 | -0.33 | 2.8 |
|  |  |  | 5 | -8.55 | -2.51 | 0.5 |
|  |  | 3 | 0 | -5.28 | -0.02 | 2.19 |
|  |  |  | 0.5 | -5.6 | -0.08 | 2.16 |
|  |  |  | 1 | -5.97 | -0.36 | 2.04 |
|  |  |  | 5 | -5.61 | -2.3 | -0.26 |
|  |  | 5 | 0 | -2.95 | -0.15 | 1.75 |
|  |  |  | 0.5 | -3.58 | -0.06 | 1.83 |
|  |  |  | 1 | -3.65 | -0.2 | 1.43 |
|  |  |  | 5 | -4.6 | -2.26 | -0.76 |

Table 10: Simulation results for FR2, TDL-A75, UE1 and UE2 same timing offset, UE1 and UE2 same cyclic shift, measurement bandwidth 40 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 60 | 1 | 1 | 0 | -4.3 | -0.64 | 2.85 |
|  |  |  | 0.5 | -4.29 | -0.62 | 2.87 |
|  |  |  | 1 | -4.31 | -0.69 | 2.85 |
|  |  |  | 5 | -5.6 | -1.7 | 1.87 |
|  |  | 3 | 0 | -3.64 | -0.54 | 2.46 |
|  |  |  | 0.5 | -3.63 | -0.52 | 2.44 |
|  |  |  | 1 | -3.59 | -0.53 | 2.45 |
|  |  |  | 5 | -4.8 | -1.62 | 1.52 |
|  |  | 5 | 0 | -3.19 | -0.41 | 2.09 |
|  |  |  | 0.5 | -3.17 | -0.4 | 2.14 |
|  |  |  | 1 | -3.14 | -0.42 | 2.09 |
|  |  |  | 5 | -4.46 | -1.53 | 1.17 |
|  | -1 | 1 | 0 | -4.5 | -0.72 | 2.84 |
|  |  |  | 0.5 | -4.56 | -0.71 | 2.81 |
|  |  |  | 1 | -4.49 | -0.73 | 2.81 |
|  |  |  | 5 | -5.69 | -1.82 | 1.79 |
|  |  | 3 | 0 | -3.8 | -0.63 | 2.47 |
|  |  |  | 0.5 | -3.75 | -0.66 | 2.43 |
|  |  |  | 1 | -3.77 | -0.63 | 2.42 |
|  |  |  | 5 | -5 | -1.71 | 1.44 |
|  |  | 5 | 0 | -3.39 | -0.5 | 2.07 |
|  |  |  | 0.5 | -3.33 | -0.5 | 2.02 |
|  |  |  | 1 | -3.28 | -0.51 | 2.05 |
|  |  |  | 5 | -4.69 | -1.52 | 1.09 |
|  | -3 | 1 | 0 | -4.75 | -0.8 | 2.78 |
|  |  |  | 0.5 | -4.83 | -0.82 | 2.79 |
|  |  |  | 1 | -4.7 | -0.81 | 2.77 |
|  |  |  | 5 | -6.14 | -1.92 | 1.86 |
|  |  | 3 | 0 | -3.9 | -0.75 | 2.34 |
|  |  |  | 0.5 | -3.9 | -0.73 | 2.35 |
|  |  |  | 1 | -3.91 | -0.71 | 2.38 |
|  |  |  | 5 | -5.31 | -1.81 | 1.39 |
|  |  | 5 | 0 | -3.44 | -0.59 | 2.01 |
|  |  |  | 0.5 | -3.42 | -0.61 | 2.03 |
|  |  |  | 1 | -3.47 | -0.63 | 1.95 |
|  |  |  | 5 | -4.8 | -1.64 | 1.14 |
| 120 | 1 | 1 | 0 | -4.03 | -0.57 | 2.7 |
|  |  |  | 0.5 | -4.05 | -0.6 | 2.67 |
|  |  |  | 1 | -4.37 | -0.9 | 2.42 |
|  |  |  | 5 | -6.94 | -3.11 | 0.41 |
|  |  | 3 | 0 | -3.78 | -0.56 | 2.55 |
|  |  |  | 0.5 | -3.75 | -0.55 | 2.6 |
|  |  |  | 1 | -4.3 | -0.85 | 2.33 |
|  |  |  | 5 | -6.56 | -3.15 | 0.32 |
|  |  | 5 | 0 | -3.46 | -0.46 | 2.38 |
|  |  |  | 0.5 | -3.46 | -0.46 | 2.42 |
|  |  |  | 1 | -3.78 | -0.77 | 2.14 |
|  |  |  | 5 | -6.26 | -3.04 | 0.06 |
|  | -1 | 1 | 0 | -4.31 | -0.66 | 2.65 |
|  |  |  | 0.5 | -4.38 | -0.7 | 2.67 |
|  |  |  | 1 | -4.86 | -1.01 | 2.39 |
|  |  |  | 5 | -7.15 | -3.28 | 0.45 |
|  |  | 3 | 0 | -4.11 | -0.63 | 2.45 |
|  |  |  | 0.5 | -4.18 | -0.69 | 2.49 |
|  |  |  | 1 | -4.61 | -0.97 | 2.27 |
|  |  |  | 5 | -6.88 | -3.31 | 0.39 |
|  |  | 5 | 0 | -3.74 | -0.56 | 2.3 |
|  |  |  | 0.5 | -3.78 | -0.61 | 2.29 |
|  |  |  | 1 | -4.14 | -0.89 | 2.02 |
|  |  |  | 5 | -6.33 | -3.21 | 0.16 |
|  | -3 | 1 | 0 | -4.74 | -0.9 | 2.5 |
|  |  |  | 0.5 | -4.86 | -0.88 | 2.5 |
|  |  |  | 1 | -5.12 | -1.19 | 2.3 |
|  |  |  | 5 | -7.54 | -3.35 | 0.33 |
|  |  | 3 | 0 | -4.6 | -0.81 | 2.45 |
|  |  |  | 0.5 | -4.5 | -0.81 | 2.49 |
|  |  |  | 1 | -4.85 | -1.15 | 2.21 |
|  |  |  | 5 | -7.21 | -3.32 | 0.27 |
|  |  | 5 | 0 | -4.07 | -0.77 | 2.18 |
|  |  |  | 0.5 | -4.13 | -0.77 | 2.19 |
|  |  |  | 1 | -4.38 | -1.01 | 1.96 |
|  |  |  | 5 | -6.64 | -3.24 | 0.11 |

## Simulation results for FR1, UE1 and UE2 different cyclic shifts and timing offsets

### Measurement bandwidth 20 RBs

Table 11: Simulation results for FR1, AWGN, UE1 and UE2 different cyclic shifts: 0 and 5, respectively, timing offset UE1 – UE2 2 µs, measurement bandwidth 20 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error UE1 [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 15 | 1 | 1 | 0 | -0.5 | -0.02 | 0.45 |
|  |  |  | 1 | -0.48 | 0 | 0.45 |
|  |  |  | 2 | -0.49 | -0.02 | 0.43 |
|  |  |  | 10 | -1.4 | -0.76 | -0.19 |
|  |  | 3 | 0 | -0.29 | 0 | 0.25 |
|  |  |  | 1 | -0.27 | 0 | 0.26 |
|  |  |  | 2 | -0.27 | -0.01 | 0.24 |
|  |  |  | 10 | -1.18 | -0.76 | -0.42 |
|  |  | 5 | 0 | -0.23 | -0.01 | 0.19 |
|  |  |  | 1 | -0.22 | 0 | 0.2 |
|  |  |  | 2 | -0.24 | -0.01 | 0.18 |
|  |  |  | 10 | -1.12 | -0.75 | -0.53 |
|  | -1 | 1 | 0 | -0.62 | 0.03 | 0.6 |
|  |  |  | 1 | -0.64 | 0.02 | 0.59 |
|  |  |  | 2 | -0.66 | 0.01 | 0.58 |
|  |  |  | 10 | -21 | -0.89 | -0.05 |
|  |  | 3 | 0 | -0.44 | 0.04 | 0.36 |
|  |  |  | 1 | -0.44 | 0.01 | 0.35 |
|  |  |  | 2 | -0.57 | 0.01 | 0.33 |
|  |  |  | 10 | -3.17 | -1 | -0.41 |
|  |  | 5 | 0 | -0.35 | 0.03 | 0.28 |
|  |  |  | 1 | -0.43 | 0.01 | 0.28 |
|  |  |  | 2 | -0.45 | 0.01 | 0.27 |
|  |  |  | 10 | -2.68 | -1.16 | -0.51 |
|  | -3 | 1 | 0 | -21.93 | -0.19 | 0.7 |
|  |  |  | 1 | -21.53 | -0.23 | 0.66 |
|  |  |  | 2 | -21.49 | -0.18 | 0.67 |
|  |  |  | 10 | -24.12 | -1.03 | -0.06 |
|  |  | 3 | 0 | -3.27 | -0.49 | 0.36 |
|  |  |  | 1 | -4.14 | -0.52 | 0.32 |
|  |  |  | 2 | -3.01 | -0.52 | 0.33 |
|  |  |  | 10 | -5.81 | -2.18 | -0.4 |
|  |  | 5 | 0 | -2.62 | -0.71 | 0.2 |
|  |  |  | 1 | -2.78 | -0.74 | 0.14 |
|  |  |  | 2 | -2.31 | -0.69 | 0.16 |
|  |  |  | 10 | -4.75 | -1.84 | -0.59 |
| 30 | 1 | 1 | 0 | -0.62 | -0.02 | 0.43 |
|  |  |  | 1 | -0.6 | -0.02 | 0.44 |
|  |  |  | 2 | -0.57 | -0.02 | 0.44 |
|  |  |  | 10 | -3.23 | -2.31 | -1.53 |
|  |  | 3 | 0 | -1.74 | -0.04 | 0.25 |
|  |  |  | 1 | -1.66 | -0.02 | 0.24 |
|  |  |  | 2 | -1.67 | -0.03 | 0.28 |
|  |  |  | 10 | -3.66 | -2.32 | -1.85 |
|  |  | 5 | 0 | -1.02 | -0.03 | 0.19 |
|  |  |  | 1 | -1.02 | -0.04 | 0.2 |
|  |  |  | 2 | -0.99 | -0.02 | 0.19 |
|  |  |  | 10 | -3.19 | -2.32 | -1.95 |
|  | -1 | 1 | 0 | -0.93 | -0.03 | 0.53 |
|  |  |  | 1 | -0.98 | -0.04 | 0.52 |
|  |  |  | 2 | -0.96 | -0.02 | 0.53 |
|  |  |  | 10 | -14.76 | -2.31 | -1.35 |
|  |  | 3 | 0 | -1.87 | -0.04 | 0.29 |
|  |  |  | 1 | -1.84 | -0.05 | 0.28 |
|  |  |  | 2 | -1.83 | -0.04 | 0.33 |
|  |  |  | 10 | -4.24 | -2.33 | -1.71 |
|  |  | 5 | 0 | -1.1 | -0.05 | 0.22 |
|  |  |  | 1 | -1.12 | -0.07 | 0.22 |
|  |  |  | 2 | -1.17 | -0.04 | 0.24 |
|  |  |  | 10 | -3.68 | -2.37 | -1.82 |
|  | -3 | 1 | 0 | -19.63 | -0.06 | 0.66 |
|  |  |  | 1 | -19.95 | -0.07 | 0.65 |
|  |  |  | 2 | -19.78 | -0.06 | 0.62 |
|  |  |  | 10 | -18.73 | -2.36 | -1.17 |
|  |  | 3 | 0 | -2.04 | -0.07 | 0.38 |
|  |  |  | 1 | -2.04 | -0.11 | 0.35 |
|  |  |  | 2 | -2.18 | -0.09 | 0.34 |
|  |  |  | 10 | -4.53 | -2.43 | -1.64 |
|  |  | 5 | 0 | -1.41 | -0.11 | 0.3 |
|  |  |  | 1 | -2.13 | -0.16 | 0.27 |
|  |  |  | 2 | -1.94 | -0.12 | 0.26 |
|  |  |  | 10 | -4.11 | -2.49 | -1.77 |

### Measurement bandwidth 40 RBs

Table 12: Simulation results for FR1, AWGN, UE1 and UE2 different cyclic shifts: 0 and 5, respectively, timing offset UE1 – UE2 2 µs, measurement bandwidth 40 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error UE1 [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 15 | 1 | 1 | 0 | -0.31 | 0 | 0.32 |
|  |  |  | 1 | -0.3 | 0.01 | 0.34 |
|  |  |  | 2 | -0.31 | 0.02 | 0.32 |
|  |  |  | 10 | -1.18 | -0.77 | -0.37 |
|  |  | 3 | 0 | -0.17 | 0.01 | 0.19 |
|  |  |  | 1 | -0.17 | 0.02 | 0.21 |
|  |  |  | 2 | -0.18 | 0.01 | 0.21 |
|  |  |  | 10 | -1.01 | -0.76 | -0.53 |
|  |  | 5 | 0 | -0.12 | 0 | 0.15 |
|  |  |  | 1 | -0.13 | 0.02 | 0.15 |
|  |  |  | 2 | -0.13 | 0.02 | 0.15 |
|  |  |  | 10 | -0.95 | -0.76 | -0.58 |
|  | -1 | 1 | 0 | -0.38 | 0.04 | 0.42 |
|  |  |  | 1 | -0.4 | 0.02 | 0.41 |
|  |  |  | 2 | -0.37 | 0.03 | 0.44 |
|  |  |  | 10 | -3.37 | -0.78 | -0.21 |
|  |  | 3 | 0 | -0.19 | 0.03 | 0.25 |
|  |  |  | 1 | -0.23 | 0.02 | 0.28 |
|  |  |  | 2 | -0.21 | 0.03 | 0.27 |
|  |  |  | 10 | -2.44 | -0.8 | -0.44 |
|  |  | 5 | 0 | -0.14 | 0.03 | 0.2 |
|  |  |  | 1 | -0.16 | 0.02 | 0.2 |
|  |  |  | 2 | -0.14 | 0.03 | 0.23 |
|  |  |  | 10 | -1.93 | -0.84 | -0.54 |
|  | -3 | 1 | 0 | -18.36 | -0.05 | 0.54 |
|  |  |  | 1 | -3.09 | -0.06 | 0.55 |
|  |  |  | 2 | -3.34 | -0.08 | 0.55 |
|  |  |  | 10 | -25.82 | -0.89 | -0.24 |
|  |  | 3 | 0 | -1.96 | -0.15 | 0.32 |
|  |  |  | 1 | -1.92 | -0.13 | 0.29 |
|  |  |  | 2 | -2.11 | -0.16 | 0.29 |
|  |  |  | 10 | -5.5 | -1.17 | -0.47 |
|  |  | 5 | 0 | -1.49 | -0.24 | 0.21 |
|  |  |  | 1 | -1.53 | -0.25 | 0.2 |
|  |  |  | 2 | -1.64 | -0.25 | 0.2 |
|  |  |  | 10 | -4.49 | -1.71 | -0.62 |
| 30 | 1 | 1 | 0 | -0.37 | -0.01 | 0.32 |
|  |  |  | 1 | -0.36 | -0.01 | 0.33 |
|  |  |  | 2 | -0.37 | 0 | 0.32 |
|  |  |  | 10 | -2.89 | -2.3 | -1.77 |
|  |  | 3 | 0 | -0.2 | -0.01 | 0.18 |
|  |  |  | 1 | -0.22 | -0.01 | 0.19 |
|  |  |  | 2 | -0.21 | -0.01 | 0.19 |
|  |  |  | 10 | -2.64 | -2.29 | -1.97 |
|  |  | 5 | 0 | -0.15 | -0.01 | 0.14 |
|  |  |  | 1 | -0.17 | 0 | 0.14 |
|  |  |  | 2 | -0.18 | -0.01 | 0.15 |
|  |  |  | 10 | -2.55 | -2.3 | -2.05 |
|  | -1 | 1 | 0 | -0.44 | -0.01 | 0.38 |
|  |  |  | 1 | -0.45 | -0.01 | 0.39 |
|  |  |  | 2 | -0.43 | -0.01 | 0.4 |
|  |  |  | 10 | -3.1 | -2.31 | -1.61 |
|  |  | 3 | 0 | -0.28 | -0.02 | 0.21 |
|  |  |  | 1 | -0.33 | -0.01 | 0.22 |
|  |  |  | 2 | -0.31 | -0.01 | 0.24 |
|  |  |  | 10 | -2.78 | -2.29 | -1.9 |
|  |  | 5 | 0 | -0.74 | -0.02 | 0.16 |
|  |  |  | 1 | -0.89 | -0.01 | 0.16 |
|  |  |  | 2 | -0.83 | -0.02 | 0.19 |
|  |  |  | 10 | -2.72 | -2.29 | -1.99 |
|  | -3 | 1 | 0 | -0.7 | -0.02 | 0.48 |
|  |  |  | 1 | -0.64 | -0.01 | 0.48 |
|  |  |  | 2 | -0.68 | -0.02 | 0.45 |
|  |  |  | 10 | -3.33 | -2.31 | -1.42 |
|  |  | 3 | 0 | -1.75 | -0.04 | 0.27 |
|  |  |  | 1 | -1.67 | -0.02 | 0.26 |
|  |  |  | 2 | -1.74 | -0.03 | 0.24 |
|  |  |  | 10 | -3.8 | -2.32 | -1.79 |
|  |  | 5 | 0 | -1.06 | -0.05 | 0.22 |
|  |  |  | 1 | -1 | -0.03 | 0.22 |
|  |  |  | 2 | -1.04 | -0.04 | 0.21 |
|  |  |  | 10 | -3.3 | -2.33 | -1.89 |

## Simulation results for FR2, UE1 and UE2 different cyclic shifts and timing offsets

### Measurement bandwidth 20 RBs

Table 13: Simulation results for FR2, AWGN, UE1 and UE2 different cyclic shifts: 0 and 5, respectively, timing offset UE1 – UE2 1 µs, measurement bandwidth 20 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 60 | 1 | 1 | 0 | -0.58 | -0.02 | 0.46 |
|  |  |  | 0.5 | -0.58 | -0.01 | 0.45 |
|  |  |  | 1 | -0.59 | 0.01 | 0.45 |
|  |  |  | 5 | -3.25 | -2.24 | -1.45 |
|  |  | 3 | 0 | -1.7 | -0.02 | 0.27 |
|  |  |  | 0.5 | -1.68 | 0 | 0.25 |
|  |  |  | 1 | -1.62 | -0.01 | 0.26 |
|  |  |  | 5 | -3.81 | -2.23 | -1.77 |
|  |  | 5 | 0 | -1.03 | -0.03 | 0.19 |
|  |  |  | 0.5 | -1.02 | -0.01 | 0.2 |
|  |  |  | 1 | -0.99 | -0.02 | 0.2 |
|  |  |  | 5 | -3.28 | -2.25 | -1.87 |
|  | -1 | 1 | 0 | -19.28 | -0.01 | 0.55 |
|  |  |  | 0.5 | -0.82 | -0.01 | 0.54 |
|  |  |  | 1 | -19.55 | -0.01 | 0.55 |
|  |  |  | 5 | -3.94 | -2.22 | -1.33 |
|  |  | 3 | 0 | -1.88 | -0.03 | 0.32 |
|  |  |  | 0.5 | -1.81 | -0.01 | 0.32 |
|  |  |  | 1 | -1.85 | -0.04 | 0.3 |
|  |  |  | 5 | -4.1 | -2.26 | -1.7 |
|  |  | 5 | 0 | -1.15 | -0.06 | 0.23 |
|  |  |  | 0.5 | -1.07 | -0.03 | 0.27 |
|  |  |  | 1 | -1.14 | -0.04 | 0.25 |
|  |  |  | 5 | -3.47 | -2.28 | -1.81 |
|  | -3 | 1 | 0 | -20.63 | -0.06 | 0.63 |
|  |  |  | 0.5 | -19.77 | -0.04 | 0.69 |
|  |  |  | 1 | -20.85 | -0.05 | 0.64 |
|  |  |  | 5 | -17.95 | -2.25 | -1.07 |
|  |  | 3 | 0 | -2.05 | -0.11 | 0.34 |
|  |  |  | 0.5 | -2.01 | -0.07 | 0.38 |
|  |  |  | 1 | -2.09 | -0.09 | 0.36 |
|  |  |  | 5 | -4.75 | -2.31 | -1.54 |
|  |  | 5 | 0 | -2.08 | -0.15 | 0.25 |
|  |  |  | 0.5 | -1.94 | -0.09 | 0.29 |
|  |  |  | 1 | -1.93 | -0.15 | 0.27 |
|  |  |  | 5 | -4.26 | -2.33 | -1.7 |
| 120 | 1 | 1 | 0 | -0.54 | 0.01 | 0.51 |
|  |  |  | 0.5 | -0.53 | 0 | 0.52 |
|  |  |  | 1 | -1.07 | -0.44 | 0.12 |
|  |  |  | 5 | -8.24 | -6.47 | -4.96 |
|  |  | 3 | 0 | -0.33 | 0.01 | 0.31 |
|  |  |  | 0.5 | -0.3 | 0 | 0.3 |
|  |  |  | 1 | -0.79 | -0.44 | -0.12 |
|  |  |  | 5 | -7.39 | -6.43 | -5.56 |
|  |  | 5 | 0 | -0.23 | 0 | 0.25 |
|  |  |  | 0.5 | -0.27 | 0 | 0.25 |
|  |  |  | 1 | -0.71 | -0.44 | -0.19 |
|  |  |  | 5 | -7.25 | -6.42 | -5.72 |
|  | -1 | 1 | 0 | -0.63 | 0 | 0.61 |
|  |  |  | 0.5 | -0.68 | -0.02 | 0.59 |
|  |  |  | 1 | -1.16 | -0.47 | 0.23 |
|  |  |  | 5 | -8.8 | -6.48 | -4.71 |
|  |  | 3 | 0 | -0.34 | -0.01 | 0.34 |
|  |  |  | 0.5 | -0.37 | -0.01 | 0.36 |
|  |  |  | 1 | -0.85 | -0.44 | -0.03 |
|  |  |  | 5 | -7.76 | -6.43 | -5.45 |
|  |  | 5 | 0 | -0.26 | 0.01 | 0.26 |
|  |  |  | 0.5 | -0.3 | -0.02 | 0.27 |
|  |  |  | 1 | -0.76 | -0.47 | -0.13 |
|  |  |  | 5 | -7.41 | -6.46 | -5.62 |
|  | -3 | 1 | 0 | -0.75 | 0.01 | 0.7 |
|  |  |  | 0.5 | -0.77 | 0 | 0.72 |
|  |  |  | 1 | -1.31 | -0.44 | 0.35 |
|  |  |  | 5 | -9.23 | -6.43 | -4.13 |
|  |  | 3 | 0 | -0.42 | 0.04 | 0.41 |
|  |  |  | 0.5 | -0.44 | 0.02 | 0.43 |
|  |  |  | 1 | -0.94 | -0.44 | 0.03 |
|  |  |  | 5 | -7.83 | -6.37 | -4.93 |
|  |  | 5 | 0 | -0.32 | 0.03 | 0.34 |
|  |  |  | 0.5 | -0.32 | 0.01 | 0.33 |
|  |  |  | 1 | -0.86 | -0.44 | -0.07 |
|  |  |  | 5 | -7.43 | -6.31 | -5.2 |

### Measurement bandwidth 40 RBs

Table 14: Simulation results for FR2, AWGN, UE1 and UE2 different cyclic shifts: 0 and 5, respectively, timing offset UE1 – UE2 1 µs, measurement bandwidth 40 RBs

| SCS [kHz] | SINR [dB] | Time domain filtering | Timing error [µs] | Delta SRS-RSRP [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5th ptile | Median | 95th ptile |
| 60 | 1 | 1 | 0 | -0.35 | -0.02 | 0.31 |
|  |  |  | 0.5 | -0.36 | -0.01 | 0.31 |
|  |  |  | 1 | -0.34 | -0.01 | 0.31 |
|  |  |  | 5 | -2.89 | -2.3 | -1.75 |
|  |  | 3 | 0 | -0.23 | -0.03 | 0.18 |
|  |  |  | 0.5 | -0.22 | -0.01 | 0.18 |
|  |  |  | 1 | -0.21 | -0.01 | 0.18 |
|  |  |  | 5 | -2.63 | -2.29 | -1.97 |
|  |  | 5 | 0 | -0.18 | -0.03 | 0.14 |
|  |  |  | 0.5 | -0.17 | -0.01 | 0.13 |
|  |  |  | 1 | -0.16 | -0.01 | 0.13 |
|  |  |  | 5 | -2.53 | -2.29 | -2.06 |
|  | -1 | 1 | 0 | -0.44 | -0.03 | 0.4 |
|  |  |  | 0.5 | -0.46 | 0 | 0.37 |
|  |  |  | 1 | -0.44 | -0.03 | 0.37 |
|  |  |  | 5 | -2.96 | -2.25 | -1.62 |
|  |  | 3 | 0 | -0.28 | -0.02 | 0.21 |
|  |  |  | 0.5 | -0.26 | -0.01 | 0.23 |
|  |  |  | 1 | -0.28 | -0.03 | 0.23 |
|  |  |  | 5 | -2.67 | -2.26 | -1.88 |
|  |  | 5 | 0 | -0.23 | -0.01 | 0.16 |
|  |  |  | 0.5 | -0.21 | 0 | 0.16 |
|  |  |  | 1 | -0.24 | -0.03 | 0.17 |
|  |  |  | 5 | -2.63 | -2.26 | -1.95 |
|  | -3 | 1 | 0 | -0.61 | -0.03 | 0.47 |
|  |  |  | 0.5 | -0.61 | -0.02 | 0.48 |
|  |  |  | 1 | -0.6 | -0.04 | 0.44 |
|  |  |  | 5 | -3.3 | -2.27 | -1.47 |
|  |  | 3 | 0 | -1.2 | -0.03 | 0.28 |
|  |  |  | 0.5 | -1.46 | -0.02 | 0.27 |
|  |  |  | 1 | -0.59 | -0.04 | 0.24 |
|  |  |  | 5 | -3.57 | -2.27 | -1.8 |
|  |  | 5 | 0 | -0.94 | -0.03 | 0.2 |
|  |  |  | 0.5 | -0.96 | -0.04 | 0.2 |
|  |  |  | 1 | -0.93 | -0.04 | 0.19 |
|  |  |  | 5 | -3.19 | -2.29 | -1.92 |
| 120 | 1 | 1 | 0 | -0.39 | -0.01 | 0.35 |
|  |  |  | 0.5 | -0.41 | -0.01 | 0.35 |
|  |  |  | 1 | -0.89 | -0.44 | -0.04 |
|  |  |  | 5 | -7.84 | -6.55 | -5.34 |
|  |  | 3 | 0 | -0.22 | -0.01 | 0.21 |
|  |  |  | 0.5 | -0.24 | -0.01 | 0.21 |
|  |  |  | 1 | -0.68 | -0.44 | -0.2 |
|  |  |  | 5 | -7.29 | -6.5 | -5.78 |
|  |  | 5 | 0 | -0.17 | -0.01 | 0.16 |
|  |  |  | 0.5 | -0.2 | -0.01 | 0.16 |
|  |  |  | 1 | -0.63 | -0.44 | -0.25 |
|  |  |  | 5 | -7.13 | -6.51 | -5.99 |
|  | -1 | 1 | 0 | -0.44 | -0.01 | 0.42 |
|  |  |  | 0.5 | -0.47 | -0.02 | 0.4 |
|  |  |  | 1 | -0.96 | -0.47 | 0.02 |
|  |  |  | 5 | -8.1 | -6.5 | -5.14 |
|  |  | 3 | 0 | -0.27 | -0.01 | 0.23 |
|  |  |  | 0.5 | -0.24 | -0.01 | 0.24 |
|  |  |  | 1 | -0.74 | -0.47 | -0.17 |
|  |  |  | 5 | -7.35 | -6.47 | -5.67 |
|  |  | 5 | 0 | -0.2 | -0.01 | 0.18 |
|  |  |  | 0.5 | -0.19 | -0.02 | 0.2 |
|  |  |  | 1 | -0.68 | -0.45 | -0.22 |
|  |  |  | 5 | -7.15 | -6.47 | -5.83 |
|  | -3 | 1 | 0 | -0.53 | -0.02 | 0.49 |
|  |  |  | 0.5 | -0.59 | -0.01 | 0.48 |
|  |  |  | 1 | -1.09 | -0.45 | 0.1 |
|  |  |  | 5 | -8.47 | -6.49 | -4.83 |
|  |  | 3 | 0 | -0.33 | -0.01 | 0.3 |
|  |  |  | 0.5 | -0.33 | -0.02 | 0.27 |
|  |  |  | 1 | -0.83 | -0.45 | -0.13 |
|  |  |  | 5 | -7.46 | -6.42 | -5.41 |
|  |  | 5 | 0 | -0.23 | -0.01 | 0.22 |
|  |  |  | 0.5 | -0.25 | -0.02 | 0.22 |
|  |  |  | 1 | -0.71 | -0.46 | -0.21 |
|  |  |  | 5 | -7.33 | -6.39 | -5.67 |

# Summary and Conclusions

In this contribution we have provided initial simulation results for SRS-RSRP measurements for CLI.

# References

- R1-1904805 “Simulation assumptions for CLI measurement requirements”, LG Electronics