3GPP TSG-RAN WG4 Meeting #91	R4-1907224

# Reno, USA, May 13th – 17th, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for clarification of note for B42_n77 and B42_n78 to TS 38.101-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clarification on applicability of intra-band contiguous or non-contiguous EN-DC requirements according to the agreement. Agreement: Intra-band contiguous and non--contiguous EN-DC requirements applies to DC_B42_n77 and DC_B42_n78. Note: DC_B42_n77 and DC_B42_n78 are specified under the condition that they are used with at least another LTE band and UL transmission of B42 is not allowed so far. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draft CR is based on the draft spec after RAN#90bis: R4-1905232 and changes are marked as yellow. Revision from R4-1906223 is marked as green. NOTE4 in Table 5.5B.4.1-1 is clarified to accommodate both intra-band contiguous and non-contiguous case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicability of intra contiguous or non-contiguous is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

5.5B.4	Inter-band EN-DC within FR1

5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |  |
| --- | --- | --- | --- |
| DC_1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n40A | DC_1A_n40A | No |  |
| DC_1A_n51A | DC_1A_n51A | No |  |
| DC_1A_n77A7 DC_1A_n77C7 | DC_1A_n77A | DC_1_n77 |  |
| DC_1A_n78A7 DC_1A_n78C7 | DC_1A_n78A | No |  |
| DC_1A_n79A7 DC_1A_n79C7 | DC_1A_n79A | No |  |
| DC_2A_n5A | DC_2A_n5A | No |  |
| DC_2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n71A | DC_2A_n71A | No |  |
| DC_2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_3A_n7A | DC_3A_n7A | No |  |
| DC_3A_n28A | DC_3A_n28A | No |  |
| DC_3A_n40A | DC_3A_n40A | No |  |
| DC_3A_n51A | DC_3A_n51A | No |  |
| DC_3A_n77A7 DC_3A_n77C7 | DC_3A_n77A | DC_3_n77 |  |
| DC_3A_n78A7 DC_3A_n78C7 DC_3C_n78A7 | DC_3A_n78A |  |  |
| DC_3A_n79A7 DC_3A_n79C7 | DC_3A_n79A | No |  |
|  |  | No |  |
| DC_5A_n40A | DC_5A_n40A | No |  |
| DC_5A_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A_n78A7 | DC_5A_n78A | No |  |
| DC_7A-7A_n78A7 | DC_7A_n78A | No |  |
| DC_7A_n28A | DC_7A_n28A | No |  |
| DC_7A_n51A | DC_7A_n51A | No |  |
| DC_7A_n78A7 | DC_7A_n78A | No |  |
| DC_7C_n78A7 | DC_7C_n78A | No |  |
| DC_8A_n40A | DC_8A_n40A | No |  |
| DC_8A_n77A7 | DC_8A_n77A | No |  |
| DC_8A_n78A7 | DC_8A_n78A | No |  |
| DC_8A_n79A7 | DC_8A_n79A | No |  |
| DC_11A_n77A7 | DC_11A_n77A | No |  |
| DC_11A_n78A7 | DC_11A_n78A | No |  |
| DC_11A_n79A7 | DC_11A_n79A | No |  |
| DC_12A_n5A | DC_12A_n5A | No |  |
| DC_12A_n66A | DC_12A_n66A | No |  |
| DC_18A_n77A7 | DC_18A_n77A | No |  |
| DC_18A_n78A7 | DC_18A_n78A | No |  |
| DC_18A_n79A7 | DC_18A_n79A | No |  |
| DC_19A_n77A7 DC_19A_n77C7 | DC_19A_n77A | No |  |
| DC_19A_n78A7 DC_19A_n78C7 | DC_19A_n78A | No |  |
| DC_19A_n79A7 DC_19A_n79C7 | DC_19A_n79A | No |  |
| DC_20A_n8A | DC_20A_n8A | DC_20_n8 |  |
| DC_20A_n28A8,10 | DC_20A_n28A | No |  |
| DC_20A_n51A | DC_20A_n51A | No |  |
| DC_20A_n77A7 | DC_20A_n77A | No |  |
| DC_20A_n78A7 | DC_20A_n78A | No |  |
| DC_21A_n77A7 DC_21A_n77C7 | DC_21A_n77A | No |  |
| DC_21A_n78A7 DC_21A_n78C7 | DC_21A_n78A | No |  |
| DC_21A_n79A7 DC_21A_n79C7 | DC_21A_n79A | No |  |
| DC_25A_n41A | DC_25A_n41A | No |  |
| DC_26A_n41A | DC_26A_n41A | No |  |
| DC_26A_n77A7 | DC_26A_n77A | No |  |
| DC_26A_n78A7 | DC_26A_n78A | No |  |
| DC_26A_n79A7 | DC_26A_n79A | No |  |
| DC_28A n51A | DC_28A_n51A | No |  |
| DC_28A_n77A7 DC_28A_n77C7 | DC_28A_n77A | No |  |
| DC_28A_n78A7 DC_28A_n78C7 | DC_28A_n78A | No |  |
| DC_28A_n79A7 DC_28A_n79C7 | DC_28A_n79A | No |  |
| DC_30A_n5A | DC_30A_n5A | No |  |
| DC_30A_n66A | DC_30A_n66A | No |  |
| DC_38A_n78A | N/A | No |  |
| DC_39A_n78A5,7 | DC_39A_n78A | No |  |
| DC_39A_n79A7 | DC_39A_n79A | No |  |
| DC_40A_n77A | N/A | No |  |
| DC_41A_n77A DC_41C_n77A | DC_41A_n77A DC_41C_n77A | No |  |
| DC_41A_n78A DC_41C_n78A | DC_41A_n78A DC_41C_n78A | No |  |
| DC_41A_n79A6,7 DC_41C_n79A6,7 | DC_41A_n79A DC_41C_n79A | No |  |
|  |  | No |  |
|  |  | No |  |
|  |  | No |  |
| DC_42A_n51A | DC_42A_n51A | No |  |
| DC_42A_n77A3,4,9 DC_42A_n77C3,4,9 DC_42C_n77A3,4,9 DC_42C_n77C3,4,9 DC_42D_n77A3,4,9 DC_42E_n77A3,4,9 | N/A | N/A |  |
| DC_42A_n78A3,4,9 DC_42A_n78C3,4,9 DC_42C_n78A3,4,9 DC_42C_n78C3,4,9 DC_42D_n78A3,4,9 DC_42E_n78A3,4,9 | N/A | N/A |  |
| DC_42A_n79A9 DC_42A_n79C9 DC_42C_n79A9 DC_42C_n79C9 DC_42D_n79A9 DC_42E_n79A9 | N/A | N/A |  |
|  | N/A |  |  |
|  | N/A |  |  |
|  | N/A |  |  |
|  | N/A |  |  |
|  | N/A |  |  |
|  | N/A |  |  |
|  | N/A |  |  |
|  | N/A |  |  |
| DC_46A_n78A2 DC_46C_n78A2 DC_46D_n78A2 DC_46E_n78A2 | N/A | N/A |  |
|  | N/A |  |  |
|  | N/A |  |  |
|  | N/A |  |  |
| DC_66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A_n71A | DC_66A_n71A | No |  |
| DC_66A_n78A | DC_66A_n78A | No |  |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC cong\figuration is part of a higher order EN-DC configuration. NOTE 4: The minimum requirements for intra-band contiguous or non-contiguous EN-DC apply. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. NOTE 10: The maximum power spectral density imbalance between downlink carriers is within [6] dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

---End of changes---