3GPP TSG-RAN4 Meeting #91	R4-1907372

# Reno, US, 13th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintanance of inter-frequency RSRP accuracy test cases for FR1 (section A.4.7.1.2, A.6.7.1.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-03-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN4#90bis, it is agreed to add TRS configuration for FR1 test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add TRS for FR1 inter-frequency RSRP accuracy test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no TRS configuration for FR1 inter-frequency RSRP accuracy test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.7.1.2, A.6.7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.4.7.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 10.1.4.1.1 and 10.1.4.1.2 for inter-frequency measurements with the testing configurations in Table A.4.7.1.2.1-1.

Table A.4.7.1.2.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.4.7.1.2.2	Test parameters

In this set of test cases there are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on a different frequency than the PSCell. The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 2 and Cell 3 are given in Table A.4.7.1.2.2-1 below. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.4.7.1.2.2-1. The inter frequency measurements are supported by a measurement gap.

Table A.4.7.1.2.2-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 3 |  | Cell 2 |  | Cell 3 |  |
| SSB ARFCN |  | 1~6 |  | freq1 |  | freq2 |  | freq1 |  | freq2 |  |
| BWchannel |  | 1,4 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2,5 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3,6 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| Duplex mode |  | 1,4 |  | FDD |  |  | FDD |  |  |
|  |  | 2,5 |  | TDD |  |  | TDD |  |  |
|  |  | 3,6 |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | 1,4 |  | N/A |  |  | N/A |  |  |
|  |  | 2,5 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3,6 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1,4 |  | SR.1.1 FDD |  | - |  | SR.1.1 FDD |  | - |  |
|  |  | 2,5 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
|  |  | 3,6 |  | SR.2.1 FDD |  |  |  | SR.2.1 FDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1,4 |  | CR.1.1 FDD |  | - |  | CR.1.1 FDD |  | - |  |
|  |  | 2,5 |  | CR.1.1 TDD |  | - |  | CR.1.1 TDD |  | - |  |
|  |  | 3,6 |  | CR.2.1 FDD |  | - |  | CR.2.1 FDD |  | - |  |
| Dedicated CORESET Reference Channel |  | 1,4 |  | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD |  | - |  |
|  |  | 2,5 |  | CCR.1.1 TDD |  | - |  | CCR.1.1 TDD |  | - |  |
|  |  | 3,6 |  | CCR.2.1 TDD |  | - |  | CCR.2.1 TDD |  | - |  |
| SSB configuration |  | 1,4 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 2,5 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 3,6 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~6 |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1,4 |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD |  | - |  |
|  |  | 2,5 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |  |
|  |  | 3,6 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |  |
| Initial BWP Configuration |  | 1~6 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~6 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| SMTC configuration |  | 1~6 |  | SMTC.1 |  |  | SMTC.1 |  |  |
| Time offset between Cell 2 and Cell 3 |  | 1~6 | s | 3 |  |  | 3 |  |  |
| EPRE ratio of PSS to SSS |  | 1~6 | dB | 0 |  | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1~6 | dBm/15kHz | -94.65 |  |  | ( for Channel 2 +8dB) |  | -117 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -116.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -116 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -115.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -115 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -114 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -113.5 |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1,2,4,5 | dBm/SSB SCS | -94.65 |  |  | ( for Channel 2 +8dB) |  | -117 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -116.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -116 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -115.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -115 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -114 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -113.5 |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | 3,6 |  | -91.65 |  |  | ( for Channel 2 +8dB) |  | -114 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | 1~6 | dB | 10 |  | 10 |  | 13 |  | -4 |  |
| SS-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1,2,4,5 | dBm/SCS | -84.65 |  |  | (RSRP for Cell 2 +25dB) |  | -121 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -120.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -119 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -118 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -117.5 |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | 3,6 |  | -81.65 |  |  | (RSRP for Cell 2 +25dB) |  | -124 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -123.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -122 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -121 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -120.5 |  |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1,2,4,5 | dBm/ 9.36MHz | -56.28 |  |  | (Io for Channel 2 +19.75dB) | -87.76 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -87.26 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -86.76 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -86.26 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -85.76 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -84.76 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -84.26 |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | 3,6 | dBm/ 38.16MHz | -50.19 |  |  | (Io for Channel 2 +19.75dB) | -84.76 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -84.26 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -83.76 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -83.26 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -82.76 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -81.76 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -81.26 |  |
|  |  | 1~6 | dB | 10 |  | 10 |  | 13 |  | -4 |  |
| Propagation condition |  | 1~6 | - | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

##### A.4.7.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 2 and Cell 3 shall fulfil the Absolute requirement in section 10.1.4.1.1 and Relative requirement in section 10.1.4.1.2.

<End of Change 1>

<Start of Change 2>

#### A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 10.1.4.1.1 and 10.1.4.1.2 for intra frequency measurements with the testing configurations for NR cells in Table A.6.7.1.2.1-1.

Table A.6.7.1.2.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |

##### A.6.7.1.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on a different frequency than the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.6.7.1.2.2-1 below. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.6.7.1.2.2-1. The inter frequency measurements are supported by a measurement gap.

Table A.6.7.1.2.2-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| SSB ARFCN |  | 1~3 |  | freq1 |  | freq2 |  | freq1 |  | freq2 |  |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| Duplex mode |  | 1 |  | FDD |  |  | FDD |  |  |
|  |  | 2 |  | TDD |  |  | TDD |  |  |
|  |  | 3 |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD |  | - |  | SR.1.1 FDD |  | - |  |
|  |  | 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
|  |  | 3 |  | SR.2.1 FDD |  |  |  | SR.2.1 FDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD |  | - |  | CR.1.1 FDD |  | - |  |
|  |  | 2 |  | CR.1.1 TDD |  | - |  | CR.1.1 TDD |  | - |  |
|  |  | 3 |  | CR.2.1 FDD |  | - |  | CR.2.1 FDD |  | - |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD |  | - |  |
|  |  | 2 |  | CCR.1.1 TDD |  | - |  | CCR.1.1 TDD |  | - |  |
|  |  | 3 |  | CCR.2.1 TDD |  | - |  | CCR.2.1 TDD |  | - |  |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD |  | - |  |
|  |  | 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| SMTC configuration |  | 1~3 |  | SMTC.1 |  |  | SMTC.1 |  |  |
| Time offset between Cell 1 and Cell 2 |  | 1~3 | s | 3 |  |  | 3 |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |  | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1~3 | dBm/15kHz | -94.65 |  |  | ( for Channel 2 +8dB) |  | -117 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -116.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -116 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -115.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -115 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -114 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -113.5 |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1,2,4,5 | dBm/SSB SCS | -94.65 |  |  | ( for Channel 2 +8dB) |  | -117 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -116.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -116 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -115.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -115 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -114 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -113.5 |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | 3 |  | -91.65 |  |  | ( for Channel 2 +8dB) |  | -114 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | 1~3 | dB | 10 |  | 10 |  | 13 |  | -4 |  |
| SS-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1,2,4,5 | dBm/SCS | -84.65 |  |  | (RSRP for Cell 2 +25dB) |  | -121 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -120.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -119 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -118 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -117.5 |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | 3 |  | -81.65 |  |  | (RSRP for Cell 2 +25dB) |  | -124 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -123.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -122 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -121 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -120.5 |  |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1,2,4,5 | dBm/ 9.36MHz | -56.28 |  |  | (Io for Channel 2 +19.75dB) | -87.76 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -87.26 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -86.76 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -86.26 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -85.76 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -84.76 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -84.26 |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | 3 | dBm/ 38.16MHz | -50.19 |  |  | (Io for Channel 2 +19.75dB) | -84.76 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -84.26 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -83.76 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -83.26 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -82.76 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -81.76 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -81.26 |  |
|  |  | 1~3 | dB | 10 |  | 10 |  | 13 |  | -4 |  |
| Propagation condition |  | 1~3 | - | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

##### A.6.7.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the Absolute requirement in section 10.1.4.1.1 and Relative requirement in section 10.1.4.1.2.

<End of Change 2>