3GPP TSG-RAN WG4 Meeting #91	R4-1907395

Reno, Nevada, US, 13 May – 17 May 2019

Source:	Ericsson, Verizon

Title:	TP to TR 36.716-02-02: Addition of CA configuration for 2-48 UL

Agenda item:	7.5.2

Document for:	Approval

# Background

This contribution provides text proposal on the CA configurations with DL_2-48 and UL_ 2-48 as defined in [1].

# Text Proposal

##### ---Start of changes---

## 5.X	DL_2A-48A_UL_2A-48A_BCS0
DL_2A-48A-48A_UL_2A-48A_BCS0
DL_2A-48A-48C_UL_2A-48A_BCS0
DL_2A-48C_UL_2A-48A_BCS0
DL_2A-48D_UL_2A-48A_BCS0
DL_2A-48A-48D_UL_2A-48A_BCS0
DL_2A-48E_UL_2A-48A_BCS0
DL_2A-48A-48E_UL_2A-48A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-48A | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-48A-48A | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  |  |  |
| CA_2A-48C | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  |  |  |
| CA_2A-48D | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  |  |  |
| CA_2A-48A-48C | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 48 | See the CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 of [1] |  |  |  |  |  |  |  |
| CA_2A-48E | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 48 | See CA_48E Bandwidth Combination Set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  |  |  |
| CA_2A-48A-48D | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-48A-48E | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 48 | See the CA_48A-46E Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  |  |  |

### 5.X.2		ΔTIB,c and ΔRIB,c values

Same ΔTIB,c and ΔRIB,c values as in TR 36.715-02-02 [2] for DL_2A-48A_UL_ 2A-48A_BCS0.

Table 6.8.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-48A | 2 | 0.6 |
|  | 48 | 0.8 |

Table 6.8.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-48A | 2 | 0.2 |
|  | 48 | 0.5 |

### 5.x.2	UE co-existence studies

Table 5.x.2-1 lists Band 2 + Band 48 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2-1: Band x and Band y UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1850 | 1640 | 5400 | 5610 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 0 | 270 | 5190 | 5550 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7250 | 7520 | 8950 | 9310 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3700 | 3280 | 10800 | 11220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1850 | 2180 | 8740 | 9250 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9100 | 9430 | 12500 | 13010 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12950 | 12290 | 4090 | 3700 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16050 | 16710 | 10950 | 11340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7400 | 6830 | 1370 | 1850 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14350 | 14920 | 12650 | 13130 |

Based on Table 5.x.2-1, 4th order IMD may also fall into Rx frequencies of bands 2 as well as that 4th and 5th order IMD may fall into Rx frequencies of band 48.

Table 5.x.2-2 summarizes frequency ranges where harmonics occur due to Band 2 and Band 48 CA.

Table 5.x.2-2: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 | 9250 | 9550 |
| 48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |

It can be seen from Table 5.x.2-2 that there is harmonic interference for band 2 UL towards its own receive band 48.

Table 5.x.2-3 lists the protected bands required for the 2UL bands CA configuration.

Table 5.x.2-3: Protected bands for the 2UL bands CA configuration

| UL E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band |  | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_2A-48A |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |

### 5.X.4	REFSENS requirements

As can be seen in 5.x.2 there is MSD needed for IMD4 falling into band 2. These need to be covered in TS 36.101 as in table below. MSD value are same as for CA_2A-49A.

Table 7.3.1A-0f: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_2A-48A | 2 | 1852.5 | 5 | 25 | 1932.5 | [12] | FDD | IMD4 |
|  | 48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |

As also can be seen in 5.x.2 there is MSD needed for band 2 2nd harmonics falling into band 48 which are already covered in TS 36.101 Table 7.3.1A-0a and Table 7.3.1A-0b. However, Table 7.3.1A-0a and Table 7.3.1A-0b also need to include the new DL configurations added: CA_2A-48A-48D, CA_2A-48E and CA_2A-48A-48E.

##### ---End of changes---

# Reference

[1]	RP-190377, “WID revision: Rel16 LTE inter-band CA for 2 bands DL with 2 bands UL”, Huawei