3GPP TSG-RAN4 Meeting #91	R4-1907443

# Reno, USA, 13 – 17 May, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | CRNum | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-2 Complete FR2 MPR/A-MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - PC1 CA MPR should not be smaller than PC3 CA MPR. For cumulative aggregated bandwidth < 400 MHz, - Change MPR for DFT-s-OFDM 64QAM outer allocations from 7.5dB to 9dB (similar to corresponding PC3 value). - Change MPR for CP-OFDM 64QAM outer allocations from 7.5dB to 9dB (similar to corresponding PC3 value). - For PC4, - Add PC4 UL-MIMO A-MPR description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For PC1, - Change MPR for DFT-s-OFDM 64QAM outer allocations from 7.5dB to 9dB (similar to corresponding PC3 value). - Change MPR for CP-OFDM 64QAM outer allocations from 7.5dB to 9dB (similar to corresponding PC3 value). - For PC4, - Add PC4 UL-MIMO A-MPR description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not correct and not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.2, 6.2D.3.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 6.2A.2.2	Maximum output power reduction for power class 1

For power class 1, MPR for UL contiguous allocations within the cumulative aggregated bandwidth is defined as:

MPRC_CA = max(MPRWT_C_CA, MPRnarrow)

Where,

MPRnarrow=14.4 dB, when BWalloc,RB is less than or equal to 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT_C_CA is the maximum power reduction due to modulation orders, transmit bandwidth configurations, and waveform types. MPRWT_contiguous is defined in Table 6.2A.2.2-1.

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1

| Waveform Type |  | Cumulative aggregated channel bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | < 400 MHz | >=400 MHz and <800 MHz | >=800 MHz and <=1400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 7.7 | [8.2] |
|  | QPSK | ≤ 6.5 | 8.7 | [9.7] |
|  | 16 QAM | ≤ 6.5 | 8.7 | [9.2] |
|  | 64 QAM | ≤ 9.0 | 10.7 | [11.2] |
| CP-OFDM | QPSK | ≤ 6.5 | 8.7 | [8.7] |
|  | 16 QAM | ≤ 6.5 | 8.7 | [8.7] |
|  | 64 QAM | ≤ 9.0 | 10.7 | [11.2] |

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRC_CA.

For non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +  [14.4])

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of simultaneously transmitted UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth

< end of changes >

< start of changes >

#### 6.2D.3.4	UE maximum output power reduction with additional requirements for UL-MIMO for power class 4

For UE with UL-MIMO, the A-MPR values specified in subclause 6.2.3 shall apply to the maximum output power specified in Table 6.2D.1.4-1. The requirements shall be met with the UL-MIMO configurations specified in sub-section 6.2D.1.4.

< end of changes >