3GPP TSG-RAN WG4 Meeting #91                                   R4-1907452

# Reno, US, 13th– 17th May, 2019

Agenda Item:		9.20.3

Source: 		CMCC

Title: 	TP for TR37.825 for SAR requirements

Document for:	Approval

- Introduction

Revised WID on Power Class 2 UE for EN-DC (1 LTE band +1 NR band) were approved in RAN#83 meeting [1], and a WF on PC2 EN-DC (1 LTE TDD band +1 NR TDD band) was agreed in last RAN4 meeting.

This TP summarizes the UE behavior related to SAR for PC2 EN-DC (TDD+TDD).

- Reference

- [1]  RP-190315  Revised WID on Power Class 2 UE for EN-DC (1 LTE band +1 NR band).CMCC

- Text Proposal

<Start of Text Proposal for TR 37.825>

## 5.6   SAR

### 5.6.1	Intra-band EN-DC

If UE supports a different power class than the default UE power class for intra-band EN-DC band combination and the supported power class enables the higher maximum output power than that of the default power class:

-	if the LTE UL/DL configuration is 0 or 6; or

-   if the LTE UL/DL configuration is 1 and special subframe configuration is 0 or 5; or

-   if the IE p-maxUE-FR1 as defined in TS 38.331 [5] is provided and set to the maximum output power of the default power class or lower;

-  apply all requirements for the default power class and set the configured transmitted power as specified in sub-clause 5.4;

-	else

-    apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 5.4;

### 5.6.2	Inter-band EN-DC

If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class:

- if the field of UE capability maxUplinkDutyCycle-EN-DC is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or

- if the field of UE capability maxUplinkDutyCycle-EN-DC is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-EN-DC as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

-  if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower:

-  shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in sub-clause 6.2B.4;

-  else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-EN-DC as defined in TS 38.331; or

-  if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-EN-DC is absent. (The exact evaluation period is no less than one radio frame):

-  shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

<Start of Text Proposal for TR 37.825>