3GPP TSG-RAN WG4 Meeting # 91	R4-1907477

# Reno, US,  13th  - 17th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-1 on maximum aggregated bandwidth for NR CA configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In NR CA configurations for intra-band contiguous CA for FR1, the ‘Aggregated bandwidth’ is used for the defintion of NR CA configurations. The aggregated bandwidth is defined as the sum of all channel bandwidths of component carriers, without providing any additional information. To align with the agreement in TS 38.101-2, it is suggested that the ‘Maximum aggregated bandwidth’ to be also applied to intra-band CA in TS 38.101-1. Thus, the bandwidth of each component carrier in the related band will consist of a possible set of bandwidths. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the ‘Aggregated bandwidth’ to ‘Maximum aggregated BW’ for intra-band contiguous CA for FR1. - Include the bandwidth of each component carrier with a possible set of bandwidths. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistency with the definition of intra-band contiguous CA for FR2. - The CA configuration table in TS 38.101-1 will be too complicated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

<< Unchanged sections omitted >>

5.5	Configurations

5.5A	Configurations for CA

5.5A.1	Configurations for intra-band contiguous CA

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA for fallback group 1

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n77C CA_n78C CA_n79C |  | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
| CA_n77D, CA_n78D, CA_n79D |  | 50 | 60, 80, 100 | 100 |  |  | 300 | 0 |
|  |  | 60 | 60, 80 | 100 |  |  |  |  |
|  |  | 80 | 80, 90, 100 | 100 |  |  |  |  |
|  |  | 90 | 100 | 100 |  |  |  |  |
|  |  | 100 | 100 | 100 |  |  |  |  |
| CA_n77E, CA_n78E, CA_n79E |  | 50 | 60, 80, 100 | 100 | 100 |  | 400 | 0 |
|  |  | 60 | 60, 80 | 100 | 100 |  |  |  |
|  |  | 80 | 80, 90, 100 | 100 | 100 |  |  |  |
|  |  | 90 | 100 | 100 | 100 |  |  |  |
|  |  | 100 | 100 | 100 | 100 |  |  |  |
| NOTE: For the NR CA configuration with more than two component carriers, the bandwidths in a BCS which may introduce combinations more than requested unintentionally should be listed in a row separately. |  |  |  |  |  |  |  |  |

==============================================================

<< End of changes >>