3GPP TSG-RAN WG4 #91 Meeting                                        	R4-1907601

Reno, USA, 13 – 17 May, 2019

Source: 	LG Electronics

Title: 	Co-existence simulation results of CLI for FR1

Agenda item:	8.2.2

Document for:	Discussion

- Introduction

- Based on the WID, RAN4 needs to perform co-existence study to identify conditions of co-existence among different operators in adjacent channels.

  - (Target no or very minimal impact on RF requirement)

- Complete core and performance requirements in time [2]

- Companies are encouraged to provide co-existence simulation results based on agreed simulation scenarios and parameters.

In RAN4 #90 meeting, WF on co-existence study of Cross-Link Interference (CLI) [1] was agreed as follows:

To see the feasibility of operating dynamic TDD without changing RF requirement, NR deployment scenarios including macro-to-macro, indoor-to-macro, macro-to-indoor, and indoor-to-indoor are considered for co-existence evaluation. In this contribution, we only focus on evaluating the co-existence scenarios for FR1.

- Simulation scenarios and assumptions

- FR1 (4 GHz) and FR2 (30 GHz)

- Macro (aggressor) → Macro (victim)

- Indoor (aggressor) → Macro (victim)

- Indoor (aggressor) → Indoor (victim)

- FR1 (4 GHz) only

- Macro (aggressor) → Indoor (victim)

- FR2 (30 GHz) only

- Micro (aggressor) → Micro (victim)

In [1], the co-existence evaluation scenarios of CLI were agreed as follows:

For the co-existence evaluation of CLI, both BS-to-BS interference and UE-to-UE interference based on traffic adaptation (full buffer and low traffic mode) were considered to see how two different traffic mode may impact their performance when operating the dynamic TDD. The ACLR, ACS requirements and RF components of FR1 are reused from [3]. Moreover, the system bandwidth of 100 MHz was used for FR1 with 30 kHz subcarrier spacing. More details of the simulation assumptions and parameters can be found in [4].

- Simulation results

In this section, we summarize our simulation results of CLI for FR1 and only focus on evaluating co-existence scenarios of macro-to-macro, indoor-to-macro, and indoor-to-indoor. For this co-existence evaluation, we have assumed that there is a different TDD configuration between the victim cell and the aggressor cell. For the indoor-to-indoor scenario, the transmission direction of Downlink (DL) was applied to the victim cell and the same transmission direction of DL was applied to the aggressor cell as a baseline. And then, the transmission direction of Uplink (UL) was applied to the aggressor while keeping the same transmission direction of the victim cell as the DL. As shown in Figure 1, there was no performance degradation found on the victim DL when there was 100% misalignment transmission direction between the victim DL and the aggressor UL.

Figure 1 Indoor-to-Indoor (victim: DL) SINR result (24 dBm Tx power)

A similar approach was applied to the case where the transmission direction of the victim cell was UL. As Figure 2, no performance degradation was found when there was 100% misalignment transmission direction between the victim UL and the aggressor DL.

Figure 2 Indoor-to-Indoor (victim: UL) SINR result (24 dBm Tx power)

Observation 1: There was no performance degradation found in the indoor-to-indoor scenario.

For the indoor-to-indoor scenario, 30 dBm (4TxRx) power of local BS was considered to see how increased Tx power of local BS may impact its SINR and throughput performance when operating the dynamic TDD. As you can see in Figure 3 and Figure 4, there was no performance degradation and the result was almost the same as the case where 24 dBm (1TxRx) power of local BS was used.

Figure 3 Indoor-to-Indoor (victim: DL) SINR result (30 dBm Tx power)

Figure 4 Indoor-to-Indoor (victim: UL) SINR result (30 dBm Tx power)

Observation 2: Even though the Tx power of local BS was increased from 24 dBm to 30 dBm, there was no performance degradation found in the indoor-to-indoor scenario.

For the macro-to-macro scenario, the transmission direction of DL was applied to the victim cell and the same transmission direction of DL was also applied to the aggressor cell as the baseline. And then, the transmission direction of UL was applied to the aggressor while keeping the same transmission direction of the victim cell as the DL. As shown in Figure 5, there was performance degradation found on the victim DL when there was 100% misalignment transmission direction between the victim DL and the aggressor UL.

Figure 5 Macro-to-Macro SINR result (victim: DL)

When the victim cell was UL in the macro-to-macro scenario, a similar approach was applied. In Figure 6, performance degradation was also found when there was 100% misalignment transmission direction between the victim UL and the aggressor DL.

Figure 6 Macro-to-Macro SINR result (victim: UL)

Observation 3: There was performance degradation found in the macro-to-macro scenario.

Even though the low traffic mode was applied to the macro-to-macro scenario, we still found that there was performance degradation between the victim cell and the aggressor.

For the indoor-to-macro scenario, a similar approach was applied to the case where the transmission direction of the aggressor cell was totally opposite of the transmission direction of both UL/DL victim cell. Also, Tx powers of local BS (24 dBm and 30 dBm) were the same as the indoor-to-indoor scenario.

Figure 7 Indoor-to-Macro (victim: DL) SINR result (24 dBm Tx power)

Figure 8 Indoor-to-Macro (victim: UL) SINR result (24 dBm Tx power)

Figure 9 Indoor-to-Macro (victim: DL) SINR result (30 dBm Tx power)

Figure 10 Indoor-to-Macro (victim: UL) SINR result (30 dBm Tx power)

There was no performance degradation found in the indoor-to-macro scenario for FR1 when the Tx power of local BS was 24 dBm (1TxRx). Although the Tx power of local BS was increased to 30 dBm (assuming 4TxRx), the result of performance was the same as the case of 24 dBm Tx power of local BS.

- Observation 4: There was no performance degradation found in the indoor-to-macro scenario.

- Observation 5: Even though the Tx power of local BS was increased from 24 dBm to 30 dBm, there was no performance degradation found in the indoor-to-macro scenario.

In RAN4 #90Bis meeting, there was an agreement that companies are encouraged to bring their simulation results with the same format in [5]. By using the format table, we gather the simulation results of the scenarios including macro-to-macro, indoor-to-macro, and indoor-to-indoor as below.

Table 1 Macro-to-Macro (full buffer case)

| Source | Observation Point | Victim UL |  |  |  | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/ 50UL | DL | 50DL/ 50UL | DL | 50DL/ 50UL | UL | 50DL/ 50UL | UL |
| LGE | 5% | - | 2.63 | - | 44.99 | - | 2.5 | - | 31.33 |
|  | 50% | - | 0.46 | - | 3.13 | - | 0.04 | - | 0.20 |
|  | 95% | - | 0.05 | - | 0.25 | - | 0.20 | - | 0.47 |

Table 2 Macro-to-Macro (low traffic case)

| Source | Observation Point | Victim UL |  |  |  | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/ 50UL | DL | 50DL/ 50UL | DL | 50DL/ 50UL | UL | 50DL/ 50UL | UL |
| LGE | 5% | - | 1.08 | - | 21.72 | - | 0.58 |  | 7.61 |
|  | 50% | - | 0.08 | - | 0.52 | - | 0.07 |  | 0.33 |
|  | 95% | - | 0 | - | -0.02 | - | 0.55 |  | 1.33 |

Table 3 Indoor-to-Macro (full buffer case - 24 dBm Tx power of local BS in the indoor)

| Source | Observation Point | Victim UL |  |  |  | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/ 50UL | DL | 50DL/ 50UL | DL | 50DL/ 50UL | UL | 50DL/ 50UL | UL |
| LGE | 5% | - | -2.54 | - | -79.15 | - | -0.83 | - | -16.30 |
|  | 50% | - | -1.24 | - | -12.03 | - | -0.13 | - | -0.69 |
|  | 95% | - | -0.04 | - | -0.20 | - | 0.84 | - | 2.14 |

Table 4 Indoor-to-Macro (full buffer case - 30 dBm Tx power of local BS in the indoor)

| Source | Observation Point | Victim UL |  |  |  | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/ 50UL | DL | 50DL/ 50UL | DL | 50DL/ 50UL | UL | 50DL/ 50UL | UL |
| LGE | 5% | - | -2.31 | - | -70.18 | - | -0.73 | - | -14.31 |
|  | 50% | - | -1.44 | - | -14.28 | - | 0.06 | - | 0.32 |
|  | 95% | - | -0.05 | - | -0.22 | - | 0.81 | - | 2.07 |

Table 5 Indoor-to-Indoor (full buffer - 24 dBm Tx power of local BS in the indoor)

| Source | Observation Point | Victim UL |  |  |  | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/ 50UL | DL | 50DL/ 50UL | DL | 50DL/ 50UL | UL | 50DL/ 50UL | UL |
| LGE | 5% | - | -0.05 | - | -0.97 | - | -0.02 | - | -0.33 |
|  | 50% | - | -0.24 | - | -2.64 | - | -0.02 | - | -0.20 |
|  | 95% | - | -0.52 | - | -2.63 | - | 0.75 | - | 2.62 |

Table 6 Indoor-to-Indoor (low traffic - 24 dBm Tx power of local BS in the indoor)

| Source | Observation Point | Victim UL |  |  |  | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/ 50UL | DL | 50DL/ 50UL | DL | 50DL/ 50UL | UL | 50DL/ 50UL | UL |
| LGE | 5% | - | 0 | - | -0.19 | - | 0 | - | 0.02 |
|  | 50% | - | -0.03 | - | -0.31 | - | 0 | - | 0 |
|  | 95% | - | -0.03 | - | -0.15 | - | 0.31 | - | 0 |

Table 7 Indoor-to-Indoor (full buffer - 30 dBm Tx power of local BS in the indoor)

| Source | Observation Point | Victim UL |  |  |  | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/ 50UL | DL | 50DL/ 50UL | DL | 50DL/ 50UL | UL | 50DL/ 50UL | UL |
| LGE | 5% | - | -0.04 | - | -0.76 | - | -0.08 | - | -1.40 |
|  | 50% | - | -0.19 | - | -2.06 | - | -0.11 | - | -1.16 |
|  | 95% | - | -0.28 | - | -1.39 | - | -0.43 | - | -1.49 |

Table 8 Indoor-to-Indoor (low traffic - 30 dBm Tx power of local BS in the indoor)

| Source | Observation Point | Victim UL |  |  |  | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/ 50UL | DL | 50DL/ 50UL | DL | 50DL/ 50UL | UL | 50DL/ 50UL | UL |
| LGE | 5% | - | 0.01 | - | -0.15 | - | 0.01 | - | -0.24 |
|  | 50% | - | -0.02 | - | -0.02 | - | 0 | - | 0 |
|  | 95% | - | 0.03 | - | 0.03 | - | 0.37 | - | 0 |

- Conclusion

We have presented the simulation results of the co-existence evaluation of CLI for FR1. For the co-existence evaluation, three possible scenarios were considered: macro-to-macro, indoor-to-macro, and indoor-to-indoor. Moreover, to see how two different traffic may impact their performance when operating the dynamic TDD in adjacent channel, traffic adaptation (full buffer and low traffic mode) was considered for the scenarios of macro-to-macro and indoor-to-indoor. Based on the simulation results above, the following observations have been made:

- Observation 1: There was no performance degradation found in the indoor-to-indoor scenario.

- Observation 2: Even though the Tx power of local BS was increased from 24 dBm to 30 dBm, there was no performance degradation found in the indoor-to-indoor scenario.

- Observation 3: There was performance degradation found in the macro-to-macro scenario.

- Observation 4: There was no performance degradation found in the indoor-to-macro scenario.

- Observation 5: Even though the Tx power of local BS was increased from 24 dBm to 30 dBm, there was no performance degradation found in the indoor-to-macro scenario.

Proposal - Consider the following summary to be captured in TR 38.828:

For the evaluated adjacent channel macro-to-macro scenario, the followings are observed for Cross link interference (CLI) based on different traffic conditions (the full buffer and the low traffic mode) without any RF requirement change or interference mitigation compared to a fixed TDD UL-DL configuration:

- Performance degradation could be observed in both DL and UL co-existence.

For the evaluated adjacent channel indoor-to-macro scenario and indoor-to-indoor scenario, the followings are observed for Cross link interference (CLI) based on different traffic conditions (the full buffer and low traffic) without any RF requirement change or interference mitigation compared to a fixed TDD UL-DL configuration.

- No Performance degradation in both DL and UL co-existences

For CLI, it seems feasible to configure TDD DL/UL dynamically for the indoor-to-indoor scenario and the indoor-to-macro scenario.

References

- R4-1902506, WF on co-existence study of CLI

- R4-1900222, Work plan of Cross Link Interference (CLI) and Remote Interference Management (RIM) for NR

- TR 36.828, Further enhancements to LTE Time Division Duplex (TDD) for Downlink-Uplink (DL-UL) interference management and traffic adaptation

- TR 38.828 v0.1.0, Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR (Rel-16)

- R4-1905095, CLI summary of simulation results for co-existence study for CLI