3GPP TSG-RAN WG4 #91 Meeting                                        	R4-1907602

Reno, USA, 13 – 17 May, 2019

Source: 	LG Electronics

Title: 	Co-existence simulation results of CLI for FR2

Agenda item:	8.2.2

Document for:	Discussion

- Introduction

- Based on the WID, RAN4 needs to perform co-existence study to identify conditions of co-existence among different operators in adjacent channels.

  - (Target no or very minimal impact on RF requirement)

- Complete core and performance requirements in time [2]

- Companies are encouraged to provide co-existence simulation results based on agreed simulation scenarios and parameters.

In RAN4 #90 meeting, WF on co-existence study of Cross-Link Interference (CLI) [1] was agreed as follows:

To see the feasibility of operating dynamic TDD without changing RF requirement, NR deployment scenarios including macro-to-macro, indoor-to-macro, macro-to-indoor, and indoor-to-indoor are considered for co-existence evaluation. In this contribution, we only focus on evaluating the co-existence scenarios for FR2.

- Simulation scenarios and assumptions

- FR1 (4 GHz) and FR2 (30 GHz)

- Macro (aggressor) → Macro (victim)

- Indoor (aggressor) → Macro (victim)

- Indoor (aggressor) → Indoor (victim)

- FR1 (4 GHz) only

- Macro (aggressor) → Indoor (victim)

- FR2 (30 GHz) only

- Micro (aggressor) → Micro (victim)

In [1], the co-existence evaluation scenarios of CLI were agreed as follows:

BS-to-BS interference and UE-to-UE interference were considered for the co-existence evaluation of CLI. Moreover, to see how two different traffic mode may impact their performance when operating the dynamic TDD, we used both full buffer mode and low traffic mode in our simulation. The ACLR, ACS requirements, and RF components of FR2 are reused from [3]. The system bandwidth of 200 MHz was used for FR2 with 30 kHz subcarrier spacing. More details of the simulation assumptions and parameters can be found in [4].

- Simulation results

In this section, we summarize our simulation results of CLI for FR2 and only focus on evaluating co-existence of the macro-to-macro, indoor-to-indoor, indoor-to-macro. For the evaluation, we have assumed that there is a different TDD configuration between the victim cell and the aggressor cell. For the indoor-to-indoor scenario, the transmission direction of Downlink (DL) was applied to the victim cell and the same transmission direction of DL was applied to the aggressor cell as a baseline. And then, the transmission direction of Uplink (UL) was applied to the aggressor while keeping the same transmission direction of the victim cell as the DL. As shown in Figure 1, there was performance degradation found on the victim DL when there was 100% misalignment transmission direction between the victim DL and the aggressor UL.

Figure 1 Indoor-to-Indoor (victim: DL) SINR result

A similar approach was applied to the case where the transmission direction of the victim cell was UL. As Figure 2, no performance degradation was found on the victim cell UL when there was 100% misalignment transmission direction between the victim UL and the aggressor DL.

Figure 2 Indoor-to-Indoor (victim: UL) SINR result

Observation 1: When the full buffer mode was considered for the indoor-to-indoor scenario, there was performance degradation found only in the DL co-existence.

Observation 2: When the low traffic mode was considered for the indoor-to-indoor scenario, there was no performance degradation found in both DL and UL co-existences.

For the macro-to-macro scenario, the transmission direction of DL was applied to the victim cell and the same transmission direction of DL was also applied to the aggressor cell as the baseline. And then, the transmission direction of UL was applied to the aggressor while keeping the same transmission direction of the victim cell as the DL. As shown in Figure 3, there was performance degradation found on the victim DL when there was 100% misalignment transmission direction between the victim DL and the aggressor UL.

Figure 3 Macro-to-Macro SINR result (victim: DL)

When the victim cell was UL in the macro-to-macro scenario, a similar approach was applied. In Figure 4, there was performance degradation found when there was 100% misalignment transmission direction between the victim UL and the aggressor DL.

Figure 4 Macro-to-Macro SINR result (victim: UL)

Observation 3: There was performance degradation found in the Macro-to-Macro scenario.

For the indoor-to-macro scenario, the transmission direction of DL was applied to the victim cell and the same transmission direction of DL was also applied to the aggressor cell as the baseline. And then, the transmission direction of UL was applied to the aggressor while keeping the same transmission direction of the victim cell as the DL. As shown in Figure 5, there was no performance degradation found on the victim DL when there was 100% misalignment transmission direction between the victim DL and the aggressor UL.

Figure 5 Indoor-to-Macro SINR result (victim: DL)

When the victim cell was UL in the indoor-to-macro scenario, a similar approach was applied. In Figure 6, there was no performance degradation found when there was 100% misalignment transmission direction between the victim UL and the aggressor DL.

Figure 6 Indoor-to-Macro SINR result (victim: UL)

- Observation 4: For the indoor-to-macro, there was no performance degradation found in the DL and UL co-existences.

In RAN4 #90Bis meeting, there was an agreement that companies are encouraged to bring their simulation results with the same format in [5]. By using the format table, we gather the simulation results of the scenarios including macro-to-macro and indoor-to-indoor as below.

Table 1 Macro-to-Macro (full buffer case)

| Source | Observation Point | Victim UL |  |  |  | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/ 50UL | DL | 50DL/ 50UL | DL | 50DL/ 50UL | UL | 50DL/ 50UL | UL |
| LGE | 5% | - | 0.56 | - | 10.04 | - | 5.50 | - | 37.08 |
|  | 50% | - | 0.79 | - | 5.18 | - | 2.90 | - | 9.50 |
|  | 95% | - | 0.01 | - | 0.08 | - | 2.19 | - | 4.96 |

Table 2 Indoor-to-Indoor (full buffer case)

| Source | Observation Point | Victim UL |  |  |  | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/ 50UL | DL | 50DL/ 50UL | DL | 50DL/ 50UL | UL | 50DL/ 50UL | UL |
| LGE | 5% | - | -0.13 | - | -2.17 | - | 1.3 | - | 19.72 |
|  | 50% | - | -0.11 | - | -1.10 | - | 0.46 | - | 4.70 |
|  | 95% | - | -0.09 | - | -0.55 | - | 0.17 | - | 1.01 |

Table 3 Indoor-to-Indoor (low traffic case)

| Source | Observation Point | Victim UL |  |  |  | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/ 50UL | DL | 50DL/ 50UL | DL | 50DL/ 50UL | UL | 50DL/ 50UL | UL |
| LGE | 5% | - | -0.12 | - | -1.98 | - | 0.16 | - | 2.61 |
|  | 50% | - | -0.08 | - | -0.78 | - | -0.02 | - | 0.19 |
|  | 95% | - | -0.04 | - | -0.27 | - | -0.09 | - | 0.55 |

Table 4. Indoor-to-Macro (full buffer case)

| Source | Observation Point | Victim UL |  |  |  | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/ 50UL | DL | 50DL/ 50UL | DL | 50DL/ 50UL | UL | 50DL/ 50UL | UL |
| LGE | 5% | - | -1.03 | - | -20.33 | - | 0.14 | - | 0.92 |
|  | 50% | - | -0.19 | - | -1.23 | - | 0.25 | - | 0.81 |
|  | 95% | - | 0 | - | -0.03 | - | 0.29 | - | 0.64 |

- Conclusion

We have presented the simulation results of the co-existence evaluation of CLI for FR2. For the co-existence evaluation, two scenarios were considered: macro-to-macro and indoor-to-indoor. Moreover, to see how two different traffic may impact their performance when operating the dynamic TDD in adjacent channel, traffic adaptation (full buffer and low traffic mode) was considered for the indoor-to-indoor scenario. Based on the simulation results above, the following observations have been made:

- Observation 1: When the full buffer mode was considered for the indoor-to-indoor scenario, there was performance degradation found only in the DL co-existence.

- Observation 2: When the low traffic mode was considered for the indoor-to-indoor scenario, there was slight performance degradation found in both DL and UL co-existences.

- Observation 3: There was performance degradation found in the macro-to-macro scenario.

- Observation 4: For the indoor-to-macro, there was no performance degradation found in the DL and UL co-existences.

Proposal - Consider the following summary to be captured in TR 38.828:

For the evaluated adjacent channel macro-to-macro scenario, the followings are observed for Cross link interference (CLI) without any RF requirement change or interference mitigation compared to a fixed TDD UL-DL configuration:

- Performance degradation could be observed in both DL and UL co-existence.

For the evaluated adjacent channel indoor-to-indoor scenario, the followings are observed for Cross link interference (CLI) based on different traffic conditions (the full buffer and the low traffic) without any RF requirement change or interference mitigation compared to a fixed TDD UL-DL configuration.

- When the full buffer mode is selected, its performance degradation could be observed only in the DL-co-existence.

- No Performance degradation in both DL and UL co-existences based on the low traffic mode.

For the evaluated adjacent channel indoor-to-macro scenario, the following is observed for Cross link interference (CLI) without any RF requirement change or interference mitigation compared to a fixed TDD UL-DL configuration:

- No performance degradation in both DL and UL co-existence.

For CLI based on the traffic adaptation, it could be possible to configure TDD DL/UL dynamically for the indoor-to-indoor scenario. Moreover, it seems feasible to configure TDD DL/UL dynamically for the indoor-to-macro scenario.

References

- R4-1902506, WF on co-existence study of CLI

- R4-1900222, Work plan of Cross Link Interference (CLI) and Remote Interference Management (RIM) for NR

- TR 38.803, Radio Frequency (RF) and co-existence aspects (Release 14)

- TR 38.828 v0.1.0, Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR (Rel-16)

- R4-1905095, CLI summary of simulation results for co-existence study for CLI