3GPP TSG RAN WG4 Meeting #91	R4-1907622

Reno, US, May 13-17, 2019

Source: 	Huawei, HiSilicon

Title: 	AH minutes on NR BS conformance testing

Agenda Item:	6.8.1

Document for:	Approval

# Time alignment error

## 11 Contributions

| Source | Contribution | Spec / Subclause | Comment |
| --- | --- | --- | --- |
| 6.7.2.4.1 - TAE [NR_newRAT-Core] |  |  |  |
| Huawei, HiSilicon | R4-1905560 | 38.104: 9.6.3.3 | Change: For intra-band non-contiguous CA, for co-located deployment TAE for FR2 is updated to “260 ns” from “3 us” |
| Huawei, HiSilicon | R4-1905561 | 38.141-2: 6.6.4.5.2 | Change: For intra-band non-contiguous CA, for co-located deployment TAE for FR2 is updated to “285 ns” from “3.025 us” |
| Ericsson | R4-1905877 | discussion | Only collocated intra-band deployment is allowed for both contiguous and non-contiguous CA in Rel-15 Update BS TAE requirement for intra-band NC CA for FR1 as 260ns Update BS TAE requirement for intra-band NC CA for FR2 as 260ns |
| Ericsson | R4-1905878 | 38.104: 6.5.3.2, 9.6.3.3 | Clarification added in Section 6.5.3.2, 9.6.3.2 and 9.6.3.3. |
| Ericsson | R4-1905881 | WF | Material presented in background |
| ZTE | R4-1906832 | Discussion | Timing circuits / implementations Provides breakdown for 3 cases, error conditions |
| ZTE | R4-1906907 | 38.104: 6.5.3.2, 9.6.3.2, 9.6.3.3 | 3 us FR1, Update the inter-band CA TAE requirement for FR1 with FR1+FR1 and FR1+FR2 cases; 3 us 3 us FR2 Update the inter-band CA for FR2 with FR1+FR2; 3 us |
| ZTE | R4-1906908 | 38.141-1: 6.5.4.5 | Update the inter-band CA TAE requirement for FR1 with FR1+FR1 and FR1+FR2 cases; 3 us |
| ZTE | R4-1906909 | 38.141-2: 6.6.4.5.1; 6.6.4.5.2 | Update the inter-band CA TAE requirement for FR1 with FR1+FR1 and FR1+FR2 cases; 3 us Update the inter-band CA for FR2 with FR1+FR2; 3 us |
| Ericsson | R4-1906979 | Unavailable? |  |
| NTT DoCoMo | R4-1906992 | discussion | Proposal 1: TAE for intra-band non-contiguous CA is applied for only intra-RRH Proposal 2: If proposal 1 is agreed, revisit TAE for intra-band non-contiguous CA to 260ns for FR1 and 130ns for FR2. Proposal 3: For UE MRTD for intra-band non-contiguous CA, consider different and larger value than TAE as much as possible to keep the flexibility of the implementation and deployment of the BS (e.g., MRTD = Half CP length). |

### Background

Ericsson WF R4-1905881 captures some of the discussion from noted document R4-1904851.

| Condition | 3 us | 260 ns | 130 ns |
| --- | --- | --- | --- |
| BS TAE for FR1 Intra-band non-contiguous CA to be [3]us | ZTE, HW, Samsung, CATT | Ericsson, Intel, Mediatek, Qualcomm |  |
| BS TAE for FR2 Intra-band non-contiguous CA to be [3]us | ZTE, HW | Nokia, Ericsson, MediaTek, Intel, Samsung | Qualcomm |

- What should value be?

- Proposal: TAE for FR1 and FR2 decoupled

- Impact to 38.133 subclause 7.6.4 (what value is proposed for intra-band NC CA)

- Impact to 38.133 subclause 7.6.3 (what value is proposed intra-band synchronous EN-DC)

From NTT

| CA configuration |  | BS TAE |  |  | UE MRTD |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | E-UTRA TS36.104 [3] | NR TS38.104 [4] |  | E-UTRA TS36.133 [5] | NR TS38.133 [6] |  |
|  |  |  | FR1 | FR2 |  | FR1 | FR2 |
| Intra-band CA | Contiguous | 130ns | 260ns | 130ns | N/A | N/A | N/A |
|  | Non-contiguous | 260ns | 3us | 3us | 30.26us | 3us | 3us |
| Inter-band CA |  | 260ns | 3us | 3us | 30.26us | 33us | 8us |

### WF

- BS TAE intra-band non-contiguous CA values for FR1 and FR2 are decoupled

  - Should phrase “co-located” in requirements be in requirements

    - Yes: Huawei

    - implied:, Ericsson, ZTE

    - (NTT) TAE for intra-band non-contiguous CA is applied for only intra-RRH

- BS TAE intra-band non-contiguous CA values for FR1

  - FR1

    - 3 us: ZTE

    - 260 ns Ericsson, ZTE (case A)

- BS TAE intra-band non-contiguous CA values for FR2

  - FR2 (impacts 38.104 9.6.3/3 , 38.141-2

    - 3 us: ZTE

    - 260 ns Ericsson

    - [260 / 285] ns: HW

- BS TAE inter-band CA values (ZTE)

  - Is clarification of CA types needed?

    - FR1: combinations of FR1+FR2, FR1+FR1

    - FR2: combinations of FR1+FR2

- What is the impact to MRTD values

## Removal of TX Diversity

### 22 Contributions

Light blue cells: tx diversity removal

| Source | Contribution | Spec / Subclause | Comment |  |
| --- | --- | --- | --- | --- |
| 6.7.2.4.1 - TAE [NR_newRAT-Core] |  |  |  |  |
| ZTE | R4-1906910 | 37.145-1: 6.5.3.5.4 | Removal of “tx diversity” in spec |  |
| ZTE | R4-1906911 | 37.145-2: 6.6.3.5.3 | Removal of tx diversity |  |
| 6.8.2.1 - eAAS specifications [NR_newRAT-Perf/Core] |  |  |  |  |
| CATT | R4-1905391 | 37.105: 6.5.3.1, 9.6.3.1 | TX diversity and CRS for NR is excluded for TAE requirement. |  |
| CATT | R4-1905392 | 37.145-1: 6.5.3.1, 6.5.3.4.2.4, 6.5.3.5.4 | TX diversity and CRS for NR is excluded for TAE requirement. |  |
| CATT | R4-1905393 | 37.145-2: 6.6.3.1, 6.6.3.4.2.3, 6.6.3.5.3 | TX diversity and CRS for NR is excluded for TAE requirement. |  |
| Huawei, HiSilicon | R4-1906717 | 37.145-1: 6.5.3.4.2.4, 6.5.3.5.4 | The text for “TX diversity” is removed, port information added |  |
| Huawei, HiSilicon | R4-1906718 | 37.145-2: 6.6.3.1, 6.6.3.4.2.3, 6.6.3.5.3 | The text for “TX diversity” is removed, port information added |  |
| Huawei, HiSilicon | R4-1906721 | Discussion paper | Discusses intention of changes (MIMO ports, no changes to 37.105) |  |
| Huawei, HiSilicon | R4-1906726 | 37.843: 5.5.3.1, 10.2.7.1 | The text for “TX diversity” is removed |  |
| 6.8.2.3 - NR conformance testing specifications |  |  |  |  |
| Huawei, HiSilicon | R4-1906725 | 38.817-02 6.5.3.1, 9.6.3, 9.6.4 | Removal of tx diversity |  |
| 6.8.3 - Common for 38.141-1 and 38.141-2 [NR_newRAT-Perf] |  |  |  |
| CATT | R4-1905388 | Discussion of implementation | Adding “Rank 2 (double layers) for NR-FR1-TM1.1” to Data content of Physical channels and Signals for NR-FR1-TM in TS38.141-1. And update the test procedure in 38.141-1 as to measure time alignment error between the different PDSCH demodulation reference signals on different antenna ports on different connectors. Adding “Rank 2 (double layers) for NR-FR2-TM1.1” to Data content of Physical channels and Signals for NR-FR2-TM in TS38.141-2. And update the test procedure in 38.141-2 as to measure time alignment error between the different PDSCH demodulation reference signals on different antenna ports on different beams. Proposal 3: Remove TX Diversity and replace CRS with PDSCH DMRS for NR for TAE requirement in TS37.105, TS37.145-1 and TS37.145-2. |
| Ericsson | R4-1906686 | Discussion of implementation | Support CDM |
| 6.8.3.3 - Test models [NR_newRAT-Perf] |  |  |  |
| ZTE | R4-1906812 | Discussion of WF | Either FDM or CDM |
| ZTE | R4-1906905 | CR: TM for 38.141-1 | Add sentence about port |
| ZTE | R4-1906906 | CR: TM for 38.141-2 | Add sentence about port |
| Huawei, HiSilicon | R4-1906947 | Discussion of TM | Ports and other mapping |
| 6.8.4.2 - TP to TS38.141-1 |  |  |  |  |
| CATT | R4-1905389 | CR: 38.141-1 | Add sentence about ports |  |
| Huawei, HiSilicon | R4-1906719 | CR: 38.141-1 | Implementation details for TM and port |  |
| Ericsson | R4-1907013 | CR: 38.141-1 | Port 0, 2 |  |
| 6.8.5.1 - TP to TS38.141-2 |  |  |  |  |
| CATT | R4-1905390 | CR: 38.141-2 | Add sentence about ports |  |
| Huawei, HiSilicon | R4-1906720 | CR: 38.141-2 | Implementation details for TM and port |  |
| 6.8.5.3.1.2 - TP to TS 38.141-2 [NR_newRAT-Perf] |  |  |  |  |
| Ericsson | R4-1907014 | CR: 38.141-2 | Port 0, 2 |  |

### Background

WF R4-1905123 (90B)

Companies are encouraged to provide implementation details for:

- Option 1. Implement TAE testing using

  - Option 1A: MIMO: Rank 2 transmission (in which CDM applied to the DMRS)

  - Option 1D: implement CDM or FDM of DMRS to emulate TX diversity

- Option 2. Introduce Tx polarization if supported for NR based specifications based on future work

  - Analysis on how TX polarization can be applied to TAE testing is encouraged by companies prior to implementation of part of the TAE requirement

  - Implement TAE testing using

    - Option 2D: implement CDM or FDM of DMRS

Apply the same changes to NR parts in 37 series in next meeting

### WF for TM design

Observation: how to implement TAE in test models affects changes in specification

Scope

- Determine type of DMRS

  - CDM: CATT, Ericsson, Huawei, ZTE

    - ports 0, 1 (1000/1001): CATT, Huawei, ZTE

  - FDM: ports 0, 2 (1000, 1002): Ericsson

  - Specify mapping rule for 2 layers

CDM of DM-RS is agreed together with the use of (1000/1001) to indicate ports

- Determine type of PT-RS

Test model

- 38.141-1 (solution to 38.141-1 applies to 38.141-2)

  - Specify port : ZTE, CATT, Huawei

    - Different interpretations of how to specify ports / layers

  - Specify mapping from codeword, layer, port (Huawei)

Test procedures

- 38.141-1

  - clarify procedure (step 4 in 6.5.4.4.2) to discuss ports / connectors

  - Clarify port / rank

R4-1906906 is revised to (note some changes to the general sections)

R4-1905389 is revised to (note some changes to the general sections)

### WF for AAS / other changes

- Technical question: should absolute or relative port number be used? Or refer to test model?

- 37.105

  - Distinguish between NR / LTE: no change (Huawei) vs detailed reference signal (CATT)

- 37.145-1:

  - Distinguish between NR / LTE: minimal (Huawei) vs detailed (CATT)

  - Removed tx diversity: ZTE (R4-1906910), CATT (R4-1905392)

  - Introduced port info: Huawei (0, 1), CATT (1000, 1001)

- 37.145-2

  - Distinguish between NR / LTE: minimal (Huawei) vs detailed (CATT)

  - Removed tx diversity: ZTE (R4-1906911), CATT (R4-1905393)

  - Introduced port info: Huawei (0, 1), CATT (1000, 1001)

- 37.843

  - Rational for removing tx diversity (Huawei)

- 38.817-02

  - Rational for removing tx diversity (Huawei)

R4-1905391 is revised to

R4-1906718 is revised to

R4-1906726 is revised to

R4-1906725 is revised to

# EVM

## Signal  Waveform

### 4 Contributions

| Source | Contribution | Spec / Subclause | Comment |
| --- | --- | --- | --- |
| 6.7.2.4.2 - Others [NR_newRAT-Core] |  |  |  |
| Nokia, Nokia Shanghai Bell | R4-1906006 | 38.104: 6.5.2.1, 9.6.2.1, B.6, C.6 | Replacing the term “reference signal” to the alternative term “reference waveform” in the sections: 6.5.2, B.5.1, B.6, C.5.1, C.6 |
| 6.8.4.2 - TP to TS38.141-1 |  |  |  |
| Nokia, Nokia Shanghai Bell | R4-1906007 | 38.141-1 | Replacing the term “reference signal” to the alternative term “reference waveform” |
| 6.8.3 - Common for 38.141-1 and 38.141-2 [NR_newRAT-Perf] |  |  |  |
| Nokia, Nokia Shanghai Bell | R4-1906005 | discussion | Proposal: For consistency, rename the term “reference signal” in the EVM context where it is not used in the sense of TS 38.211 to the alternative term “reference waveform” in the NR BS core (TS 38.104) and test specifications (TS 38.141-1 and TS 38.141-2) in Rel-15 (including NR TR 38.817-02). |
| 6.8.5.1 - Common to FR1 and FR2 radiated conformance testing [NR_newRAT-Perf] |  |  |  |
| Nokia, Nokia Shanghai Bell | R4-1906008 | 38.141-2 | Replacing the term “reference signal” to the alternative term “reference waveform” |

### Discussion

Replace “reference signal” by “reference waveform” to avoid confusion with DM-RS

Keysight: it is ok to just change “reference signal” to “reference waveform” in both 38.104 and 38.141

AH chair: this is not a good idea since it looks confusing with “Modulation quality is defined by the difference between the measured carrier signal and a reference waveform.” with this change.

### WF

Change “reference signal” to “ideal signal”

R4-1906006, R4-1906007, R4-1906008 are revised to reflect this change.

## EVM Core cleanup

### 3 Contributions

| Source | Contribution | Spec / Subclause | Comment |
| --- | --- | --- | --- |
| 6.7.2.4.2 - Others [NR_newRAT-Core] |  |  |  |
| NTT DoCoMo | R4-1907156 | 38.104: 6.5.2.3, 9.6.2.3.1 | The minimum symbol allocation of CORESET is one symbol, and PUSCH can be assigned to the second symbol of a slot. Basically, as a core requirement, EVM requirements should be complied in all PUSCH resources. Therefore, it is not necessary to describe the start symbol of EVM measurement in the core specification. Removal sentence about starting point |
| 6.8.2.3 - NR conformance testing specifications |  |  |  |
| Huawei, HiSilicon | R4-1906722 | Discussion | Motivation for evm cleanup. Examine historical aspect / comparison of 36.104 |
| Huawei, HiSilicon | R4-1906723 | 38.104 6.5.2.3, 9.6.2.3.1 | Evm core cleanup |

### Discussion

Remove procedural aspects from the core spec

### WF

WF:

- Remove “EVM measurements start on the third symbol of a slot (l=2) when the first symbol of that slot is a downlink symbol.” HW, NTT, Keysight, ZTE, Ericsson

## Annex F

### 7 Contributions

| Source | Contribution | Spec / Subclause | Comment |
| --- | --- | --- | --- |
| 6.8.4.2 - TP to TS38.141-1 |  |  |  |
| Huawei, HiSilicon | R4-1906724 | Annex F topics | See below |
| ZTE | R4-1906932 | Annex F topics | FR1 See below |
| ZTE | R4-1906933 | Annex F topics | FR2 see below |
| 6.8.3 - Common for 38.141-1 and 38.141-2 [NR_newRAT-Perf] |  |  |  |
| Nokia, Nokia Shanghai Bell | R4-1906015 | Annex F topics | see below Rename the term “reference signal” in the EVM context where it is not used in the sense of TS 38.211 to the alternative term “reference waveform” in Annexes F |
| Ericsson | R4-1906684 | Annex F topics | see below |
| Huawei | R4-1907034 | Annex F |  |
| Huawei | R4-1907035 | Annex F |  |

### Background

- Organizational issues for Annex F (avoiding “void” sections)

- WF on selected topics (R4-1905127)

  - Example NRB Values:

  - Number of DL frames in Averaged EVM calculation:

  - Moving average window for EVM:

  - TDD EVM Measurements:

- For flexible slot in TDD configuration, it is needed to be determined what flexible slots should be excluded from the EVM measurements due to insufficient PT-RS.

- The same number of measurement slots would need to be considered for measured average as some cases 10 slots of TDD is not equal to 10 ms measurement period.

  - TS TX Power (RSTP) and OFDM Symbol TX power (OSTP)

  - RB Definition

### summary on Annex F

|  | Options | positions |
| --- | --- | --- |
| Example NRB Values | - Do not list any specific transmission bandwidth configurations and leave the text general to apply for all CBW/SCS | Huawei: reference to sampling tables and how to handle longer CP ZTE: also how to handle longer CP Ericsson |
|  | - Use the largest CBW supported for each SCS |  |
|  | - Use a selected value FR1 | ZTE, Nokia: 100MHz BW, 30kHz SCS; |
|  | - Use a selected value FR2 | ZTE: 400MHz BW, 120kHz SCS Nokia 400MHz BW, 120kHz SCS |
| Number of DL frames in Averaged EVM calculation: | Use formula | Huawei: formula unclear whether parameters come from, formula not applicable to FR2 ZTE, Nokia Ericsson: use SCS instead of mu |
|  | other |  |
| Moving average window for EVM | Use 19 for FR1 | Huawei: note impact to different RB sizes, general formula provided ZTE: 19 for FR1 Nokia, Ericsson |
|  | Use 19 for FR2 | Huawei: Nokia, Ericsson |
|  | Other | ZTE: 37 for FR2 |
| TDD EVM Measurements | Skip Special slot | Huawei |
|  | Number of RE | Huawei: can be based on RNTI |
| TS TX Power (RSTP) and OFDM Symbol TX power (OSTP) | Keep RSTP |  |
|  | Remove RSTP | Huawei, ZTE, Nokia |
|  | Keep OSTP | Huawei , ZTE, Nokia (requirement) Note: use 4th symbol: Huawei, ZTE Note: ZTE proposes to scale measurement by SCS |
|  | Remove OSTP |  |
| RB Definition | Keep number of RE/slot | Huawei: can be based on RNTI |
|  | Other | ZTE: do not use pair of RBs/slot Ericsson Keep ideology RB bundling for EVM definition from LTE, and adopt to the NR structure. |

  - It is agreed to capture some example NRB Values, i.e. 100MHz BW, 30kHz SCS for FR1 and 400MHz BW, 120kHz SCS

  - Number of DL frames in Averaged EVM calculation: agreed to use the formula. Exact formula is FFS.

  - Moving average window for EVM: 19 for both FR1 and FR2

  - TDD EVM Measurements:

- It is agreed to include the flexible slots in the EVM measurements.

  - It is agreed to remove TS TX Power (RETP) and keep OFDM Symbol TX power (OSTP)

    - [Use the fourth symbol] to take the measurements.

  - Agreed to clarify RB Definition

R4-1907034 and R4-1907035 are revised to (taking into account comments from Keysignt and ZTE)

Keysight to provide a CR for 38.104 (tdoc number to be assigned)

ZTE to provide a CR to TR38.817-02 (tdoc number to be assigned)

# Test model

## EVM measurement

### 3 Contributions

| Source | Contribution | Spec / Subclause | Comment |
| --- | --- | --- | --- |
| Keysight Technologies | R4-1906666 | Evm discussion |  |
| Keysight Technologies | R4-1906670 | 38.141-1 4.9.2.2, 4.9.2.3, 6.5.3.5 | EVM and test model |
| Keysight Technologies | R4-1906672 | 38.141-2 4.9.2.2, 4.9.2.3, 6.6.3.5 | EVM and test model |

### Open topics

- Is filler RNTI used for measurements

  - TM3.3: should filler pdsch be excluded from EVM measurements

- Are symbols 0 / 1 excluded from EVM measurements

Proposal is to remove text from 6.5.3.5 test requirements subclause which is for 1st and 2nd symbol exclusion

•	Change RNTI value to be 0 (zero) for PDSCH defined in TM2 and 2a (Table 4.9.2.3.2-1 for 38.141-1, text for 38.141-2) for simplification of specifying EVM measurement target PDSCH.

•	specify EVM measurement target as all PDSCH resource blocks with RNTI=0, add this text instead of exclusion text for 1st and 2nd symbol

- Are TDD slots with fewer number of downlink symbols excluded from measurements

Remove text to exclude shorter length slot where less number of reference signals exist than other slot from EVM measurement in 6.5.3.5 Test requirements subclause of TS38.141.-1 and 6.6.3.5 in TS38.141-2

### WF

For EVM measurements:

- Include symbols 0 and 1 in the PDSCH measurement excluding the PDCCH, for all test cases except TM2

- For TM3.1 and TM3.1a, include filler PDSCH from the 3rd symbol to the last symbol for PRBs 0, 1, 2, excluding the cases where boosting/deboosting is applied. The filler bits should have the same modulation as RNTI 0

R4-1906670 and R4-1906672 are revised to

## RBG

### 3 Contributions

| Source | Contribution | Spec / Subclause | Comment |
| --- | --- | --- | --- |
| Keysight Technologies | R4-1906592 | Discussion paper for RBG | Propose using RBG alignment for TM1.2, TM3.2, TM3.3 |
| Keysight Technologies | R4-1906594 | CR |  |
| Huawei, HiSilicon | R4-1907106 | Discussion paper for RBG | Based on discussions with Keysight, paper examines the technical aspects of TBG alignment |

### Background

The introduction of filler PDSCH complicated the use of RBGs. Keysight’s concern is about implementation

### WF

R4-1906594 is revised taking into account proposals from R4-1907106

Keysight to provide a CR for 38.141-2 as well

## Correction

### 2 Contributions

| Source | Contribution | Spec / Subclause | Comment |
| --- | --- | --- | --- |
|  |  |  | 6.8.3.3 - Test models [NR_newRAT-Perf] |
| Nokia, Nokia Shanghai Bell | R4-1906009 |  | “with n_"RNTI" =0"” added to value in third raw in table 4.9.2.2.1-1 In subclause 4.9.2.3.2 following bullet is deleted: “PDSCH resource allocation type 1 according to TS 38.214 [18]”. |
| Nokia, Nokia Shanghai Bell | R4-1906010 |  | “with n_"RNTI" =0"” added to value in third raw in table 4.9.2.2.1-1 In subclause 4.9.2.3 “ = 0” is deleted. In subcluase 4.9.2.3.1 space added after “ = 0” In subclause 4.9.2.3.2 following bullet is deleted: “PDSCH resource allocation type 1 according to TS 38.214 [18]”. |

### Discussion

- Update 4.9.2.2.1-1 to specify RNTI of filler PDSCH (Nokia, HW)

- Remove resource allocation type

### WF

R4-1906009 and R4-1906010 are noted.

# FR2 off power

## 8 contributions

| R4-1905409 | Discussion on MU for FR2 OTA Tx transient period measurement | CMCC | Discussion paper – requests that MU be kept as low as possible and that TE vendors supply reasoning for MU values. I think Keysight have done this and latest proposed values are not significantly different to the previous square bracket values. |
| --- | --- | --- | --- |
| R4-1905898 | CR to TR38.817-02 on TT and MU tables for FR2 OFF power | Huawei | CR to implement current agreement – will need updating if we agree new keysight values |
| R4-1905899 | draft CR to TS 38.141-2 - FR2 TX OFF update procedure and MU and TT tables. | Huawei | Draft CR to implement current agreement – will need updating if we agree new keysight values |
| R4-1906599 | FR2 Tx Off power measurement MU table | Keysight Technologies UK Ltd, Rohde & Schwarz | Test equipment values changes slightly from previous square bracket agreement TE values: 24.25<f<29.5GHz from 1.4 to 1.25 37<f<40GHz from 1.4 to 1.45 MU values: 24.25<f<29.5GHz from 3.2 to 2.9 37<f<40GHz from 3.2 to 3.25 |
| R4-1906641 | Draft CR to TS38.141-2 on TT and MU tables for FR2 Tx OFF power measurement (4.1.2.2, C.1) | Keysight Technologies UK Ltd, Rohde & Schwarz | Update the TS with proposed values – same as ours (but with different values) – this draft CR misses the test req. section |
| R4-1906664 | Draft CR to TR38.817-02 on MU budget table for FR2 Tx OFF power (12.3.6) | Keysight Technologies UK Ltd, Rohde & Schwarz | Update the TR with proposed values – same as ours (but with different values) |
| R4-1906955 | CR to TR 38.817-02: updating the FR2 OTA transmit ON/OFF column in Table 12.10.1-1 (12.10.1) | Nokia, Nokia Shanghai Bell | remove TX OFF from the TRP table |
| R4-1906167 | Draft CR to TS 38.141-2: Editorial cleanup of OTA transmit ON/OFF power in subclause 6.5 | Ericsson | 1. Align structure and text according to TS 38.141-1, subclause 6.4 2. Removal of [] 3. Removal of TT |

## Summary

Keysight have updated the TE MU values,

for 24.25 to 29.5GHz the value is lower so the final MU value is lower

for 37 to 40GHz the value is 0.05dB higher, the final MU value is also 0.05dB higher, rounded to 1d.p. this is 0.1dB higher than existing value (3.3 rather than 3.2)

## Discussion

Need to capture the reason why in the test specs FR2 OTA tx on/off power is not captured. One way is to capture some clarification text in the TR.

## WF

Keysight values are acceptable

R4-1906167 is revised to contain only the structural and text changes.

R4-1905898 is revised to

R4-1906641 is revised to

R4-1906955 is revised to remove the whole FR2 OTA transmit ON/OFF column in Table 12.10.1-1