3GPP TSG-RAN WG4 Meeting #91	R4-1907713
Reno, USA, 13 - 17 May 2019

Agenda item:	10.2.4

Source:	Keysight Technologies, Spirent Communications

Title:	Channel model parameters for UMi and InO CDL-A and C for FR2

Document for:	Approval

# Introduction

In [1], it was encouraged to provide channel model tables for NR MIMO OTA. This contribution presents scaled channel model tables for UMi and InO scenarios for both CDL-A and CDL-C models at FR2.

The angular scaling procedure presented in [2] is augmented first by clarifying the scaling procedure.  Subsequently, we tabulate scaled channel model tables for each scenario.

# Clarifications related to Scaling Procedure

Discussions prior to the meeting highlighted the need to clarify some of the scaling procedures [2].

TR25.996 [3] describes μθ:

As indicated in [2]: “The values are calculated for the AOD, AOA, ZOD, and ZOA angles after removing the mean angle following the definition of rms angular spread in TR25.996, without finding the minimum over circular shifts”.

It should be clarified that the calculation is performed after removing the mean angle and then we follow the TR. 25.996 procedure as such, i.e., the following equation A-2 from Annex A of TR38.901 [4]

is used to rotate  to zero (and also wrap AOAs within +/-180) and subsequently equations A-3

and A-1 of TR 25.996 [3]

are used to calculate the ASmodel. Note that eq. A-2 of TR 25.996 is not applied to ASmodel calculations, , the following equation is used instead

# Channel model tables

Channel model parameter tables for CDL-A and C for UMi and InO at 28 GHz are presented in this contribution. Note that the channel model tables presented here are only the propagation parameters, i.e., without the effect of base station antenna filtering. Thus, if someone needs to see the impact of used antenna array on the channel model parameters, it should be done explicitly.

It should be noted thatClusters 2-4 and 5-7 for CDL A, and clusters 2-4 and 6-8 for CDL C have been treated as midpaths as outlined in [5]. The spatial distribution of the rays for each midpath is given in Table 7.5-5 in [4].

## UMi CDL-A and C Models

Table 1 and Table 2 shows the model parameters, UMi CDL-A and UMi CDL-C models, respectively. For the determination of desired zenith spread of departure (ZSDdesired) from table 7.5-8 of [4], the following parameters are used hBS  = 10 m, hUT = 1.5 m, and d2d = 100 m.

Table 1. Channel model parameters for UMi CDL-A at 28 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -13.4014 | -39.363 | 112.1365 | 95.9936 | 100.6889 |
| 2 | 22.914 | 0 | -2.5086 | -157.634 | 97.1624 | 88.9311 |
| 3 | 24.15 | -2.2185 | -2.5086 | -157.634 | 97.1624 | 88.9311 |
| 4 | 35.208 | -3.9794 | -2.5086 | -157.634 | 97.1624 | 88.9311 |
| 5 | 27.66 | -5.9799 | 17.4974 | 126.7698 | 97.9452 | 89.8621 |
| 6 | 32.25 | -8.1984 | 17.4974 | 126.7698 | 97.9452 | 89.8621 |
| 7 | 40.248 | -9.9593 | 17.4974 | 126.7698 | 97.9452 | 89.8621 |
| 8 | 34.5 | -10.5014 | 24.1308 | -70.3543 | 98.7118 | 73.6909 |
| 9 | 45.708 | -7.5014 | -18.9331 | -93.5479 | 96.1295 | 76.7596 |
| 10 | 92.25 | -15.9014 | 31.951 | 136.9496 | 95.3467 | 67.8293 |
| 11 | 113.868 | -6.6014 | -19.2086 | 112.4835 | 98.0648 | 77.7251 |
| 12 | 133.452 | -16.7014 | 26.9494 | -136.349 | 99.2935 | 66.4156 |
| 13 | 130.308 | -12.4014 | -34.0436 | 97.8501 | 98.7444 | 74.415 |
| 14 | 149.652 | -15.2014 | -38.0703 | 126.7698 | 98.902 | 106.7919 |
| 15 | 150.714 | -10.8014 | -29.1481 | -73.3619 | 95.912 | 97.9305 |
| 16 | 183.492 | -11.3014 | -30.4408 | -82.6162 | 95.7272 | 99.7579 |
| 17 | 244.86 | -12.7014 | 33.4345 | -96.6134 | 95.806 | 99.9303 |
| 18 | 267.474 | -16.2014 | 29.8317 | 146.3196 | 99.0271 | 68.6913 |
| 19 | 274.17 | -18.3014 | 26.5044 | 166.0428 | 94.9226 | 66.8293 |
| 20 | 287.796 | -18.9014 | -26.7533 | 172.2895 | 95.083 | 62.6917 |
| 21 | 300.396 | -16.6014 | -34.2767 | 141.4611 | 99.2962 | 107.7918 |
| 22 | 318.258 | -19.9014 | 25.1904 | -157.113 | 95.2461 | 109.4124 |
| 23 | 579.516 | -29.7014 | -13.5289 | 114.3922 | 98.5677 | 111.2743 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.0596 | 6.3623 | 0.0815 | 1.0344 | 10 |  |

Proposal 1: Use channel model parameters of Table 1 for the FR2 UMi CDL-A scenario.

Table 2. Channel model parameters for UMi CDL-C at 28 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -33.0198 | -136.447 | 99.0189 | 83.1027 |
| 2 | 12.594 | -1.25 | -23.6428 | 132.2125 | 99.2836 | 72.391 |
| 3 | 13.314 | -3.4684 | -23.6428 | 132.2125 | 99.2836 | 72.391 |
| 4 | 13.974 | -5.2294 | -23.6428 | 132.2125 | 99.2836 | 72.391 |
| 5 | 13.056 | 0 | -30.6953 | -153.861 | 99.6618 | 71.0089 |
| 6 | 38.196 | -2.2185 | -14.5417 | 165.3318 | 99.3971 | 74.6025 |
| 7 | 38.688 | -3.9794 | -14.5417 | 165.3318 | 99.3971 | 74.6025 |
| 8 | 39.36 | -3.9215 | -14.5417 | 165.3318 | 99.3971 | 74.6025 |
| 9 | 39.504 | -7.4215 | 14.1407 | 89.7621 | 100.5316 | 69.1429 |
| 10 | 47.61 | -7.1215 | -40.0722 | 97.0562 | 98.6596 | 66.6551 |
| 11 | 49.278 | -10.7215 | 16.938 | -101.685 | 100.7018 | 71.9073 |
| 12 | 56.016 | -11.1215 | -52.9162 | 84.1765 | 98.3192 | 64.3745 |
| 13 | 73.71 | -5.1215 | -36.4475 | 98.1076 | 100.248 | 85.176 |
| 14 | 78.498 | -6.8215 | -39.9934 | -115.221 | 100.3425 | 64.0981 |
| 15 | 130.224 | -8.7215 | -45.588 | 106.9131 | 98.2247 | 64.72 |
| 16 | 162.63 | -13.2215 | 25.8028 | 73.531 | 100.3425 | 92.1559 |
| 17 | 255.534 | -13.9215 | 24.4238 | 42.5803 | 98.584 | 65.6184 |
| 18 | 276.018 | -13.9215 | 20.3264 | 55.8543 | 98.2436 | 68.6592 |
| 19 | 329.412 | -15.8215 | -54.8074 | 44.3545 | 98.0734 | 59.1223 |
| 20 | 336.462 | -17.1215 | 21.6659 | 59.7313 | 100.8153 | 65.2729 |
| 21 | 378.39 | -16.0215 | 22.0993 | 57.0371 | 98.2247 | 58.4312 |
| 22 | 398.244 | -15.7215 | 27.3394 | 53.8172 | 98.2058 | 65.2038 |
| 23 | 422.562 | -21.6215 | 32.4218 | 38.9661 | 100.5316 | 62.6468 |
| 24 | 519.138 | -22.8215 | -63.4358 | 75.4367 | 101.0233 | 61.9557 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.788 | 9.8569 | 0.5673 | 4.8376 | 7 |  |

Proposal 2: Use channel model parameters of Table 2 for the FR2 UMi CDL-C scenario.

## InO CDL-A and C Models

Table 3 and Table 4 tabulate channel model parameters for InO CDL-A and InO CDL-C models at 28 GHz, respectively.

Table 3. Channel model parameters for InO CDL-A at 28 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -13.4014 | -101.633 | 110.3637 | 77.4554 | 114.0008 |
| 2 | 11.457 | 0 | -3.2678 | -157.49 | 95.5583 | 90.2073 |
| 3 | 12.075 | -2.2185 | -3.2678 | -157.49 | 95.5583 | 90.2073 |
| 4 | 17.604 | -3.9794 | -3.2678 | -157.49 | 95.5583 | 90.2073 |
| 5 | 13.83 | -5.9799 | 50.1288 | 125.3079 | 107.6831 | 92.0912 |
| 6 | 16.125 | -8.1984 | 50.1288 | 125.3079 | 107.6831 | 92.0912 |
| 7 | 20.124 | -9.9593 | 50.1288 | 125.3079 | 107.6831 | 92.0912 |
| 8 | 17.25 | -10.5014 | 67.8333 | -68.3566 | 119.5552 | 59.3663 |
| 9 | 22.854 | -7.5014 | -47.105 | -92.0429 | 79.5604 | 65.5763 |
| 10 | 46.125 | -15.9014 | 88.7055 | 135.7039 | 67.4357 | 47.5044 |
| 11 | 56.934 | -6.6014 | -47.8403 | 110.7181 | 109.5355 | 67.5301 |
| 12 | 66.726 | -16.7014 | 75.3564 | -135.753 | 128.5646 | 44.6436 |
| 13 | 65.154 | -12.4014 | -87.4352 | 95.7739 | 120.0604 | 60.8316 |
| 14 | 74.826 | -15.2014 | -98.1824 | 125.3079 | 122.5022 | 126.3512 |
| 15 | 75.357 | -10.8014 | -74.3689 | -71.4282 | 76.1924 | 108.4188 |
| 16 | 91.746 | -11.3014 | -77.8193 | -80.879 | 73.3297 | 112.1169 |
| 17 | 122.43 | -12.7014 | 92.665 | -95.1735 | 74.5505 | 112.4658 |
| 18 | 133.737 | -16.2014 | 83.0491 | 145.2729 | 124.4388 | 49.2488 |
| 19 | 137.085 | -18.3014 | 74.1685 | 165.4151 | 60.8681 | 45.4809 |
| 20 | 143.898 | -18.9014 | -67.9771 | 171.7944 | 63.352 | 37.1078 |
| 21 | 150.198 | -16.6014 | -88.0574 | 140.3112 | 128.6067 | 128.3747 |
| 22 | 159.129 | -19.9014 | 70.6615 | -156.959 | 65.878 | 131.6542 |
| 23 | 289.758 | -29.7014 | -32.6811 | 112.6674 | 117.3239 | 135.4221 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 2.8282 | 6.4975 | 1.263 | 2.0933 | 10 |  |

Proposal 3: Use channel model parameters of Table 3 for the FR2 InO CDL-A scenario.

Table 4. Channel model parameters for InO CDL-C at 28 GHz.

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -47.7555 | -135.004 | 92.8739 | 93.4014 |
| 2 | 6.297 | -1.25 | -22.7283 | 131.7154 | 96.9741 | 71.7246 |
| 3 | 6.657 | -3.4684 | -22.7283 | 131.7154 | 96.9741 | 71.7246 |
| 4 | 6.987 | -5.2294 | -22.7283 | 131.7154 | 96.9741 | 71.7246 |
| 5 | 6.528 | 0 | -41.5513 | -152.787 | 102.8315 | 68.9276 |
| 6 | 19.098 | -2.2185 | 1.5629 | 165.5381 | 98.7313 | 76.1998 |
| 7 | 19.344 | -3.9794 | 1.5629 | 165.5381 | 98.7313 | 76.1998 |
| 8 | 19.68 | -3.9215 | 1.5629 | 165.5381 | 98.7313 | 76.1998 |
| 9 | 19.752 | -7.4215 | 78.1168 | 88.3632 | 116.3034 | 65.1516 |
| 10 | 23.805 | -7.1215 | -66.5785 | 95.8123 | 87.3094 | 60.117 |
| 11 | 24.639 | -10.7215 | 85.5829 | -99.5032 | 118.9393 | 70.7456 |
| 12 | 28.008 | -11.1215 | -100.86 | 82.659 | 82.0378 | 55.5019 |
| 13 | 36.855 | -5.1215 | -56.9041 | 96.886 | 111.9104 | 97.5969 |
| 14 | 39.249 | -6.8215 | -66.3682 | -113.328 | 113.3748 | 54.9425 |
| 15 | 65.112 | -8.7215 | -81.3004 | 105.8786 | 80.5734 | 56.2012 |
| 16 | 81.315 | -13.2215 | 109.2431 | 71.7874 | 113.3748 | 111.7218 |
| 17 | 127.767 | -13.9215 | 105.5626 | 40.1793 | 86.1379 | 58.0192 |
| 18 | 138.009 | -13.9215 | 94.6263 | 53.7352 | 80.8663 | 64.1727 |
| 19 | 164.706 | -15.8215 | -105.907 | 41.9912 | 78.2305 | 44.8733 |
| 20 | 168.231 | -17.1215 | 98.2016 | 57.6946 | 120.6965 | 57.32 |
| 21 | 189.195 | -16.0215 | 99.3584 | 54.9432 | 80.5734 | 43.4748 |
| 22 | 199.122 | -15.7215 | 113.3442 | 51.6548 | 80.2806 | 57.1801 |
| 23 | 211.281 | -21.6215 | 126.9094 | 36.4883 | 116.3034 | 52.0057 |
| 24 | 259.569 | -22.8215 | -128.936 | 73.7336 | 123.918 | 50.6072 |
| Per-Cluster Parameters |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 2.1031 | 10.0663 | 8.7861 | 9.7895 | 7 |  |

Proposal 4: Use channel model parameters of Table 4 for the FR2 InO CDL-C scenario.

# Conclusion

Proposal 1: Use channel model parameters of Table 1 for the FR2 UMi CDL-A scenario.

Proposal 2: Use channel model parameters of Table 2 for the FR2 UMi CDL-C scenario.

Proposal 3: Use channel model parameters of Table 3 for the FR2 InO CDL-A scenario.

Proposal 4: Use channel model parameters of Table 4 for the FR2 InO CDL-C scenario.

# References

- R4-1904160, “Way Forward on NR MIMO OTA”, CAICT, 3GPP TSG-RAN WG4 Meeting #90bis, Xi’an, China, April 2019

- R4-1904197, “On channel model implementations”, Keysight, 3GPP TSG-RAN WG4 Meeting #90bis, Xi’an, China, April 2019

- 3GPP TR 25.996, “Spatial channel model for Multiple Input Multiple Output (MIMO) simulations”, V15.0.0 (2018-06)

- 3GPP TR 38.901 “Study on channel model for frequencies from 0.5 to 100 GHz” V15.0.0 (2018-06)

- R4-1907109, CDL modification proposal, Spirent Communications, 3GPP TSG RAN WG4 Meeting #91

# Annex A

Tables 2 and 4 from [2] with 4 significant digits are provided below

Table A-1. Original (non-circular) angle spreads of CDL models.

| Model | [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| CDL-A | 73.6985 | 85.2676 | 28.5575 | 21.0831 |
| CDL-B | 41.5917 | 59.3326 | 5.9633 | 10.3818 |
| CDL-C | 39.0949 | 71.1175 | 4.0666 | 10.4245 |
| CDL-D | 18.9859 | 21.0747 | 2.9629 | 1.8735 |
| CDL-E | 13.1544 | 37.5640 | 1.4577 | 2.4601 |

Table A-2. Desired AS for UMi and InO at 28 GHz (FR2).

| Model | [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| UMi NLOS (CDL-A, B, C) | 15.6188 | 49.3183 | 0.7762 | 7.2695 |
| UMi LOS (CDL-D, E) | 13.7050 | 41.0212 | 0.6166 | 3.8350 |
| InO NLOS (CDL-A, B, C) | 41.6869 | 50.3659 | 12.0226 | 14.7109 |
| InO LOS (CDL-D, E) | 39.8107 | 31.8526 | 1.3702 | 11.4756 |