3GPP TSG-RAN WG4 Meeting #91	R4-1907755

# Reno, Nevada, USA, 13th – 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: |  | CR on TS38.133 for CSSF outside gap when SFTD measurement is configured (Section 9.1.5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: |  | NR_NewRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: |  | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: | - The UE cell identification and measurement periods should be extended when UE is required to conduct the SFTD measurement without measurement gaps. - Requirement of measurement within gap should include Section 9.2.5.1, 9.2.5.2. |  |  |  |  |  |  |  |  |  |  |
| Summary of change: | - The UE cell identification and measurement periods derived based on CSSFoutside_gap,i may be extended for measurement objects their cell identification and measurement periods are overlapped with SFTD measurement time. - Editorical changes for clarifying the impact of detection of inter-RAT RSTD LTE reference cell on CSSF calculation. Add Section 9.2.5.1, 9.2.5.2 into the changes of previous endorsed CR R4-1904687. |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | - The searcher number for UE to conduct the cell identification and measurement will be extended. - Section 9.2.5.1, 9.2.5.2 are not included in endorsed CR R4-1904687. |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: | Section 9.1.5 |  |  |  |  |  |  |  |  |  |  |
|  | Y |  |  | N |  |  |  |  |  |  |  |
| Other specs |  |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------- Beginning of Change ------------------------------------------------------------

9.1.5	Carrier-specific scaling factor

This clause specifies the derivation of carrier-specific scaling factor (CSSF) values, which scale the measurement delay requirements given in Section 9.2, 9.3 and 9.4 when UE is configured to monitor multiple measurement objects. The CSSF values are categorized into CSSFoutside_gap,i and CSSFwithin_gap,i, for the measurements conducted outside measurement gaps and within measurement gaps, respectively.

9.1.5.1	Monitoring of multiple layers outside gaps

The carrier-specific scaling factor CSSFoutside_gap,i for measurement object i derived in this chapter is applied to following measurement types:

-	Intra-frequency measurement with no measurement gap in Section 9.2.5, when none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Intra-frequency measurement with no measurement gap in Section 9.2.5, when part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

UE is expected to conduct the measurement of this measurement object i only outside the measurement gaps.

If the higher layer signaling in TS 38.331 [2] signaling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFoutside_gap_i and requirements derivied from CSSFoutside_gap_i are not specified.

The UE cell identification and measurement periods derived based on CSSFoutside_gap,i in sections 9.2.5.1, 9.2.5.2 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with Tmeasure_SFTD1 specified in section 8.1.2.4.25 and section 8.1.2.4.26.

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE configured with the E-UTRA-NR dual connectivity operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.1-1.

Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2 | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| EN-DC with FR1 only CA | 1 | Number of configured FR1 SCell(s) | N/A | N/A | N/A |
| EN-DC with FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCells |
| EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1 | 1 | 2×(Number of configured SCell(s)-1) | N/A | 2 | 2×(Number of configured SCell(s)-1) |
| EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1 | N/A | Number of configured SCell(s) | 1 | N/A | Number of configured SCell(s) |
| Note 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows section 9.2.3.2. |  |  |  |  |  |

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps without NR-DC operation

For UE not configured with NR-DC operation in SA mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.2-1, which shall also be applied for a UE configured with NE-DC operation.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor without NR-DC operation in SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1 | Number of configured FR1 SCell(s) | N/A | N/A | N/A |
| FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCell(s) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1 | 2×(Number of configured SCell(s)-1) | N/A | 2 | 2×(Number of configured SCell(s)-1) |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows section 9.2.3.2. |  |  |  |  |  |

9.1.5.1.3	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps with NR-DC operation

For UE configured with NR-DC operation in SA mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.3-1.

Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor with NR-DC operation in SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- |
| FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) Note 1 | 1 | 2×(Number of configured SCell(s)) | 2 | 2×(Number of configured SCell(s)) |
| Note 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. |  |  |  |  |

9.1.5.2	Monitoring of multiple layers within gaps

The carrier-specific scaling factor CSSFwithin_gap,i for measurement object i derived in this chapter is applied to following measurement types:

-	Intra-frequency measurement with no measurement gap in Section 9.2.5, when all of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Intra-frequency measurement with measurement gap in Section 9.2.6.

-	Inter-frequency measurement in Section 9.3

-	Inter-RAT measurement in Section 9.4

UE is expected to conduct the measurement of this measurement object i only within the measurement gaps.

If the higher layer signaling in TS 38.331 [2] signaling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFwithin_gap_i and requirements derivied from CSSFoutside_gap_i are not specified.

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed within gaps

Editor’s note: The scaling value CSSFwithin_gap,i below has been derived without considering GSM inter-RAT carriers.

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this section.

If measurement object i refers to an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, CSSFwithin_gap,i=1. Otherwise, the the CSSFwithin_gap,i for other measurement objects (including RSTD measurement with periodicity Tprs=160ms) participate in the gap competition are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intrafrequency measurement objects and interfrequency/interRAT measurement objects which are candidates to be measured within the gap j.

-	An NR carrier is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intrafrequency NR carriers, if the high layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An interRAT measurement object is a candidate to be measured in all meausrement gaps.

Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 1280ms period.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

Per gap j:

Mintra,i,j: Number of intrafrequency measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

Minter,i,j : Number of NR interfrequency measurement objects or NR interRAT measurement objects configured by E-UTRA PCell, EUTRA interfrequency measurement objects configured by E-UTRA PCell, UTRA inter-RAT measurement objects and GSM interRAT measurement objects configured by E-UTRA PCell which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter,i,j  equals 0.

Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intrafrequency, interfrequency and interRAT measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intrafrequency measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an interfrequency or interRAT measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this section.

If measurement object i refers to an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, CSSFwithin_gap,i=1. Otherwise, the the CSSFwithin_gap,i for other measurement objects (including RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intrafrequency measurement objects and interfrequency/interRAT measurement objects which are candidates to be measured within the gap j.

-	An NR carrier is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intrafrequency NR carriers, if the high layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An interRAT measurement object is a candidate to be measured in all meausrement gaps.

Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 1280ms period.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

Per gap j:

Mintra,i,j: Number of intrafrequency measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

Minter,i,j : Number of NR interfrequency and EUTRA interRAT measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter,i,j  equals 0.

Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intrafrequency, interfrequency and interRAT measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intrafrequency measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an interfrequency or interRAT measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1

CSSFwithin_gap,k=1 during TDetect, E-UTRAN FDD specified in section 9.4.4.1.2.2 and TDetect, E-UTRAN TDD specified in section 9.4.4.2.2.2, where k is the carrier frequency where the UE is performing cell detection of the inter-RAT E-UTRA OTDOA assistance data reference cell when acquiring the subframe and slot timing of the cell according to Section 9.4.4. In this case, the UE cell identification and measurement periods derived based on CSSFwithin_gap,i in sections 9.2.5.1, 9.2.5.2, 9.2.6.2, 9.2.6.3, 9.3.4, 9.3.5, 9.4.2.2, and 9.4.2.3 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with TDetect, E-UTRAN FDD and TDetect, E-UTRAN TDD.

----------------------------------------------------------- End of Change ----------------------------------------------------------------