3GPP TSG-RAN4 Meeting #91	R4-1907770

# Reno, USA, May 13th - 17th, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on correcting the conditions in section B.2.4 B.2.5 and B.2.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The conditions for CSI-RS based L1-RSRP reporting with SCS=60kHz in FR1 are not correctly defined. The conditions for RRC connection release with redirection to NR in FR2 are not defined. The conditions for UE transmit timing in FR1 are not are not correctly defined. The conditions for UE transmit timing in FR2 are not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To correct the values of minimum CSI-RS_RP for SCSCSI-RS=60 kHz in Table 2.4.2-1. - To update the Table B.2.5-2 used in the conditions for RRC connection release with redirection to NR in FR2. - To correct the values of minimum SSB_RP in Table 2.6.1-1 and the values of minimum CSI-RS_RP in Table 2.6.2-1. - To update the Tables B.2.6.1-2 and B.2.6.2-2 used in the conditions for UE transmit timing in FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conditions for L1-RSRP reporting, RRC connection release with redirection and UE transmit timing are not properly defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | section B.2.4, B.2.5, B.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## B.2.4	Conditions for NR L1-RSRP reporting

B.2.4.1	Conditions for SSB based L1-RSRP reporting

This section defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.1-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.1-2 for FR2 NR cells.

Table B.2.4.1-1: Conditions for SSB based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

Table B.2.4.1-2: Conditions for SSB based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -113.3 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -113.3 | -109.1 | -124.8+Y4 |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -113.3 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -102.3 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -102.3 | -98.2 | -115.8+Z4 |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -102.3 | -98.2 | -115.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in TS 38.101-2 [19] clause 7.3.4. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in TS 38.101-2 [19] clause 6.2.1. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.4.1-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1 and 4 respectively

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting

This section defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.2-2 for FR2 NR cells.

Table B.2.4.2-1: Conditions for CSI-RS based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 |  |  |  |  | Minimum CSI-RS_RP |  |  |  |  | CSI-RS Ês/Iot |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | dBm / SCSCSI-RS |  |  |  |  | dB |  |
|  |  |  |  | SCSCSI-RS = 15 kHz |  |  |  | SCSCSI-RS = 30 kHz |  | SCSCSI-RS = 60 kHz |  |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A |  |  | -124 |  |  |  | -121 |  | -118 | -3 |  |
|  | NR_FDD_FR1_B |  | -123.5 | -120.5 | -117.5 |  |
|  | NR_TDD_FR1_C |  |  | -123 |  |  |  | -120 |  | -117 |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -122.5 |  |  |  | -119.5 |  | -116.5 |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  | -122 |  |  |  | -119 |  | -116 |  |  |
|  | NR_FDD_FR1_G |  |  | -121 |  |  |  | -118 |  | -115 |  |  |
|  | NR_FDD_FR1_H |  |  | -120.5 |  |  |  | -117.5 |  | -114.5 |  |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |  |  |

Table B.2.4.2-2: Conditions for CSI-RS based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum CSI-RS_RP Note 2, Note 3 |  |  |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSCSI-RS |  |  |  |  | dB |
|  |  |  | SCSCSI-RS = 60 kHz |  |  |  | SCSCSI-RS = 120 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -116.3 | -112.1 | -127.8+Y4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -128.3+Y1 | -116.3 | -112.1 | -127.8+Y4 |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -116.3 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -105.3 | -101.2 | -118.8+Z4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -120.3+Z1 | -105.3 | -101.2 | -118.8+Z4 |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -105.3 | -101.2 | -118.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in TS 38.101-2 [19] clause 7.3.4. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum CSI-RS Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in TS 38.101-2 [19] clause 6.2.1. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.4.2-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1 and 4 respectively

B.2.5	Conditions for RRC connection release with redirection to NR

This section defines the following conditions for RRC connection release with redirection to NR: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.5-1 for FR1 NR cells.

The conditions are defined in Table B.2.5-2 for FR2 NR cells.

Table B.2.5-1: Conditions for for RRC connection release with redirection to NR in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

Table B.2.5-2: Conditions for RRC connection release with redirection to NR in FR2

| Parameter | Angle of arrival |  | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  |  | SSB Ês/Iot |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | dBm / SCSSSB |  |  |  |  |  |  |  | dB |  |
|  |  |  |  | SCSSSB = 120 kHz |  |  |  |  |  | SCSSSB = 240 kHz |  |  |  |
|  |  |  |  | UE Power class |  |  |  |  |  | UE Power class |  |  |  |
|  |  |  |  | 1 | 2 |  | 3 | 4 |  | 1, 2, 3, 4 |  |  |  |
| Conditions | Rx Beam Peak |  | n257 | -126.3+Y1 | -114.3 |  | -110.1 | -125.8+Y4 |  | (Value for SCSSSB = 120 kHz) +3dB |  | ≥-4 |  |
|  |  |  | n258 | -126.3+Y1 | -114.3 |  | -110.1 | -125.8+Y4 |  |  |  |  |  |
|  |  |  | n260 | -123.3+Y1 |  |  | -107.5 | -123.8+Y4 |  |  |  |  |  |
|  |  |  | n261 | -126.3+Y1 | -114.3 |  | -110.1 | -125.8+Y4 |  |  |  |  |  |
|  | Spherical coverage Note 1 |  | n257 | -118.3+Z1 | -103.3 |  | -99.2 | -116.8+Z4 |  | (Value for SCSSSB = 120 kHz) +3dB |  | ≥-4 |  |
|  |  |  | n258 | -118.3+Z1 | -103.3 |  | -99.2 | -116.8+Z4 |  |  |  |  |  |
|  |  |  | n260 | -115.3+Z1 |  |  | -94.9 | -111.8+Z4 |  |  |  |  |  |
|  |  |  | n261 | -114.3 | -103.3 |  | -99.2 | -116.8+Z4 |  |  |  |  |  |
| Note 1: Values based on EIS spherical coverage as defined in TS 38.101-2 [19] clause 7.3.4. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in TS 38.101-2 [19] clause 6.2.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.5.2-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1 and 4 respectively

B.2.6	Conditions for UE transmit timing

B.2.6.1	Conditions for SSB based UE transmit timing

This section defines the following conditions for UE transmit timing adjustment performed based on SSBs: SSB_RP and SSB Ês/Iot and applicable for a corresponding operating band.

The conditions are defined in Table B.2.6.1-1 for FR1 SSB.

Table B.2.6.1-1: Conditions for SSB based UE transmit timing in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB=15 kHz | SCSSSB=30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | [-3] |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

The conditions are defined in Table B.2.6.1-2 for FR2 SSB.

Table B.2.6.1-2: Conditions for SSB based UE transmit timing in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak |  | n257 |  | -125.3+Y1 |  | -113.3 |  |  | -109.1 |  | -124.8+Y4 |  | (Value for SCSSSB = 120 kHz) +3dB |  | ≥[-3] |
|  |  |  | n258 |  | -125.3+Y1 |  | -113.3 |  |  | -109.1 |  | -124.8+Y4 |  |  |  |  |
|  |  |  | n260 |  | -122.3+Y1 |  |  |  |  | -106.5 |  | -122.8+Y4 |  |  |  |  |
|  |  |  | n261 |  | -125.3+Y1 |  | -113.3 |  |  | -109.1 |  | -124.8+Y4 |  |  |  |  |
|  | Spherical coverage Note 1 |  | n257 |  | -117.3+Z1 |  | -102.3 |  |  | -98.2 |  | -115.8+Z4 |  | (Value for SCSSSB = 120 kHz) +3dB |  | ≥[-3] |
|  |  |  | n258 |  | -117.3+Z1 |  | -102.3 |  |  | -98.2 |  | -115.8+Z4 |  |  |  |  |
|  |  |  | n260 |  | -114.3+Z1 |  |  |  |  | -93.9 |  | -110.8+Z4 |  |  |  |  |
|  |  |  | n261 |  | -117.3+Z1 |  | -102.3 |  |  | -98.2 |  | -115.8+Z4 |  |  |  |  |
| Note 1: Values based on EIS spherical coverage as defined in TS 38.101-2 [19] clause 7.3.4. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in TS 38.101-2 [19] clause 6.2.1. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.6.1-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1 and 4 respectively

B.2.6.2	Conditions for CSI-RS based UE transmit timing

This section defines the following conditions for UE transmit timing adjustment based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot and applicable for a corresponding operating band.

The conditions are defined in Table B.2.6.2-1 for FR1 CSI-RS.

Table B.2.6.2-2: Conditions for CSI-RS based UE transmit timing in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS RSRP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS=15 kHz | SCSCSI-RS=30 kHz |  | SCSCSI-RS=60 kHz |  |  |  |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 |  | -118 |  | [-3] |  |  |
|  | NR_FDD_FR1_B | -123.5 |  | -120.5 |  | -117.5 |  |  |  |
|  | NR_TDD_FR1_C | -123 | -120 |  | -117 |  |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  | -116.5 |  |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  | -116 |  |  |  |  |
|  | NR_FDD_FR1_G | -121 | -118 |  | -115 |  |  |  |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  | -114.5 |  |  |  |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

The conditions are defined in Table B.2.6.2-2 for FR2 CSI-RS.

Table B.2.6.2-2: Conditions for CSI-RS based UE transmit timing in FR2

| Parameter |  | Angle of arrival |  | NR operating bands | Minimum CSI-RS_RP Note 2, Note 3 |  |  |  |  |  |  |  | CSI-RS Ês/Iot |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | dBm / SCSCSI-RS |  |  |  |  |  |  |  | dB |  |
|  |  |  |  |  | SCSCSI-RS = 60 kHz |  |  |  |  |  | SCSCSI-RS = 120 kHz |  |  |  |
|  |  |  |  |  | UE Power class |  |  |  |  |  | UE Power class |  |  |  |
|  |  |  |  |  | 1 | 2 |  | 3 |  | 4 | 1, 2, 3, 4 |  |  |  |
| Conditions |  | Rx Beam Peak |  | n257 | -128.3+Y1 | -116.3 |  | -112.1 |  | -127.8+Y4 | (Value for SCSCSI-RS = 60 kHz) +3dB |  | ≥[-3] |  |
|  |  |  |  | n258 | -128.3+Y1 | -116.3 |  | -112.1 |  | -127.8+Y4 |  |  |  |  |
|  |  |  |  | n260 | -125.3+Y1 |  |  | -109.5 |  | -125.8+Y4 |  |  |  |  |
|  |  |  |  | n261 | -128.3+Y1 | -116.3 |  | -112.1 |  | -127.8+Y4 |  |  |  |  |
|  |  | Spherical coverage Note 1 |  | n257 | -120.3+Z1 | -105.3 |  | -101.2 |  | -118.8+Z4 | (Value for SCSCSI-RS = 60 kHz) +3dB |  | ≥[-3] |  |
|  |  |  |  | n258 | -120.3+Z1 | -105.3 |  | -101.2 |  | -118.8+Z4 |  |  |  |  |
|  |  |  |  | n260 | -117.3+Z1 |  |  | -96.9 |  | -113.8+Z4 |  |  |  |  |
|  |  |  |  | n261 | -120.3+Z1 | -105.3 |  | -101.2 |  | -118.8+Z4 |  |  |  |  |
| Note 1: Values based on EIS spherical coverage as defined in TS 38.101-2 [19] clause 7.3.4. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum CSI-RS Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in TS 38.101-2 [19] clause 6.2.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.6.2-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1, 2 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1, 2, 3 and 4 respectively

<End of Change 1>