3GPP TSG-RAN WG4 Meeting #91	R4-1906287

Reno, US, 13th– 17th May, 2019

Agenda item:	6.10.7.2.1

Source: 		NTT DOCOMO, INC.

Title: 	SCell activation delay

Document for:	Discussion

1. Introduction

At the previous meeting, there was intensive discussion on SCell activation delay in FR2. Following agreements were reached after the discussion [1, 2].

R4-1904696 Way forward on FR2 SCell activation delay CR- rev Cat: () v Source: Qualcomm Abstract: This contribution provides the way forward on Discussion: Agreement: - Implication of known cell definition - SCell activation delay will not include the L1-RSRP reporting time for Power Class 1 - FFS for UE supporting Power Class 2/3/4 Decision: Approved

In this contribution, we discuss further details on SCell activation delay.

2. Discussion

Followings are agreements regarding the definition of SCell activation delay reached in the last meeting.

Agreement: - Implication of known cell definition - SCell activation delay will not include the L1-RSRP reporting time for Power Class 1 - FFS for UE supporting Power Class 2/3/4

One remaining issue is whether or not the SCell activation delay should include the L1-RSRP reporting time for PC2-4. A discussion point is whether we can use measurement results for L3-RSRP to UE procedure after activation such as PDSCH demodulation and CSI acquisition. At this point, it seems to have no difference between PC1 and PC2-4. Thus we propose to apply same rules for PC2-4.

Proposal: As an implication of known cell, SCell activation delay doesn’t include the L1-RSRP reporting time for PC2-4.

3. Conclusion

In this contribution, we discuss further details on SCell activation delay. Our proposals are presented as follows.

Proposal: As an implication of known cell, SCell activation delay doesn’t include the L1-RSRP reporting time for PC2-4.

References

[1] 3GPP, “Draft meeting report for RAN4 #90bis,” May 2019.

[2] R4-1904696, Qualcomm, Way forward on FR2 SCell activation delay, Apr. 2019.