3GPP TSG-RAN WG4 Meeting #92	R4-1909166

Ljubljana, Slovenia, 26 – 30 August 2019

Agenda Item:	9.17.1.1

Source:	Ericsson

Title:	Discussion on UE demodulation requirements for NR-DC

Document for:	Discussion

# 1	Introduction

RAN plenary #84 approved a new WID on NR performance requirement improvement [1], and one of the objectives is to introduce a new PDSCH demodulation requirements for NR-DC.

UE demodulation and CSI reporting requirements: - NR CA PDSCH normal demodulation requirements for NR CA, EN-DC, NE-DC, NR-DC - No additional LTE requirements will be introduced.

We discuss the view on PDSCH demodulation requirements for NR-DC.

# 2	Discussion

As we discussed in our discussion paper on CA/EN-DC/NE-DC [2], there are several parameters related to CA/DC as follows and we propose 10 combinations for 2CC CA scenario.

- Frequency range: FR1, FR2, FR1+FR2

- DC type: Intra-band contiguous DC, Intra-band non-contiguous DC, Inter-band DC

- Duplex mode: FDD only, TDD only, FDD+TDD

- SCS: 15 and 30kHz for FR1, 120kHz for FR2

For NR-DC, when we review the specified band combinations in TS38.101-1/2/3, there are three band combinations specified in Rel-15, i.e., DC_n77-n257, DC_n78-n257, and DC_n79-n257. Since n77/n78/n79 are TDD bands in FR1 and n257 is a TDD band in FR2, RAN4 can consider one combination for NR-DC as shown in Table 1.

Table 1	Possible NR-DC parameter combination.

| Test number | Frequency range | DC Type | Duplex mode | SCS |
| --- | --- | --- | --- | --- |
| 1 | FR1+FR2 | Inter-band | TDD+TDD | 30+120 |

# 3	Summary

Proposal: RAN4 specify the PDSCH demodulation requirements for NR-DC with the following scenario:

| Test number | Frequency range | DC Type | Duplex mode | SCS |
| --- | --- | --- | --- | --- |
| 1 | FR1+FR2 | Inter-band | TDD+TDD | 30+120 |

The more detailed parameters such as CBW and MCS to be discussed further.

# References

- RP-191587, “New WID on NR performance requirement enhancement”, China Telecom.

- R4-1909165, “Discussion on UE demodulation requirements for CA, EN-DC, and NE-DC”, Ericsson.