3GPP TSG-RAN WG4 Meeting #92                                                               R4-1907998

# Ljubljana, Slovenia, 26th– 30th August, 2019

# Source:    Verizon, Nokia, Ericsson, [Samsung], Qualcomm

# Tile: 	Correction of n260 and n261 intra-band CA configuration

Agenda item:			7.5.2

Document for:	Approval

- Discussion

As part of channel arrangement requirement for CA, the mmWave nominal channel spacing for intra-band contiguous and non-contiguous carrier aggregation are introduced in section 5.4A.1of [1] and illustrated in below, in which the nominal channel spacing (NCS) is a spacing between the center frequencies of two adjacent component carriers for the contiguous intra-band CA, and a larger NCS for non- contiguous intra-band CA. The requirements shall be applied to multiple mmWave deployment scenarios.

Still, a contradictive information in “NOTE” is maintained in multiple configuration tables of [1], [2], and [3] for both n260 and n261 intra-band non-contiguous carrier aggregations. It introduces a minimum 50MHz or larger frequency gap in between adjacent component carriers. And, this misinforms an additional spacing, as additional “non-contiguous gap”, on top of the required spacing in CA configuration to n260 and n261 intra-band non-contiguous carrier aggregations. Obviously, this doesn’t align on the specification requirement and will result in uncooperative CA configurations. Moreover, it could create a large variety of fragmented spectrum arrangements and lower down the spectrum efficiency.

For avoiding the issues, thus, this “non-contiguous gap” should be removed from the existing specifications, including renovation of WID [4], to ensure the accurate configurations to n260 and n261 intra-band non-contiguous carrier aggregations.

Proposal 1

Delay the text in footnotes which upholds the maximum bandwidth of band n260 and n261 and statement of “a 50 MHz gap is in between non-contiguous CCs” from configuration tables of [1], [2] and [3]

Proposal 2

Same correction should be applied to both Table 4-2 and Table 4-3 of WID [4]

In fact, the section 5.4A.1 of [1] didn’t provide exact spacing value of frequency spacing needed for the intra-band non-contiguous CA, but a statement as it “… shall be larger than the nominal channel spacing defined in this subclause”. For aligning on the requirement, it is important to explain what the extra frequency spacing shall be in addition from the nominal channel spacing. For minimizing impact to Rel.15 spec, our suggestion is to tolerate 50MHz out of entire band n261 bandwidth (850MHz) and 200MHz out of entire band n260 channel bandwidth (3000MHz) as totality of the quantified range of “larger than the nominal channel spacing” specified in 5.4A.1 for the non-contiguous carrier aggregation. As there might be some implementation out there already, a new BCS would be used for the change to avoid potential impact.

Proposal 3:

Update the maximum aggregated bandwidth for n261 to 800MHz, and the maximum aggregated bandwidth for n260 to 2800MHz in the configuration tables

Proposal 4:

A new BCS “1” is used for to reflect the updates above

Attached is an updated WID [4] for approval. And, distinct contributions are submitted for [1], [2], and [3] on the same corrections above.

- Proposal

In order to aovid incorrect NR CA configurations, we would like RAN4 to approval follow proposals,

Proposal 1

Modefy the footnote Note(1, 2, or 4) and remove the “a 50MHz gap is in between non-contiguous CCs” from configuration tables of [1], [2] and [3]

Proposal 2

Update footnotes in both Table 4-2 and Table 4-3 of WID [4] and remove the “a 50MHz gap is in between non-contiguous CCs”

Proposal 3:

Update the maximum aggregated bandwidth for n261 to 800MHz, and the maximum aggregated bandwidth for n260 to 2800MHz in configuration tables

Proposal 4:

A new BCS “1” is used for to reflect the updates above

Reference

[1] 38.101-2, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Rel.15 and 16)

[2] 37.865-01-01, NR Carrier Aggregation for intra-band (m Down Link (DL)/1 Up Link (UL) bands) and inter-band (n          Down Link (DL) /1 Up Link (UL) bands)

[3] 38.716-01-01, NR intra band Carrier Aggregation (CA) Rel-16 for xCC Down Link (DL) / yCC Up Link (UL) including contiguous and non-contiguous spectrum (x >= y)

[4] RP-191194, Revised WID on Rel-16 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y)