3GPP TSG-RAN WG4 Meeting #92                 	R4-1908127

Ljubljana, Slovenia, 26th – 30th August 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR UCI over PUSCH simulation results

Agenda item:	7.12.2.2.1

Document for:	Discussion

Introduction

In the RAN4#91 meeting, the latest way forward detailing how to contribute the input for NR UCI over PUSCH demodulation performance requirements was agreed [1]. It was decided to add the test cases of:

- Add test case of DMRS configuration with 1+1 for FR2 with MCS 16

- Add test case of without PTRS configuration for FR2 with MCS 16

In this contribution we restate our previous results and amend them with the new test cases. New results are marked with green colour.

Simulation Results

Simulation settings

The following results were obtained using the initial simulation assumptions, as captured in [1]. It should be remarked that the definition of the test metrics was changed w.r.t. RAN4#90b.

Table 1: Simulation assumption for UCI over PUSCH results

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | FR1 | FR2 |
| General System | Transform precoding | Disabled | Disabled |
|  | Number of Tx | 1 | 1 |
|  | Number of Rx | 2 | 2 |
|  | Number of layers | 1 | 1 |
|  | [Uplink-downlink allocation for TDD] | 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
|  | Code block group based PUSCH transmission | Disabled | Disabled |
|  | MCS index | 16 | 16 |
|  | Propagation Condition | MCS 16: TDL-C 300ns, 100 Hz | MCS 16: TDL-A 30ns, 300Hz |
| HARQ | Maximum number of HARQ transmission | 1 | 1 |
|  | RV | 0 | 0 |
| DMRS | DM-RS configuration type | 1 | 1 |
|  | Maximum number of OFDM symbols for front loaded DM-RS | 1 | 1 |
|  | Number of additional DM-RS symbols | 1 | 0, 1 |
|  | Number of DM-RS CDM group(s) without data | 2 | 2 |
|  | EPRE ratio of PUSCH to DM-RS | -3 dB | -3 dB |
|  | DM-RS port | 0 | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | A, B | B |
|  | PUSCH starting symbol index | 0 | 0 |
|  | PUSCH symbol length | 14 | 10 |
| Frequency domain resource | Carrier frequency (GHz) | 4 | 30 |
|  | SCS and BW | 30kHz: 10 MHz | 120kHz: 50 MHz |
|  | RB assignment | Full applicable test bandwidth | Full applicable test bandwidth |
|  | Frequency hopping | Disabled | Disabled |
| PTRS configuration | Frequency density (KPT-RS) | no PT-RS | 2, no PT-RS |
|  | Time density (LPT-RS) | no PT-RS | 1, no PT-RS |
| UCI | UL-SCH | With | With |
|  | Payload (bits) | Case 1: csi_part1= 5, csi_part2 =2 Case 2 : csi_part1=20,csi_part2=20 | Case 1: csi_part1 = 5 ,csi_part2 = 2 Case 2: csi_part1=20, csi_part2=20 |
|  | uci-on-pusch-scaling | 1 | 1 |
|  | I^HARQ_ACK | 11 | 11 |
|  | I^CSI-1 and I^CSI-2 | 13 for CSI part1 and part2 | 13 for CSI part1 and part2 |
| Test metric |  | 0.1% BLER of UCI with CSI part1, 1% BLER of UCI with CSI part2 | 0.1% BLER of UCI with CSI part1, 1% BLER of UCI with CSI part2 |

Simulation results

Summary

The summary simulation results with the parameters defined in the last section, are shown in Table 2.

Table 2: Simulation results summary

|  |  |  |  |  |  |  |  |  | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH 1T2R |  |  |  |  | UCI playload | CSI part1 | CSI part2 | Test metric | Ideal | Impairment |
| CP-OFDM | FR1 Mappying type A | 30kHz, 10MHz | MCS16 | TDL300-100 | 7 | 5 | 2 | 0.1% BLER of CSI part1 | 2,66 | 5,16 |
|  |  |  |  |  |  |  |  | 1% BLER of CSI part2 | -2,52 | -0,02 |
|  |  |  |  |  | 40 | 20 | 20 | 0.1% BLER of CSI part1 | 2,50 | 5,00 |
|  |  |  |  |  |  |  |  | 1% BLER of CSI part2 | 0,46 | 2,96 |
|  | FR1 Mappying type B | 30kHz, 10MHz | MCS16 | TDL300-100 | 7 | 5 | 2 | 0.1% BLER of CSI part1 | 3,38 | 5,88 |
|  |  |  |  |  |  |  |  | 1% BLER of CSI part2 | -2,51 | -0,01 |
|  |  |  |  |  | 40 | 20 | 20 | 0.1% BLER of CSI part1 | 2,40 | 4,90 |
|  |  |  |  |  |  |  |  | 1% BLER of CSI part2 | 0,46 | 2,96 |
|  | FR2 Mappying type B | 120KHz, 50MHz | MCS16 | TDL30-300 | 7 | 5 | 2 | 0.1% BLER of CSI part1 | 5,06 | 8,16 |
|  |  |  |  |  |  |  |  | 1% BLER of CSI part2 | -2,12 | 0,98 |
|  |  |  |  |  | 40 | 20 | 20 | 0.1%BLER of CSI part1 | 2,87 | 5,97 |
|  |  |  |  |  |  |  |  | 1% BLER of CSI part2 | 1,01 | 4,11 |
|  | FR2 Mappying type B | 120KHz, 50MHz | MCS16, DMRS 1+0, No PT-RS | TDL30-300 | 7 | 5 | 2 | 0.1% BLER of CSI part1 | 5.00 | 8.10 |
|  |  |  |  |  |  |  |  | 1% BLER of CSI part2 | -2.00 | 1.10 |
|  |  |  |  |  | 40 | 20 | 20 | 0.1%BLER of CSI part1 | 2.83 | 5.93 |
|  |  |  |  |  |  |  |  | 1% BLER of CSI part2 | 0.84 | 3.94 |
|  | FR2 Mappying type B | 120KHz, 50MHz | MCS16, DMRS 1+1, PT-RS | TDL30-300 | 7 | 5 | 2 | 0.1% BLER of CSI part1 | 6.33 | 9.60 |
|  |  |  |  |  |  |  |  | 1% BLER of CSI part2 | -1.15 | 1.95 |
|  |  |  |  |  | 40 | 20 | 20 | 0.1%BLER of CSI part1 | 3.46 | 6.56 |
|  |  |  |  |  |  |  |  | 1% BLER of CSI part2 | 1.63 | 4.73 |
|  | FR2 Mappying type B | 120KHz, 50MHz | MCS16, DMRS 1+1, No PT-RS | TDL30-300 | 7 | 5 | 2 | 0.1% BLER of CSI part1 | 5.00 | 8.10 |
|  |  |  |  |  |  |  |  | 1% BLER of CSI part2 | -2.00 | 1.10 |
|  |  |  |  |  | 40 | 20 | 20 | 0.1%BLER of CSI part1 | 2.91 | 6.01 |
|  |  |  |  |  |  |  |  | 1% BLER of CSI part2 | 1.05 | 4.15 |

30KHz_DMRS1+1_typeA_40bit

Table 3: 30KHz_DMRS1+1_typeA_40bit

| SNR [dB] | BLER |  |
| --- | --- | --- |
|  | CSI part1 | CSI part2 |
| -8 | 0,8827 | 0,8829 |
| -7 | 0,7765 | 0,7763 |
| -6 | 0,6302 | 0,6319 |
| -5 | 0,4616 | 0,4590 |
| -4 | 0,3028 | 0,3018 |
| -3 | 0,1803 | 0,1739 |
| -2 | 0,0930 | 0,0894 |
| -1 | 0,0405 | 0,0405 |
| 0 | 0,0169 | 0,0162 |
| 1 | 0,0063 | 0,0056 |
| 2 | 0,0019 | 0,0018 |
| 3 | 0,0005 | 0,0006 |
| 4 | 0,0001 | 0,0001 |
| 5 |  |  |
| 6 |  |  |

30KHz_DMRS1+1_typeA_7bit

Table 4: 30KHz_DMRS1+1_typeA_7bit

| SNR [dB] | BLER |  |
| --- | --- | --- |
|  | CSI part1 | CSI part2 |
| -8 | 0,5348 | 0,1556 |
| -7 | 0,4449 | 0,1115 |
| -6 | 0,3536 | 0,0757 |
| -5 | 0,2632 | 0,0465 |
| -4 | 0,1830 | 0,0268 |
| -3 | 0,1177 | 0,0141 |
| -2 | 0,0698 | 0,0069 |
| -1 | 0,0381 | 0,0031 |
| 0 | 0,0194 | 0,0014 |
| 1 | 0,0090 | 0,0005 |
| 2 | 0,0040 | 0,0001 |
| 3 | 0,0015 | 0,0001 |
| 4 | 0,0005 |  |
| 5 | 0,0002 |  |
| 6 | 0,0001 |  |

30KHz_DMRS1+1_typeB_40bit

Table 5: 30KHz_DMRS1+1_typeB_40bit

| SNR [dB] | BLER |  |
| --- | --- | --- |
|  | CSI part1 | CSI part2 |
| -8 | 0,8797 | 0,8851 |
| -7 | 0,7723 | 0,7778 |
| -6 | 0,6241 | 0,6275 |
| -5 | 0,4561 | 0,4624 |
| -4 | 0,2974 | 0,3038 |
| -3 | 0,1702 | 0,1757 |
| -2 | 0,0878 | 0,0901 |
| -1 | 0,0414 | 0,0409 |
| 0 | 0,0163 | 0,0163 |
| 1 | 0,0058 | 0,0057 |
| 2 | 0,0016 | 0,0017 |
| 3 | 0,0005 | 0,0004 |
| 4 | 0,0001 | 0,0001 |
| 5 | 0,0001 |  |
| 6 |  |  |

30KHz_DMRS1+1_typeB_7bit

Table 6: 30KHz_DMRS1+1_typeB_7bit

| SNR [dB] | BLER |  |
| --- | --- | --- |
|  | CSI part1 | CSI part2 |
| -8 | 0,4574 | 0,1567 |
| -7 | 0,3691 | 0,1119 |
| -6 | 0,2849 | 0,0755 |
| -5 | 0,2051 | 0,0469 |
| -4 | 0,1390 | 0,0268 |
| -3 | 0,0883 | 0,0143 |
| -2 | 0,0511 | 0,0068 |
| -1 | 0,0270 | 0,0030 |
| 0 | 0,0125 | 0,0010 |
| 1 | 0,0052 | 0,0004 |
| 2 | 0,0020 | 0,0001 |
| 3 | 0,0007 |  |
| 4 | 0,0003 |  |
| 5 |  |  |
| 6 |  |  |

120KHz_DMRS1_40bit

Table 7: 120KHz_DMRS1_40bit

| SNR [dB] | BLER |  |
| --- | --- | --- |
|  | CSI part1 | CSI part2 |
| -8 | 0,8569 | 0,8565 |
| -7 | 0,7516 | 0,7539 |
| -6 | 0,6204 | 0,6245 |
| -5 | 0,4721 | 0,4802 |
| -4 | 0,3296 | 0,3351 |
| -3 | 0,2092 | 0,2130 |
| -2 | 0,1178 | 0,1236 |
| -1 | 0,0589 | 0,0625 |
| 0 | 0,0264 | 0,0272 |
| 1 | 0,0099 | 0,0101 |
| 2 | 0,0031 | 0,0029 |
| 3 | 0,0008 | 0,0008 |
| 4 | 0,0002 | 0,0002 |
| 5 | 0,0001 | 0,0001 |
| 6 |  |  |

120KHz_DMRS1_7bit

Table 8: 120KHz_DMRS1_7bit

| SNR [dB] | BLER |  |
| --- | --- | --- |
|  | CSI part1 | CSI part2 |
| -8 | 0,7764 | 0,1570 |
| -7 | 0,6976 | 0,1152 |
| -6 | 0,6025 | 0,0788 |
| -5 | 0,4985 | 0,0508 |
| -4 | 0,3924 | 0,0310 |
| -3 | 0,2884 | 0,0175 |
| -2 | 0,2015 | 0,0092 |
| -1 | 0,1288 | 0,0043 |
| 0 | 0,0738 | 0,0019 |
| 1 | 0,0385 | 0,0007 |
| 2 | 0,0195 | 0,0002 |
| 3 | 0,0087 | 0,0001 |
| 4 | 0,0032 |  |
| 5 | 0,0011 |  |
| 6 | 0,0003 |  |

120KHz_DMRS_bit

Table 9: 120KHz_DMRS1_NoPTRS_7bit

| SNR [dB] | BLER |  |
| --- | --- | --- |
|  | CSI part1 | CSI part2 |
| -8 | 0.7759 | 0.1592 |
| -7 | 0.6970 | 0.1177 |
| -6 | 0.6035 | 0.0816 |
| -5 | 0.4959 | 0.0525 |
| -4 | 0.3858 | 0.0321 |
| -3 | 0.2825 | 0.0181 |
| -2 | 0.1937 | 0.0100 |
| -1 | 0.1248 | 0.0052 |
| 0 | 0.0721 | 0.0020 |
| 1 | 0.0377 | 0.0007 |
| 2 | 0.0186 | 0.0003 |
| 3 | 0.0084 | 0.0001 |
| 4 | 0.0031 | 0.0001 |
| 5 | 0.0010 | 0.0000 |
| 6 | 0.0002 | 0.0000 |

120KHz_DMRS_bit

Table 10: 120KHz_DMRS1+1_withPTRS_7bit

| SNR [dB] | BLER |  |
| --- | --- | --- |
|  | CSI part1 | CSI part2 |
| -8 | 0.9095 | 0.2409 |
| -7 | 0.8578 | 0.1807 |
| -6 | 0.7835 | 0.1287 |
| -5 | 0.6879 | 0.0852 |
| -4 | 0.5773 | 0.0537 |
| -3 | 0.4560 | 0.0324 |
| -2 | 0.3391 | 0.0181 |
| -1 | 0.2338 | 0.0090 |
| 0 | 0.1492 | 0.0044 |
| 1 | 0.0901 | 0.0020 |
| 2 | 0.0497 | 0.0009 |
| 3 | 0.0239 | 0.0003 |
| 4 | 0.0107 | 0.0001 |
| 5 | 0.0044 | 0.0001 |
| 6 | 0.0015 | 0.0000 |

120KHz_DMRS_bit

Table 11: 120KHz_DMRS1+1_NoPtrs_7bit

| SNR [dB] | BLER |  |
| --- | --- | --- |
|  | CSI part1 | CSI part2 |
| -8 | 0.7766 | 0.1592 |
| -7 | 0.6979 | 0.1177 |
| -6 | 0.6034 | 0.0815 |
| -5 | 0.4954 | 0.0525 |
| -4 | 0.3853 | 0.0321 |
| -3 | 0.2828 | 0.0181 |
| -2 | 0.1935 | 0.0100 |
| -1 | 0.1240 | 0.0053 |
| 0 | 0.0719 | 0.0020 |
| 1 | 0.0379 | 0.0007 |
| 2 | 0.0184 | 0.0002 |
| 3 | 0.0083 | 0.0001 |
| 4 | 0.0031 | 0.0001 |
| 5 | 0.0010 | 0.0000 |
| 6 | 0.0002 | 0.0000 |

120KHz_DMRS_bit

Table 12: 120KHz_DMRS1_NoPtrs_40bit

| SNR [dB] | BLER |  |
| --- | --- | --- |
|  | CSI part1 | CSI part2 |
| -8 | 0.8484 | 0.8483 |
| -7 | 0.7432 | 0.7417 |
| -6 | 0.6112 | 0.6088 |
| -5 | 0.4620 | 0.4620 |
| -4 | 0.3210 | 0.3176 |
| -3 | 0.2013 | 0.1988 |
| -2 | 0.1138 | 0.1134 |
| -1 | 0.0565 | 0.0573 |
| 0 | 0.0242 | 0.0234 |
| 1 | 0.0088 | 0.0085 |
| 2 | 0.0029 | 0.0024 |
| 3 | 0.0008 | 0.0006 |
| 4 | 0.0001 | 0.0001 |
| 5 | 0.0000 | 0.0001 |
| 6 | 0.0000 | 0.0000 |

120KHz_DMRS_bit

Table 13: 120KHz_DMRS1+1_WithPTRS_40bit

| SNR [dB] | BLER |  |
| --- | --- | --- |
|  | CSI part1 | CSI part2 |
| -8 | 0.9074 | 0.9070 |
| -7 | 0.8273 | 0.8265 |
| -6 | 0.7121 | 0.7135 |
| -5 | 0.5775 | 0.5762 |
| -4 | 0.4278 | 0.4298 |
| -3 | 0.2898 | 0.2919 |
| -2 | 0.1784 | 0.1787 |
| -1 | 0.0979 | 0.0973 |
| 0 | 0.0475 | 0.0471 |
| 1 | 0.0188 | 0.0199 |
| 2 | 0.0063 | 0.0067 |
| 3 | 0.0018 | 0.0018 |
| 4 | 0.0005 | 0.0005 |
| 5 | 0.0001 | 0.0001 |
| 6 | 0.0000 | 0.0000 |

120KHz_DMRS_bit

Table 14: 120KHz_DMRS1+1_NoPTRS_40bit

| SNR [dB] | BLER |  |
| --- | --- | --- |
|  | CSI part1 | CSI part2 |
| -8 | 0.8545 | 0.8575 |
| -7 | 0.7497 | 0.7554 |
| -6 | 0.6206 | 0.6268 |
| -5 | 0.4744 | 0.4841 |
| -4 | 0.3337 | 0.3414 |
| -3 | 0.2110 | 0.2190 |
| -2 | 0.1213 | 0.1249 |
| -1 | 0.0625 | 0.0631 |
| 0 | 0.0273 | 0.0283 |
| 1 | 0.0102 | 0.0106 |
| 2 | 0.0028 | 0.0030 |
| 3 | 0.0009 | 0.0008 |
| 4 | 0.0001 | 0.0001 |
| 5 | 0.0000 | 0.0000 |
| 6 | 0.0000 | 0.0000 |

Conclusion

In this contribution we have provided our simulation results for the new NR UCI over PUSCH test cases.

- References

- R4-1907240, Way forward on performance requirements of UCI multiplexing on PUSCH, Samsung, RAN4#91.