3GPP TSG-RAN WG4 Meeting #92	R4-1908282

Ljubljana, Slovenia, 26 – 30 August 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on maximum Doppler shift in uplink for Rel-16 NR high speed train scenarios

Agenda item:	9.16.2.3

Document for:	Discussion

# 1	Introduction

A new WI on NR support for high speed train scenario [1] was approved at the RAN#84 meeting. One objective of the WI is to investigate and determine the maximum Doppler shift based on the operating frequency, speed and NR design limitations for all UL/DL physical channels. In the WI, the considered target vehicular speed and operating frequency are up to 500 km/h and 3.6 GHz, respectively.

This document determines the theoretical maximum Doppler shift, which can be derived from reference signal design structures of uplink NR physical channels.

# 2	Discussion

DM-RS

For time-varying channel as in the high-speed trains scenario, the accuracy of Doppler shift estimation depends on the number of DM-RS symbols distributed in the time domain. The minimum time interval ∆t between 2 DM-RS symbols in one slot sets the theoretical upper limit of Doppler shift fd that can be accurately estimated. The theoretical maximum Doppler shift can be calculated as

fd=12∆t

Referring to Section 6.4 in TS 38.211, 2 types of DM-RS design are supported for PUSCH, namely PUSCH mapping type A and B. The main difference between these 2 types is in the location of the first DM-RS symbol. Table 1 shows the DM-RS structure of PUSCH mapping type A and B, which is taken from Table 6.4.1.1.3-3 in TS 38.211. For PUSCH mapping type A:

-	 is defined relative to the start of the slot if frequency hopping is disabled and relative to the start of each hop in case frequency hopping is enabled

-	 is given by the higher-layer parameter dmrs-TypeA-Position

and for PUSCH mapping type B:

-	 is defined relative to the start of the scheduled PUSCH resources if frequency hopping is disabled and relative to the start of each hop in case frequency hopping is enabled

-

From Table 1, it can be seen that PUSCH supports the DM-RS density per slot can increase from one up to four symbols.

Tables 2-4 show the achievable maximum Doppler shift for different DM-RS density corresponding to Table 1 for PUSCH mapping type A and SCS = 15 kHz, 30 kHz and 60 kHz.

Tables 5-7 show the maximum achievable Doppler shift for PUSCH mapping type B.

Table 1: PUSCH DM-RS positions  within a slot for single-symbol DM-RS and intra-slot frequency hopping disabled (Table 6.4.1.1.3-3, TS 38.211)

| ld in symbols | DM-RS positions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUSCH mapping type A |  |  |  | PUSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| <4 | - | - | - | - |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  | , 4 | , 4 | , 4 |
| 6 |  |  |  |  |  | , 4 | , 4 | , 4 |
| 7 |  |  |  |  |  | , 4 | , 4 | , 4 |
| 8 |  | , 7 | , 7 | , 7 |  | , 6 | , 3, 6 | , 3, 6 |
| 9 |  | , 7 | , 7 | , 7 |  | , 6 | , 3, 6 | , 3, 6 |
| 10 |  | , 9 | , 6, 9 | , 6, 9 |  | , 8 | , 4, 8 | , 3, 6, 9 |
| 11 |  | , 9 | , 6, 9 | , 6, 9 |  | , 8 | , 4, 8 | , 3, 6, 9 |
| 12 |  | , 9 | , 6, 9 | , 5, 8, 11 |  | , 10 | , 5, 10 | , 3, 6, 9 |
| 13 |  | , 11 | , 7, 11 | , 5, 8, 11 |  | , 10 | , 5, 10 | , 3, 6, 9 |
| 14 |  | , 11 | , 7, 11 | , 5, 8, 11 |  | , 10 | , 5, 10 | , 3, 6, 9 |

Table 2: Maximum Doppler shift fd for PUSCH mapping type A for SCS = 15 kHz and  = ‘pos2’

| ld in symbols | PUSCH mappnig type A dmrs-AdditionalPosition |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| <4 | - | - | - | - |
| 4 | fd = 500 Hz | fd = 500 Hz | fd = 500 Hz | fd = 500 Hz |
| 5 | fd = 500 Hz | fd = 500 Hz | fd = 500 Hz | fd = 500 Hz |
| 6 | fd = 500 Hz | fd = 500 Hz | fd = 500 Hz | fd = 500 Hz |
| 7 | fd = 500 Hz | fd = 500 Hz | fd = 500 Hz | fd = 500 Hz |
| 8 | fd = 500 Hz | fd = 1400 Hz | fd = 1400 Hz | fd = 1400 Hz |
| 9 | fd = 500 Hz | fd = 1400 Hz | fd = 1400 Hz | fd = 1400 Hz |
| 10 | fd = 500 Hz | fd = 1000 Hz | fd = 2333 Hz | fd = 2333 Hz |
| 11 | fd = 500 Hz | fd = 1000 Hz | fd = 2333 Hz | fd = 2333 Hz |
| 12 | fd = 500 Hz | fd = 1000 Hz | fd = 2333 Hz | fd = 2333 Hz |
| 13 | fd = 500 Hz | fd = 778 Hz | fd = 1750 Hz | fd = 2333 Hz |
| 14 | fd = 500 Hz | fd = 778 Hz | fd = 1750 Hz | fd = 2333 Hz |

Table 3: Maximum Doppler shift fd for PUSCH mapping type A for SCS = 30 kHz and  = ‘pos2’

| ld in symbols | PUSCH mappnig type A dmrs-AdditionalPosition |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| <4 | - | - | - | - |
| 4 | fd = 1000 Hz | fd = 1000 Hz | fd = 1000 Hz | fd = 1000 Hz |
| 5 | fd = 1000 Hz | fd = 1000 Hz | fd = 1000 Hz | fd = 1000 Hz |
| 6 | fd = 1000 Hz | fd = 1000 Hz | fd = 1000 Hz | fd = 1000 Hz |
| 7 | fd = 1000 Hz | fd = 1000 Hz | fd = 1000 Hz | fd = 1000 Hz |
| 8 | fd = 1000 Hz | fd = 2800 Hz | fd = 2800 Hz | fd = 2800 Hz |
| 9 | fd = 1000 Hz | fd = 2800 Hz | fd = 2800 Hz | fd = 2800 Hz |
| 10 | fd = 1000 Hz | fd = 2000 Hz | fd = 4666 Hz | fd = 4666 Hz |
| 11 | fd = 1000 Hz | fd = 2000 Hz | fd = 4666 Hz | fd = 4666 Hz |
| 12 | fd = 1000 Hz | fd = 2000 Hz | fd = 4666 Hz | fd = 4666 Hz |
| 13 | fd = 1000 Hz | fd = 1556 Hz | fd = 3500 Hz | fd = 4666 Hz |
| 14 | fd = 1000 Hz | fd = 1556 Hz | fd =3500 Hz | fd = 4666 Hz |

Table 4: Maximum Doppler shift fd for PUSCH mapping type A for SCS = 60 kHz (normal cyclic prefix) and  = ‘pos2’

| ld in symbols | PUSCH mappnig type A dmrs-AdditionalPosition |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| <4 | - | - | - | - |
| 4 | fd = 2000 Hz | fd = 2000 Hz | fd = 2000 Hz | fd = 2000 Hz |
| 5 | fd = 2000 Hz | fd = 2000 Hz | fd = 2000 Hz | fd = 2000 Hz |
| 6 | fd = 2000 Hz | fd = 2000 Hz | fd = 2000 Hz | fd = 2000 Hz |
| 7 | fd = 2000 Hz | fd = 2000 Hz | fd = 2000 Hz | fd = 2000 Hz |
| 8 | fd = 2000 Hz | fd = 5600 Hz | fd = 5600 Hz | fd = 5600 Hz |
| 9 | fd = 2000 Hz | fd = 5600 Hz | fd = 5600 Hz | fd = 5600 Hz |
| 10 | fd = 2000 Hz | fd = 4000 Hz | fd = 9332 Hz | fd = 9332 Hz |
| 11 | fd = 2000 Hz | fd = 4000 Hz | fd = 9332 Hz | fd = 9332 Hz |
| 12 | fd = 2000 Hz | fd = 4000 Hz | fd = 9332 Hz | fd = 9332 Hz |
| 3 | fd = 2000 Hz | fd = 3112 Hz | fd = 7000 Hz | fd = 9332 Hz |
| 14 | fd = 2000 Hz | fd = 3112 Hz | fd = 7000 Hz | fd = 9332 Hz |

Table 5: Maximum Doppler shift fd for PUSCH mapping type B for SCS = 15 kHz and  = 0

| ld in symbols | PUSCH mappnig type A dmrs-AdditionalPosition |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| <4 | fd = 500 Hz | fd = 500 Hz | fd = 500 Hz | fd = 500 Hz |
| 4 | fd = 500 Hz | fd = 500 Hz | fd = 500 Hz | fd = 500 Hz |
| 5 | fd = 500 Hz | fd = 1750 Hz | fd = 1750 Hz | fd = 1750 Hz |
| 6 | fd = 500 Hz | fd = 1750 Hz | fd = 1750 Hz | fd = 1750 Hz |
| 7 | fd = 500 Hz | fd = 1750 Hz | fd = 1750 Hz | fd = 1750 Hz |
| 8 | fd = 500 Hz | fd = 1167 Hz | fd = 2333 Hz | fd = 2333 Hz |
| 9 | fd = 500 Hz | fd = 1167 Hz | fd = 2333 Hz | fd = 2333 Hz |
| 10 | fd = 500 Hz | fd = 875 Hz | fd = 1750 Hz | fd = 2333 Hz |
| 11 | fd = 500 Hz | fd = 875 Hz | fd = 1750 Hz | fd = 2333 Hz |
| 12 | fd = 500 Hz | fd = 700 Hz | fd = 1400 Hz | fd = 2333 Hz |
| 13 | fd = 500 Hz | fd = 700 Hz | fd = 1400 Hz | fd = 2333 Hz |
| 14 | fd = 500 Hz | fd = 700 Hz | fd = 1400 Hz | fd = 2333 Hz |

Table 6: Maximum Doppler shift fd for PUSCH mapping type B for SCS = 30 kHz and  = 0

| ld in symbols | PUSCH mappnig type A dmrs-AdditionalPosition |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| <4 | fd = 1000 Hz | fd = 1000 Hz | fd = 1000 Hz | fd = 1000 Hz |
| 4 | fd = 1000 Hz | fd = 1000 Hz | fd = 1000 Hz | fd = 1000 Hz |
| 5 | fd = 1000 Hz | fd = 3500 Hz | fd = 3500 Hz | fd = 3500 Hz |
| 6 | fd = 1000 Hz | fd = 3500 Hz | fd = 3500 Hz | fd = 3500 Hz |
| 7 | fd = 1000 Hz | fd = 3500 Hz | fd = 3500 Hz | fd = 3500 Hz |
| 8 | fd = 1000 Hz | fd = 2334 Hz | fd = 4666 Hz | fd = 4666 Hz |
| 9 | fd = 1000 Hz | fd = 2334 Hz | fd = 4666 Hz | fd = 4666 Hz |
| 10 | fd = 1000 Hz | fd = 1750 Hz | fd = 3500 Hz | fd = 4666 Hz |
| 11 | fd = 1000 Hz | fd = 1750 Hz | fd = 3500 Hz | fd = 4666 Hz |
| 12 | fd = 1000 Hz | fd = 1400 Hz | fd = 2800 Hz | fd = 4666 Hz |
| 13 | fd = 1000 Hz | fd = 1400 Hz | fd = 2800 Hz | fd = 4666 Hz |
| 14 | fd = 1000 Hz | fd = 1400 Hz | fd = 2800 Hz | fd = 4666 Hz |

Table 7: Maximum Doppler shift fd for PUSCH mapping type B for SCS = 60 kHz (normal cyclic prefix) and  = 0

| ld in symbols | PUSCH mappnig type A dmrs-AdditionalPosition |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| <4 | fd = 2000 Hz | fd = 2000 Hz | fd = 2000 Hz | fd = 2000 Hz |
| 4 | fd = 2000 Hz | fd = 2000 Hz | fd = 2000 Hz | fd = 2000 Hz |
| 5 | fd = 2000 Hz | fd = 7000 Hz | fd = 7000 Hz | fd = 7000 Hz |
| 6 | fd = 2000 Hz | fd = 7000 Hz | fd = 7000 Hz | fd = 7000 Hz |
| 7 | fd = 2000 Hz | fd = 7000 Hz | fd = 7000 Hz | fd = 7000 Hz |
| 8 | fd = 2000 Hz | fd = 4668 Hz | fd = 9332 Hz | fd = 9332 Hz |
| 9 | fd = 2000 Hz | fd = 4668 Hz | fd = 9332 Hz | fd = 9332 Hz |
| 10 | fd = 2000 Hz | fd = 3500 Hz | fd = 7000 Hz | fd = 9332 Hz |
| 11 | fd = 2000 Hz | fd = 3500 Hz | fd = 7000 Hz | fd = 9332 Hz |
| 12 | fd = 2000 Hz | fd = 2800 Hz | fd = 5600 Hz | fd = 9332 Hz |
| 13 | fd = 2000 Hz | fd = 2800 Hz | fd = 5600 Hz | fd = 9332 Hz |
| 14 | fd = 2000 Hz | fd = 2800 Hz | fd = 5600 Hz | fd = 9332 Hz |

The following observations can be made:

Observation 1: The Doppler shift per slot is proportional to the SCS irrespective of the PUSCH mapping type. SCS = 60 kHz gives the highest Doppler shift value which is 4 and 2 times that of SCS = 15 kHz and 30 kHz, respectively.

Observation 2: The maximum Doppler shift achievable by PUSCH mapping type A and B is the same (e.g.,  fd = 2333 Hz for SCS = 15 kHz).

Observation 3: Both 3- and 4-DM-RS symbol densities per slot give the same maximum Doppler shift (e.g., fd = 2333 Hz for SCS = 15 kHz).

Observation 4: Given the vehicular speed 500 km/h and operating frequency 3.6 GHz, the Doppler shift is within the theoretical limit of 3 DM-RS symbols or higher per slot for PUSCH using SCS = 30 kHz.

PT-RS

According to Section 6.4.1.2 in TS 38.211, the number of PT-RS symbols per slot is determined by LPT-RS, which is defined in Table 8. The table also shows the achievable maximum Doppler shift based on the minimum time interval between 2 PT-RS symbols in a slot for each LPT-RS.

Table 8: Maximum Doppler shift fd for different PT-RS configurations

| Schedules MCS | Time Density LPT-RS | SCS |  |  |
| --- | --- | --- | --- | --- |
|  |  | 15 kHz | 30 kHz | 60 kHz |
| ptrs-MCS3 IMCS < ptrs-MCS4 | 1 | fd = 7000 Hz | fd= 14000 Hz | fd= 28000 Hz |
| ptrs-MCS2 IMCS < ptrs-MCS3 | 2 | fd = 3500 Hz | fd = 7000 Hz | fd = 14000 Hz |
| ptrs-MCS1 IMCS < ptrs-MCS2 | 4 | fd = 1750 Hz | fd = 3500 Hz | fd = 7000 Hz |

Observation 5: As expected, LPT-RS = 1 gives the highest Doppler shift value as compared with LPT-RS = {2, 4} and 4 DM-RS symbols per slot.

# 3	Conclusions

The document has derived the theoretical maximum Doppler shift based on the reference signal design structures of PUSCH and PT-RS. The following are our observations:

Observation 1: The Doppler shift per slot is proportional to the SCS irrespective of the PUSCH mapping type. SCS = 60 kHz gives the highest Doppler shift value which is 4 and 2 times that of SCS = 15 kHz and 30 kHz, respectively.

Observation 2: The maximum Doppler shift achievable by PUSCH mapping type A and B is the same (e.g.,  fd = 2333 Hz for SCS = 15 kHz).

Observation 3: Both 3- and 4-DM-RS symbol densities per slot give the same maximum Doppler shift (e.g., fd = 2333 Hz for SCS = 15 kHz).

Observation 4: Given the vehicular speed 500 km/h and operating frequency 3.6 GHz, the Doppler shift is within the theoretical limit of 3 DM-RS symbols or higher per slot for PUSCH using SCS = 30 kHz.

Observation 5: As expected, LPT-RS = 1 gives the highest Doppler shift value as compared with LPT-RS = {2, 4} and 4 DM-RS symbols per slot.

# References

- RP-191512, New WID on NR support for high speed train scenario, CMCC