3GPP TSG-RAN WG4 Meeting #92	R4-1908284

Ljublajana, Slovenia 26th – 30th Aug 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0054 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new channel bandwidths for n7 into TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_n7_BW |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of new channel bandwidths for n7 into the specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A-MPR and REFSENS is defined for new channel bandwidths for n7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New channel bandwidths for n7 are not usable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5, 6.2.3.1, 6.2.3.14, 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

******************************* Start of changes**************************************************

### 5.3.5	UE channel bandwidth per operating band

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the TX and RX path.

Table 5.3.5-1 Channel bandwidths for each NR band

|  | NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 904 MHz | 100 MHz |
| n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
| n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
| n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n39 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |
| n50 | 15 | Yes | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |  |
| n51 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| n70 | 15 | Yes | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |  |
| n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n75 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n76 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |
| n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |
| n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |
|  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |
|  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |
| n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n81 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n82 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n83 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n84 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| n86 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |

******************************* Un-changed section***********************************************

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2 In absense of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK  is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms, the allowed A-MPR is TBD.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Subclause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 |  |  | Subclause 6.2.3.7 |
| NS_04 | 6.5.2.3.2, 6.5.3.3.1 | n41 | 10, 15, 20, 40, 50, 60 80, 90, 100 |  | Subclause 6.2.3.2 |
| NS_05 | 6.5.3.3.4 | n1, n84 | 5, 10, 15, 20 (NOTE 2) |  | Subclause 6.2.3.4 |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n84 | 5, 10, 15, 20 |  | Subclause 6.2.3.4 |
| NS_06 | 6.5.2.3.4 | n12 | 5, 10, 15 |  | N/A |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3-1 | Table 6.2.3.3-1 |
| NS_17 | 6.5.3.3.2 | n28, n83 | 5,10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 |
| NS_35 | 6.5.2.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.6 | n74 (NOTE 3) | 10, 15 | Table 6.2.3.8-1 | Table 6.2.3.8-1 |
| NS_38 | 6.5.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.9-1 | Table 6.2.3.9-1 |
| NS_39 | 6.5.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.10-1 | Table 6.2.3.10-1 |
| NS_40 | 6.5.3.3.9 | n51 | 5 |  | Table 6.2.3.5-1 |
| NS_41 | 6.5.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.11-1 |
| NS_42 | 6.5.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.12-1 |
| NS_43 | 6.5.3.3.5 | n8, n81 | 5, 10, 15 |  | Subclause 6.2.3.6 |
| NS_43U | 6.5.3.3.5, 6.5.2.4.2 | n8, n81 | 5, 10, 15 |  | Subclause 6.2.3.6 |
| NS_46 | 6.5.3.2 | n7 | 25, 30, 40, 50 | Table 6.2.3.14-1 | Table 6.2.3.14-2 |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n25, n66, n80, n81, n84, n86 (NOTE 1) |  |  | Table 6.2.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed NOTE 2: No A-MPR is applied for 5 MHz CBW where the lower channel edge is ≥ 1930 MHz,10 MHz CBW where the lower channel edge is ≥ 1950 MHz and 15 MHz CBW where the lower channel edge is ≥ 1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 |  |  |  |  |  |  |  |
| n39 | NS_01 |  |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 |  |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in subclause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

******************************* Un-changed section***********************************************

#### 6.2.3.14	A-MPR for NS_46

Table 6.2.3.14-1: A-MPR regions for NS_46

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 15 MHz | 2558.5 ≤ FC ≤ 2562.5 |  | Note 1 | A1 |
| 20 MHz | 2547 ≤ FC ≤ 2560 |  | Note 1 | A2 |
| 25 MHz | 2534.5 ≤ FC < 2557.5 |  | Note 1 | A3 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.14 | >0 | A4 |
|  |  | ≥4.14, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
| NOTE 1: >9.72MHz for DFT-s-OFDM, >16.02MHz for CP-OFDM. CP-OFDM threshold only applies for 20 and 25MHz bandwidths. |  |  |  |  |

Table 6.2.3.14-2: A-MPR for NS_46

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 3.5 | 4 | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | QPSK | 3.5 | 4 | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 16 QAM | 3.5 | 4 | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 64 QAM | 3.5 | 4 | 4.5 | 5 |  | 3.5 | 6 | 10 |
|  | 256 QAM |  |  |  |  |  |  | 6 | 10 |
| CP-OFDM | QPSK | 3.5 | 5.5 | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 16 QAM | 3.5 | 5.5 | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 64 QAM | 3.5 | 5.5 | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 256 QAM |  |  | 6 |  |  |  | 7 | 11 |

******************************* Un-changed section***********************************************

### 7.3.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1 and Table 7.3.2-2.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |  |  |  |
| n2 | 15 | -98.0 | -94.8 | -93.0 | -91.8 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  |  |  |  |  |  |  |  |  |
| n3 | 15 | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 |  |  |  |  |  |  |  |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 |  |  |  |  |  |  |  |
| n5 | 15 | -98.0 | -94.8 | -93.0 | -90.8 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 | -93.1 | -91.0 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.8 | -90.7 | -89.9 | -88.6 | [-77.6] |  |  |  |  | FDD |
|  | 30 |  | -95.1 | -93.1 | -92.0 | -90.8 | -90.0 | -88.7 | [-77.7] |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 | -91.0 | -90.1 | -88.9 | [-77.8] |  |  |  |  |  |
| n8 | 15 | -97.0 | -93.8 | -92.0 | -90.0 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 | -92.1 | -90.2 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 | -84.1 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 | -91.1 | -90.0 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 | -93.3 | -91.5 | -90.3 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -93.6 | -91.6 | -90.5 |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -94.0 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.6 | -93.6 | -91.0 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | -100.0 | -96.8 | -95.0 |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | -97.1 | -95.1 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 |  |  |  |  |  |  |  |  |  |  |
| n381 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |  |  |  |
| n39 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 |  |  |  |  |  | TDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.9 |  |  |  |  |  |  |
| n40 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 | -89.6 |  |  |  |  | TDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 | -89.7 | -88.9 | -87.6 |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.9 | -89.8 | -89.1 | -87.6 |  |  |  |
| n411 | 15 |  | -94.8 | -93.0 | -91.8 |  |  | -88.6 | -87.6 |  |  |  |  | TDD |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  |  | -88.7 | -87.7 | -86.9 | -85.6 | -85.1 | -84.7 |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  |  | -88.9 | -87.8 | -87.1 | -85.6 | -85.1 | -84.7 |  |
| n50 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  | -90.6 | -89.6 |  |  |  |  | TDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  | -90.7 | -89.7 | -88.9 | -87.6 |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  | -90.9 | -89.8 | -89.1 | -87.6 |  |  |  |
| n51 | 15 | -100.0 |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  | -90.1 |  |  |  |  |  | FDD |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  | -90.2 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  | -90.4 |  |  |  |  |  |  |
| n70 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 |  |  |  |  |  |  |  |  |
| n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | -99.53 | -96.33 | -94.53 | -93.33 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.63 | -94.63 | -93.53 |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.03 | -94.93 | -93.73 |  |  |  |  |  |  |  |  |  |
| n771,4 | 15 |  | -95.3 | -93.5 | -92.2 |  |  | -89.1 | -88.1 |  |  |  |  | TDD |
|  | 30 |  | -95.6 | -93.6 | -92.4 |  |  | -89.2 | -88.2 | -87.4 | -86.1 | -85.6 | -85.1 |  |
|  | 60 |  | -96.0 | -93.9 | -92.6 |  |  | -89.4 | -88.3 | -87.5 | -86.2 | -85.7 | -85.2 |  |
| n781 | 15 |  | -95.8 | -94.0 | -92.7 |  |  | -89.6 | -88.6 |  |  |  |  | TDD |
|  | 30 |  | -96.1 | -94.1 | -92.9 |  |  | -89.7 | -88.7 | -87.9 | -86.6 | -86.1 | -85.6 |  |
|  | 60 |  | -96.5 | -94.4 | -93.1 |  |  | -89.9 | -88.8 | -88.0 | -86.7 | -86.2 | -85.7 |  |
| n791 | 15 |  |  |  |  |  |  | -89.6 | -88.6 |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  | -89.7 | -88.7 | -87.9 | -86.6 |  | -85.6 |  |
|  | 60 |  |  |  |  |  |  | -89.9 | -88.8 | -88.0 | -86.7 |  | -85.7 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1 shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n1, n2, n3, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n77, n78, n79 | -2.2 |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.2-1 and Table 7.3.2-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n1 | 15 | 25 | 501 | 751 | 1001 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 |  |  |  |  |  |  |  |  |  |
| n2 | 15 | 25 | 501 | 501 | 501 |  |  |  |  |  |  |  |  | FDD |
|  | 30 | 101 | 24 | 241 | 241 |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 1011 |  |  |  |  |  |  |  |  |  |
| n3 | 15 | 25 | 501 | 501 | 501 | 501 | 501 |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 241 | 241 | 241 | 241 |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |  |
| n5 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 | 25 | 501 | 751 | 751 | 721 | 641 | 451 | 451 |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 361 | 361 | 321 | 201 | 201 |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 181 | 181 | 161 | 101 | 101 |  |  |  |  |  |
| n8 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | 201 | 201 | 201 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | 25 | 201 | 202 | 202 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 102 | 102 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | 25 | 501 | 501 | 501 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 241 | 241 |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |
| n28 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 | 24 |  |  |  |  |  |  |  |  |  |
| n39 | 15 | 25 | 50 | 75 | 100 | 128 | 160 | 216 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 | 100 |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 | 50 |  |  |  |  |  |  |
| n40 | 15 | 25 | 50 | 75 | 100 | 128 | 160 | 216 | 270 |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 | 100 | 128 | 162 | 216 |  |  |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 | 50 | 64 | 75 | 100 |  |  |  |
| n41 | 15 |  | 50 | 75 | 100 |  |  | 216 | 270 |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  |  | 100 | 128 | 162 | 216 | 243 | 270 |  |
|  | 60 |  | 10 | 18 | 24 |  |  | 50 | 64 | 75 | 100 | 120 | 135 |  |
| n50 | 15 | 25 | 50 | 75 | 100 |  |  | 216 | 270 |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  |  | 100 | 128 | 162 | NOTE 3 |  |  |  |
|  | 60 |  | 10 | 18 | 24 |  |  | 50 | 64 | 75 | NOTE 3 |  |  |  |
| n51 | 15 | 25 |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | 25 | 501 | 751 | 1001 |  |  | 216 |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 |  |  | 1001 |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 |  |  | 501 |  |  |  |  |  |  |
| n70 | 15 | 25 | 501 | 751 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |
| n71 | 15 | 25 | 251 | 201 | 201 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 51 | 51 | 51 |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  | 50 | 75 | 100 |  |  | 216 | 270 |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  |  | 100 | 128 | 162 | 216 | 243 | 270 |  |
|  | 60 |  | 10 | 18 | 24 |  |  | 50 | 64 | 75 | 100 | 120 | 135 |  |
| n78 | 15 |  | 50 | 75 | 100 |  |  | 216 | 270 |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  |  | 100 | 128 | 162 | 216 | 243 | 270 |  |
|  | 60 |  | 10 | 18 | 24 |  |  | 50 | 64 | 75 | 100 | 120 | 135 |  |
| n79 | 15 |  |  |  |  |  |  | 216 | 270 |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  | 100 | 128 | 162 | 216 |  | 270 |  |
|  | 60 |  |  |  |  |  |  | 50 | 64 | 75 | 100 |  | 135 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). NOTE 2: For Band 20; for 15 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4; NOTE 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest duplex distance shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Unless given by Table 7.3.2-4, the minimum requirements specified in Tables 7.3.2-1 and 7.3.2-2 shall be verified with the network signalling value NS_01 (Table 6.2.3-1) configured.

Table 7.3.2-4: Network signaling value for reference sensitivity

| Operating band | Network Signalling value |
| --- | --- |
| n2 | NS_03 |
| n12 | NS_06 |
| n25 | NS_03 |
| n66 | NS_03 |
| n70 | NS_03 |
| n71 | NS_35 |

******************************* End of changes ***********************************************