3GPP TSG-RAN WG4 Meeting #92 	R4-1908555

# Ljubljana, Slovenia, August 26th – 30th, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.810 | CR |  | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 38.810: Modifications of TX beam peak direction search and EIRP spherical coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current EIRP test procedure as defined in TR 38.810 where EIRP determined from the maximum value between EIRP(PolLink = ) and EIRP(PolLink = Ф) upon beam correspondence may not always realize the UL polarization gain under the black box measurement approach. This draft CR proposes to modify the test procedures for Tx beam peak direction search, peak EIRP, EIRP spherical coverage, and beam correspondence tolerance based on total EIRP = EIRP(PolMeas=, PolLink=) + EIRP(PolMeas=Ф, PolLink= Ф) in order to assure that the UE UL polarization gain as budgeted in the EIRP specifications can be properly accounted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change EIRP test procedures in sub-clauses 5.2.1.3.2, 5.2.1.3.7, 5.2.1.3.9, 5.2.3.3.2, and 5.2.4.2.2 based on EIRP = EIRP(PolMeas=, PolLink=) + EIRP(PolMeas=Ф, PolLink= Ф) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may fail the peak EIRP and EIRP coverage requirements as the UL polarization gain may not always be realized based on the current EIRP test procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.3.2, 5.2.1.3.7, 5.2.1.3.9, 5.2.3.3.2, 5.2.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changed sections>>>

##### 5.2.1.3.2	Peak EIRP Measurement Procedure

The TX beam peak direction is where the maximum total component of EIRP is found, including the respective polarization of the measurement antenna used to form the TX beam, according to 5.2.1.3.7.

The measurement procedure includes the following steps:

1)	Connect the SS (System Simulator) with the DUT through the measurement antenna with polarization reference PolLink= to form the TX beam towards the TX beam peak direction.

2)	Lock the beam.

3)	Measure the mean power Pmeas(PolMeas= PolLink=) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator).

4)	Calculate EIRP(PolMeas= PolLink= by adding the composite loss of the entire transmission path for utilized signal path, LEIRP,θ, and frequency to the measured power Pmeas(PolMeas= PolLink=

5) Unlock the beam.

6) Connect the SS (System Simulator) with the DUT through the measurement antenna with polarization reference PolLink= to form the TX beam towards the TX beam peak direction.

7) Lock the beam.

8)	Measure the mean power Pmeas(PolMeas= PolLink=) of the modulated signal arriving at the power measurement equipment.

9)	Calculate EIRP(PolMeas= PolLink=) by adding the composite losses of the entire transmission path for utilized signal path, LEIRP,ϕ, and frequency to the measured power Pmeas(PolMeas= PolLink=).

10)	Calculate total EIRP = EIRP(PolMeas= PolLink= + EIRP(PolMeas= PolLink=

<< Unchanged sections omitted >>

##### 5.2.1.3.7	TX Beam Peak direction search and EIRP Spherical Coverage

The beam peak search and spherical coverage test procedure apply to DUTs with different beam correspondence capability, as defined in TS 38.306 [19]. For the purpose of DUT refining TX beam with beam correspondence capability, the System Simulator shall assure the presence of downlink reference signals including both SSB and CSI-RS and Type D QCL shall be maintained between SSB and CSI-RS.

The TX beam peak direction is found with a 3D EIRP scan (separately for each orthogonal downlink polarization). The TX beam peak direction search grid points for this single grid approach are defined in Annex G.2. Alternatively, a coarse and fine grid approach could be used according to the definition in Annex G.2.4.

The measurement procedure includes the following steps for each of the points in the grid:

1)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= polarization to form the TX beam towards the measurement antenna.

2)	DUT refines its TX beam toward that direction depending on DUT’s beam correspondence capability which shall match OEM declaration: if DUT’s beam correspondence capability is [bit-1], then DUT autonomously chooses the corresponding TX beam for PUSCH transmission using downlink reference signals to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping; if DUT’s beam correspondence capability is [bit-0], then DUT chooses the TX beam for PUSCH transmission which is based on beam correspondence with relying on both DL measurements on downlink reference signals and network-assisted uplink beam sweeping.

3)	Lock the beam.

4)	Measure the mean power Pmeas(PolMeas=PolLink=) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator).

5)	Calculate EIRP(PolMeas=PolLink=) by adding the composite loss of the entire transmission path for utilized signal path, LEIRP,θ, and frequency to the measured power Pmeas(PolMeas=PolLink=).

6)	Unlock the beam.

7)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink=polarization to form the TX beam towards the measurement antenna.

8) Lock the beam.

9)	Measure the mean power Pmeas(PolMeas=PolLink=) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator).

10) Calculate EIRP(PolMeas=PolLink=) by adding the composite loss of the entire transmission path for utilized signal path, LEIRP,, and frequency to the measured power Pmeas(PolMeas=PolLink=).

For beam correspondence capability [bit-0] DUT, the above step 2) can be further clarified as following sub-steps:

2.1) DUT uses downlink reference signals to select proper RX beam and uses autonomous beam correspondence to select the TX beam.

2.2) SS configures M=8 SRS resources to DUT, with the field spatialRelationInfo omitted. In case DUT supports less than 8 SRS resources, SS configures the number of SRS resources according to the maximum number of SRS resources indicated by UE capability signalling.

2.3) Based on the TX beam autonomously selected by DUT, DUT chooses TX beams to transmit SRS resources configured by SS.

2.4) Based on measurement of the received SRS, SS chooses the best SRS beam which is indicated in the field of SRS Resource Indicator (SRI) in the scheduling grant for PUSCH.

2.5) DUT transmits PUSCH corresponding to the SRS resource indicated by the SRI.

The TX beam peak direction is where the maximum of EIRP(PolMeas=PolLink=) + EIRP(PolMeas=PolLink=) is found.

The EIRP results from the TX beam peak search using the minimum number of grid points as described in Annex G.2 can be re-used for EIRP spherical coverage. In case a coarse beam peak grid is used for TX beam peak search, using the minimum number of grid points defined in Annex G.3.3.2.3, the EIRP results can be re-used for EIRP spherical coverage.

In case a separate test is performed for EIRP spherical coverage, the procedure above shall be followed using the minimum number of grid points defined in Annex G.3.3.2.3 for spherical coverage.

The EIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using EIRP(PolMeas=PolLink=) + EIRP(PolMeas=PolLink=) for all grid points. When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(θ).

<< Unchanged sections omitted >>

##### 5.2.1.3.9	Beam Correspondence Tolerance

The beam correspondence requirement is fulfilled if the DUT satisfies one of the following conditions, depending on the beam correspondence capability, as defined in TS38.306 [19]:

-	If [bit-1], the DUT shall meet the minimum peak EIRP requirement and spherical coverage requirement with its autonomously chosen UL beams and without uplink beam sweeping. Such a DUT is considered to have met the beam correspondence tolerance requirement.

-	If [bit-0], the DUT shall meet the minimum peak EIRP requirement and spherical coverage requirement with uplink beam sweeping. Such a DUT shall meet the beam correspondence tolerance requirement and shall support uplink beam management, as defined in TS 38.306 [19].

Thus, the beam correspondence tolerance test is only applicable to the DUT that has beam correspondence capability as [bit-0] (which shall match OEM declaration), such that DUT relies on uplink beam sweeping to fulfil the minimum peak EIRP and spherical coverage requirements.

ΔEIRPBC is introduced for beam correspondence tolerance based on two EIRP measurements (EIRP1 and EIRP2). EIRP1 is the measured total EIRP based on the beam which DUT chooses autonomously (corresponding beam) to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping. EIRP2 is the measured total EIRP based on the beam yielding highest EIRP in a given direction, which is based on beam correspondence with relying on UL beam sweeping. ΔEIRPBC shall be calculated over the link angles spanning a subset of the spherical coverage grid points which are corresponding to the top Nth percentile of the EIRP2 measurement points in the grid, where the value of N is according to EIRP spherical coverage requirement of DUT’s power class defined in TS 38.101-2 [16] clause 6.2.1, e.g., N=50 for power class 3 DUT.

For each of the points in the grid:

1)	Follow the test procedures specified in subclause 5.2.1.3.7 with uplink beam sweeping disabled, obtain EIRP1(PolMeas=PolLink=) and EIRP1(PolMeas=PolLink=EIRP1 is calculated by EIRP1 = EIRP1(PolMeas=PolLink=) + EIRP1(PolMeas=PolLink=).

2)	Follow the test procedures specified in subclause 5.2.1.3.7, with uplink beam sweeping enabled (SS does not configure the spatialRelationInfo to DUT) during DUT TX beam refinement, obtain EIRP2(PolMeas=PolLink=) and EIRP2(PolMeas=PolLink=EIRP2 is calculated by EIRP2 = EIRP2(PolMeas=PolLink=) + EIRP2(PolMeas=PolLink=).

3)	Calculate the ΔEIRPBC = EIRP2 – EIRP1.

The ΔEIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using ΔEIRPBC for each of all top Nth percentile of the EIRP2 measurement points in the grid. When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(θ).

Editor’s Note: The side conditions for downlink reference signals SSB and CSI-RS in beam correspondence tolerance test are FFS.

<< Unchanged sections omitted >>

##### 5.2.3.3.2	Peak EIRP Measurement Procedure

The TX beam peak direction is where the maximum total component of EIRP is found, including the respective polarization of the measurement antenna used to form the TX beam, according to 5.2.1.3.7.

1)	Connect the SS (System Simulator) with the DUT through the measurement antenna with polarization reference PolLink= to form the TX beam towards the TX beam peak direction.

2)	Lock the beam.

3)	Measure the mean power Pmeas(PolMeas= PolLink=) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator).

4)	Calculate EIRP(PolMeas= PolLink= by adding the composite loss of the entire transmission path for utilized signal path, LEIRP,θ, and frequency to the measured power Pmeas(PolMeas= PolLink=.

5) Unlock the beam.

6) Connect the SS (System Simulator) with the DUT through the measurement antenna with polarization reference PolLink= to form the TX beam towards the TX beam peak direction.

7) Lock the beam.

8)	Measure the mean power Pmeas(PolMeas= PolLink=) of the modulated signal arriving at the power measurement equipment.

9)	Calculate EIRP(PolMeas= PolLink=) by adding the composite losses of the entire transmission path for utilized signal path, LEIRP,ϕ and frequency to the measured power Pmeas(PolMeas= PolLink=).

10)	Calculate total EIRP = EIRP(PolMeas= PolLink= + EIRP(PolMeas= PolLink=.

<< Unchanged sections omitted >>

##### 5.2.4.2.2	Peak EIRP Measurement Procedure

The TX beam peak direction is where the maximum total component of EIRP is found, including the respective polarization of the measurement antenna used to form the TX beam, according to 5.2.1.3.7.

The measurement procedure includes the following steps:

1)	Connect the SS (System Simulator) to the DUT through the measurement antenna with polarization reference PolLink to form the TX beam towards the TX beam peak direction.

2)	Lock the beam.

3)	Determine the EIRP for both polarization towards the TX beam peak direction by using a Near Field to Far Field transform.

4)	Calculate total EIRP = EIRP(PolMeas= PolLink= + EIRP(PolMeas= PolLink=.

<<< End of changed sections>>>