3GPP TSG-RAN WG4 Meeting #92	R4-1908660

Ljubljana, SI, 26th– 30th Aug, 2019

Agenda item:	9.11.2	Feasibility Study 	[NR_DL256QAM_FR2]

Source: 		NTT DOCOMO, INC.

Title: 	Feasibility evaluation for NR FR2 DL256QAM

Document for:	Approval

- Introduction

In RAN4#91, simulation assumptions of feasibility study for FR2 DL 256QAM was agreed [1]. In this contribution, we provide link level simulation results compared between 64QAM and 256QAM and discuss on the feasibility.

2.	Discussion

Based on the agreed simulation assumption [1], the following simulation assumptions are used:

Table 1: Simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257) |
| CBW | 100MHz |
| SCS | 120 kHz |
| Allocated RBs | Full allocation |
| Propagation | Static TDL-A 30ns delay spread, 35Hz Doppler frequency |
| MCS | fixed MCSs 64QAM: MCS 24, 25, 26, 28 256QAM: MCS 21, 23, 25, 27 |
| Precoding | Precoding configuration defined in 38.101-4 Section 7.2 for fading channels and Section 7.5 for static channel; follow PMI |
| Symbol type | CP-OFDM |
| HARQ | 8 |
| Antenna configuration | Fading channel: 2x2 for Rank2, Low correlation Static channel: 2x2 for Rank2 |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Ideal CPE compensation |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): PN model config1: example1 (BS) + example1(UE) |

Figure 1 and 2 show link level simulation results compared between 64QAM and 256QAM. On static channel with antenna configuration 2x2 and Rank 2, even with  3% Tx EVM and 3% Rx EVM, the performance gain compared to 64QAM modulation is obtained over 21dB SNR. This SNR is a realistic value that can be achieved in the expected deployment (e.g., Small cell scenario).

Observation 1: From the evaluation results, FR2 DL 256QAM modulation has a better performance than 64QAM modulation with realistic SNR.

a) Tx EVM 0% and Rx EVM 0%						 b) Tx EVM 3% and Rx EVM 3%

Figure 1: Simulation results on static channel

To be added

Figure 2: Simulation results on TDL-A channel

From above evaluations, we can conclude that DL 256QAM in FR2 can provide certain system performance gain in realistic network. Therefore, we propose to introduce the requirement of FR2 DL 256QAM.

Regarding Tx EVM, our simulation assumed 3% considering the feasible value. As a requirement for BS Tx EVM, it is sufficient to define 3.5% which is the same requirement as FR1. Therefore, it is proposed to define 3.5% as a requirement for BS Tx EVM for 256QAM modulation.

Proposal 1: Introduce RAN4 requirements for FR2 DL 256QAM modulation.

Proposal 2: Define 3.5% as a requirement for BS Tx EVM with 256QAM modulation in FR2.

3.	Conclusion

In this contribution, we provide simulation results for the feasibility study of FR2 DL 256QAM. The following observation and proposals are obtained.

Observation 1: From the evaluation results, FR2 DL 256QAM modulation has a better performance than 64QAM modulation with realistic SNR..

Proposal 1: Introduce RAN4 requirements for FR2 DL 256QAM modulation.

Proposal 2: Define 3.5% as a core requirement for BS Tx EVM with 256QAM modulation in FR2.

References

- R4-1907711, “Simulation assumption of feasibility study for FR2 DL 256QAM”, RAN4 #91, May 2019