3GPP TSG-RAN WG4 #92	R4-1908875

Ljubljana, Slovenia, 26th – 30th August 2019

Agenda item:	9.1.1.1

Source: 	Qualcomm Incorporated

Title: 	Channel raster in NR-U

Document for:	Approval

# Introduction

A study item on NR based access to unlicensed spectrum has been concluded in RAN#82 plenary in December 2018 [1]. At the same time, a new WI has been approved on NR-U in 5 and 6 GHz [2] which involves RAN4.

In this contribution, we treat the channel raster for NR-U in band n46. An approach similar to definition of LAA channel raster will be followed and proposals on specific frequency position of the raster will be made in this contribution.

# Discussion

Differently than LAA, NR-U will be able to support larger channel bandwidths multiple of 20MHz. Because of this, it is not possible to re-use the channel raster as defined for LAA and additional frequency points will have to be defined. For example, we would not be able to fit a 40MHz or 80MHz channel to the current channel raster defined for LAA. For this reason, in Section 2.1 we make a proposal for the definition of the channel raster in band n46.

## Channel raster for NR-U in band n46

We believe that there is no need to down-sample the currently defined global frequency channel raster with 15kHz granularity. This will allow more flexible NR-U operation, especially in the cases where NR-U does not need to co-exist with other technologies. However, care should be taken in the case NR-U is deployed in an area where another technology (Wi-Fi/LAA) may be operating, in order to guarantee optimal co-channel co-existence.

Proposal: NR-U operation in band n46 will be based on the global frequency channel raster defined in Table 5.4.2.1-1 in [3] with a granularity of 15kHz.

The entry in channel raster Table 5.4.2.3-1 in [3] would read:

| NR Operating Band | ∆FRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n46 (NOTE) | 15 | 744000 - <1> - 794333 | 744000 - <1> - 794333 |

NOTE: The values of NREF between 756001 (5340.015 MHz) - <1> - 765333 (5479.995 MHz) are not allowed for operation in band n46

# Conclusions

In this contribution we presented our views on the definition of channel raster for NR-U operation in unlicensed band n46.

Based on the above observations, we made the following proposals:

Proposal: NR-U operation in band n46 will be based on the global frequency channel raster defined in Table 5.4.2.1-1 in [3] with a granularity of 15kHz.

The entry in channel raster Table 5.4.2.3-1 in [3] would read:

| NR Operating Band | ∆FRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n46 (NOTE) | 15 | 744000 - <1> - 794333 | 744000 - <1> - 794333 |

NOTE: The values of NREF between 756001 (5340.015 MHz) - <1> - 765333 (5479.995 MHz) are not allowed for operation in band n46

# Reference

- RP-181339, “Revised SID on NR-based Access to Unlicensed Spectrum”, Qualcomm Incorporated.

- RP-182878, “NR-based Access to Unlicensed Spectrum”, Qualcomm Incorporated.

- 3GPP TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, v15.4.0.

- R4-1906697, “Observations on synchronization raster in NR-U”, Qualcomm Incorporated.

- 3GPP TS 36.101, “User Equipment (UE) radio transmission and reception”, v16.0.0