3GPP TSG-RAN WG4 Meeting #92				  												R4-1909256

Ljubljana, Slovenia, 26th Aug. 2019 - 30th Aug. 2019

Source: 	Nokia, Nokia Shanghai Bell, T-Mobile USA

Title: 						TP to TR 38.716-02-00: CA_n41-n261

Agenda Item: 			10.2.3 [NR_CADC_R16_2BDL_xBUL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 38.716-02-00 [1] to introduce the combinations listed in the table below following the WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2) [2].

| CA configuration | Uplink Configuration | BCS | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41C-n261A | CA_n41A | 0 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | New | (ongoing) DL_n41A-n261A_UL_n41A_BCS0 (completed) DL_n41C_UL_n41A_BCS0 |
| CA_n41(2A)-n261A | CA_n41A | 0 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | New | (ongoing) DL_n41A-n261A_UL_n41A_BCS0 (ongoing) DL_n41(2A)_UL_n41A_BCS1 |
| CA_n41(2A)-n261(2A) | CA_n41A | 0 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | New | (ongoing) DL_n41A-n261(2A)_UL_n41A_BCS0 (new) DL_n41(2A)-n261A_UL_n41A_BCS0 |
| CA_n41C-n261(2A) | CA_n41A | 0 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | New | (ongoing) DL_n41A-n261(2A)_UL_n41A_BCS0 (new) DL_n41C-n261A_UL_n41A_BCS0 |

The TP updates section 8.3 in TR 38.716-02-00 especially the content of Table 8.3.1.2-1.

# 2	Text Proposal

*********************************** Start of the TP ***********************************

## 8.3	CA_n41-n261

### 8.3.1	Common for 1 band UL and 2 bands UL CA

#### 8.3.1.1		Operating bands for CA

Table 8.3.1.1-1: CA band combination of band n41+n261

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2469 MHz | – | 2690 MHz | 2469 MHz | – | 2690 MHz | TDD |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

#### 8.3.1.2		Channel bandwidths per operating band for CA

Table 8.3.1.2-1: Supported bandwidths per CA band combination of band n41+n261

| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n261A | - | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n41A-n261(2A) | - | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See Table 5.5A.2-1 CA_ n261(2A) BCS0 in 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41C-n261A | - | n41 | See Table 5.5A.1-1 CA_ n41C BCS0 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n261 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n41(2A)-n261A | - | n41 | See Table 5.5A.2-1 CA_ n41(2A) BCS1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n261 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n41(2A)-n261(2A) | - | n41 | See Table 5.5A.2-1 CA_ n41(2A) BCS1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n261 | See Table 5.5A.2-1 CA_ n261(2A) BCS0 in 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41C-n261(2A) | - | n41 | See Table 5.5A.2-1 CA_ n41(2A) BCS1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n261 | See Table 5.5A.2-1 CA_ n261(2A) BCS0 in 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.1.3		UE co-existence studies

Harmonic relation between FR1 and FR2 band is not analyzed due to large frequency separation.

The additional spurious emission level to coexisting bands is specified in Table 8.3.1.3-1.

Table 8.3.1.3-1: Requirements for uplink inter-band CA (two bands)

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n41-n261 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |

#### 8.3.1.4		∆TIB and ∆RIB values

For CA_n41-n261, the TIB,c and RIB,c values are all zero and thus do not need to be explicitly specified in TS 38.101-3.

#### 8.3.1.5		REFSENS requirements

There are no specific REFSENS requirements.

*********************************** End of the TP ***********************************

# 3	References

[1] 	R4-1905855 TR 38.716-02-00 v0.5.0, ZTE.

[2] 	RP-191350 Revised WID:  NR inter-band CA/DC for 2 bands DL with up to 2 bands UL, ZTE.