3GPP TSG-RAN WG4 Meeting #92	R4-1909533

Ljubljana, Slovenia, 26 August – 30 August 2019

Source:	ZTE

Title:	Discussion on one shot timing adjustment requirements

Agenda item:	7.10.6.1.2

Document for:	Discussion

1. Introduction

In the RAN4#91 meeting, there was good progress on uplink timing adjustment due to beam switch. The requirements have been captured in section 7.1.2.2 in TS38.133. It is copied blow. The threshold H for UE reception time change has not been finalized yet.

#### 7.1.2.2 One shot timing adjustment When the magnitude of the T exceeds H then the UE shall adjust its transmission timing in one adjustment only once provided that the following conditions are met at the UE. Otherwise when the the magnitude of the T ≤ H then the UE shall adjust its transmission timing according to the rules defined in clause 7.1.2.1. - SSB_RP and SSB Ês/Iot according to Annex B.2.6.1 for a corresponding operating Band, The UE transmit timing immediately after applying the one shot timing adjustment shall be: T2-(NTA+NTA offset)+2(T1-T2). After applying the one shot timing adjustment the UE shall adjust its transmission timing according to the rules defined in clause 7.1.2.1. Where: T= T1-T2 - T1 is the reception time at the UE just before the one shot timing adjustment, - T2 is the reception time to be used at the UE just after the one shot timing adjustment, - H is defined in table 7.1.2.2-1. Table 7.1.2.2-1: The value of H Frequency Range; SCS of SSB signals (KHz); SCS of uplink signals s(KHz); H [Tc]. 1; 15; 15; TBD. 30; TBD. 60; TBD. 30; 15; TBD. 30; TBD. 60; TBD. 2; 120; 60; TBD. 120; TBD. 240; 60; TBD. 120; TBD.

In agreed way forward [1] on UE timing adjustment with beam switching, it was agreed that interruption due to UE autonomous timing adjustment needs to be further studied.

- Interruption due to UE autonomous timing adjustment: - When UE perform one shot timing adjustment, interruption is allowed for both DL and UL transmission. - Depending on the value of H, the interruption duration is FFS. - The location of the interruption in time is FFS.

In this contribution we provide our views on threshold H for one shot timing adjustment and interruption due to UE autonomous timing adjustment.

2. Discussion

2.1. Threshold H

Though the issue was triggered due to beam switch, the requirements were specified in a general way. When there is large change of UE downlink reception timing that exceeds certain threshold H, no matter what the reason is, the UE should perform one shot timing adjustment. This would reduce adjustment time period for UE to converge uplink transmission timing error within Te.

The uplink timing adjustment is to ensure uplink signals arrive at BS in an expected timing range which would not degrade BS demodulation performance. There are unavoidable uplink timing errors due to downlink reception timing estimation error, radio propagation delay estimation error and timing adjustment accuracy. The uplink timing error for NR FR1 and FR2 are summarized in Table 1.

Table 1. Uplink timing error

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals (KHz) | Te | Step of uplink TA command | UE Timing Advance adjustment accuracy | Summation of uplink timing error |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc | 16*64*Tc | 4*64*Tc | 32*64*Tc |
|  |  | 30 | 10*64*Tc | 8*64*Tc | 4*64*Tc | 22*64*Tc |
|  |  | 60 | 10*64*Tc | 4*64*Tc | 2*64*Tc | 16*64*Tc |
|  | 30 | 15 | 8*64*Tc | 16*64*Tc | 4*64*Tc | 28*64*Tc |
|  |  | 30 | 8*64*Tc | 8*64*Tc | 4*64*Tc | 20*64*Tc |
|  |  | 60 | 7*64*Tc | 4*64*Tc | 2*64*Tc | 13*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc | 4*64*Tc | 2*64*Tc | 9.5*64*Tc |
|  |  | 120 | 3.5*64*Tc | 2*64*Tc | 0.5*64*Tc | 6*64*Tc |
|  | 240 | 60 | 3*64*Tc | 4*64*Tc | 2*64*Tc | 9*64*Tc |
|  |  | 120 | 3*64*Tc | 2*64*Tc | 0.5*64*Tc | 5.5*64*Tc |

When there is large timing change, e.g. due to beam switch, UE needs to perform one shot timing adjustment. The threshold H should at least be no less than Tq for maximum autonomous time adjustment step, as in table 2 below. Otherwise UE can do gradual timing adjustment to ensure the uplink timing accuracy.

Table 2. Tq maximum autonomous time adjustment step

| Frequency Range | SCS of uplink signals (KHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2 | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

On the other hand UE may do one shot adjustment frequently if the threshold H is too small. In some cases it may not be necessary. When downlink reception timing change is between Tq and H, UE needs to do one step timing adjustment with Tq immediately in the uplink. However this is not specified in current spec. UE may do this Tq adjustment in 200ms period. In this case the threshold H is an extra uplink timing error to be considered.

The threshold H should not be too larger either. If it is too large, e.g. as half CP, the uplink signal will arrive at BS outside of expected timing range. The BS demodulation performance will be highly degraded.

The total uplink timing error when threshold H is taken into account is summarized in Table 3 with proposed threshold H, which is around 10 percent of UL CP length. It is noted in FR1 for 60 kHz SCS the uplink timing error without H is 16*64*Tc. It is already consumed almost half CP length (18*64*Tc). So Tq is proposed as H in this case. For FR2, similar to 60 kHz SCS in FR1 that uplink timing error without considering threshold H is already close to half CP length, there is no room for large threshold H.

Table 3. Uplink timing error with threshold H

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals (KHz) | Summation of uplink timing error | Threshold H | Total uplink timing error with H | UL CP length |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 15 | 32*64*Tc | 16*64*Tc | 48*64*Tc | 144*64*Tc |
|  |  | 30 | 22*64*Tc | 8*64*Tc | 30*64*Tc | 72*64*Tc |
|  |  | 60 | 16*64*Tc | 5.5*64*Tc | 21.5*64*Tc | 36*64*Tc |
|  | 30 | 15 | 28*64*Tc | 16*64*Tc | 44*64*Tc | 144*64*Tc |
|  |  | 30 | 20*64*Tc | 8*64*Tc | 28*64*Tc | 72*64*Tc |
|  |  | 60 | 13*64*Tc | 5.5*64*Tc | 18.5*64*Tc | 36*64*Tc |
| 2 | 120 | 60 | 9.5*64*Tc | 3.5*64*Tc | 13*64*Tc | 36*64*Tc |
|  |  | 120 | 6*64*Tc | 2.5*64*Tc | 8.5*64*Tc | 18*64*Tc |
|  | 240 | 60 | 9*64*Tc | 3.5*64*Tc | 12.5*64*Tc | 36*64*Tc |
|  |  | 120 | 5.5*64*Tc | 2.5*64*Tc | 8*64*Tc | 18*64*Tc |

It can be seen from Table 3 that with proposed threshold H there is some implementation margin, considering half CP timing alignment, for lower SCS, but the margin is very small for higher SCS in FR1 and FR2. So the threshold H should not be larger than the value in Table 3.

Proposal 1. The threshold H is proposed in Table 4.

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | H [Tc] |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 16*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 5.5*64*Tc |
|  | 30 | 15 | 16*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 5.5*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 2.5*64*Tc |
|  | 240 | 60 | 3.5*64*Tc |
|  |  | 120 | 2.5*64*Tc |

2.2. Interruptions due to UE autonomous timing adjustment

In the last meeting there is concern that in some cases there may be interruptions due to UE one shot autonomous timing adjustment. The one shot timing adjustment is depicted as in Figure 1.

Figure 1. One shot adjustment

It can be seen from Figure 1 that when the timing change Tdelta is larger enough the UE Rx-Tx switching timing may be compromised. This may be the only extreme case that interruptions may happen. However in our view such case should happen in practical network rarely, if it could happen. The UE Rx-Tx switching time, BS Tx-Rx switching time and propagation delay in the cell coverage are all taken into account when guard period is configured. The guard period should be long enough to cover all UEs in the cell coverage. It is also noted that guard period is configured in symbol granularity which means there is usually enough margin for timing advance.

If large downlink timing change happens, as long as the UE is still in the coverage of the cell the timing advance after one shot timing adjustment could not consume all of the guard period except the part for UE and BS Rx/Tx switching. Therefore no interruption is foreseen during UE one shot timing adjustment.

Proposal 2. No interruption is allowed during one shot timing adjustment.

3. Conclusion

In this contribution, we provided our views on threshold H for one shot timing adjustment and interruption due to UE autonomous timing adjustment. Based on the observations following proposals are present.

Proposal 1. The threshold H is proposed in Table 4.

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | H [Tc] |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 16*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 5.5*64*Tc |
|  | 30 | 15 | 16*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 5.5*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 2.5*64*Tc |
|  | 240 | 60 | 3.5*64*Tc |
|  |  | 120 | 2.5*64*Tc |

Proposal 2. No interruption is allowed during one shot timing adjustment.

4. References

- R4-1907203	Way forward on UE timing adjustment with Beam switching, Ericsson, et al