3GPP TSG-RAN WG4 Meeting #92	R4-1909549

# Ljubljana, Slovenia,  26 August, 2019 - 31 August, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Gap sharing between intra- and inter-frequency for NR-DC in section 9.1.5.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  | Date: |  |  | 2019/16/8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Gap sharing for NE-DC was agreed to be based on configured measurement objects in RAN4#91 Agreement: - Whether to account for FR2 intra-frequency measurements in Pool A (intra-frequency MOs) or Pool B (inter-frequency/RAT MOs) - In Pool B when there are no inter-frequency/RAT MOs configured, otherwise in Pool A. However, in the agreed CR, the decision on how to count the MO is done on a gap by gap basis In the case of Minter,i,j =0 for UE configured with per-UE gap, FR2 intrafrequency measurement objects shall be counted as interfrequency measurement objects in calculating Minter,i,j, while only FR1 intrafrequency measurement shall be counted as intrafrequency measurement objects in calculating Mintra,i,j. This creates several issues - Minter,i,j may be zero in some gaps and non zero in others depending on SMTC configurations which is not aligned with RAN4 agreements - The definition may become confusing/circular because intra FR2 MO are being counted as if they are inter MO according to this sentence. But then Minter,i,j will no longer be equal to zero, and the condition no longer applies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Explicitly define pool A and pool B. Define the conditions for membership of pool A and pool B, and define CSSFwithin_gap,I according to the competing measurement objects in pool A and pool B. Delete the editor notes: FFS if better description exists for the case when the number of configured interfrequency and interRAT measurement objects is zero and the UE is configured with per-UE gap |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing description remains in specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Change 1

##### 9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this section.

If measurement object i refers to an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, CSSFwithin_gap,i=1. Otherwise, the the CSSFwithin_gap,i for other measurement objects (including RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intrafrequency measurement objects and interfrequency/interRAT measurement objects which are candidates to be measured within the gap j.

-	An NR carrier is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intrafrequency NR carriers, if the high layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An interRAT measurement object is a candidate to be measured in all meausrement gaps.

Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 1280ms period.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

Per gap j:

If the number of configured interfrequency and interRAT measuerement objects is non-zero and the UE is configured with per UE gaps, or if the UE is configured with per FR gaps:

FR1 and FR2 intrafrequency measurement objects belong to group A

Interfrequency and interRAT measurement objects belong to group B

MgroupA,i,j: Sum of the number of FR1 intrafrequency measurement objects Mintra-FR1,i,j and the number of FR2 intrafrequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. I MgroupA,i,j  equals 0.

MgoupBi,j : Number of NR interfrequency and EUTRA interRAT measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

If the number of configured interfrequency and interRAT measuerement objects is zero and the UE is configured with per UE gaps:

FR1 intrafrequency measurement objects belong to group A

FR2 intrafrequency measurement objects belong to group B

MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is a group A measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×MgroupA,i,j) in gaps where MgroupA,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupA,i,j) in gaps where MgroupA,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an group B measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×MgroupAi,j) in gaps where MgroupB,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupB,i,j) in gaps where MgroupB,i,j=0, where j=0…(160/MGRP)-1