3GPP TSG-RAN WG4 Meeting #92-Bis	R4-1910754

# Chongqing, China, 14th – 18th October, 2019

Source: 	Qualcomm Inc.

Title: 	General Spectrum Emission Mask for NR UE 7-24

Agenda Item:	10.4.7.2

Document for:	Approval

# Introduction

Table 1-2 to Table 1-5 show general spectrum emission mask and spurious emission limits for FR1 and FR2, respectively. Figure 1-1 summarizes this information for a 50MHz BW mode. Note that depending on frequency of the upper edge of FR1 UL operating band, -30dBm/MHz requirements can extend all the way up to 26GHz. For FR2, -30dBm requirement is applicable all the way down to 12.75GHz.

Observation 1: Spurious emissions from FR1/FR2 bands in 7-24GHz range is limited to -30dBm/MHz

Spurious emission limits are dictated by ITU-R recommendations and are unlikely to change for 7-24GHz range. On the other hand, general spectrum emission mask depends on RF technology and will most certainly differ between the 3 example ranges of 7-24GHz.

# Discussion

It has been widely accepted that example ranges 1 and 3 will use implementations similar to FR1 and FR2, respectively. We propose that the same general spectrum emission mask is used for example ranges 1 and 2.

Proposal 1: Example Range 1 should use similar general spectrum emission mask and spurious emissions limits as FR1

Proposal 2: Example Range 3 should use similar general spectrum emission mask and spurious emissions limits as FR2

The situation for example range 2 is less certain and requires further study.

Table 1-1: Example Ranges [1]

| Example range | Frequency Range (GHz) | Example Frequency (GHz) |
| --- | --- | --- |
| 1 | 7.125 to [10-13] | 10 |
| 2 | [10-13] to [16-18] | 15 |
| 3 | [16-18] to 24.250 | 20 |

Table 1-2: FR1 General spectrum emission mask

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 | -25 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  | -25 |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  | -25 |  |

Table 1-3: FR1 spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz |  |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. |  |  |  |

Table 1-4: FR2 General spectrum emission mask

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
| 0-5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | 1 MHz |
| 40-100 | -13 | -13 | -13 | -13 | 1 MHz |
| 100-200 |  | -13 | -13 | -13 | 1 MHz |
| 200-400 |  |  | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | 1 MHz |
| NOTE 1: Void |  |  |  |  |  |

Table 1-5: FR2 spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| 12.75 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 1 MHz |

Figure 1-1: FR1/FR2 General spectrum emission mask and spurious emissions limits for 50MHz BW mode

# Conclusion

Observation 1: Spurious emissions from FR1/FR2 bands in 7-24GHz range is limited to -30dBm/MHz

Proposal 1: Example Range 1 should use similar general spectrum emission mask and spurious emissions limits as FR1

Proposal 2: Example Range 3 should use similar general spectrum emission mask and spurious emissions limits as FR2

# References

- R4-1907798, TP to TR 38.820: Addition of example frequencies in subclause 6.2, RAN4 #91