3GPP TSG-RAN4  #92-Bis																	     	R4-1910764

Chongqing, CN, 14-18 Oct. 2019

Source: 	Qualcomm Incorporated

Title: 	TP to TR38.831: UE Architectures for FR2 DL CA BW Enhancement

Agenda item:	8.14.4

Work item:	NR_RF_FR2_req_enh

Release:	16

Document for:	Approval

# Introduction

In [1], we discussed the rel. 16 DL CA BW enhancement feature in context of practical UE architecture choices. In this contribution, we propose a TP to TR based on the UE architecture study in [1].

# Text Proposal

## 4.3	FR2 UE requirements for contiguous intra-band DL CA for aggregated bandwidth larger than 1200 MHz

4.3.1	General

Enhanced DL CA aggregated BW is gated by:

- DL data pipe size in the digital domain

- Signal-handling bandwidth of the ‘front end’, which include RF electronics, ADCs, filters, etc. This parameter is identical to frequency separation, which is defined as the frequency span between the lower edge of the lowest component carrier and the upper edge of the highest component carrier that a UE can support simultaneously.

DL data pipe size enhancement is left to UE implementation. UE frequency separation enhancement is addressed in the UE architecture study in the next subsection. The following assumptions apply:

- The UE’s UL electronics (frequency separation ability) and data handling capacity (‘data pipe’) are left untouched while the DL CA capability is enhanced

- The UE’s DL CA frequency spectrum coverage, i.e the band where a UE can support placing DL CCs is contiguous. This assumption does not mean DL CA must also be contiguous.

- All DL CCs have the same AoA

    - UE Architecture Choices

4.3.2.1		Single receive chain with expanded capability

The receiver design comprises a single chain with enhanced capability relative to rel. 15.

Figure 4.3.2.1-1: Single enhanced receiver to support wider frequency separation

The UL architecture in a UE with independent T/R LOs allows UL coverage to match DL coverage with retuning of TxLO. In the topology with common LOs, the enhanced DL coverage will extend past the UL coverage equally on both sides.

Figure 4.3.2.1-2: UL options for UE with single enhanced receiver chain

4.3.2.2		Multiple parallel receive chains

This implementation arranges multiple receiver chains to work in parallel to deliver CA BW enhancement.

Figure 4.3.2.2-1: Multiple receiver chains to support wider frequency separation

For UEs with independent Rx and Tx LOs, the UE’s UL coverage can be made to match it with retuning of TxLO. For UEs that utilize a common LO, the UL coverage must be closely related to DL coverage of one of the receivers.

Figure 4.3.2.2-2: UL options for UE with multiple receiver chain

4.3.2.3		UE architecture summary

The UE topologies above exhibit differing UL and DL coverage, summarized in table 4.3.2.3-1.

|  | UL Coverage in Relation to enhanced DL coverage type |  |
| --- | --- | --- |
|  | Independent R/T LOs | Common R/T LOs |
| Single Enhanced receiver | Matches DL coverage | DL coverage extends past UL coverage equally on both sides |
| Multiple Rel. 15 receivers | Matches DL coverage | DL coverage extends past UL coverage on one side |

Table 4.3.2.3-1: UL and DL coverage comparison for UEs

# Reference

- R4-1908164, ‘UE Architecture for DL CA BW Enhancement for Rel. 16 FR2’, Qualcomm, RAN4#92, Ljubljana, Sl, Aug 2019