3GPP TSG-RAN WG4 Meeting #92b                 	 R4-1910815

Chongqing, China, 14th - 18th Oct, 2019

Agenda Item:	6.10.6.1.1

Source:	MediaTek Inc.

Title:	UE UL timing adjustment due to Rx beam change

Document for:		Discussion

# Introduction

UE will take either gradual or one-shot UL Tx timing adjustment, depending on how large the timing difference UE observed. There are still some open issues left, e.g.,

- The threshold H

- The accuracy (Te1) of one-shot adjustment

- interruption requirement (duration and location)

In last RAN4 meeting, the issue of UE UL Tx timing adjustment due to Rx beam change was further discussed without reaching conclusion. Analyses from [1] and [2] provided views from the aspect of BS, while papers [3][4][5] are from UE’s perspective. In this paper, we discuss above remaining issues.

# Discussion

Based on the agreed CR [1], UE’s behavior on how to adjust its UL timing can be illustrated as Figure 1.

- When the timing difference between new and old beams are smaller than Te, UE does not need to take any actions

- When the timing difference is larger than Te but smaller than H, UE adjustment its UL Tx timing based on legacy requirement (gradual small-step adjustment)

- When the timing difference is larger than H, UE will conduct a one-shot big-step timing adjustment along the opposite direction of DL timing change and with twice the amount of difference between new and old DL timing. After this one-shot adjustment, UE fallbacks to gradual small-step adjustment. In the same time, it is expected that BS will also send TA commands to maintain a proper UL Rx timing at BS’s receiver.

Figure 1. Three different UE behaviors on UL timing adjustment to deal with DL timing change.

The threshold H

From network side, the basic idea from [1] and [2] is that the threshold H is somehow to largest DL timing difference that UE will still perform small step UL Tx timing adjustment. When such a difference is resulted from the change of DL propagation delay, then one can also expect similar change in UL propagation delay. One-shot adjust is one efficient way to directly compensate the change in both DL and UL propagation delay. However, for small step adjustment, network will still see some UL timing jump at its receiver side. If this UL timing jump is too large, e.g., larger than CP, then the UL decoding performance cannot guaranteed, leading some degradation in UL throughput as well as the waste of UE’s transmitting power.

The overall timing error of UL reception is calculated in Table 1. (The detail can be found in [1] and [2]). We add some column (F) to consider the impact of different H on the overall BS error. We see that if H is somehow within the range of 25~30% of the UL CP, then the overall BS error could be roughly controlled around half of CP.

Table 1. Calculation of overall BS timing error in reception

| SSB SCS [kHz] | UL SCS [kHz] | A: UE timing error [Ts] | B: TA command resolution [Ts] | C: TA adjustment accuracy [Ts] | D: Overall BS error without H [Ts] | UL CP [Ts] | E: D over UL CP length | F: Overall BS error with H over CP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | H=50% | H=33% | H=30% | H=25% |
| 15 | 15 | 12 | 8 | 4 | 24 | 144 | 17% | 67% | 50% | 47% | 42% |
|  | 30 | 10 | 4 | 4 | 18 | 72 | 25% | 75% | 58% | 55% | 50% |
|  | 60 | 10 | 2 | 2 | 14 | 36 | 39% | 89% | 72% | 69% | 64% |
| 30 | 15 | 8 | 8 | 4 | 20 | 144 | 14% | 64% | 47% | 44% | 39% |
|  | 30 | 8 | 4 | 4 | 16 | 72 | 22% | 72% | 55% | 52% | 47% |
|  | 60 | 7 | 2 | 2 | 11 | 36 | 31% | 81% | 64% | 61% | 56% |
| 120 | 60 | 3.5 | 2 | 2 | 7.5 | 36 | 21% | 71% | 54% | 51% | 46% |
|  | 120 | 3.5 | 1 | 0.5 | 5 | 18 | 28% | 78% | 61% | 58% | 53% |
| 240 | 60 | 3 | 2 | 2 | 7 | 36 | 19% | 69% | 52% | 49% | 44% |
|  | 120 | 3 | 1 | 0.5 | 4.5 | 18 | 25% | 75% | 58% | 55% | 50% |
| Note |  |  |  |  | D = A+B+C |  | E=D/CP | F = E+H |  |  |  |

Observation 1: When H is somehow within the range of 25~30% of the UL CP, then the overall BS error could be roughly controlled around half of CP.

From UE side, the threshold H should be at least larger than the sum of DL timing estimation error on serving Tx-Rx beam pair and target Tx-Rx beam pair, as shown in Figure 2. So that we can guarantee that the UL timing accuracy will not introduce problem in UE’s behaviour on choosing between small-step or one-shot timing adjustments.

Figure 2. Criterion to determine H from UE perspective

Table 2 provides the ratios of H over UL CP are calculated for different cases base on H=2*Te. The largest ratio is 56% when DL SCS is 15KHz and UL SCS is 60KHz. For the other cases, the largest ratio is no larger than 40% of CP. Therefore, from UE perspective, reasonable H is within the range of 40~56%.

Table 2. H and Ratio of H over CP

| FR | SCS of SSB [KHz] | SCS of UL signals [KHz] | Te | H = 2* Te | UL CP | Ratio: H over CP |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [Ts] | [Ts] | [Ts] |  |
| 1 | 15 | 15 | 12 | 24 | 144 | 17% |
|  |  | 30 | 10 | 20 | 72 | 28% |
|  |  | 60 | 10 | 20 | 36 | 56% |
|  | 30 | 15 | 8 | 16 | 144 | 11% |
|  |  | 30 | 8 | 16 | 72 | 22% |
|  |  | 60 | 7 | 14 | 36 | 39% |
| 2 | 120 | 60 | 3.5 | 7 | 36 | 19% |
|  |  | 120 | 3.5 | 7 | 18 | 39% |
|  | 240 | 60 | 3 | 6 | 36 | 17% |
|  |  | 120 | 3 | 6 | 18 | 33% |

Observation 2: From UE’s perspective, reasonable H is within the range of 40~56%.

Based on above analysis, we can see that the range of wanted H is different from network and UE perspectives. Some compromise in both sides are needed. Our suggestion is to try to agree on 33% of the CP for all SCSs. If this is not possible, then SCS-specific H can be considered.

Proposal 1: RAN4 tries to agree on H to be 33% of the CP for all SCSs. If not possible, try to work on SCS-specific H.

The accuracy (Te1) of one-shot adjustment

From above discussion of H, we can see that the DL timing estimation accuracy requirement of non-serving beam is already implicitly introduced when we decide the value U. Therefore, we suggest that there is no need to further explicitly specify the accuracy requirement for Te1.

Proposal 2: No explicit accuracy requirement is specified for UL Tx transmit timing on non-serving beam, because it is already implicitly considered in the threshold H.

Interruption requirement

The one-shot timing adjustment could happen upon TCI-state switch or UE’s autonomous Rx beam change. For the former one, the interruption requirements are already address in the TCI-state switch section 8.10 of TS38.133. For the later one, we are thinking that perhaps the interruption requirements is not needed. The time that UE will take autonomous Rx beam change is completely unknown to the network. Therefore, there is no corresponding network action to handle this autonomous interruption. Similar discussions and conclusions happened when RAN4 was discussing the interruption requirements of BWP switch due to RACH.

Proposal 3: No requirements are specified for one-shot UL timing adjustment due to UE’s autonomous Rx beam change.

# Summary

In this paper, we discuss the issue of UE UL timing change due to Rx beam change. We have the following observation and proposal:

Observation 1: When H is somehow within the range of 25~30% of the UL CP, then the overall BS error could be roughly controlled around half of CP.

Observation 2: From UE’s perspective, reasonable H is within the range of 40~56%.

Proposal 1: RAN4 tries to agree on H to be 33% of the CP for all SCSs. If not possible, try to work on SCS-specific H.

Proposal 2: No explicit accuracy requirement is specified for UL Tx transmit timing on non-serving beam, because it is already implicitly considered in the threshold H.

Proposal 3: No requirements are specified for one-shot UL timing adjustment due to UE’s autonomous Rx beam change.

# References

[1] R4-1909533, “Discussion on one shot timing adjustment requirements”, ZTE

[2] R4-1909504, “Analysis of one shot timing adjustment requirements”, Ericsson

[3] R4-1908179, “Remaining issues for beam switching on timing requirements for FR2”, MeidaTek inc.

[4] R4-1909950, “UL one shot timing adjustment”, Qualcomm Incorporated

[5] R4-1909609, “Discussion on remaining issues on UE one-shot timing adjustment requirements”, Huawei, HiSilicon